3GPP TSG-RAN WG4 Meeting#112bis							         		R4-2416379

Hefei, China, 14 – 18 October 2024

Title: 	On RAN4 PRD scope for Release19 and beyond

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.1.1.1 PRD

Document for:	Discussion

# 1	Background

In RAN#112, the RAN4 group agreed to discuss the scope of a Permanent Reference Document (PRD) on band combinations. In this contribution, we provide our view on the expected content for band combination which seems the consensus for Release 19. We also provide our views on what topics and content RAN4 PRD(s) should aim at longer term.

# Discussion

## 2.1 Way forward inputs

The way forward [1] captured the agreement seeking for companies’ inputs on the scope of the first RAN4 PRD on band combinations.

Agreement:

- Companies come up with concrete proposals on the scope of the first PRD for band combinations with PRD skeleton, if possible.

  - Ericsson can lead the discussions for the first PRD.

- Based on the initial version of PRD, RAN4 will decide whether to have PRD and how to organize the discussions.

In this contribution, we provide our view on the detailed content that a band combination PRD should cover, while also providing our view of which topics/procedures may be worth capturing in a future PRD.

## 2.2 Proposed scope for the RAN4 band combination PRD.

There is a strong consensus that the first RAN4 PRD should tackle the band combinations topic. This is the obvious first choice given the simplification and documentation effort spent in the last two releases and the fact that this is an ever-growing effort in terms of number of baskets, volume, and complexity.

To be a solid reference for the band combinations related work, the PRD content and structure should document the following for FR1 band combinations:

- The current band combination baskets with corresponding valid DL and UL configurations.

- The band combination completeness criteria for DL and UL fallbacks (especially for BCS 4 and 5).

- The band combination request process.

- The band combination TP/TR content and requirement (when available based on the templates recently approved) for:

  - Intra-band UL/DL CA

    - When needed, capture additional emissions requirement for ULCA and initiate A-MPR evaluation work.

    - Add single CC UL and ULCA MSD detection tables.

    - Intra-band ULCA Power class table

    - Add MSD test point rules and if feasible/agreeable MSD calculation example.

  - 2 band DL inter-band CA with 1 or 2 band UL

    - 1UL 1/2CC and 2UL 2/3CC MSD detections tables

    - Delta T/R

    - Inter-band ULCA Power class table

    - Add MSD test point rules and if feasible/agreeable MSD calculation example.

  - 3 band DL inter-band CA with 1 or 2 band UL

    - 2UL 2/3CC MSD detections tables

    - Delta T/R

    - Add MSD test point rules and if feasible/agreeable MSD calculation example.

  - >3 DL bands inter-band CA: only Delta T/R rule constructing from lower orders.

- To our understanding there is no band combination specific work on the BS or RRM side, but it may be worth adding some aspects related to additional aspects to consider for band combinations:

  - Simultaneous Tx/Rx operation or not.

  - Potential interruptions.

- When appropriate, the above may also cover HPUE band combinations.

- We realize that all the content is not yet available, and this should be contribution driven but we think the PRD skeleton should be compatible with the above content to be future proof.

## 2.3 More general scope for UERF RAN4 PRD(s) in the future

Beyond the first PRD for Release 19 on band combination, here we would like to discuss here what kind of topics/content is worth capturing in the PRD.

In general, we think that the content of the PRD(s) should enable any RAN4 delegate (at least for UERF) to understand how the specification is derived, what work is required to complete the specification, and also understand other companies’ contributions.

Our RAN4 specifications are based on a number of commonly agreed assumptions (some from one decade ago) that are not spelled out and make the specification difficult to understand by people outside 3GPP, but also by a large number of delegates that are not familiar enough to contribute on the topic or even just forget about these assumptions (I confess it happens to me at times).

A few examples of topics that may be a worth some content in future PRD(s):

- Single band REFSENS assumptions and calculations and related HPUE RSD(s).

- What to do to introduce a new band, a new CBW.

- Single CC MPR and A-MPR assumptions, PA calibration, table format for default power class and HPUE. Related allocation types.

- Intra-band ULCA MPR and A-MPR assumptions, PA calibration, table format for default power class and HPUE. Related allocation types.

- Band Co-existence assumptions and rules

- PCmax equations and mechanism?

- In general, any 38.101-1/2/3 topic/clause that has a large number of tables inputs and little explanation on how the table content is derived.

Hopefully, with further specification simplification, the above list will be reduced, but also work towards establishing the PRD, may allow for automating a number of RAN4 UERF group tasks.

# Conclusion

In this contribution we provide our suggestion for the Release 19 PRD but also provide guidelines for what kind of topics/content may be worth capturing in PRD(s) in the future.

To be a solid reference for the band combinations related work, the PRD content and structure should document the following for FR1 band combinations:

- The current band combination baskets with corresponding valid DL and UL configurations.

- The band combination completeness criteria for DL and UL fallbacks (especially for BCS 4 and 5).

- The band combination request process.

- The band combination TP/TR content and requirement (when available based on the templates recently approved) for:

  - Intra-band UL/DL CA

    - When needed, capture additional emissions requirement for ULCA and initiate A-MPR evaluation work.

    - Add single CC UL and ULCA MSD detection tables.

    - Intra-band ULCA Power class table

    - Add MSD test point rules and if feasible/agreeable MSD calculation example.

  - 2 band DL inter-band CA with 1 or 2 band UL

    - 1UL 1/2CC and 2UL 2/3CC MSD detections tables

    - Delta T/R

    - Inter-band ULCA Power class table

    - Add MSD test point rules and if feasible/agreeable MSD calculation example.

  - 3 band DL inter-band CA with 1 or 2 band UL

    - 2UL 2/3CC MSD detections tables

    - Delta T/R

    - Add MSD test point rules and if feasible/agreeable MSD calculation example.

  - >3 DL bands inter-band CA: only Delta T/R rule constructing from lower orders.

- To our understanding there is no band combination specific work on the BS or RRM side, but it may be worth adding some aspects related to additional aspects to consider for band combinations:

  - Simultaneous Tx/Rx operation or not.

  - Potential interruptions.

- When appropriate, the above may also cover HPUE band combinations.

- We realize that all the content is not yet available, and this should be contribution driven but we think the PRD skeleton should be compatible with the above content to be future proof.

A few examples of topics that may be a worth some content in future PRD(s):

- Single band REFSENS assumptions and calculations and related HPUE RSD(s).

- What to do to introduce a new band, a new CBW.

- Single CC MPR and A-MPR assumptions, PA calibration, table format for default power class and HPUE. Related allocation types.

- Intra-band ULCA MPR and A-MPR assumptions, PA calibration, table format for default power class and HPUE. Related allocation types.

- Band Co-existence assumptions and rules

- PCmax equations and mechanism?

- In general, any 38.101-1/2/3 topic/clause that has a large number of tables inputs and little explanation on how the table content is derived.

# References

[1] R4-2414430 WF on UE RF Specification quality improvement, Qualcomm, RAN4#112