3GPP TSG-RAN WG4 Meeting # 112bis                                R4-2416383

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	 6.4.3

Source:        ZTE Corporation, Sanechips

Title:           Discussion on RRM impacts for less than 5MHz BW

Document for:  Approval

Introduction

The work item on NR support for dedicated spectrum less than 5MHz for FR1 was approved at RAN#103[1]. according to the WID, RAN4 shoud define RRM requirements for inter-band CA and DC for combinations introduced in RF part and main RAN4 scopes are summarized as following:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

In this contribution, we will provide our views on RRM impacts for less than 5MHz BW.

Discussion

In Rel-18, the impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz has been discussed for single carrier and relative RRM requirements were updated. In Rel-19, RAN4 should define RRM requirements for inter-band CA and DC cases.

As per the WID in Rel-19, CA and DC are supported and the RRM requirements maybe impact due to introduction of NR in less than 5 MHz bandwidth. RANP agreed to use a PBCH transmission bandwidth is 20 PRBs for 5MHz channel bandwidth and a PBCH transmission bandwidth is 12 PRBs for 3MHz channel bandwidth. So, there is no impact when CBW equals to 5MHz. For the case that CBW equals to 3MHz, the transmission bandwidth for PBCH is smaller and RAN4 should further discuss whether to update relative requirements or not.

  - SCell configured with less than 5MHz bandwidth

In RAN4#112 meeting, we have discussed the RRM scope for SCell configured with less than 5MHz bandwidth. And there is a remain issue on whether to consider intra-band CA in R19, copy as follows [2].

Issue 2-1-3: Whether intra-band CA is in the scope: - Proposals - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19. - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations.

According to the WID, RAN4 should define RRM requirements for inter-band CA and DC for combinations introduced in RF part. It is clearly stated that we should discuss the scenario of inter-band. So, for this issue, we prefer to focus on inter-band CA and wait until RF room has concrete conclusion on intra-band CA combinations.

Proposal 1: Focus on inter-band CA and wait until RF room has concrete conclusion on intra-band CA combinations.

According to the WF, we should update the related requirements to support less than 5MHz bandwidth in the target SCell in FR1.

- SCell activation delay requirements

- Measurement requirements for deactivated SCC

For SCell activation delay, we should start from single SCell activation and whether consider multiple SCell activation is up to RF session. In legacy, the requirements of SCell activation delay in FR1 are listed as blew.

| Scenario# |  | Tactivation_time |
| --- | --- | --- |
| Known SCell | The activated SCell measurement period is no larger than 2400ms | - TFirstSSB+ 5ms |
|  | The activated SCell measurement period is larger than 2400ms | - TFirstSSB_MAX + Trs + 5ms |
| Unknown SCell | When one of the following conditions is met: - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation | - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms, if UE supports shortMeasInterval-r18 and conditions are met - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms, if UE supports shortMeasInterval-r18 and conditions are not met - TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if UE not supports shortMeasInterval-r18 and conditions are met - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, if UE not supports shortMeasInterval-r18 and conditions are not met |
|  | When both of the following conditions are not met: - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation | If UE supports shortMeasInterval-r18 - 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. If UE not supports shortMeasInterval-r18 - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. |

Based on the above table, we should discuss whether and how to update components which may be impacted by reducing bandwidth. In R18, more time is specified to read the SSB due to the puncturing. With the same principle, we think more time maybe needed for SCell activation delay to support less than 5MHz bandwidth in the target SCell in FR1.

Observation 1: In R18, more time is specified to read the SSB due to the puncturing. With the same principle, more time maybe needed for SCell activation delay to support less than 5MHz bandwidth in the target SCell in FR1.

Proposal 2: Need to study whether the SCell activation delay should be updated, for example, more time maybe needed for SCell activation delay to support less than 5MHz bandwidth in the target SCell in FR1.

As to Measurement requirements for deactivated SCC, RAN4 needs to specify SSB index identification delay requirements for UE with deactivated SCell measurements on a target cell with less than 5MHz SSB BW in FR1, taking into consideration the impacts from PBCH puncturing.

- In Rel-18, RAN4 has discussed SSB index identification delay for intra-frequency measurement and inter-frequency measurement. Finially, RAN4 has reached concensus that 3+4 samples for intra-frequency measurement and 3+3 samples for inter-frequency measurement to acquire the index of the SSB being measured. The definiation of time period for time index detection for a UE operating on a target cell with 12 PRB SSB has been captured in clause 9.2.5.1, 9.2.6.2, 9.3.4, and 9.3.9, applicable to FR1 intra-f without gap, FR1 intra-f with gap, FR1 inter-f with gap and FR1 inter-f without gap, respectively.

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 |  |

Observation 2: In Rel-18, SSB index identification delay

  - Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples

  - Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples

- In our understanding, same requirements can be reused for deactivated SCell. In current spec., time period for time index detection for deactivated SCell as follow.

Table 9.2.5.1-6: Time period for time index detection, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(3 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(3 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Therefore, for SSB index identification delay, we prefer to reuse R18 requirements to compensate the losses caused by punching.

Proposal 3: For the SSB index identification delay requirements for deactivated SCC, time index detection for a UE operating on a target cell with 12 PRB SSB defined in R18 can be used as baseline.

2.2PSCell configured with less than 5MHz bandwidth

According to the spec, Tsearch is the time for AGC settling and PSS/SSS detection. If BW is reduced from 20PRB to 12PRB, there is no inpact on AGC settling and PSS/SSS detection. So, we can use the same statement as legacy for PSCell addition. As to Tdelta, time for fine time tracking and acquiring full timing information of the target cell, which will be impact by puncturing and additional Trs may be needed.

Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR1 or FR2 no later than in slot n+Tconfig_PSCellNR slot lengtℎ: where: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms ... Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 3*N* Trs ms. N = 1 when target cell is in FR1, N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2 T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell. ...

As to PSCell addition and release, we can refer to the discussion on side condition for handover in Rel-18. In Rel-18, the value of T_delta has be updated and additional Trs has be added in handover requirements due to PBCH BW of 12PRBs for 3MHz channel bandwidth. In our understanding, the same principle can be reused for PSCell addition and release delay.

Observation 3: In the latest version of TS 38.133, T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 4: Samilar as HO requirements, T∆ should be updated and additional Trs should be added for PSCell addition delay.

Meanwhilie, the same revision should be done for Conditional PSCell Addition Delay and Subsequent Conditional PSCell Addition Delay.

Proposal 5: The extension for conditional PSCell Addition delay and subsequent conditoinal PSCell Delay delay should be consisered, additional Trs should be added for T∆.

In the last meeting, we have discussed and there was no consensus on whether to study the impacts of less than 5MHz, in particular PBCH puncturing, to PSCell change delay requirements. Some company proposed that PScell change requirement is based on PScell addition with additional margin. After checking, we can observe the requirements for PSCell change is same as PSCell addition in clause 8.9.2. Therefore, just need to consider the extension for PSCell addition, and no need to update PScell change requirement.

TS 38.133 clause 8.11 The UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than specified in clause 8.9.2 for the case of NR-DC and in TS 36.133 clause 7.31.2 for the case of EN-DC

Observation 4: PScell change requirement is based on PScell addition and no need to update PScell change requirement.

However, as to the conditional PSCell change delay and subsequent conditoinal PSCell change delay requirements, we should consider the extension as well.

TS 38.133 clause 8.11B.2 The UE shall be capable to transmit PRACH preamble towards the new target PSCell no later than in slot n + Tconfig_PSCell_Conditional: Where: - Slot n is the last slot overlapping with the PDSCH containing conditional PSCell change. - Tconfig_PSCell_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms ... - Tmeasure is the measurements time stated in clause 8.11B.2.1. ... - T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms. ... TS 38.133 clause 8.11E.2 The UE shall be capable to transmit PRACH preamble towards the new target PSCell no later than in slot n + Tconfig_PSCell_Subsequent_Change_Conditional: Where: - Slot n is the time when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change. - Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms ... - Tmeasure is the measurements time stated in clause 8.11E.2.1. ... - T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms. ...

Proposal 6: The extension for conditional PSCell change delay and subsequent conditoinal PSCell change delay requirements should be consisered, additional Trs should be added for T∆.

For handover with PSCell, the requirements for PCell interruption and PSCell change are based on handover and PSCell addition. Therefor, no further extension is needed.

When handover with PSCell from NR-DC to NR-DC is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: - Tsearch, TIU, T∆ and Tmargin are the same as defined in clause 6.1.1.2.2. ... When handover with PSCell from NR-DC to NR-DC is commanded, the PSCell change time shall be less than DHOwithPSCell_PSCell: - DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_PCell + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms Where: - TRRC_delay, Tsearch_PSCell, T∆ and TPSCell_ DU are the same as defined in clause 8.9.2.

Observation 5: Partial components of handover with PSCell requirement is based on PScell addition and handover, maybe no need to update handover with PSCell requirement.

2.3Other RRM impacts

Issue 2-3-1: NW indication on PBCH BW in MO configuration and/or HO command: - Proposals - Option 1: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell. - Option 2: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command. Issue 2-3-3: EMR requirements: - Proposals - Option 1: RAN4 to Study the Cell detection requirements in EMR measurements for less than 5MHz. - Option 2: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading. Further discuss: - Option 2: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading. - Option 3: not define IDLE mode EMR requirement

For the first issue, RF session has reach comsensus that NW indication on PBCH BW in MO configuration is needed, so we don’t need to discuss and just follow the RF conclusion.

R4-2414451 Issue 2-3: Network signaling on Scell transmission bandiwdth - Agreement (online session) - No new signalling from network to inform UE the number of PRB transmission is required in the RF scope of this item

For the second issue, we prefer to support option2. RAN4 should study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading.

For EMR measurement, the requirement for legacy UE is N2 = 3 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1[2].

Table 4.4.2.2-1: TSSB_index,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | TSSB_index,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |
| 0.32 | 1 | 8 | N2 x 1.28 x N1 x 1.5 (N2 x 4 x N1 x 1.5) |
| 0.64 |  | 5 | N2 x 1.28 x N1 (N2 x 2 x N1) |
| 1.28 |  | 4 | N2 x 1.28 x N1 (N2 x 1 x N1) |
| 2.56 |  | 3 | N2 x 2.56 x N1 (N2 x 1 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. NOTE 2: N2 = 3 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1, and N2 = 3, 5 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR2. |  |  |  |

- As mentioned above, RAN4 has discussed SSB index identification delay for unknown intra-frequency cell and unknown inter-frequency cell in Rel-18. Finially, RAN4 has reached concensus that 3+4 samples for intra-frequency and 3+3 samples for inter-frequency. In our understanding, same principle should be used for EMR measurement. That’s to say, N2=[3+3] if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1 and PBCH bandwidth for 3MHz channel bandwidth.

- Proposal 7: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading.

Conclusions

In this contribution, we provide our views on the RRRM impacts for less than 5MHz BW and the proposals are:

Proposal 1: Focus on inter-band CA and wait until RF room has concrete conclusion on intra-band CA combinations.

Observation 1: In R18, more time is specified to read the SSB due to the puncturing. With the same principle, more time maybe needed for SCell activation delay to support less than 5MHz bandwidth in the target SCell in FR1.

Proposal 2: Need to study whether the SCell activation delay should be updated, for example, more time maybe needed for SCell activation delay to support less than 5MHz bandwidth in the target SCell in FR1.

Observation 2: In Rel-18, SSB index identification delay

  - Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples

  - Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples

Proposal 3: For the SSB index identification delay requirements for deactivated SCC, time index detection for a UE operating on a target cell with 12 PRB SSB defined in R18 can be used as baseline.

Observation 3: In the latest version of TS 38.133, T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 4: Samilar as HO requirements, T∆ should be updated and additional Trs should be added for PSCell addition delay.

Proposal 5: The extension for conditional PSCell Addition delay and subsequent conditoinal PSCell Delay delay should be consisered, additional Trs should be added for T∆.

Observation 4: PScell change requirement is based on PScell addition and no need to update PScell change requirement.

Proposal 6: The extension for conditional PSCell change delay and subsequent conditoinal PSCell change delay requirements should be consisered, additional Trs should be added for T∆.

Observation 5: Partial components of handover with PSCell requirement is based on PScell addition and handover, maybe no need to update handover with PSCell requirement.

- Proposal 7: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading.

References

- RP-240832, WI resulting from discussion on NR channel BW less than 5MHz for TN, vivo

- R4-2413883, WF for [112][214] NR_FR1_lessthan_5MHz_BW_Ph2, Intel