3GPP TSG-RAN WG4 Meeting # 112bis                                R4-2416385

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.24.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Conditional Intra-CU LTM

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to layer3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

Discussion

Compared to R18 LTM, conditional Intra-CU LTM should be supported in R19.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

LTM as introduced in Rel-18 offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures. In Rel-19, enhancements should be specified so that the system can benefit from both the high robustness and short interruption. It’s benefical to combine LTM and conditional L3 mobility procedures and RAN4 needs to define requirements based on RAN2’s design.

From the objective, UE evaluated conditions will be specified by other groups to trigger LTM. RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need be considered. Meanwhile, the requirements are applicable to conditional cell switch to change the NR PCell to another NR cell, including FR1 to FR1, FR1 to FR2, FR2 to FR1 and FR2 to FR2. The relevant requirements can refer to the conditional handover of L3 in section 6.1.4.

6.1.4.2 NR FR1 – NR FR1 conditional handover The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR1 cell. ##### 6.1.4.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover Tmeasure is the measurements time stated in clause 6.1.4.2.2. TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3. Tinterrupt is the interruption time stated in clause 6.1.4.2.4.

Proposal 1: RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need to be considered:

- NR FR1 – NR FR1

- NR FR1 – NR FR2

- NR FR2 – NR FR1

- NR FR2 – NR FR2

As to the conditional LTM including subsequent LTM, there is no subequent HO in current spec. And just subsequent CPAC has introduced in section 8 in Rel 18. RAN4 should defined RRM requirements for conditional LTM including subsequent LTM. Maybe the requirement of subsequent CPAC can be used as baseline for conditional including subsequent LTM.

Proposal 2: RAN4 should defined RRM requirements for conditional LTM including subsequent LTM.

However, it’s the first meeting to discuss this part and there is no progress in RAN2. Therefore, we should wait for RAN2 input to further discuss RRM requirements.

Proposal 3: Wait for RAN2 input to further discuss RRM requirements.

Conclusions

In this contribution, we put forward the following proposals on conditional LTM.

Proposal 1: RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need to be considered:

- NR FR1 – NR FR1

- NR FR1 – NR FR2

- NR FR2 – NR FR1

- NR FR2 – NR FR2

Proposal 2: RAN4 should defined RRM requirements for conditional LTM including subsequent LTM.

Proposal 3: Wait for RAN2 input to further discuss RRM requirements.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom