3GPP TSG-RAN WG4 Meeting #112bis	                               R4-2416386

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.5.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on intra-band non-collocated EN-DC and NR-CA

Document for:	Approval

Introduction

For intra-band non-collocated EN-DC/NR-CA deployment, RAN4 has diasussed and achieved agreements on MRTD/MTTD requirements, interruption requirements and scheduling restriction requirements for Type 2 UE in Rel-18. In Rel-19, we should focus on Type 4 UE accroding to WID [1] to better support the intra-band non-collocated EN-DC/NR-CA deployment scenarios.

The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

In this contribution, we will provide the discussion on RRM impacts for intra-band non-collocated EN-DC/NR-CA, including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption.

Discussion

Based on the agreements in [2], Type 3a/3b and Type 4a/4b are assumed for 4-layer MIMO case. The assumed UE architectures for different Types are listed as Table.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

It can be observed from the table that Type4 UE is assumed to have separate Rx chains for different CCs, similar with Type 2 UE. Different from Type2 UE, Type4 UE is up to 4-layer DL MIMO per CC. Therefore, the main difference between Type2 and Type4 is the supported MIMO layers per CC. In our view, the requirements of Type2 UE can be used as baseline for Type4 UE.

As to the difference between type 4a and type 4b, we can observe from above table that type 4a supports 2 DL MIMO on one CC and 4 DL MIMO on the other CC, while type 4b supports 4 DL MIMO on both CCs. In our understanding, there is no distingush between two sub types from RRM perspective.

In the last meeting, we have discussed the related requirements for Type4 UE [3], and reached consensus that the requirements list blew defined for type 2 UE in non-collocated condition in R18 can be applied.

- MRTD and MTTD for Type 4 capable UE in non-collocated scenario

- MRTD and MTTD for Type 4 capable UE in collocated scenario

- Interruption requirements for Type 4 capable UE when UE operates with separate RF chain in non-collocated scenario

- SCell activation delay requirements for Type 4 capable UE when UE operates with separate RF chain in non-collocated condition

- Scheduling availability for Type 4 capable UE when UE operates with separate RF chain in non-collocated condition

Besides, there are some issues need further discussion. According to WF[], at least the following issues need to be resolved.

- Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

- Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated scenario

- SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

- Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition

- SCell BFD/CBD requirements for Type 4 capable UE

  - MRTD/MTTD requirements

- Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

In the last meeting, MRTD/MTTD requirements defined for Type 4 capable UE in non-collocated condition and  collocated condition (depend on RF progress), which are same as Type 2 UE due to the similar RF structure. Meanwhile, some company proposed whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19. In Rel 18, the requirements for non-collocated FR1 intra-band asynchronous NR-CA and non-collocated FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations for Type 2 UE was not defined.

RAN4#106 Issue 1-1-4: MRTD/MTTD requirements for FR1 inter-band asynchronous EN-DC with overlapping DL band for Type-2 UE - Do not consider MRTD/MTTD requirements for non-collocated FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in the scope of this work item Issue 1-1-5: MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 2 UE - Do not introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 2 UE

Since it was agreed in RRM that not to introduce MRTD/MTTD requirements for non-collocated intra-band asynchronous NR-CA Type 2 UE in Rel-18, we think Type 4 UE can use the same method to handle this scenario.

Proposal 1: Similar as Type 2 UE, for Type 4 UE

  - Do not consider MRTD/MTTD requirements for non-collocated FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in the scope of this work item

  - Do not introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE

2.2 SCell operation related delay and interruption

- Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated scenario

- SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

- Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition

In this part, we should discuss the requirements about Interruption, SCell activation delay and scheduling availability for Type 4 UE. In the last meeting, we have discussed the requirements for Type 4 capable UE when UE operates with separate RF chain in non-collocated scenario and reached consensus to reuse the SCell activation delay defined for type 2 UE in non-collocated condition in R18.

As to interruptions requirements, separate Rx chains are assumed For inter-band case and intra-band non-collocated (Type 2 UE) case, and there is no interruption due to AGC settling. For intra-band case collocated case, shared Rx chain is assumed and the interruption due to AGC settling is included. In R19, Type 4 UE with separate Rx chains are assumed for intra-band non-contiguous case, which is same as the assumption for inter-band case and intra-band non-collocated for Type 2 UE. Therefore, the current interruption requirements for inter-band CA also can be applied for Type 4 UE.

For SCell activation delay, the same requirements are applied for FR1 intra-band collocated non-contiguous CA and FR1 inter-band CA cases. Meanwhile, MRTD for intra-band non-collocated non-contiguous CA is extended from 3us to 33us, which is same as that for inter-band CA. Therefore, the SCell activation delay requirements for FR1 inter-band CA case have been reused for FR1 intra-band non-collocated non-contiguous case.

For current scheduling availability requirements, the scheduling restrictions on one FR1 serving cell shall also apply to other serving cells in the same band based on the assumption that shared Rx chain is used for intra-band CA, and there is no scheduling restrictions on serving cells in other bands based on the assumption that separate Rx chains are used for inter-band CA or intra-band CA. Due to separate Rx chains assumed for intra-band non-contiguous CA for supporting non-collocated deployment, there is no scheduling restrictions on non-contiguous serving cells in the same band.

In this meeting, we should focus on the collocated scenario and discuss whether the requirements defined for type 2 UE in collocated scenario in R18 can be applied.

| Type 4 UE | separate RF chain ( non-collocated scenario) | shared RF chains (collocated scenario) |
| --- | --- | --- |
| Interruption requirements | Reuse (same as type 2 UE in non-collocated condition in R18) | FFS |
| SCell activation delay requirements | Reuse (same as type 2 UE in non-collocated condition in R18) | FFS |
| Scheduling availability | Reuse (same as type 2 UE in non-collocated condition in R18) | FFS |

As to FFS, there are two options could be considered and both are reasonable in our view.

  - When type 4 capable UE operates with shared RF chain in collocated condition, scheduling availability defined for type 2 UE in collocated condition in R18 can be applied

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

For Type 4 UE, if UE is in non-collocated scenario, UE shall work with separate RF chains. And if UE is in collocated scenario, UE would change its hardware to use shared RF chains, which is same as Type 2 UE. Therefore, it’s reasonable to reuse the requirements defined for type 2 UE in collocated condition. However, BS signalling was introduced to indicate that UE is operating under Type 1 or Type 2 CA/DC in R18, and the releated RRM requirements mentioned above have been revised due to new BS signalling.

RAN4#109 Issue 1-3: the impacted RRM requirement for UE supporting [intraBandNRCA-NonCollocated-r18] due to new BS signalling Agreement: The impacted RRM requirement applicability for UE supporting [intraBandNRCA-NonCollocated-r18] due to new BS signalling - MRTD/MTTD, interruption, [SCell activation delay, BFD/CBD, scheduling restrictions and measurement restrictions requirements.] Issue 1-4: the impacted RRM requirement for R18 UE supporting interBandMRDC-WithOverlapDL-Bands-r16 due to new BS signalling Agreement: The impacted RRM requirement applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 due to new BS signalling - MRTD/MTTD and interruption requirements.

Observation 1: For type 4 capable UE, MRTD/MTTD and the other RRM requirements need to be adapted based on the eventual UE type in operation which is indicated by the BS signaling.

In our understanding, the requirements defined for type 2 UE in collocated scenario in R18 can be applied when Type 4 capable UE operates with shared RF chains in collocated scenario. However, new BS Signaling to switch between Type 4a/4b and Type 1 UE is discussing in RF session [4]. To avoid modifying the requirements again, it’s fine for us to wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

RAN4#112 < Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)> Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. < Issue 2-3-5: New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting.

Proposal 2: Requirements for Type 4 capable UE when UE operates with shared RF chains should be discussed based on RF progress on UE type transition and design of new UE capabilities/NW signalling.

2.3 SCell BFD/CBD requirements

- SCell BFD/CBD requirements for Type 4 capable UE

Similar as Type2 UE, we should revised BFD/CBD measurements requirements for FR1 intra-band non-contiguous CA supporting non-collocated deployment for 4-layer MIMO case. As mentioned in previous meeting, it is assumed that the serving cells in the same band have the similar link quality in co-located deploymen for link recovery procedure, and the link qualities of serving cells in the same band could be different for Type 2 UE in non-collocated deployment. Consequently, there is no need to limit BFD/CBD requirements only on one serving cell. The same principle can be applied to type4 UE.

Ran4#106bis Issue 1-5-2: Impacts on SCell BFD requirements - For non-collocated FR1 intra-band non-contiguous CA, Type 2 UE can be configured with BFD/CBD measurements on up to two serving cells in the same band

Proposal 3: It is suggested that Type 4 UE can be configured with BFD/CBD measurements on more than one serving cells in the same band.

Conclusions

In this contribution, we put forward the following proposals:

Proposal 1: Similar as Type 2 UE, for Type 4 UE

- Do not consider MRTD/MTTD requirements for non-collocated FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in the scope of this work item

- Do not introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE

Observation 1: For type 4 capable UE, MRTD/MTTD and the other RRM requirements need to be adapted based on the eventual UE type in operation which is indicated by the BS signaling.

Proposal 2: Requirements for Type 4 capable UE when UE operates with shared RF chains should be discussed based on RF progress on UE type transition and design of new UE capabilities/NW signalling.

Proposal 3: For Type 4 UE with FR1 intra-band CA, there are no scheduling restrictions on carriers in the same band.

References

- RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA, Moderator (Apple)

- R4-2220537, WF on the requirement for intra-band non-collocated CA/EN-DC, KDDI

- R4-2413973, WF on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Huawei

- R4-2414453, WF on Rel-19 non-collocated scenario, KDDI