3GPP TSG RAN WG4 #112bis													     R4-2416392

Hefei, CN, October 14th – 18th, 2024

Agenda Item:		6.12.2.2

Source:				Inmarsat, Viasat

Title:	Operator views on Usage Modes for NTN Smartphones

Document for:	Discussion

# Background

The intent of this paper is to provide a perspective on the anticipated usage modes of NTN devices, in particular smartphones, from an NTN operator’s perspective. At the RAN4#112 WG in Maastricht it was decided that we would follow a smaller subset of usage modes, and that NR handsets will be baselined on the full sphere coverage, whereas NB-IoT handsets will be baselined on partial sphere with a directional bias.

# Discussion

## 2.1 Mobile Handset – NR

Our perspectives on the anticipated usage modes / use cases for NTN NR Mobile Handsets are as follows:

- Smartphones and other handsets supporting NR NTN are expected to support a broad portfolio of services, with closer alignment and user experience to that of current terrestrial services, but with reduced throughput due to the more challenging link budget.

- Therefore, users should not be expected to operate their device in a fashion that is significantly different from how a TN UE would be operated, except for the fact that the user must be outside or potentially in “light-indoor” (e.g. a car, a tent or behind a large window).

  - This means that in all possible orientations the UE should be able to close the link, and no specific orientation should be required in order to facilitate satellite access.

  - Making the user hold the UE in a strict orientation for specific functions (e.g. making a voice call, browsing, tethering) may put NTN UEs at a disadvantage from an ergonomic standpoint.

  - However, we can expect that in some scenarios, some level of “user education” could be leveraged, if correctly implemented, to improve an otherwise challenging link.

- The use cases for NR NTN UEs extend beyond voice calls taken from a ‘browsing mode’, if the above point is to be fulfilled, head and hand must thus be considered.

- In addition to voice, the UE may be used for video calling and video streaming also, which requires a variety of other orientations.

- Tethering is also a possible use case, where the UE is used to provide connectivity to other devices. The orientation of the device in this scenario is unpredictable, but in many cases it can be expected to correspond to the device lying on a surface.

- A device might also be used for map consultation and navigation on a frontal chest mount, for example by a public safety, law enforcement or similar user, such as a reporter in the field. In this case, the flat horizontal scenario, facing up, with partial body blockage, would be realistic.

All of the above use cases highlight the need for an antenna pattern with a strong spherical, omni-directional performance, as a baseline.

However, given the challenging link budget, that there are a number of modes in which the device could still benefit greatly from a directional bias of the coverage performance, and that to some extent the user can be “educated” to assist the connection in certain scenarios, we think that other techniques that could be adopted.

Technniques based on multi-antennas/multi-TX, such as antenna switching/selection or even simple UL beamforming, could be considered, since the device has most likely knowledge of its own orientation, as well as the direction of the network, at least for NTN, where the NTN ephemeris are available to the UE.

Observation 1: NTN NR devices are expected to be operated in a range of applications including voice, browsing, video calls, video streaming and tethering.

Observation 2: NTN NR devices are expected to be operated in a similar manner to that of a TN handset, and to be operated using usage modes that a TN handset would use also, including head-and-hand, browsing and lying on surfaces or horizontal mounts (e.g. for tethering or map navigation)

Proposal 1: Further consider additional techniques to improve the directional coverage by leveraging multi-antenna TX, antenna switching, selection and simple beamforming, based on UE awareness and satellite assistance information.

## 2.2 Mobile Handset – NB-IoT

Our perspectives on the anticipated usage modes for NTN NB-IoT Mobile handsets are as follows:

- NB-IoT primarily supports low data rate communication, as such, it’s applicability to smartphones is expected to focus primarily on messaging, SOS and wireless emergency alerts, with some limited capabilities voice calls.

- In addition, currently, NB-IoT is being deployed on existing GEO satellites or low-power NGSO satellites, due to its narrow band properties, which were not originally designed for smartphone support.

- Since user unawareness is not of paramount importance in an emergency, the user can be guided in adopting a specific orientation of the handset, in order to facilitate connection with the satellite.

- Specifically, for messaging, which is arguably the primary use case, users tend to naturally adopt a mode very similar to browsing mode.

- In emergency situations a more reliable link is necessary. Hence, a high gain portion of the antenna pointing towards the satellite would provide a decent margin in the link budget allowing for a more reliable link.

- The main additional scenarios in which coverage in other directions would be required is mostly limited to paging and reselection.  In those cases, since the phone is likely to be stored, the link would already be quite challenging to close.

- With future higher power constellations, NB-IoT link budget would be significantly better than NR, thus helping to close the gap also for paging and re-selection, for those devices that may have a directional coverage bias.

- Future dual-mode devices that adopt both NB-IoT NTN and NR NTN, may either support both antenna types (if they have multiple antennas, or generally align with the NR antenna pattern, but the narrow band nature would help provide consistent coverage across the orientations.

The above use cases / usage modes show the justification for an antenna with directional bias when concerned with NB-IoT NTN UE.

Observation 3: NTN NB-IoT devices are expected to prioritise low data applications in particular emergency / critical calling / messaging.

Observation 4: NTN NB-IoT devices will have a specific set of usage modes to improve the reliability of the link, which enhances the reliability of critical communications.

Observation 5: For future high-power space segment, the coverage gap outside of the preferential direction could be in part mitigated by the space segment capabilities.

Proposal 2: For dual mode devices that will support both NR and NB-IoT NTN, the device is either expected to support both spherical and partial coverage, or to support at least full spherical coverage.

# Conclusions

Observation 1: NTN NR devices are expected to be operated in a range of applications including voice, browsing, video calls, video streaming and tethering.

Observation 2: NTN NR devices are expected to be operated in a similar manner to that of a TN handset, and to be operated using usage modes that a TN handset would use also, including head-and-hand, browsing and lying on surfaces or horizontal mounts (e.g. for tethering or map navigation)

Proposal 1: Further consider additional techniques to improve the directional coverage by leveraging multi-antenna TX, antenna switching, selection and simple beamforming, based on UE awareness and satellite assistance information.

Observation 3: NTN NB-IoT devices are expected to prioritise low data applications in particular emergency / critical calling / messaging.

Observation 4: NTN NB-IoT devices will have a specific set of usage modes to improve the reliability of the link, which enhances the reliability of critical communications.

Observation 5: For future high-power space segment, the coverage gap outside of the preferential direction could be in part mitigated by the space segment capabilities.

Proposal 2: For dual mode devices that will support both NR and NB-IoT NTN, the device is either expected to support both spherical and partial coverage, or to support at least full spherical coverage.