3GPP TSG RAN WG4 Meeting #112bis	R4-2416394

Hefei, China, 14th – 18th October 2024

Source:		Rohde & Schwarz

Title:	Discussion on methodology for doppler and delay modelling for NGSO satellites

Agenda Item:	6.8.4

Document for:	Approval

# Introduction

During the last RAN4 meeting the discussion on a realistic doppler/delay profile for NGSO satellites has started. In this paper we share our view on how the doppler/delay profile should be modelled as part of the test system.

# Discussion

## Background

At the end of the last meeting, a way forward [1] has been agreed where several options for the methodology of modelling the doppler/delay profile have been mentioned:

Issue 1-2-1 Methodology for Time varying Doppler and Delay shifts modelling

- Agreement

- RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following candidate options:

  - Option 1: Eckstein-Hechler model

  - Option 2: Keplerian model

  - Option 3: Existing model from TR 38.811 5.3.4.3

  - Option 4: Proposal 2 from R4-2412552

dht=modvNGSO⋅t, 2dh,max-dh,max Tt=dtc=dh(t)2+hNGSO2c Ft=-fddhtd(t)=-fddh(t)dht2+hNGSO2 where - Tt: Propagation delay of the signal from the satellite at time t, - F(t): Doppler (frequency) shift of the signal from the satellite at time t. - dh(t): the horizontal distance between the satellite and UE at time t, - d(t): the distance between the satellite and UE at time t, - dh,max: the maximum horizontal distance between the satellite and UE, i.e., hNGSO/tan⁡(E), - c: the speed of light, - fd: Maximum Doppler shift (Hz)

  - Option 5: Proposal 2 from R4-2412783

- The instant satellite speed v is given as: v=μR+h - The initial angle β of vector geocentric to UE relative to the x axis is given as: β=π-α-arcsin⁡R∙cosαR+h - The satellite position s is given as: s=j∙R+h - The UE position u is given as: u=R∙ej∙β+v∙tR+h - The Doppler fd is given as: fd=vc∙fc∙realu-su-s - The delay T is given as: T=¦u-s¦c where μ=398600.4418×109m3s2 is standard gravitational parameter, R=6371008.7714m is earth average radius, h=600000m is satellite altitude for LEO-600, α is initial angle of vector UE to satellite relative to vector UE to clockwise side horizon, t is the time, c=299792458ms is the speed of light, fc=2×109Hz is the carrier frequency.

  - Other options not precluded

Note: Companies are encouraged to bring more information for option 1 and option2.

- Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model

  - FFS on the initial ephemeris information and parameters

## Discussion

We discuss the proposed options and evaluate the utilization of these models for the test purposes according to the following criteria:

- Accuracy

- Scalability

- Compatibility

- Computation complexity

- Feasibility and impact on test procedure

Option 1: Eckstein-Hechler model

The Eckstein-Hechler model is a semi-analytical method used to propagate satellite orbits in the presence of Earth's gravitational perturbations. It considers higher-order gravitational effects, primarily from Earth's non-spherical shape i.e. oblateness and other irregularities in the Earth's gravitational field, modeled by its geopotential. The model is particularly suited for medium-accuracy predictions over short to medium timescales.

It includes the effects of Earth's gravitational field beyond the simple two-body problem, such as Earth's J2 and higher-order zonal harmonics.

- Accuracy

  - Accurate for low orbits

  - Accounts for the Earth’s non-spherical shape and is more precise than simple models

  - It does not account for for all types of perturbations, such as solar radiation pressure or third-body influences (e.g., the Moon or Sun).

  - Assumes a circular orbit and is not suitable for highly elliptical orbits

  - Maximum possible propagation time is 900s

- Scalability

  - The model is limited for low satellite orbits

  - Limited to quasi-circular orbits

- Compatibility

  - UE uses the Keplerian model as baseline, see [1]

  - Scarcely used in systems

- Computation complexity

  - It offers faster computations than fully numerical methods. However, it can be more computationally demanding than models with comparable accuracy.

- Feasibility and impact on test procedure

  - Non-high-end devices will not implement it, meaning that most of the UEs that will be tested, will have a different model than the model implemented in the TE (in case adopted). This will impose additional effort and adaptations to the test procedure.

  - Does not work for inclinations close to the critical inclinations (~63.43 deg and ~116.57 deg), which will impose a workaround and additional test complications on the test procedure, if the orbits inclinations are beyond those inclinations.

Option 2: Keplerian model

The Keplerian model is based on Johannes Kepler's laws of planetary motion, which describe the orbits of celestial bodies around a central mass in the form of elliptical paths. The model assumes that satellites move in a simple elliptical orbit, with the central body located at one of the foci of the ellipse.

- Accuracy

  - Only considers the gravitational force of the central body, ignoring perturbations from other forces

  - Its accuracy has proven to be sufficient for high precision positioning and satellite orbit applications

  - Maximum possible propagation time is 900s

- Scalability

  - The model can be also used for higher orbits

  - Easy to implement in new devices, as most UE vendors are already deploying it in the devices for positioning applications.

  - Extendable to a more precise model by considering additional perturbation parameters such as Argument of Latitude Correction, Radial Correction, Inclination Correction etc.

- Compatibility

  - Compatible to devices with positioning applications

  - Most UEs will implement it due to its feasibility and good performance

- Computation complexity

  - The computational complexity is low compared to Eckstein-Heckler making it suitable for UE and TE implementation.

- Feasibility and impact on test procedure

  - The model is relatively simple and easy to use. Keplerian elements provide a concise description of an orbit using only six parameters.

  - It is widely used in e.g. GNSS applications and is considered as the reference implementation for UEs [1]

Options 3-5

We address options 3-5 together in the following, as the models’ approach is similar and is based on analytical models of an assumed satellite path with varying calculation complexities.

The proposed options vary in their complexity, where option 4 is with the lowest computational complexity and is a very simple emulation of a straight path that highly diverges from the intended satellite orbit. The other options add computation complexity to reduce the divergence from the satellite orbit. However, the additional computation complexity makes it in total not far from the Keplerian model computational complexity, with the Keplerian model having a significant advantage in accuracy.

Furthermore, as these analytical models are not implemented in the UE, the UE must be put into a special test mode to be tested with these models.

- Accuracy

  - These models are simplified models in which parameters of the models in options 1 and 2 are not considered, e.g. perturbation parameters.

- Scalability

  - Models are fixed are not scalable

  - Parameter values as e.g. height can be changed.

  - Emulated route remains fixed and cannot consider different orbits and additional effects such as gravitational perturbations, elliptical orbit, orbit tilt etc.

- Compatibility

  - The models are not compatible with UE baseline model, the Keplerian model.

- Computation complexity

  - Relatively low, however less performing than comparable complexity models, i.e. Keplerian

- Feasibility and impact on test procedure

  - The UE has to be put into a special test mode in order to be tested with any of the models in options 3-5.

  - Limited emulation time, as handover scenarios cannot be simulated properly if the satellite orbit is not emulated with sufficient accuracy.

  - In case of UE failure, it will be difficult to trace back to the origin of the error as the UE is put into a special test mode

Observation 1:  Options 3-5 are highly simplified models with a varying computational complexity. With comparable computational complexity the Keplerian model offers higher precision orbit emulation.

Observation 2:  Options 3-5 will impose test procedure complications as the UE will need to put into a special test mode to be able to follow the less realistic path.

Observation 3: Eckstein-Hechler is a more precise model, however it is significantly more complex to implement than the Keplerian model.

As it has been suggested and discussed in [2] Issue 1-2-3, the UE ephemeris information is updated with a fixed rate during the Test Case run in order to compensate for its internal model divergence from the satellite orbit. The advantage of the TE applying a more precise model than the UE baseline is to leave the option for UE vendors to implement a more precise model. However, if the UE position is updated with a sufficient rate (e.g. as proposed (at least) every 10.24 s), the divergence between the UE and TE models, if different, is negligible as such, having a more precise model in the TE will not be measurable in the test.

In case multiple satellites (e.g. for RRM applications) need to be emulated, the doppler/delay profile needs to be computed for every satellite independently. Thus, the computational advantage of the Keplerian model becomes even more crucial.

Observation 4: The Keplerian model provides a good trade-off between accuracy and computational complexity, especially in multi-satellite scenarios.

Furthermore, the Keplerian model is widely used for high precision positioning and orbit modelling (e.g. GNSS applications) and provides a solid framework for testing and validation procedures. In these GNSS applications the propagation time can be up to 2 hours with a required accuracy of ca. 1m. Therefore, with a given propagation time of 900s for NTN, the accuracy of the Keplerian model can be considered sufficient.

Observation 5: The Keplerian model is widely used for high precision positioning and orbit modelling and provides a solid framework for testing and validation procedures.

Observation 6: The propagation time for NTN is 900s compared to up to 2 hours for GNSS applications.

Observation 7: The Keplerian model can be considered sufficient for RF/Demod/RRM testing.

In the evolution of the GNSS system the accuracy of the orbit propagation has been continuously improved over the decades, because it is clearly specified how the UE receivers propagate the satellite position based on the provided Kepler elements. This optimization would have not been possible if the propagation on the UE side would have been left up to implementation. Henceforth, it would be beneficial to standardize a common propagation model for all NTN UEs.

Proposal 1: A common propagation model for all NTN UEs should be standardized, to allow the optimization of the NTN performance.

Keplerian model would also be extendable for a more precise modelling with perturbation parameters such as, Argument of Latitude Correction, Radial Correction and Inclination Correction.

Proposal 2: RAN4 to adopt the Keplerian model for the TE-emulated time varying doppler shift and delay shift model for satellite motion.

# Summary

In this contribution we shared our views on the different methods to calculate the doppler and delay profiles for NGSO satellites.

Observation 1:  Options 3-5 are highly simplified models with a varying computational complexity. With comparable computational complexity the Keplerian model offers higher precision orbit emulation.

Observation 2:  Options 3-5 will impose test procedure complications as the UE will need to put into a special test mode to be able to follow the less realistic path.

Observation 3: Eckstein-Hechler is a more precise model, however it is significantly more complex to implement than the Keplerian model.

Observation 4: The Keplerian model provides a good trade-off between accuracy and computational complexity, especially in multi-satellite scenarios.

Observation 5: The Keplerian model is widely used for high precision positioning and orbit modelling and provides a solid framework for testing and validation procedures.

Observation 6: The propagation time for NTN is 900s compared to up to 2 hours for GNSS applications.

Observation 7: The Keplerian model can be considered sufficient for RF/Demod/RRM testing.

Proposal 1: A common propagation model for all NTN UEs should be standardized, to allow the optimization of the NTN performance.

Proposal 2: RAN4 to adopt the Keplerian model for the TE-emulated time varying doppler shift and delay shift model for satellite motion.

# References

R4-2413516; “Way Forward for [112][329] NTN_testing_NGSO_channel_model”; Samsung; RAN4#112 August 2024

R4-2413429; “Topic summary for [112][329] NTN_testing_NGSO_channel_model”; Samsung; RAN4#112 August 2024