3GPP TSG RAN WG4 #112bis													     R4-2416396

Hefei, CN, October 14th – 18th, 2024

Agenda Item:		6.8.2.1

Source:				Inmarsat, Viasat

Title:	Key aspects on HPUE for NTN

Document for:	Discussion and Decision

# Background

The intent of this paper is to provide information on NTN HPUE and guide the definition of the general framework of operation that in our view is common to the vast majority (or all) of the existing NTN bands, to provide some thoughts and rationale on a methodology by which certain RF performance metrics can be derived and to provide input on coexistence scenario parameters.

In general we do not think that TN-NTN coexistence alone is sufficient in order to define the NTN HPUE general parameters such as SEM, ACLR, ACS, etc.

# Regulatory Information

This section describes the regulatory information associated with bands 253, 255 and band 256. For NB-IoT band 253 has been defined as extended L-band (UL = 1668-1675MHz, DL = 1518-1525MHz), band 253 is in the process of being defined for NR with the same band number.

## 2.1 ETSI EN 301 681

The ETSI EN 301 681 harmonized standard provides regulatory requirements for band 255 and band 253 for operation within Europe. Depending on which frequency band is explicitly used, different tables within the standard are mandated. In this harmonized standard, devices are separated into two categories, those which are only capable of transmitting in n255, and those which are capable of transmitting within all or any part of n255 or n253.

Table 1 ETSI EN 301 681 Requirements vs Band

| Band | Uplink [MHz] | Downlink [MHz] | In-band SEM Table | Out-of-band SEM Table |
| --- | --- | --- | --- | --- |
| n255 | 1626.5-1660.5 | 1525-1559 | Table 4a | Table 3 |
| n253 | 1668-1675 | 1518-1525 | Table 4a | Table 3a |

Proposal 1: ETSI EN 301 681 shall be used to assess the regulatory compliance of HPUE operating in bands n253 and n255, including ACLR and SEM.

ETSI EN 301 681 covers devices which radiate a maximum EIRP of 15dBW. This means the ETSI EN 301 681 regulatory requirements are valid for the following peak antenna gains for each power class:

Table 2 ETSI EN 301 681 Maximum Antenna Applicability

| Power Class | Power [dBm] | Max Antenna Gain [dBi] |
| --- | --- | --- |
| 3 | 23 | 22 |
| 2 | 26 | 19 |
| 1.5 | 29 | 16 |
| 1 | 31 | 14 |

For mobile handset applications the expected antenna gain is well under this even for PC1, as such EN 301 681 is again suitable to assess the regulatory compliance for mobile handset devices operating in bands 253 and 255.

Observation 1: ETSI EN 301 681 is applicable for UEs with an EIRP of less than 15dBW which is highly likely not tbe exceeded by mobile handsets.

ETSI EN 301 681 also provides information on the protection of radioastronomy bands, 1660-1660.5MHz and 1668-1670MHz, mandating a mute function for parts of the band when in the presence of a radioastronomy station that is registered in the ITU master international frequency register..

Proposal 2: ETSI EN 301 681 can be used to inform system design features for the protection of the radio astronomy bands at 1660-1660.5MHz and 1668-1670MHz.

## 2.2 ETSI EN 302 574 Part 2

The ETSI EN 302 574 Part 2 harmonized standard provides regulatory requirements for band 256 for devices which operate with a channel bandwidth greater than 1MHz (i.e. NR). Different out of band regulatory spectrum emissions masks are used, depending on how far away from the device’s transmitted signal is from the frequency of interest.

Observation 2: ETSI EN 301 574 Part 2 is only applicable for UE with a channel bandwidth greater than 1MHz.

The emissions requirements for this standard are given in tables, including spectrum emissions masks given in Table 4 and spurious emissions requirements given in Table 5.

Proposal 3: ETSI EN 301 574 Part 2 shall be used to assess the regulatory compliance of NR HPUE operating in band n256, including ACLR and SEM.

## 2.3 ETSI EN 302 574 Part 3

The ETSI EN 302 574 Part 3 harmonized standard provides regulatory requirements for band 256 for devices which operate with a channel bandwidth less than 1MHz (e.g. NB-IoT). Different out of band regulatory spectrum emissions masks are used, depending on how far away from the device’s transmitted signal is from the frequency of interest.

Observation 3: ETSI EN 301 574 Part 3 is only applicable for UE with a channel bandwidth less than 1MHz.

The spectrum emissions mask for frequencies less than 2.5MHz from the UE centre frequency is table 3b. The spectrum emissions mask for frequencies more than 2.5MHz from the UE centre frequency is table 3a.

For the In-band spectrum emissions mask case, either Table 4a or Table 4b can be used, though the manufacturer must declare which table was used to assess regulatory compliance.

Proposal 4: ETSI EN 301 574 Part 3 shall be used to assess the regulatory compliance of NB-IoT HPUE operating in band n256, including ACLR and SEM.

## 2.4 FCC Part 25

The FCC part 25 specification clause 202 covers the FCC requirements for bands 253, 255 and 256. Condensed into a table the requirements are as follows, for an authorised bandwidth B, with a transmitter power W:

Table 3 FCC Part 25 Emissions Mask

| Frequency from edge of authorised bandwidth | Attenuation [dB] | Measurement BW [kHz] |
| --- | --- | --- |
| 0.5B - B | 25 | 4 |
| B – 2.5B | 35 | 4 |
| >=2.5B | 43-10log10(W) | 4 |

Proposal 5: FCC Part 25 clause 202 shall be used to assess the regulatory compliance of bands 253, 255 and 256, and shall be used to inform decisions on spectrum emissions related requirements such as ACLR and SEM.

# RF Performance

## 3.1 ACLR Methodology

When HPUE was defined for the TN NR, the ACLR limits were defined as follows:

Table 4 TS 36.101-1 ACLR Requirement for HPUE

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

As can be seen the ACLR requirement is only increased by 1dB for PC1.5 and PC2 from the baseline PC3. PC1 however is increased by 7dB, this is because the device type of a TN PC1 device is expected to be of a larger  form factor to a PC1.5 and below device, thus an RFFE with a better filter response is able to be used, which allows for a more strict ACLR requirement.

Observation 4: The ACLR requirements defined for PC1 for TN NR UE are based on a form factor which is larger than that of a mobile handset, thus allowing a higher quality RFFE to be used. This much higher ACLR may not be achievable in a mobile handset form factor at least for certain transmission configurations, such as NR, but might be achievable for NB-IoT.

For the form factor expected for NTN HPUE, i.e. a mobile handset, an ACLR requirement that is 7dB higher than the PC3 requirement may be impossible. On the other hand, if we impose a lower ACLR, then the UE may meet the 3GPP spectrum emissions mask, but A-MPR would be required in order to meet the regulatory requirements, thus rendering the increase in power class pointless.

Observation 5: If we define an ACLR that is too low, the UE may not meet regulatory spectrum emissions masks, meaning a higher A-MPR would have to be defined, rendering the increase in power class pointless.

Thus, it is suggested that the baseline ACLR requirements for NTN HPUE, for both NR and NB-IoT, are to be derived from the regulatory spectrum emissions masks and the existing 3GPP spectrum emission masks since this is the absolute minimum requirement the UE has to meet to be allowed to legally operate over the licensed spectrum.

Proposal 6: The spectrum emissions masks in the regulatory standards will be used to derive the minimum ACLR requirements for NB-IoT and NR NTN, as this allows us to retain the benefit of power gains made from HPUE without mandating a high value for A-MPR.

## 3.2 Spectrum Emissions Mask Methodology

For TN NR UE, the spectrum emissions mask does not change between power classes. As a result of the high ACLR specified for the PC1 devices in the TN specification as shown above, there is no issue when operating at higher power classes when considering a violation of the spectrum emissions mask.

Observation 6: For PC1 using the TN ACLR specification, the UE will not struggle to meet the SEM due to the high ACLR value.

For NTN UE, the ACLR of PC1 is expected to be much lower. As a result, the UE may struggle to meet the same SEM as the lower power classes. Therefore it may be necessary to derive a new power-class-dependent spectrum emissions mask.

As a baseline, once the minimum ACLR value has been derived based on the regulatory requirements, a new spectrum emissions mask can be derived from the expected emissions of a device of each power class.

Proposal 7: Depending on the findings of the ACLR study based on regulatory requirements, a new spectrum emissions mask can be derived depending on whether the existing spectrum emissions mask is met with the expected ACLR requirement at each power class.

# Coexistence Study Input

## Band 255 Coexistence Analysis

Consider the following band plan for band 255 and band 253:

Figure 1 Band 255 / Band 253 band plan

For HPUE, the only thing changing with respect to coexistence scenarios is the UE UL power. As seen in Figure 1, for UE UL in band 255 / band 253, there is only one TN band, n70, which is potentially of concern 35MHz away from band 255 edge and 20MHz away from band 253 edge. On the other hand there are several TN bands very close to the NTN downlink. Therefore, the only coexistence scenarios worth considering for bands 255/256 are scenarios 2 and 4 as per TR 38.863.

Observation 7: for band 255/253 UL, there is only one TN band within 50MHz from the band edge which must be protected. Therefore, the coexistence study limits for HPUE in these bands can be more relaxed, and the UE TX requirements derived will likely not be based on the TN-NTN coexistence.

Observation 8: In case UE ACS values are studied, for bands 255/253, the only scenarios that would need to be considered are those where NTN is the victim, such as scenario 1 and a new scenario that has not yet been captured in TR 38.863, which would include a TN UL as the aggressor.

Proposal 8: for band 255/253 HPUE coexistence scenario only consider scenarios 2 and 4 as per TR 38.863.

Proposal 9: For bands 255/253, HPUE UL/TX requirements should not be driven by TN-NTN coexistence.

Comparing this band plan with the band plan for band 256:

Figure 2 Band 256 band plan

It can be seen that there are many overlapping / adjacent bands that are much closer to the band 256 uplink. Therefore many scenarios for coexistence must be considered, and the requirements will not be as relaxed when compared to bands 255/253.

Observation 9: for band 256 there are a number of  TN bands overlapping / adjacent to the NTN UL band, therefore when compared to bands 255/253 the general requirements may be driven by the coexistence analysis.

# Conclusions

Proposal 1: ETSI EN 301 681 shall be used to assess the regulatory compliance of HPUE operating in bands n253 and n255, including ACLR and SEM.

Proposal 2: ETSI EN 301 681 can be used to inform system design features for the protection of the radio astronomy bands at 1660-1660.5MHz and 1668-1670MHz.

Proposal 3: ETSI EN 301 574 Part 2 shall be used to assess the regulatory compliance of NR HPUE operating in band n256, including ACLR and SEM.

Proposal 4: ETSI EN 301 574 Part 3 shall be used to assess the regulatory compliance of NB-IoT HPUE operating in band n256, including ACLR and SEM.

Proposal 5: FCC Part 25 clause 202 shall be used to assess the regulatory compliance of bands 253, 255 and 256, and shall be used to inform decisions on spectrum emissions related requirements such as ACLR and SEM.

Proposal 6: The spectrum emissions masks in the regulatory standards will be used to derive the minimum ACLR requirements for NB-IoT and NR NTN, as this allows us to retain the benefit of power gains made from HPUE without mandating a high value for A-MPR.

Proposal 7: Depending on the findings of the ACLR study based on regulatory requirements, a new spectrum emissions mask can be derived depending on whether the existing spectrum emissions mask is met with the expected ACLR requirement at each power class.

Proposal 8: for band 255/253 HPUE coexistence scenario only consider scenarios 2 and 4 as per TR 38.863.

Proposal 9: For bands 255/253, HPUE UL/TX requirements should not be driven by TN-NTN coexistence.

Observation 1: ETSI EN 301 681 is applicable for UEs with an EIRP of less than 15dBW which is highly likely not tbe exceeded by mobile handsets.

Observation 2: ETSI EN 301 574 Part 2 is only applicable for UE with a channel bandwidth greater than 1MHz.

Observation 3: ETSI EN 301 574 Part 3 is only applicable for UE with a channel bandwidth less than 1MHz.

Observation 4: The ACLR requirements defined for PC1 for TN NR UE are based on a form factor which is larger than that of a mobile handset, thus allowing a higher quality RFFE to be used. This much higher ACLR may not be achievable in a mobile handset form factor at least for certain transmission configurations, such as NR, but might be achievable for NB-IoT.

Observation 5: If we define an ACLR that is too low, the UE may not meet regulatory spectrum emissions masks, meaning a higher A-MPR would have to be defined, rendering the increase in power class pointless.

Observation 6: For PC1 using the TN ACLR specification, the UE will not struggle to meet the SEM due to the high ACLR value.

Observation 7: for band 255/253 UL, there is only one TN band within 50MHz from the band edge which must be protected. Therefore, the coexistence study limits for HPUE in these bands can be more relaxed, and the UE TX requirements derived will likely not be based on the TN-NTN coexistence.

Observation 8: In case UE ACS values are studied, for bands 255/253, the only scenarios that would need to be considered are those where NTN is the victim, such as scenario 1 and a new scenario that has not yet been captured in TR 38.863, which would include a TN UL as the aggressor.