# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416412

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	Huawei, HiSilicon

Title: 		Discussion on IMT parameters for 15GHz range

Agenda Item:	6.2.4.2

Document for:	Discussion

# Introduction

RAN WG4 has already replied to ITU-R WP 5D on the parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, for the following two frequency ranges:

- 4400-4800 MHz frequency range parameters in [2], and

- 7125 and 8400 MHz frequency range parameters in [3].

Discussion on parameters for 14800 to 15350 MHz frequency range was already initiated.

In this contribution, we provide further inputs on proposed parameters for 14800 to 15350 MHz frequency range.

# Discussion

## System parameters

For system parameters, the duplex mode was already agreed to be TDD as baseline (keeping SBFD as potential candidate). Signal bandwidth for NR is basically frequency range agnostic, and for the LS reply it is sufficient to reuse the formula (Signal bandwidth = NRB × SCS × 12) as done already for the other frequency ranges.

The only remaining parameter is the channel bandwidth, which is discussed below.

### Channel bandwidth

The following agreement was captured in WF in [4] during RAN4#112:

Agreement: For LS about 15GHz, choose the single value out of 100MHz, 200MHz or 400MHz as baseline with understanding that the other channel bandwidths are not precluded.

So far, a typical channel bandwidth of 100 MHz was decided for 7125 and 8400 MHz frequency range.

When it comes to the selection of typical channel bandwidth for 14800 to 15350 MHz frequency range, we need to consider that this range has not been assigned to FR1 nor to FR2 so far. Currently, there is no IMT spectrum allocation in 14800 to 15350 MHz, so decision on FR1 vs. FR2 assignment is not needed for now, and the purpose of the ITU-R WP 5D task is to evaluate possibility of future IMT allocations towards WRC-27.

Referring back to the Rel-16 RAN4 study item on 7-24 GHz frequency range (as documents in TR 38.820), RAN4 was investigating where could be practical boundary among the FR1 and FR2. This was tightly related to the decision on the conducted vs. radiated requirements applicability, related RF architectures reuse (FR1-like, vs FR2-like), etc. While it was not possible to conclude on concrete FR1/FR2 frequency boundary at that time, it was commonly understood that such boundary is going to be somewhere within 10 – 15 GHz range.

Now we should also consider what are the existing NR channel bandwidths for both FR1 and FR2-1. While 100MHz channel is defined for both FR1 and FR2-1, the 200MHz and 400MHz channels are only supported by FR2-1. This observation has the following consequences:

- If we would select 100 MHz as typical channel bandwidth for 14800 to 15350 MHz frequency range, it would still allow to decide on either FR1, or FR2-1 assignment of 14800 to 15350 MHz in future;

- If we would select 200 MHz as typical channel bandwidth for 14800 to 15350 MHz frequency range, it would implicitly mean that 14800 to 15350 MHz frequency range shall follow FR2-1 system parameters (SCS selection etc.) as 200MHz is currently not supported in FR1.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264.

While the current RAN4 task it to provide parameters for IMT sharing and compatibility studies in preparation for WRC-27, decision on the typical channel bandwidth for 14800 to 15350 MHz frequency range shall not have any impact on future RAN4 decision on FR1 vs. FR2 assignment. With this, we formulate the following proposal:

Proposal 1: Decision on typical channel bandwidth for 14800 to 15350 MHz frequency range shall not influence future decision on 14800 to 15350 MHz assignment to FR1 or FR2, i.e. it shall not exclude FR1 from future discussion.

14800 to 15350 MHz frequency range providing “only” 550MHz, selection of 400 MHz channel bandwidth is not seen as suitable to provide sufficient spectrum utilization flexibility. It is suggested to exclude 400 MHz channel bandwidth from consideration.

Proposal 2: Exclude 400 MHz channel bandwidth from consideration.

Even though we would be supportive to decide on 200 MHz channel bandwidth for 14800 to 15350 MHz frequency range, the above discussion motivates to the following proposal:

Proposal 3: Select 100 MHz as typical channel bandwidth for 14800 to 15350 MHz frequency range (200 MHz channel bandwidths not precluded).

## BS parameters

WF on BS parameters was agreed in [4] during RAN4#112. Below we provide further inputs on general aspects and BS-related parameters.

### FR1-like vs FR2-like requirement baseline

Most of the parameters provided already in LS’ in [2, 3] for lower frequency ranges (4400 to 4800 MHz and for 7125 to 8400 MHz) were based on references to the existing NR BS specifications. Due to the nature of those ranges, they were obviously referring to the FR1 specifications.

For the 14800 to 15350 MHz frequency range this is no longer the case, and it is not obvious whether related FR1, or FR2 requirements shall be used as baseline for ITU-R LS reply. Therefore, we suggest to explicitly decouple discussion on the IMT-2030 LS reply, from any future decisions on 14800 to 15350 MHz frequency range assignment to FR1 or FR2. There is no need to indicate this in the LS itself, but the TR shall capture a disclaimer, so when the final decision is taken on potential spectrum allocations (around WRC-27 timeline), with the technology and specifications advancements, RAN4 would be able to open discussion on 14800 to 15350 MHz RF requirements and FR1 vs FR2 comparisons. In other words: we shall refrain from discussions on RF requirements definitions for 14800 to 15350 MHz at this point of time.

Proposal 4: Capture the following disclaimer in the TR 38.922:

Agreements on parameters and requirements for 14800 to 15350 MHz frequency range shall not be considered decisive for FR1 vs FR2 assignment of this range, nor on related RF requirements limits. Those aspects are expected to be decided in future 6G work item, pending WRC-27 decision.

Another issue identified is potential divergence of decisions among various UE/BS requirements, where e.g. one requirement is decided to be FR1-based, while some other may be decided to be FR2-based (we are already discussing on requirement, with no fundamental decision taken on the FR1 vs FR2 assignment of the 14800 to 15350 MHz frequency range). To address this, the following is proposed:

Proposal 5: Clarify in the LS on 14800 to 15350 MHz, that “Parameters for the frequency range 14800 to 15350 MHz were derived based on [FR1/FR2/mix of FR1 and FR2] RF requirements.”. Text in [] to be updated once all BS and UE requirements are settled.

### BS antenna array and sub-array size

The following agreement was captured in WF in [4]:

Issue 2-1: BS antenna array and sub-array size Recommended for further discussions - 3072 AEs dual polarization: - 16x24 array and 4x1 sub-array - 4096 AEs dual polarization: - 8x32 array and 8x1 sub-array - 16x32 array and 4x1 sub-array - 2048 AEs dual polarization: - 8x32 array and 4x1 sub-array - 1536 AEs dual polarization: - 8x16 and 6x1 sub-array - Other options are not precluded.

Based on the above listed options, our preferred option is 16x32 array and 4x1 sub-array, which has largest AEs number (4096 AEs dual polarization) and better coexistence performance with satellite in the air as shown in the following figure.

Figure 1: IMT BS vertical antenna patters: 16x32 vs 8x32 AE comparison

Proposal 6: For BS antenna array and sub-array size, select 4096 AEs dual polarization case with 16x32 array and 4x1 sub-array.

### AAS vs. non-AAS BS

Template provided by ITU-R WP 5D requests for parameters to be provided for both non-AAS and for AAS BS.

Based on previous discussions, there were proposals to consider 4x4 array for Indoor scenarios – with such antenna array as well as any higher order AA configurations, the non-AAS BS architecture does not seen to be a workable solution. Therefore:

Proposal 7: For 14800 to 15350 MHz frequency range, do not consider non-AAS BS architecture in the LS reply.

### Spectral mask

During RAN4#112, some contribution were suggesting to refer to TR 38.921 (IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz) for the derivation of the BS spectral emission mask for 14800 to 15350 MHz frequency range. Related Cat.A limits are extracted below for reference:

Table 2.2.3-1: WA BS OBUE limits for 10.0 - 10.5 GHz, Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -13 dBm | 1MHz |

While this approach would be good starting point if we decide FR1-like approach, one could question whether FR2-like approach shall be also considered. Related OTA OBUE emission limits (Category A) for FR2 band 24.25 – 33.4 GHz are extracted below:

Table 2.2.3-2: OBUE limits, 24.25 – 33.4 GHz, Category A

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |

One fundamental difference among FR1 and FR2, is lack of basic limit and emissions scaling in FR2 – still, emissions scaling shall not be needed for the purpose of the ITU-R WP 5D sharing and compatibility studies.

Proposal 8: For Category A OBUE limits, consider table 2.2.3-1 and 2.2.3-2 as baseline for spectral mask adjustment for 14800 to 15350 MHz frequency range.

One crucial aspect found in TR 38.921 OBUE section, is the following clarification:

It is agreed that it does not mean that 50 MHz will be the minimum channel bandwidth when RAN4 specified the channel bandwidth and emission requirements.

The above statement clarifies that irrespective of the agreements captured in TR 38.921, there may be another decision in future, defining other value of the minimum channel bandwidth and emissions requirements.

### ACLR /ACS

ACLS and ACS requirement levels are to be derived from the adjacent channel coexistence analyses.

Proposal 9: BS ACLS and BS ACS requirement levels for 14800 to 15350 MHz frequency range are to be derived from adjacent channel coexistence analyses.

### Spurious emissions

Referring to the TS 38.104, one can notice that for FR1 and for FR2 spurious emissions requirements, Category A limits are basically following ITU-R SM.329. According to SM.329, for the 14800 to 15350 MHz fundamental frequency, the spurious emissions requirement shall be defined from 30 MHz to the 2nd	harmonic (include the entire harmonic band and not be truncated at the precise upper frequency limit stated).

Table 2.2.4-1: BS spurious emission limits for 14800 to 15350 MHz for Category A

| Frequency range | Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz |
| 1 GHz – 2nd harmonic |  | 1 MHz |

Proposal 10: For Category A spurious emissions limits in the LS reply, follow SM.329 limit of -13 dBm, for the range of 30 MHz to 2nd harmonic.

Referring to the previous LS for 4400 to 4800 MHz and for 7125 to 8400 MHz [2, 3], both Category A and Category B limits were referred. However, it seems that for the purpose of IMT sharing and compatibility studies it should be sufficient to provide Category A limits only, which are considered to be global limits (as opposed to regional Category B limits), as per SM.329 definitions.

### Out of band emissions

ΔfOBUE = 100 MHz was agreed for 7125 to 8400 MHz range, reusing n104 value of FR1. If we would follow FR2 approach for 14800 to 15350 MHz frequency range, we would be using ΔfOBUE = 1500 MHz as baseline, which seems to be excessing for 15 GHz range. Irrespective of the FR1 vs. FR2 discussion, it is proposed to re-evaluate ΔfOBUE value based on the estimated filtering capabilities.

Proposal 11: For ΔfOBUE value, re-evaluate estimated filtering capabilities before proceeding to FR1, or FR2 value reuse.

### Output power

Following the arrangement for 7125 to 8400 MHz range, it is proposed to focus on the maximum output power, and not to provide inputs on the average output power:

Proposal 12: Agree that the average output power won’t be mentioned in the reply LS for 14800 to 15350 MHz frequency range.

### Sensitivity

Proposal 13: for the sensitivity inputs to the LS table, reuse the approach from previous LS, i.e. To be specified, as the sensitivity is not a critical parameter for sharing and compatibility studies.

### SINR operating range

Proposal 14: for the SINR operating range inputs to the LS table, reuse the approach from previous LS, i.e. refer to “SINR operating range and mapping function”.

## UE parameters

WF on UE parameters was agreed in [5] during RAN4#112. Below we provide further inputs on the UE-related parameters.

### UE type

The following was captured in the WF in [5]:

Issue 3-1: UE Type assumption for comparison of FR1 Vs FR2 like UE performance - FFS the below options for UE types: - Revisit RAN4#111 UE type assumption with the following options - Companies are encouraged to clarify their assumption when they provide simulation results. - FR1-like UE: - Option 1: 1TX assumption with omnidirectional antenna - Option 2: Two discrete elements following Table 1-1 below with maximum-gain across elements selection criterion. - Option 3: For [N] Tx/Rx, [G] dBi gain for link of interest regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel links regardless the AoA. - Companies to clearly state what value of N and G assumed. - FR2-like UE: - FR2-like UE: Two panels with 1x2 array at each panel following Table 1-2 below with maximum gain across panels criterion.

When it comes to Issue 3-1 on UE type assumptions, our preference is FR1-like option 2, i.e. two discrete elements following Table 1-1 with maximum-gain across elements selection criterion.

Proposal 15: For Issue 3-1 on UE type assumptions, select option 2 for the FR1-like alternative (i.e. two discrete elements following Table 1-1 with maximum-gain across elements selection criterion).

Consideration of FR2-like UE in this frequency range presents an attractive opportunity for potential study towards IMT-2030., which would require to consider cost, power consumption analyses, etc. Therefore, we suggest to continue analysis and postpone the final decision:

Proposal 16: Postpone the final decision on the UE type assumption at least until November 2024, to collect more inputs on the analysis of the FR2-like option before taking the decision.

### UE noise figure in the LS reply

The following was captured in the WF in [5]:

At last meeting the following agreement was captured in the WF (R4-2410741): Agreement: - 11dB for simulations. Actual noise factor for reply will be decided based on feasibility. Note that RAN4 agreed on 13 dB for UE in 7.125 – 8400 MHz frequency range.. Issue 3-3: UE noise factor for both simulation and LS reply - FFS the below options: - Option 1: 11 dB i.e. follow simulation assumption - Option 2: 8 dB - Option 3: More than 11 dB

To provide consistent inputs towards ITU-R WP 5D, in our view 11 dB value for the UE NF in 14800 to 15350 MHz frequency range is the most reasonable, as it reflects the value use for the co-existence simulation assumptions. Considering the fact that RAN4 has already agreed on 13 dB NF for UE in 7.125 – 8400 MHz frequency range, it also would make sense to look into higher NF numbers.

Proposal 17: UE noise figure for the LS reply on 14800 to 15350 MHz frequency range to be 11 dB.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Decision on typical channel bandwidth for 14800 to 15350 MHz frequency range shall not influence future decision on 14800 to 15350 MHz assignment to FR1 or FR2, i.e. it shall not exclude FR1 from future discussion.

Proposal 2: Exclude 400 MHz channel bandwidth from consideration.

Proposal 3: Select 100 MHz as typical channel bandwidth for 14800 to 15350 MHz frequency range (200 MHz channel bandwidths not precluded).

Proposal 4: Capture the following disclaimer in the TR 38.922:

Agreements on parameters and requirements for 14800 to 15350 MHz frequency range shall not be considered decisive for FR1 vs FR2 assignment of this range, nor on related RF requirements limits. Those aspects are expected to be decided in future 6G work item, pending WRC-27 decision.

Proposal 5: Clarify in the LS on 14800 to 15350 MHz, that “Parameters for the frequency range 14800 to 15350 MHz were derived based on [FR1/FR2/mix of FR1 and FR2] RF requirements.”. Text in [] to be updated once all BS and UE requirements are settled.

Proposal 6: For BS antenna array and sub-array size, select 4096 AEs dual polarization case with 16x32 array and 4x1 sub-array.

Proposal 7: For 14800 to 15350 MHz frequency range, do not consider non-AAS BS architecture in the LS reply.

Proposal 8: For Category A OBUE limits, consider table 2.2.3-1 and 2.2.3-2 as baseline for spectral mask adjustment for 14800 to 15350 MHz frequency range.

Table 2.2.3-1: WA BS OBUE limits for 10.0 - 10.5 GHz, Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -13 dBm | 1MHz |

Table 2.2.3-2: OBUE limits, 24.25 – 33.4 GHz, Category A

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |

Proposal 9: BS ACLS and BS ACS requirement levels for 14800 to 15350 MHz frequency range are to be derived from adjacent channel coexistence analyses.

Proposal 10: For spurious emissions limits in the LS reply, follow SM.329 limit of -13 dBm, for the range of 30 MHz to 2nd harmonic.

Proposal 11: For ΔfOBUE value, re-evaluate estimated filtering capabilities before proceeding to FR1, or FR2 value reuse.

Proposal 12: Agree that the average output power won’t be mentioned in the reply LS for 14800 to 15350 MHz frequency range.

Proposal 13: for the sensitivity inputs to the LS table, reuse the approach from previous LS, i.e. To be specified, as the sensitivity is not a critical parameter for sharing and compatibility studies.

Proposal 14: for the SINR operating range inputs to the LS table, reuse the approach from previous LS, i.e. refer to “SINR operating range and mapping function”.

Proposal 15: For Issue 3-1 on UE type assumptions, select option 2 for the FR1-like alternative (i.e. two discrete elements following Table 1-1 with maximum-gain across elements selection criterion).

Proposal 16: Postpone the final decision on the UE type assumption at least until November 2024, to collect more inputs on the analysis of the FR2-like option before taking the decision.

Proposal 17: UE noise figure for the LS reply on 14800 to 15350 MHz frequency range to be 11 dB.

Based on the above proposals, TP to TR 38.922 in [6] was drafted for the 14800 to 15350 MHz frequency range parameters.

Proposal 18: It is proposed to be use a Draft running LS for 14800 to 15350 MHz frequency range parameters in [7] as starting point to collect all the agreements so far. One revised, it is proposed for Endorsement.

# References

[1]		RP-241326	Study on IMT parameters for NR for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz 	FS_NR_IMT_4400_7125_14800MHz, SID

[2]		R4-2410576	LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, RAN4#111

[3]		R4-2414449	LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, RAN4#112

[4]		R4-2414283	WF on 15GHz BS parameters, RAN4#112

[5]		R4-2414284	WF on 15GHz UE parameters, RAN4#112

[6]		R4-2416414	TP to TR 38.922: Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27	Huawei, HiSilicon

[7]		R4-2416416	Draft running LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27	Huawei, HiSilicon