# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416414

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	Huawei, HiSilicon

Title: 		TP to TR 38.922: BS parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

Agenda Item:	6.2.4.2

Document for:	Approval

# Introduction

In this contribution, we provide TP to TR 38.922 for clause 6 on 14800 - 15350 MHz frequency range.

# Conclusions

Based on the discussion in [1], the following proposals were formulated:

Proposal 1: Approve the attached TP to TR 38.922 for clause 6 on 14800 - 15350 MHz frequency range.

# References

[1]		R4-2416412	Discussion on IMT parameters for 15GHz range 	Huawei, HiSilicon

# TP to TR 38.922 v0.2.0

------------------------------ Modified section ------------------------------

## 6.2	General parameters

### 6.2.1	General

At the time of writing, there were no NR operating bands defined in the 14800 - 15350 MHz frequency range.

Agreements on parameters and requirements for 14800 to 15350 MHz frequency range shall not be considered decisive for FR1 vs FR2 assignment of this range, nor on related RF requirements limits. Those aspects are expected to be decided in future 6G work item, pending WRC-27 decision.

BS and UE parameters in clause 6.3 and 6.4, were derived based on [FR1/FR2/mix of FR1 and FR2] RF requirements.

### 6.2.2	Duplex mode

Considering decision on 7125 - 8400 MHz frequency range in clause 5.1.1, and observing that all FR2 bands are TDD, TDD duplex mode was agreed for 14800 - 15350 MHz frequency range.

### 6.2.3	Channel Bandwidth

Considering 550 MHz of contiguous spectrum available in 14800 - 15350 MHz frequency range, it was agreed that [100] MHz channel bandwidth can be considered as typical value for the purpose of IMT sharing and compatibility studies in preparation for WRC-27. Selection of this value allows to consider both FR1-like, as well as FR2-like assignment of future operating bands in 14800 - 15350 MHz frequency range. Wider channel bandwidths (i.e. 200 MHz and 400 MHz) were not precluded.

### 6.2.4	Signal Bandwidth

NR signal bandwidth calculation is the same for all considered frequency ranges. For more details refer to clause 4.1.3.

## 6.3	BS parameters

### 6.3.1	General

For 14800 to 15350 MHz frequency range, it was agreed not to consider non-AAS BS architecture (for both indoor and outdoor scenarios) due to practical aspects of the antenna designs. Therefore, all transmitter and receiver characteristics are based on AAS requirements (i.e. [BS type 1-O / BS type 2-O]).

### 6.3.1	Transmitter characteristics

#### 6.3.1.1	Power dynamic range

There is no power control in downlink and fixed power per resource block is assumed during the study phase. Hence 0 dB power dynamic range was agreed for the 14800 - 15350 MHz.

#### 6.3.1.2	Spectral mask

TBD

#### 6.3.1.3	ACLR

TBD

#### 6.3.1.4	Spurious emissions

For Category A spurious emissions limits in 14800 - 15350 MHz frequency range, follow SM.329 [x] and the limit of -13 dBm, for the range of 30 MHz to 2nd harmonic.

Table 6.3.1.4-1: General BS transmitter spurious emission limits in 14800 - 15350 MHz, Category A

Table 6.3.1.4-2: General BS transmitter spurious emission limits in 14800 - 15350 MHz, Category B

#### 6.3.1.5	Maximum output power

The maximum BS output power/sector (e.i.r.p.) was provided in the antenna parameter table of the LS [6], as extracted in Table 6.3.1.5-1. It was agreed to be aligned with antenna characteristics.

Table 6.3.1.5-1: Maximum BS output power in 14800 - 15350 MHz

The Total Radiated Power (TRP) for two polarizations was agreed as shown in Table 6.3.1.5-2 below.

Table 6.3.1.5-2: Total Radiated Power in 14800 - 15350 MHz

#### 6.3.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS on 14800 - 15350 MHz frequency range in [6].

### 6.3.2	Receiver characteristics

#### 6.3.2.1	Noise figure

The BS noise figure relevant for 14800 - 15350 MHz frequency range is listed in Table 6.3.2.1-1.

Table 6.3.2.1-1: BS noise figure for 14800 - 15350 MHz frequency range

| BS class | Noise figure (dB) |
| --- | --- |
| Wide Area | 8 |
| Medium Range | 13 |
| Local Area | 16 |

#### 6.3.2.2	Sensitivity

The sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter in the reply LS on 14800 - 15350 MHz frequency range in [6].

#### 6.3.2.3	Blocking response

TBD

#### 6.3.2.4	ACS

TBD

------------------------------ End of modified section -------------------------