3GPP TSG-RAN WG4 Meeting #112bis	R4-2416417

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Nokia

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	6.16.2

Document for:	Discussion

Introduction

In RAN4#112 the topic of intercell and intracell inter user interference for 8Rx configuration were discussed for the first time with the following main agreements made:

- RAN4 to introduce UE demodulation requirements for both TDD and FDD

- Not to cover PDSCH absolute physical layer throughput requirements for 8Rx

- Not to define different set of requirements for FWA, vehicular and industrial devices

- Only consider MMSE-IRC for requirements with 8Rx intracell inter user interference.

In the following we will discuss the remaining open issues and provide our view with observations and proposals to enable further progress on the topic

Discussion

General

Test scope

In RAN4#112 the test scope was discussed. Some agreements were made and one open issue to be discussed further (see [1]):

Test scope - RAN4 to introduce UE demodulation requirements for both TDD and FDD - Not to cover PDSCH absolute physical layer throughput requirements for 8Rx - Candidate options on whether to cover CQI requirements for ICI scenario: - Option 1: CQI reporting requirements under ICI should be cover for 8Rx - Option 2: No need to cover such requirements

CQI Requirements were previously defined for 8Rx under AWGN conditions. In addition, CQI requirements were also defined in Rel-17 for inter-cell interference under fading channel conditions for 2Rx and 4Rx. We see no reason to not follow precedence and also introduce CQI requirements for 8Rx under inter-cell interference.

Cover CQI reporting requirements under inter-cell interference scenarios for 8Rx (Option 1)

8Rx UE MMSE-IRC receiver assumption

In RAN4#112 it was discussed if requirements shall be defined for inter-cell interference cases using both baseline and simplified 8Rx receiver (as was done previously when defining 8Rx requirements) (see [1][3])

8Rx UE MMSE-IRC receiver assumption - Candidate options - Option 1: Cover both Baseline SU-MIMO 8Rx receiver and Simplified SU-MIMO 8Rx receiver - Option 2: Define requirements only for SU-MIMO baseline receiver - Interested companies are encouraged to bring simulation results to show the gap between the two types of receivers.

In Rel-18 requirements were introduced for both baseline and simplified receiver based on the simulation results showing enough SNR difference between the two receiver implementations. Hence, an implementation using simplified receiver would not be able to pass requirements for baseline receiver.

As highlighted by companies in RAN4#112, 2Rx and 4Rx interference requirements exist in [3] which could potentially cover the inter-cell interference performance for the 4Rx joint receiver. Thus, 8Rx UEs with simplified receiver implementation, which are passing these 4Rx inter-cell interference requirements, can be expected to also have similar performance also for 8Rx.

It must however be noted, that “8Rx-only” UEs with simplified receiver implementation may come to market, which will not be tested, if only baseline receiver requirements are defined (i.e., requirements which are only valid for UEs with baseline receiver).

To avoid such a non-testable devices scenario, we propose to define one set of 8Rx CQI requirements for inter-cell interference, that is not related to a specific receiver type. Though, alternatively, we can be open to defining two sets of requirements (baseline and simplified) for target UE with rank 2 and/or 4, if simulation results indicate that a simplified receiver may not be able to pass interference requirements for baseline receiver.

If 8Rx requirements are defined for baseline receivers only, this means that simplified receiver performance must be secured by 2Rx/4Rx requirements. This will not cover possible “8Rx-only” UEs with simplified receiver, unless it is required that such receiver is tested in an 4Rx configuration.

Define one set of 8Rx requirements, that is not based on a specific receiver type. If not feasible, either define two sets of requirements or require that an “8Rx-only” UE with simplified receiver will also be tested in a 4Rx configuration against existing 4Rx intercell interference requirements.

PDSCH mapping type

In RAN4#112 it was discussed which PDSCH mapping types to cover when defining PDSCH requirements (see [1]):

PDSCH mapping type - Candidate options: - Option 1: Include both PDSCH type A and type B requirements. - Option 2: Reuse the same configuration for Rel-17 2/4Rx, i.e., PDSCH mapping type A only

One of the WI objectives [4] is to enable 8Rx for industrial devices. The industrial use case assumes a very reliable environment with low latency transmissions. This implies short transmissions with short TDRA. This is possible using the PDSCH type B feature. Hence, we see it relevant to also include the PDSCH mapping type B in the 8 Rx UE demod requirements.

Type B will fit to the industrial use case due to the need for very reliable environment with low latency.

RAN4 to include requirements for PDSCH type A. If time permits also consider defining requirements with type B.

PDSCH requirements with inter cell interference

Interference modelling and power levels

In RAN4#112 the interference modelling and power levels were discussed (see [1]):

Interference modelling and power levels - Candidate options: - Option 1: Use the Intercell Interference model, specified in Clause B.6.2 of TS 38.101-4 - Option 1A: Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx - Option 1B: Only consider HomNet scenario with 2 interference cells and reuse the same power levels (i.e., INR values) defined for Rel-17 2/4Rx - Option 2: RAN4 to discuss if it is still reasonable to use 16QAM based randomly modulated data for each interfering cell

We see no reason to reduce the test scope to only HomNet scenario with 2 interference cells, hence we support for 8Rx to analyze both one and two interference cell configurations. In addition, for initial simulation analysis, we see using the Rel-17 defined INR values is a good starting point.

Use the Intercell Interference model, specified in Clause B.6.2 of TS 38.101-4 to cover both HomNet and Hetnet scenarios (Option 1A). As a starting point for initial analysis use 16QAM based random modulated data for each interfering cell with INR values also defined for Rel-17 2/4Rx requirements.

Network Type and SCS/CBW

In RAN4#112 the Network Type and SCS/CBW were not concluded with one option available (see [1]):

Network Type and SCS/CBW - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) - Other options are not precluded

Existing 8Rx requirements were based on 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) and the same applies for existing inter-cell interference requirements, hence it is reasonable to continue using the same SCS/CBW configurations for 8Rx inter-cell interference requirements.

Using 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD for SCS/CBW as a starting point is reasonable and align with existing inter-cell interference requirements.

Reuse the same SCS/CBW defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) for initial evaluation (Option 1)

Propagation condition

In RAN112 the propagation conditions needed was discussed. Specifically, if only TDLA30-10 shall be considered (see [1]):

Propagation condition - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet) - Option 2: Consider TDLA30-10 as the starting point

It is our understanding that TDLA 30-10 represents a relatively benign multipath environment with low doppler that can be expected of a FWA environment whereas TDLC 300-100 represents a complex multipath environment (with a LOS component) with some mobility that can be expected in an industrial environment deployment.
Therefore using the two different propagation conditions for requirement definition seems reasonable to cover different use-case related to the WID description (see [4]).

In addition, as TDLC300-100 and TDLA30-10 has been used in Rel-17 inter-cell interference requirements it is also a good starting point for 8Rx.

TDLC300-100 and TDLA-30-10 can represent industrial and FWA environments and have already been used in Rel-17 inter-cell interference requirements making both channel models a good starting point for further analysis for 8Rx.

Consider both TDLC300-100 and TDLA30-10 to represent both HomNet (2 interfering cells) and HetNet (1 interfering cell) (Option 1).

Rank

In RAN4#112 the rank to be used for requirement definition was brought up (see [1]):

Rank - Candidate options: - Option 1: Consider higher rank for 8Rx - Option 1A: RAN4 to investigate rank 1, 2, 4 and choose a reasonable requirement value - Option 1B: Rank 2 and Rank 4 as the starting point - Option 1C: Rank 2 and Rank 4 and Rank 8 - Option 1D: Rank 4 - Option 1E: Consider 2 layers, 4 layers and 6 layers - Option 2: Same as Rel-17 2/4Rx, i.e., Rank 1

As a starting point lower and higher rank for requirement definition shall not be excluded unless found unfeasible. As it was agreed in Rel-18 to not include requirements for Rank 1, RAN4 should focus on defining requirements for inter-cell interference with rank 2 and above.

Consider Rank 2 and Rank 4 and Rank 8 (Option 1C). In case Rank 8 is found not feasible for requirement definition, then also consider rank 6.

MCS

In RAN4#112 the candidates for MCS were discussed (see [1]):

MCS - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., MCS 13 - Option 2: MCS 16 - Option 3: MCS 19 for rank 2, MCS 17 for rank 4 and 8 - Option 4: Define different parameters for FWA, Vehicular and Industrial deployments respectively

Existing requirements for Rank 8 are defined with quite high SNR (22.5dB/23.6dB) for 8Rx using MCS19. For Inter-cell interference cases, it is expected that the resulting requirements will be higher than the existing requirements possible resulting in a too high SNR. We would prefer to keep similar modulation order for Inter-cell interference as for non-interference cases (i.e. 64QAM), however as just indicated MCS19 is likely unfeasible for higher Rank with ICI so a lower MCS can be selected.
This leads us to the following:

Existing SU-MIMO Rank 8 requirements are defined with quite high SNR (22.5dB/23.6dB) and addition of interference will possibly increase the required SNR to a level where requirement definition is not feasible. For lower rank (i.e. rank 2), the existing 8Rx requirements are expected to be low enough to sustain the increase due to Inter-cell interference and is therefore still expected to be feasible for requirement definition.

As a starting point use the following configurations:
- Rank 2: MCS 19
- Rank 4,6,8: MCS17

Antenna and MIMO correlation

In RAN4#112 the Antenna and MIMO correlation was discussed (see [1]):

Antenna and MIMO correlation - Candidate options: - Option 1: 2T8R ULA low - Option 2: 4T8R - Option 3: - 2T8R ULA medium B - 4T8R ULA low - 8T8R ULA low

Option 3 from above was used to define existing 8Rx requirements in 38.101-4 [3], hence we see it as a good starting point for further analysis of intercell interference in RAN4.

For initial simulation analysis use the following antenna and MIMO correlation settings (Option 3):
- 2T8R ULA medium B
- 4T8R ULA low
- 8T8R ULA low

Other parameters

In RAN4#112 it was proposed to reuse same configuration for Rel-17 2/4Rx (see [1])

Other parameters - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx - Other options are not precluded

For 8Rx we do not see an issue using the existing Rel-17 configurations as baseline with modifications needed to support 8Rx (i.e. configuration parameters not specifically dictated by this WID can be re-used from existing Rel-17 configurations)

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC (Option 1) as a starting point for initial evaluation.

PDSCH requirements with intra cell inter user interference

Network Type and SCS/CBW

In RAN4#112 the Network Type and SCS/CBW were not concluded with one option available (see [1]):

Network Type and SCS/CBW - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U - Other options are not precluded

Existing 8Rx requirements were based on 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) and the same applies for existing intra-cell inter-user interference requirements, hence it is reasonable to continue using the same SCS/CBW configurations for 8Rx intra-cell inter-user interference requirements.

Using 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD for SCS/CBW as a starting point is reasonable and align with previously used SCS/CBW.

Reuse the same SCS/CBW defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) for initial evaluation (Option 1)

Interference modelling

In RAN4#112 the interference modelling were brought up for discussion with focus on beamforming model and number of co-UEs (see [1]):

Interference modelling - Candidate options on MU-MIMO Beamforming Model: - Option 1: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4 - Option 2: Include in a new clause of Annex B.6 of TS 38.101-4 - Candidate options on co-UE number: - Option 1: Same as Rel-17 2/4Rx requirements, i.e., 1 target UE + 1 co-UE - Option 2: Consider 1 target UE and more than 1 co-UEs

Beamforming model:

Current model is covering how to apply MU precoding in TE and we do not see the need for a new capability to be applied, hence the current approach should be sufficient/fit for the purpose of this WID. Based on our understanding we see it reasonable to reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4

We are open to understand better the view of proponents of option 2.

Current model is covering how to apply MU precoding in TE and we do not see the need for a new capability to be applied, hence the current approach should be sufficient/fit for purpose of this WID.

Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4 (Option 1)

Number of coUEs:

Having more than one coUE is more likely to occur in deployment for 8Rx compared to 4Rx, which was maximum number of Rx in the Rel-17 discussions. We do not, however, see a 2 coUE configuration in deployment as often occurring, hence it is not the highest priority to define requirement for 2 coUE setup. RAN4 should however not dismiss the configuration from the simulation analysis.

In 8Rx it is more likely that 2 co-UEs will be seen in deployment, hence we see it relevant for further simulation analysis.

Consider 1 target UE and more than 1 co-UEs (Option 2).

Propagation condition

In RAN4#112 the propagation conditions to be used for requirement definition was discussed (see [1]):

Propagation condition - Candidate options: - Option 1: Reuse the same precoding method as MMSE-IRC, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2 - Option 2: Consider TDLA30-10

It is our understanding that TDLA 30-10 represents a relatively benign multipath environment with low doppler that can be expected of a FWA environment whereas TDLC 300-100 represents a complex multipath environment (with a LOS component) with some mobility that can be expected in an industrial environment deployment.
Therefore using the two different propagation conditions for requirement definition seems reasonable to cover different use-case related to the WID description (see [4]).

In addition, as TDLC300-100 and TDLA30-10 has been used in Rel-17 intra-cell inter-user interference requirements it is also a good starting point for 8Rx.

TDLC300-100 and TDLA-30-10 can represent industrial and FWA environments and have already been used in Rel-17 intra-cell inter-user interference requirements making both channel models a good starting point for further analysis for 8Rx.

Consider both TDLC300-100 and TDLA30-10 for initial analysis. Decide which channel to use for specific rank combinations when rank combinations are agreed.

Precoding method for the co-scheduled UE

In RAN4#112 the precoding method for the co-UE(s) were brough up for discussion (see [1]):

Precoding method for the co-scheduled UE - Candidate options: - Option 1: Reuse the same precoding method as MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests. - Option 2: Orthogonal precoding with the target UE, i.e., any column of matrix of any serving UE or co-scheduled UE is orthogonal to any column of matrix of any other UE

As seen before we also here expect that random precoding will be required for low rank, as the use of orthogonal precoding with low rank will not provide enough diversity in the system to show high gain compared to reference MMSE-MRC receiver.

We expect random precoding will be necessary in rank 1+1 to introduce enough diversity for showing high enough gain compared to reference receiver.

Reuse the same precoding method as MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for all other rank combinations (i.e. not 1+1).

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation

In RAN4#112 the antenna, rank and MIMO correlation configurations were brought up for discussion (see [1]):

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation - Candidate options: - Option 1: 2T8R ULA low - Option 2: Rank 4+2 with 8T8R - Option 3: - Rank 2+2 for 2T8R - Rank 4+4 for 4T8R - Rank 8+8 for 8T8R when 1 paired UE scheduled - Option 4: - Rank 1+1 for 2T8R - Rank 2+2 for 4T8R - Rank 2+6 or rank 6+6 for 8T8R - Option 5: - Rank 2+2+2 for 8T8R - Rank 4+4 for 4T8R - Option 6: - Rank 2+2 - Rank 4+4

For initial evaluation the combinations already used in Rel-17 (i.e. 1+1 and 2+2) shall be done also for 8Rx. In addition, 8Rx now provides new configurations due to the availability of 8 transmission layers.
As part of the additional combinations, it is possible to define a setup with 2 interference UEs in for example a 2+2+2 configuration, however for initial evaluation we do not see such configuration as a priority as they are less likely to occur in deployment. In case it is selected to use a 2+2+2 configuration, the two co-UEs must not be configured with the same modulation order.
Additionally, we see the proposed combinations of 6+2 or 2+6 as relevant combinations for real network deployments, where the 6+2 configuration is prioritized as being relevant for FWA.

We see the need to secure requirements for 8Rx with both Rel-17 rank configurations (1+1 and 2+2) as well as the 8Rx additional configurations in the following order of priority: (4+4, 6+2), (2+2+2, 2+6).

For initial evaluation focus on the following antenna configuration and rank (target UE + coUE) as starting point:
1st priority:
- Rank 1+1 for 2Tx8Rx
- Rank 2+2 for 4Tx8Rx
- Rank 4+4 for 8Tx8Rx
- Rank 6+2 for 8Tx8Rx
2nd propriety:
- Rank 2+2+2 for 8Tx8Rx
- Rank 2+6 for 8Tx8Rx

MCS

In RAN4#112 the decision of MCS for the target UE was brought up (see [1]):

MCS - Candidate options for the target UE: - Option 1: Same as Rel-17 2/4Rx requirements for MMSE-IRC, i.e., MCS 13 - Option 2: - MCS 19 Table 1 for Rank 2 - MCS 17 Table 1 for Rank 4 - MCS 17 Table 1 for Rank 8 - Option 3: Consider MCS 17 and MCS 13 - Option 4: MCS 19 for rank 2, MCS 13 for rank 4

In existing Rel-17 requirements MCS13 was used for both 1+1 and 2+2 cases. For 8Rx we see it possible to select higher MCS for both 1+1 and 2+2 configurations, however for higher rank (f.ex. 4+4 and 6+2) it is likely that a lower MCS will be required. With 8Rx we see it possible for lower rank setup to now include MCS with 64QAM configuration.

Use MCS17/19 for rank 1 and rank 2 and MCS13/17 for Rank4 and Rank 6 as starting point. Further discuss down selecting after initial simulation analysis.

Other parameters

In RAN4#112 it was proposed to reuse same configuration for Rel-17 2/4Rx (see [1]):

Other parameters - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC - Other options are not precluded

For 8Rx we do not see an issue using the existing Rel-17 configurations as baseline with modifications needed to support 8Rx (i.e. configuration parameters not specifically dictated by this WID can be re-used from existing Rel-17 configurations)

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC (Option 1) as a starting point for initial evaluation.

Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm

In RAN4#112 it was brought up if there is a need to introduce two CQI calculations assumptions (baseline and simplified receiver) (see [1])

CQI calculation algorithm - Candidate options: - Option 1: RAN4 to consider whether to introduce two CQI calculation assumptions to match two 8Rx receiver assumptions. - Baseline CQI calculation: UE perform CQI calculation with 8Rx joint processing - Simplified CQI calculation: UE perform CQI calculation with separate 4Rx processing - Other options not precluded

In Rel-18 CQI requirements were only defined for 8Rx AWGN case without any distinction between simplified or baseline receiver, hence there were found no need to distinguish between CQI calculation assumptions.

We support only introducing one CQI calculation assumption independent of the receiver types unless it is found that introduction of two CQI calculation assumptions is needed possible if agreed to introduce CQI requirements for fading conditions.

For now, introduce one CQI calculation assumption independent of receiver type. Reconsider if later proven that two CQI calculation assumptions matching the two receiver types are needed.

Test metric

Test metric - Candidate options: - Option 1: Reuse the same test metric for Rel-17 2/4Rx - a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g . - b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.. - Other options are not precluded

For 8Rx we do not see any reason to change the test metric from Rel-17, hence we are fine with reusing the same test metric from Rel-17 2/4Rx.

As a starting point, reuse the same test metric as for Rel-17 2/4Rx (Option 1) for initial evaluation

Propagation condition

Propagation condition - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell - Option 2: Use AWGN static channel as the baseline, further study whether to involve fading channel

Fading channel was used in Rel-17 CQI requirements for inter-cell interference. No CQI requirements were defined for inter-cell interference for 8Rx in Rel-18 and we could not fully re-construct the reason for including AWGN and not including fading channel conditions.

We could not fully re-construct the reason for not including fading channel conditions and it does not seem to be captured in the meeting notes.

Further study whether to also include fading channel conditions. In case it is decided to not include fading channel conditions then reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Interference power level

In RAN4#112 the question of interference power level was brought up (see [1]):

Interference power level - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1 - Other options are not precluded

In existing Rel-17 CQI requirements for inter-cell interference an interference level of 10.04dB was used for both 2Rx and 4Rx. For initial evaluation we are fine with using the same interference level as a starting point.

Re-use the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1 for initial evaluation.

Antenna configuration and MIMO correlation for the serving cell

Antenna configuration and MIMO correlation for the serving cell - Candidate options: - Option 1: 2T8R ULA Low - Option 2: Consider 4T8R

Since existing 8Rx CQI requirements are defined with 4T8R, the definition of CQI requirements for 8Rx with inter-cell interference shall also be using antenna configuration 4T8R for the serving cell unless initial evaluation shows it not feasible to define requirements for 4T8R. As such, we see no reason to change the baseline from the prior 8Rx CQI requirements.

Existing CQI requirements for 8Rx is defined in a 4T8R antenna configuration and we see no reason changing the baseline from the prior 8Rx requirements.

Use 4T8R ULA Low antenna configuration for the serving cell (Option 2)

Rank

In RAN4#112 the Rank for CQI requirement definition was discussed (see [1])

Rank - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., rank 1 - Option 2: Rank 2 - Option 3: Rank 4

Existing CQI requirements for 8Rx is defined using Rank 4 (Table A.4-3, TBS.3-2 in [3]), hence we see no reason to not use rank 4 as a starting point also for 8Rx with intercell interference.

If 4T8R configuration is agreed, use Rank 4 as a starting point for initial evaluation of CQI requirements.

Tested SINR value, throughput ratio g  and BLER X%

In RAN4#112 it was brough up how to decide the SINR, throughput and BLER for CQI requirements (see [1]):

Tested SINR value, throughput ratio g and BLER X% - Candidate options: - Option 1: Should be decided based on simulation results - Other options are not precluded

It has been the usual practice in previous requirement definitions to decide the SINR, throughput ratio and BLER based on companies’ simulation results. We see no reason to change this now.

Decide the tested SINR value, throughput ratio and BLER based on simulation results.

CSI-IM and NZP CSI-RS configurations

In RAN4#112 it was brought up if the CSI-IM and NZP CSI-RS configurations from Rel-17 could be reused in Rel-19 for 8Rx (see [1]):

CSI-IM and NZP CSI-RS configurations - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx - Other options are not precluded

We are fine to re-use the Rel-17 configurations for further simulation alignment.

Reuse the same configuration for Rel-17 2/4Rx (Option 1)

Other parameters

Other parameters - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx - Option 2: For the serving cell, reuse the setup from 8Rx CQI reporting requirement in Rel-18

For 8Rx we do not see an issue using the existing Rel-17 configurations as baseline with modifications needed to support 8Rx (i.e. configuration parameters not specifically dictated by this WID can be re-used from existing Rel-17 configurations)

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC (Option 1) as a starting point for initial evaluation.

Applicability rule and release independent

Test applicability

Test applicability - Candidate options: - Option 1: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule - Option 2: Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined

At this point it is not possible to determine if future 8Rx requirements will fully cover the same UE functionality as existing 2Rx/4Rx requirements, hence we do not at this time see it reasonable to discuss test applicability. Our view at this point is that the new requirements should be mandatory requirements for all 8Rx UEs without additional test applicability rules. However the discussion on test applicability can be postponed until it is more clear, which requirements are to be defined.

Baseline assumption: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule (option 1)

Release independent

The question of release independency was brought up in RAN4#112 (see [1]):

Release independent - Candidate options: - Option 1: The new requirements for 8Rx should be release independent from Rel-18 - Other options are not precluded

Since there are already 8Rx requirements defined in Rel.18 and it is expected that a Rel.18 UE will perform equal in interference scenarios to a Rel-19 UE, we see no issue for new requirements for 8Rx to also be mandatory from Rel-18.

We expect an Rel-18 UE will also be able to pass the Rel-19 added interference requirements for 8Rx.

Conclusion

In this paper we have provided our view on the open issues related to requirement definition for 8Rx inter-cell and inter-cell inter-user requirements.

The following Observations and Proposals were made:

General

Test scope

- Cover CQI reporting requirements under inter-cell interference scenarios for 8Rx (Option 1)

8Rx UE MMSE-IRC receiver assumption

- If 8Rx requirements are defined for baseline receivers only, this means that simplified receiver performance must be secured by 2Rx/4Rx requirements. This will not cover possible “8Rx-only” UEs with simplified receiver, unless it is required that such receiver is tested in an 4Rx configuration.

Define one set of 8Rx requirements, that is not based on a specific receiver type. If not feasible, either define two sets of requirements or require that an “8Rx-only” UE with simplified receiver will also be tested in a 4Rx configuration against existing 4Rx intercell interference requirements.

PDSCH mapping type

Type B will fit to the industrial use case due to the need for very reliable environment with low latency.

RAN4 to include requirements for PDSCH type A. If time permits also consider defining requirements with type B.

PDSCH requirements with inter cell interference

Interference modelling and power levels

Use the Intercell Interference model, specified in Clause B.6.2 of TS 38.101-4 to cover both HomNet and Hetnet scenarios (Option 1A). As a starting point for initial analysis use 16QAM based random modulated data for each interfering cell with INR values also defined for Rel-17 2/4Rx requirements.

Network Type and SCS/CBW

Using 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD for SCS/CBW as a starting point is reasonable and align with existing inter-cell interference requirements.

Reuse the same SCS/CBW defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) for initial evaluation (Option 1)

Propagation condition

TDLC300-100 and TDLA-30-10 can represent industrial and FWA environments and have already been used in Rel-17 inter-cell interference requirements making both channel models a good starting point for further analysis for 8Rx.

Consider both TDLC300-100 and TDLA30-10 to represent both HomNet (2 interfering cells) and HetNet (1 interfering cell) (Option 1).

Rank

Consider Rank 2 and Rank 4 and Rank 8 (Option 1C). In case Rank 8 is found not feasible for requirement definition, then also consider rank 6.

MCS

Existing SU-MIMO Rank 8 requirements are defined with quite high SNR (22.5dB/23.6dB) and addition of interference will possibly increase the required SNR to a level where requirement definition is not feasible. For lower rank (i.e. rank 2), the existing 8Rx requirements are expected to be low enough to sustain the increase due to Inter-cell interference and is therefore still expected to be feasible for requirement definition.

As a starting point use the following configurations:
- Rank 2: MCS 19
- Rank 4,6,8: MCS17

Antenna and MIMO correlation

For initial simulations used the following antenna and MIMO correlation settings (Option 3):
- 2T8R ULA medium B
- 4T8R ULA low
- 8T8R ULA low

Other parameters

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC (Option 1) as a starting point for initial evaluation.

PDSCH requirements with intra cell inter user interference

Network Type and SCS/CBW

Using 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD for SCS/CBW as a starting point is reasonable and align with previously used SCS/CBW.

Reuse the same SCS/CBW defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) for initial evaluation (Option 1)

Interference modelling – Beamforming model

Current model is covering how to apply MU precoding in TE and we do not see the need for a new capability to be applied, hence the current approach should be sufficient/fit for purpose of this WID.

Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4 (Option 1)

Interference modelling – Number of coUEs

In 8Rx it is more likely that 2 co-UEs will be seen in deployment, hence we see it relevant for further simulation analysis.

Consider 1 target UE and more than 1 co-UEs (Option 2).

Propagation condition

TDLC300-100 and TDLA-30-10 can represent industrial and FWA environments and have already been used in Rel-17 intra-cell inter-user interference requirements making both channel models a good starting point for further analysis for 8Rx.

Consider both TDLC300-100 and TDLA30-10 for initial analysis. Decide which channel to use for specific rank combinations when rank combinations are agreed.

Precoding method for the co-scheduled UE

We expect random precoding will be necessary in rank 1+1 to introduce enough diversity for showing high enough gain compared to reference receiver.

Reuse the same precoding method as MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for all other rank combinations (i.e. not 1+1).

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation

We see the need to secure requirements for 8Rx with both Rel-17 rank configurations (1+1 and 2+2) as well as the 8Rx additional configurations in the following order of priority: (4+4, 6+2), (2+2+2, 2+6).

For initial evaluation focus on the following antenna configuration and rank (target UE + coUE) as starting point:
1st priority:
- Rank 1+1 for 2Tx8Rx
- Rank 2+2 for 4Tx8Rx
- Rank 4+4 for 8Tx8Rx
- Rank 6+2 for 8Tx8Rx
2nd propriety:
- Rank 2+2+2 for 8Tx8Rx
- Rank 2+6 for 8Tx8Rx

MCS

Use MCS17/19 for rank 1 and rank 2 and MCS13/17 for Rank4 and Rank 6 as starting point. Further discuss down selecting after initial simulation analysis.

Other parameters

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC (Option 1) as a starting point for initial evaluation.

Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm

For now, introduce one CQI calculation assumption independent of receiver type. Reconsider if later proven that two CQI calculation assumptions matching the two receiver types are needed.

Test metric

As a starting point, reuse the same test metric as for Rel-17 2/4Rx (Option 1) for initial evaluation

Propagation condition

We could not fully re-construct the reason for not including fading channel conditions and it does not seem to be captured in the meeting notes.

Further study whether to also include fading channel conditions. In case it is decided to not include fading channel conditions then reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Interference power level

Re-use the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1 for initial evaluation.

Antenna configuration and MIMO correlation for the serving cell

Existing CQI requirements for 8Rx is defined in a 4T8R antenna configuration and we see no reason changing the baseline from the prior 8Rx requirements.

Use 4T8R ULA Low antenna configuration for the serving cell (Option 2)

Rank

If 4T8R configuration is agreed, use Rank 4 as a starting point for initial evaluation of CQI requirements.

Tested SINR value, throughput ratio g  and BLER X%

Decide the tested SINR value, throughput ratio and BLER based on simulation results.

CSI-IM and NZP CSI-RS configurations

Reuse the same configuration for Rel-17 2/4Rx (Option 1)

Other parameters

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC (Option 1) as a starting point for initial evaluation.

Applicability rule and release independent

Test applicability

Baseline assumption: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule (option 1)

Release independent

We expect an Rel-18 UE will also be able to pass the Rel-19 added interference requirements for 8Rx.

- References

- R4-2413570 - WF on NR demodulation performance: Phase 5.

- TS38.212 - Multiplexing and channel coding (Release 18)

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

- RP-241297, “NR demodulation performance: Phase 5”, RAN#104, June 2024