3GPP TSG-RAN WG4 Meeting #112bis	R4-2416432
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.12.3.2

Source:	CAICT

Title:	Framework and time plan for Rel-19 MIMO OTA performance requirements development (Oct 2024)

Document for:	Approval

1	Introduction

This contribution provides a framework and time plan for Rel-19 MIMO OTA performance requirements development, based on the WID [1], the workplan [2], the recent agreements in WF [3], Rel-17 and Rel-18 MIMO OTA lab alignment outcomes [4, 5], and the Rel-18 approved framework [6].

This contribution can be revised according to discussion outcomes of this meeting.

2	Discussion

One of the performance part objectives of Rel-19 TRP_TRS_MIMO_OTA_Ph3 WI is to specify MIMO OTA performance requirements for several FR1 bands, as described in the WID as below [1].

- Specify MIMO OTA requirements: - Specify FR1 MIMO OTA requirements for new bands for handheld UE based on operators demand (2 layers or 4 layers) - Band n3, n8, n77, n79 are first priority - The requirements are specified based on Rel-17 static MPAC test method

Bands n3, n8, n77, n79 are listed as first priority in Rel-19 MIMO OTA requirements work. In Rel-17 and Rel-18, RAN4 specified TRMS minimum performance requirements for bands n1, n5, n28, n41, n78.  The FR1 MIMO OTA performance requirements work in Rel-17~19 is summarized in Table 1.

Table 1. Summary of the FR1 MIMO OTA performance requirements work in Rel-17~19

| NR bands | Duplex Mode | Channel Bandwidth [MHz] | Carrier center of Mid channel [MHz] | MIMO layer | Channel model | TRMS requirements Status |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | FDD | 10 | 2140 | 4x4 | FR1 UMa CDL-C | Defined in Rel-18 |
| n5 | FDD | 10 | 881.5 | 2x2 | FR1 UMi CDL-C |  |
| n28 | FDD | 10 | 780.5 | 2x2 | FR1 UMi CDL-C |  |
| n41 | TDD | 40 | 2592.99 | 4x4 | FR1 UMa CDL-C | Defined in Rel-17 |
| n78 | TDD | 40 | 3549.99 | 4x4 | FR1 UMa CDL-C |  |
| n3 | FDD | 10 | 1842.5 | to be discussed | Depends on MIMO layer | Rel-19 objectives in the WID |
| n8 | FDD | 10 | 942.5 | 2x2 | FR1 UMi CDL-C |  |
| n77 | TDD | 40 | 3750 | 4x4 | FR1 UMa CDL-C |  |
| n79 | TDD | 40 | 4700.01 | 4x4 | FR1 UMa CDL-C |  |

In TS 38.101-1, it is stated that:

7.2 Diversity characteristics The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination.

For bands n3 and n8, the UE is required to be equipped with a minimum of two Rx antenna ports. n8 is a low frequency band where it is difficult to equip four Rx antenna ports within a handheld UE due to the relatively large wavelength. However, for band n3, a proportion of commercial smart phones are actually with four Rx antenna ports. A precedent in Rel-18 is band n1, although 3GPP spec. does not require 4Rx UE in this band, RAN4 still specified 4x4 MIMO OTA requirement rather than 2x2 MIMO OTA requirement because 4Rx UEs at band n1 is the majority on the market and mandatory in some countries/regions. Thus, RAN4 should make decision on which requirement(s) should be specified for band n3 – 2x2 MIMO OTA or 4x4 MIMO or both?  Besides, the workplan [2] is to finalize the framework no later than RAN4#113 (Nov. 2024) and then start the MIMO OTA measurement campaign (including prerequisite channel model validations), thus, RAN4 should make the decision no later than RAN4#113.

Proposal 1: RAN4 should make the decision on defining which MIMO OTA requirement for band n3, no later than RAN4#113 (Nov. 2024).

- Option 1: 4x4 MIMO OTA requirement

- Option 2: 2x2 MIMO OTA requirement

- Option 3: both

The frequency range of band n77 (3300 MHz – 4200 MHz) covers that of n78 (3300 MHz – 3800 MHz). To check whether the n78 MIMO OTA requirement can be reused to n77, we collected TRMS measurement results in both n77 and n78 from 10 different commercial smart phones. The testing was performed at the Mid channels of n77 and n78 respectively, while the measurement setup, channel model, and test parameters are identical.  Comparison of the TRMS measurement results between n77 and n78 is listed in Table 2. All the 10 DUTs show worse MIMO OTA performance at n77 when compared to n78, the TRMSaverage,70 gap ranges from 1.93 to 4.61 dBm/30kHz. Thus, it is not feasible to reuse the MIMO OTA requirement for n78 to n77.

Table 2. Comparison of the TRMS measurement results between n77 and n78 [dBm/30kHz]

| UE | TRMSaverage,70 @ n77 (3750MHz) | TRMSaverage,70 @ n78 (3549.99MHz) | TRMS gap between n77 and n78 |
| --- | --- | --- | --- |
| UE#1 | -94.90 | -97.48 | 2.58 |
| UE#2 | -94.48 | -96.82 | 2.34 |
| UE#3 | -95.95 | -98.33 | 2.38 |
| UE#4 | -96.26 | -98.55 | 2.29 |
| UE#5 | -96.04 | -98.02 | 1.98 |
| UE#6 | -91.75 | -94.77 | 3.02 |
| UE#7 | -93.46 | -96.82 | 3.36 |
| UE#8 | -96.64 | -98.57 | 1.93 |
| UE#9 | -94.15 | -98.14 | 4.00 |
| UE#10 | -92.72 | -97.33 | 4.61 |

Observation 1: All the 10 DUTs show worse MIMO OTA performance at n77 when compared to n78, the TRMSaverage,70 gap ranges from 1.93 to 4.61 dBm/30kHz. It is not feasible to reuse the n78 MIMO OTA requirement to n77.

At RAN4#112 meeting, it was agreed that no static MIMO OTA lab alignment is need in Rel-19, as captured in the WF [3]:

Issue 5-2-1: Further FR1 MIMO OTA lab alignment activity Agreements: - No need to perform static MPAC lab alignment activities in Rel-19.

In Rel-17, RAN4 successfully completed the FR1 MIMO OTA lab alignment activities for bands > 1GHz, with 6 labs (Apple, CAICT, CMCC&BUPT, Huawei, MediaTek, Xiaomi) were confirmed as aligned [4]; in Rel-18, the FR1 MIMO OTA lab alignment activity for bands < 1GHz was completed with 6 aligned labs (Apple, CAICT, CMCC&BUPT, MediaTek, OPPP, Xiaomi) [5]. In Rel-19, the measurement data for deriving TRMS requirements can be collected from Rel-17 aligned labs for bands > 1GHz, and from Rel-18 aligned labs for bands < 1GHz.

In Section 3 of this contribution, we propose an initial framework for Rel-19 MIMO OTA performance requirement development. The framework can be updated based on discussion outcomes at this meeting and the next meeting.

Proposal 2: Approve the framework in Section 3 of this contribution for Rel-19 MIMO OTA performance requirements development. The framework can be updated based on discussion outcomes at this meeting and the next meeting.

3	Framework for Rel-19 MIMO OTA performance requirements development (for approval)

3.1 Overall work flow

The overall work flow of Rel-19 MIMO OTA performance requirements development for bands <1GHz (n8) and bands >1GHz (n3/n77/n79) are illustrated in Figs. 1 (a) and (b), respectively.

(a) for bands <1GHz (n8)                           (b) for bands >1GHz (n3/n77/n79)

Fig. 1.  Work flow of Rel-19 MIMO OTA performance requirements development

In general, Rel-19 MIMO OTA performance requirements will be derived based on a measurement data pool of commercial devices per band. To establish valid and trustable measurement data pools for defining FR1 MIMO OTA performance requirements, the following activities are required.

  - For bands <1GHz (n8)

    - Channel Model Validation: Companies shall complete channel model validation before submitting measurement results, validation results should be submitted to RAN4 for review. Working procedures of the channel model validation are provided in Section 3.2.1.

      - Note: Labs that submitted channel model validation results for band n8 in Rel-18 (as captured in TR 38.761) do not need to submit the results again.

    - Measurement Campaign: Collect measurement data from Rel-18 aligned labs (Apple, CAICT, CMCC&BUPT, MediaTek, OPPP, Xiaomi) for defining FR1 MIMO OTA requirements for bands < 1GHz. Working procedures of the measurement campaign are provided in Section 3.2.2.

  - For bands >1GHz (n3/n77/n79)

    - Channel Model Validation: Companies shall complete channel model validation before submitting measurement results, validation results should be submitted to RAN4 for review. Working procedures of the channel model validation are provided in Section 3.2.1.

      - Note: Rel-17 aligned labs do not need to submit channel model validation results for band n77, because the channel model validation results for band n78 are applicable to band n77 due to the same channel model and validation frequency.

    - Measurement Campaign: Collect measurement data from Rel-17 aligned labs (Apple, CAICT, CMCC&BUPT, Huawei, MediaTek, Xiaomi) for defining FR1 MIMO OTA requirements for bands > 1GHz. Working procedures of the measurement campaign are provided in Section 3.2.2.

Detailed working procedures for Rel-19 MIMO OTA performance requirements development are described in Section 3.2.

3.2 Detailed working procedures

3.2.1 Channel Model Validation

- The purpose of Channel Model Validation is to ensure that the channel models are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the test zone of the MPAC system.

- The channel model validation measurements shall be performed as described in Annex C.3 of TS 38.151, including:

  - Power Delay Profile (PDP)

  - Doppler/Temporal correlation

  - Spatial correlation

  - Cross-polarization

  - Power validation

- Test band: n3, n8, n79; Test frequency: as specified in Tables C.3.1-1 and C.3.1-2 of TS 38.151

- Channel model: as specified in Annex C.1 of TS 38.151

  - For band n8: FR1 UMi CDL-C (2x2 MIMO)

  - For band n79: FR1 UMa CDL-C (4x4 MIMO)

  - For band n3: TBD

- Pass/fail limits: as defined in Annex C.4 of TS 38.151

3.2.2 Measurement Campaign

- The purpose of Measurement Campaign is to collect measurement results of commercial devices from permitted labs for specifying FR1 MIMO OTA performance requirements.

- Test cases for Rel-19 FR1 MIMO OTA Measurement Campaign:

  - Test bands: discuss whether to further prioritize the bands in the WID, and whether to introduce more bands; in the WID: n3, n8, n77, n79 are 1st priority

  - MIMO layer:

    - For band n8: 2x2 MIMO (with FR1 UMi CDL-C channel model)

    - For bands n77, n79: 4x4 MIMO (with FR1 UMa CDL-C channel model)

    - For band n3: TBD

  - Use scenario: Free space

  - Operation mode: SA (first stage)

  - Power class: PC3 (first stage)

- Commercial device (Smartphone) selection criteria for FR1 MIMO OTA Measurement Campaign:

  - DUT capability: supporting all the bands n3, n8, n77, n79 listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - DUT variety: the selection of commercial devices should cover various of devices in the market. The following selection criteria can also be considered:

    - Year of production: 2021-2025

    - Brand variety

    - Price range (to cover different price ranges, including High/Mid/Low-end products)

    - Popularity

    - Number of bands supported

  - For band n3, UEs equipped with [TBD, 4 or 2] Rx antenna ports shall be selected for the Measurement Campaign. Labs can try to identify 4Rx or 2Rx UEs in any methods listed below:

    - Method 1: If a UE can be connected to BS simulator with 4x4 MIMO, the UE can be confirmed as a 4Rx UE

    - Method 2: Obtain the MIMO layer information from BS simulator, e.g., check the IE maxNumberMIMO-LayersPDSCH

    - Method 3: Directly collect the information from OEMs.

    - Other methods are not precluded

- Commercial devices provision:

  - Test labs can prepare and collect commercial devices by themselves based on the above selection criteria.

  - Any 3GPP member can work with the selected test labs to provide devices

    - A test lab shall measure only one UE model in case different samples are provided

    - Same UE model supporting different sets of bands can be measured. For this case, the UE model should be marked as different model, e.g., model A-1, model A-2. (guidance on how to manage this case are provided in the spreadsheet in R4-2321191)

    - For band n1, make sure the devices are [TBD, 4Rx or 2Rx] UEs

    - The 3GPP member providing the DUTs should contact one of the selected labs to check their availability to receive the DUTs and define together the related provisioning aspects. Any issue should be reported to the rapporteur in a timely manner to discuss for an alternative solution

- Measurement results submission:

  - RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure

  - UE information disclosure: labs use the spreadsheet in R4-2321191 to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

  - Labs use the worksheet template in [TBD, use Rel-18 template R4-2403019 as starting point] to submit the measurement results for Rel-19 3GPP FR1 MIMO OTA performance data pool. Do NOT apply any systematic offset to the measurement results.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed):

    - The minimum number of submitted devices from each lab is 3, the maximum number is 15. Meanwhile, labs are encouraged to provide as much data as possible within 15

    - Volunteer labs provide the device information sheet ONLY to the RAN4 Secretary and the sheet used to submit measurement results to 3GPP RAN4

  - RAN4 Secretary ONLY publishes to 3GPP RAN4 the following summary of statistical information after anonymizing the sensitive UE information data, i.e., UE model name and vendor name:

    - Total number of devices

    - Total number of models

    - Total number of devices vendors

    - Percentage of devices per vendor

    - Percentage of devices per Power Class

    - Percentage of devices per each supported band

    - Percentage of devices per year of production

    - Percentage of the devices that are certified by at least one of certification bodies as following: PTCRB, GCF, NAL/CTA (Chinese Network Access Licensed Test), FCC, CE

      - Once the device gets the above certification, for RAN4 discussion that means the device is not a prototype

- Percentage of the devices that are certified for each certification body (for information only)

3.2.3 Specifying Performance Requirements

- Only measurement data from the aligned labs with channel model validation completed will be considered for specifying requirements, as described in Section 3.1.

- Thresholds for specifying FR1 MIMO OTA performance requirement:

  - Minimum amount of data per band: 15

  - Minimum amount of UE models per band: 12

- Method of requirements derivation: per-band Data driven approach

- The value at [85%] percentile of the CDF curve can be selected as the starting point for requirement discussion

- Performance part of the work will proceed in a contribution-driven manner.

3.3 Time plan

1. Finalize the working procedures and time plan for Rel-19 MIMO OTA requirements development no later than RAN4 #113 (Nov 2024).

2. Measurement Campaign for bands n77/n8 can start at an early stage with the Rel-17/Rel-18 aligned labs that completed channel model validations for these bands.

3. Labs should submit channel model validation results for bands n3/n8/n79 by formal T-docs no later than RAN4 #114-bis (Apr 2025) to participate in the Measurement Campaign at the corresponding bands. Earlier submission is encouraged.

4. To properly plan the Measurement Campaign, the following actions are requested no later than RAN4#115 (May 2025):

  - The rapporteur checks with the volunteer labs the amount of DUTs (minimum 3, maximum 15) they expect to be able to measure AND how many DUTs they can accommodate from 3GPP members

  - The 3GPP members providing devices for the measurement campaign (if any) check how many samples they intend to provide (in terms of maximum number)

  - Planning of the measurement campaign could be reviewed based on the above points

5. The 3GPP members (if any) should provide the devices to volunteer labs no later than RAN4#116 (Aug 2025).

6. Labs should submit measurement results of commercial devices by formal T-docs no later than RAN4#117 (Nov 2025), also submit the devices information to RAN4 Secretary.

7. Conclude FR1 MIMO OTA performance requirements no later than RAN4#118 (Feb 2026).

4	Conclusion

In this contribution, we share our proposals and an initial framework for Rel-19 FR1 MIMO OTA performance requirements development.

Proposal 1: RAN4 should make the decision on defining which MIMO OTA requirement for band n3, no later than RAN4#113 (Nov. 2024).

- Option 1: 4x4 MIMO OTA requirement

- Option 2: 2x2 MIMO OTA requirement

- Option 3: both

Observation 1: All the 10 DUTs show worse MIMO OTA performance at n77 when compared to n78, the TRMSaverage,70 gap ranges from 1.93 to 4.61 dBm/30kHz. It is not feasible to reuse the n78 MIMO OTA requirement to n77.

Proposal 2: Approve the framework in Section 3 of this contribution for Rel-19 MIMO OTA performance requirements development. The framework can be updated based on discussion outcomes at this meeting and the next meeting.

References

- RP-241623, Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, 3GPP RAN#104, Jun. 2024.

- R4-2412056, Updated Workplan of Rel-19 FR1 OTA WI, vivo, CAICT, 3GPP RAN#112, Aug. 2024.

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo, 3GPP RAN#112, Aug. 2024.

- R4-2214360, WF on NR MIMO OTA (Rel-17), CAICT, vivo, 3GPP RAN4#104-e, Aug. 2022.

- R4-2406083, Way Forward for [110bis][337] NR_MIMO_OTA_enh, CAICT, 3GPP RAN4#110-bis, Apr. 2024.

- R4-2405468, Updated Framework and time plan for FR1 MIMO OTA performance requirements development (Apr 2024), CAICT, 3GPP RAN4#110-bis, Apr. 2024.