3GPP TSG-RAN WG4 Meeting #112bis	R4- 2416438

Hefei, China, October 14 – 18, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SSB adaptation

Agenda item:	6.22.2.2

Document for:	Discussion

Introduction

The Rel-19 WID on Enhancements of network energy savings for NR, revised in [1], specifies for the objective “adaptation of common signal/channel transmissions”, among other enhancements:

- SSB adaptation in time domain, e.g. adapting periodicity”.

At RAN4#112 meeting, the RAN4 scope of the WI was first discussed, and the agreements were captured in the WF [2]. In this contribution, we continue the discussion on the RRM impact from SSB adaptation.

Discussion

  - Progress status in RAN4 and RAN1

At RAN4#112 meeting, the following has been agreed on RRM requirements for SSB adaptation [2].

- Sub-topic 3-1: SSB Adaptation requirements Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

Open issues for SSB adaptation are contained in [3].

RAN1 has discussed this enhancement in their last meetings. At RAN1#118 meeting, the following agreements were reached:

Agreements (RAN1#118) [Adaptation of SSB in time domain] Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

At RAN1#117 meeting, the following agreement was reached:

Agreement (RAN1#117) For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g., two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

  - Scenarios for SSB adaptation

RAN4 #112 agreed to discuss SSB periodicity adaptation in time domain as a starting point as listed in the WID [1]. It is noted, RAN1#118 did not achieve an agreement to also consider other types of SSB adaptation e.g. related to beam transmissions. Thus, it is proposed start working on the impact from SSB periodicity adaptation in time domain to RRM requirements. In case RAN1 achieves agreement on other types of SSB adaptation, RAN4 can work on related RRM requirements.

Proposal 1: 	RAN4 to start working on the impact from SSB periodicity adaptation in time domain to RRM requirements.

SSB adaptation in time domain is being studied in RAN1 for Rel-19 NES capable UEs in a PCell and/or an SCell in connected mode.

- In a PCell scenario, adaptation of SSB burst periodicity can improve energy efficiency. For instance, SSB transmissions other than CD-SSB can be reduced in PCell for higher NES gain (more sleep opportunities can be achieved with fewer SSB transmissions), e.g., longer SSB periodicity in the PCell when procedures related to SCell addition or activation are not needed. Or SSB transmissions can be increased by applying shorter SSB periodicity e.g. for RRM measurements.

- In an SCell scenario, SSB transmissions can be more frequent to facilitate UE measurements, e.g., for the activation of the SCell. In this case, a trade-off between benefiting the UE with faster measurements vs. benefiting the network for NES gain may be considered. Or less frequent in case there is no need for RRM measurements.

According to RAN1 #118 agreement, SSB adaptation in the PCell for CD-SSB is not to be considered. RAN1 studies the options of SSB adaptation for SSB not being a CD-SSB and SSB other than CD-SSB not on the sync raster. This means that RAN4 would have to study measurement impact for the scenario of SSB adaptation in the PCell for NCD-SSB, either located on sync-raster or not located on sync-raster.

Proposal 2: 	RAN4 to study the RRM impact for the scenario of SSB adaptation in the PCell for NCD-SSB, regardless of whether on sync-raster or not.

Regarding SSB adaptation in SCell, RAN1 agreed to consider SSB adaptation for NCD-SSB, regardless of whether located on sync raster or not. For CD-SSB in SCell, RAN1 is still studying whether to apply SSB adaptation.

Proposal 3: 	RAN4 to study the RRM impact for the scenario of SSB adaptation in the SCell for NCD-SSB, regardless of whether on sync-raster or not.

Related to the above RAN1#117 agreement on SSB adaptation in time domain, in our understanding the agreed Option 1 is related to adaptation of SSB burst periodicity for a configured single SSB pattern by defining a set of SSB burst periodicities for the SSB transmission. Option 2 is based on definition of two SSB configurations, where either one or both can be active. It is currently not precluded realizing Option 1 using option 2, i.e. single active SSB pattern. However, there is an open issue for Option 2, that the two SSB configurations may differ in the SSB burst periodicity and suitable other parameters such as SSB type or others, the latter requiring further study in RAN1. For Option 2 it is also unclear, whether it is related to single active SSB pattern or two concurrent active SSB patterns. As RAN1 is discussing the change of SSB burst periodicity before paging to improve the UE synchronization when being paged (although a Rel-18 solution based on TRS exists), it is assumed that both Options 1 and 2 are related to alternating between SSB burst periodicities or between two SSB configurations with different SSB burst periodicities which allows to activate up to two SSB patterns at a time (e.g. resulting in one pattern for legacy UEs and two patterns for Rel-19 NES UEs). Although, RAN1 clarification of these scenarios for Option 2 is needed here.

Another scenario for SSB adaptation is considered by RAN1 related to on-demand SSB, namely Option 4 of the RAN4#116 agreement:

Agreement (RAN1#116) … Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. …

Regarding Option 4 in the RAN1#116 agreement, based on our understanding, it is intended for Case-2 with always-ON periodic SSB transmission in SCell, where the NW triggers on-demand denser SSB transmission with shorter SSB periodicity from time instance A, i.e. when NW sends SCell activation command, and later NW starts sending SSB with longer periodicity after time instance B, i.e. when NW receives UE measurement report. The Option 4 can thus be jointly considered with the RAN1#117 agreement about adaptation of SSB burst periodicity in time-domain.

Observation #1: Options 1 and 2 of the RAN1#117 agreement are discussing the alternating between different SSB periodicities and the operation of up to two SSB configurations, respectively.

Observation #2: For Option 1, only one SSB pattern is active at a time, and this may have different SSB burst periodicities.

Observation #3: Option 4 of the RAN1#116 agreement is intended for Case-2 with always-ON periodic SSB transmission in SCell, where the NW triggers on-demand denser SSB transmission with shorter SSB periodicity from time instance A, i.e. when NW sends SCell activation command, and later NW starts sending SSB with longer periodicity after time instance B, i.e. when NW has received UE measurement report.

In summary, until RAN1 concludes the discussion on Option 2 in view of applicable scenarios, e.g. if this is related to CD-SSB and NCD-SSB configurations, and whether two SSB configurations can be active at a time, RAN4 should focus on Option 1. Moreover, RAN1 in our view needs also to discuss applicable scenarios for Option 1 triggering SSB adaptation in greater detail. Based on the above RAN1 agreements, RAN4 can start to investigate RRM impacts for SSB adaptation in the time domain for a single active SSB pattern, due to usage of different SSB burst periodicities.

Proposal 4: 	RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single active SSB pattern, due to usage of different SSB burst periodicities. Further scenarios can be considered based on further RAN1 agreements.

Regarding the range of SSB periodicities, we observe that 160ms is the longest periodicity in current spec. It is noted in the RAN1 study item [4] few companies have investigated NW energy saving potential for SSB periodicities beyond 160ms, such as 640ms and 1280ms, but the majority of companies provided results for NW energy savings for SSB periodicities up to and including 160 ms. Thus, RAN4 should maintain the existing specified range for SSB burst periodicity.

Proposal 5: 	RAN4 to maintain the specified range for SSB burst periodicity i.e. 5ms to 160ms.

The UE needs to process the signaling message, depending on the signaling method, which will be defined by RAN1 and RAN2 (e.g., based on DCI, MAC-CE or RRC). Hence some processing time needs to be defined, until the UE is ready to receive SSBs with the new adjusted burst periodicity.

Proposal 6: 	RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity.

  - Layer 3 measurement impact

Furthermore, due to the change in SSB burst periodicity, SSB measurements consisting of a number of measurement samples will be faster or slower for shorter or longer SSB burst periodicity, respectively. Thus, there is an impact on ongoing measurements due to SSB burst periodicity adaptation, e.g. whether or not to combine measurement samples stemming from different SSB burst periodicities to generate a L3 measurement. Thus, RAN4 needs to consider the L3 measurement impact.

SSB based L3 measurements in RRC CONNECTED state are the following ones:

- NR intra-frequency NR measurements,

- NR inter-frequency NR measurements,

- inter-RAT NR measurements in E-UTRAN.

The impact to these measurements is analyzed in this section.

According to TS 38.133, NCD-SSB based L3 measurements can be used for serving cell, intra-frequency and inter-frequency neighbor cell measurements in RRC_CONNECTED state, if the NW indicates it as reference SSB.

For NR intra-frequency measurements in RRC_CONNECTED state, RRM impacts exist for non-serving cell detection delay and the measurement delay for known non-serving cell, as listed below in further detail, referring to existing RRM requirements in TS 38.133.

Intra-frequency cell detection

Requirements for intra-frequency cell detection, i.e. for intra-frequency PSS/SSS detection in RRC_CONNECTED state without measurement gaps, are based on the configured SMTC window. If the SMTC period (highlighted) is changed according to the SSB periodicity, the cell detection delay increases proportionally up to the defined limit in the max calculation as depicted in Table 9.2.5.1-1 for the case of FR1.

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Similar requirements apply for cell detection delay for UEs indicating support for “no-gap-with-interruption”.

For the case of configured measurement gap, the cell detection delay increases proportionally to the SMTC period up to the defined limit as depicted in Table 9.2.6.2-1 for the case of FR1.

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil( 5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

For the case of deactivated SCell, the cell detection delay is proportional to the measCycleSCell value (highlighted) up to a defined limit as depicted in Table 9.2.5.1-4 for the case of FR1. This value also depends on the SSB periodicity, i.e. it cannot be lesser.

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Similar requirements apply for cell detection delay related to deactivated PSCells.

For cell detection using NCSG, the delay requirements are also defined proportional to SMTC period up to the defined limit in the max calculation as depicted in Table 9.2.7.1-1 for the case of FR1.

Table 9.2.7.1-1: Time period for PSS/SSS detection with NCSG (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5 x KNCSG) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Intra-frequency Cell identification

Additionally, for the UE to perform a cell identification, a round of measurements would be needed when considering cell identification without index reading. If index reading is needed, then an additional delay for time index detection is added.

Intra-frequency Time index detection

Requirements for intra-frequency time index detection in RRC_CONNECTED state without measurement gaps, are based on the configured SMTC window. If the SMTC period (highlighted) is changed according to the SSB periodicity, the time index detection delay increases proportionally up to the defined limit in the max calculation as depicted in Table 9.2.5.1-3 for the case of FR1.

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

For the case of configured measurement gap, the time index detection delay increases proportionally to the SMTC period up to the defined limit as depicted in Table 9.2.6.2-3 for the case of FR1.

Table 9.2.6.2-3: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil(3 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(M2Note 1x 3 x Kgap) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | Ceil(3 x Kgap )x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

For the case of deactivated SCell, the time index detection delay is proportional to the measCycleSCell value (highlighted) up to a defined limit as depicted in Table 9.2.5.1-6 for the case of FR1. This value also depends on the SSB periodicity, i.e. it cannot be lesser.

Table 9.2.5.1-6: Time period for time index detection, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(3 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(3 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

For the case of deactivated SCell, the time index detection delay using NCSG is proportional to the measCycleSCell value up to a defined limit as depicted in Table 9.2.7.1-6 for the case of FR1.

Table 9.2.7.1-6: Time period for time index detection with NCSG (deactivated SCell) (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | 3 x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320ms | 3 x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | 3 x KNCSG x max(measCycleSCell, VIRP,DRX cycle) x CSSFintra |

Intra-frequency measurements

Requirements for intra-frequency measurements in RRC_CONNECTED state without measurement gaps are based on the configured SMTC window. If the SMTC period is changed according to the SSB periodicity, the intra-frequency measurement period increases proportionally up to the defined limit as depicted in Table 9.2.5.2-1 for the case of FR1.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Similar requirements apply for intra-frequency measurement period for UEs indicating support for “no-gap-with-interruption”.

For the case of configured measurement gap, the intra-frequency measurement period increases proportionally to the SMTC period up to the defined limit as depicted in Table 9.2.6.3-1 for the case of FR1.

Table 9.2.6.3-1: Measurement period for intra-frequency measurements with gaps(FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil(5 x Kgap )x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. |  |

For the case of deactivated SCell, the intra-frequency measurement period is proportional to the measCycleSCell value up to a defined limit as depicted in Table 9.2.5.2-3 for the case of FR1.

Table 9.2.5.2-3: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCel |  |

Similar requirements apply for intra-frequency measurement period related to deactivated PSCells.

Requirements for intra-frequency measurements period in RRC_CONNECTED state with NCSG, are also based on the configured SMTC window. If the SMTC period is changed according to the SSB periodicity, the intra-frequency measurement period increases proportionally up to the defined limit as depicted in Table 9.2.7.2-1 for the case of FR1.

Table 9.2.7.2-1: Measurement period for intra-frequency measurements with NCSG (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x KNCSG) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |

Inter-frequency cell detection

For NR inter-frequency cell detection in RRC_CONNECTED state, similar RRM impacts exist for non-serving cell detection delay, as listed above for intra-frequency measurements.

Inter-frequency measurements

For NR inter-frequency measurement in RRC_CONNECTED state, similar RRM impacts exist for the measurement period for non-serving cell, as listed above for intra-frequency measurements.

Inter-RAT NR cell detection

For Inter-RAT NR cell detection when UE camps on E-UTRAN, CD-SSB is assumed to be configured for an NR candidate cell. Hence this scenario is not applicable for SSB adaptation, unless RAN1 achieves agreement on it.

Inter-RAT NR measurements

For Inter-RAT NR measurements when UE camps on E-UTRAN, CD-SSB is assumed to be configured for an NR candidate cell. Hence this scenario is not applicable for SSB adaptation, unless RAN1 achieves agreement on it.

Proposal 7: 	RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra- / inter-frequency cell detection delay and to NR intra- / inter-frequency measurement period, but not to inter-RAT NR cell detection delay or inter-RAT NR cell measurement delay when UE camps on E-UTRAN.

  - Layer 1 measurement impact

Besides L3 measurement impact, RAN4 needs to study the L1 measurement impact due to different SSB burst periodicities to generate an L1 measurement.

SSB based L1 measurements in RRC CONNECTED state are the following ones:

- L1-RSRP,

- L1-SINR.

The impact to these measurements is analyzed in this section.

According to TS 38.133, clause 9.2.1, NR intra-frequency measurements are based on the reference SSB, which may be CD-SSB or NCD-SSB. The same applies for serving cell L1 measurements, i.e. NCD-SSB based L1 measurements can be used to derive serving cell L1-RSRP and L1-SINR measurements in RRC_CONNECTED state, if the NW indicates NCD-SSB as reference SSB.

For NR intra-frequency measurements in RRC_CONNECTED state, RRM impacts exist for non-serving cell detection delay and the measurement delay for detected non-serving cell, as listed below in further detail, referring to existing RRM requirements in TS 38.133.

SSB based radio link monitoring

According to clause 8.1.2.2 for SSB based radio link monitoring requirements, the out-of-sync evaluation period TEvaluate_out_SSB and the in-sync evaluation period TEvaluate_in_SSB both depend on TSSB being the periodicity of the SSB configured for RLM, except for DRX cycles > 320ms, as shown below for FR1 and FR2 for serving cell. For the deactivated PSCell, both evaluation periods depend on the configured measCyclePSCell parameter independent of DRX enabled/disabled and or the DRX cycle length. However, the measCyclePSCell period is configured based on the SSB periodicity, i.e. it cannot be lesser and measCyclePSCell cannot be shorter than 160ms.

Table 8.1.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 ´ P) ´ TSSB) | Max(100, Ceil(5 ´ P) ´ TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 ´ P) ´ Max(TDRX,TSSB)) | Max(100, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 ´ P) ´ TDRX | Ceil(5 ´ P) ´ TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 ´ P ´ N) ´ TSSB) | Max(100, Ceil(5 ´ P ´ N) ´ TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 ´ P ´ N) ´ Max(TDRX,TSSB)) | Max(100, Ceil(7.5 ´ P ´ N) ´ Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 ´ P ´ N) ´ TDRX | Ceil(5 ´ P ´ N) ´ TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1.2.2-4: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1(deactivated PSCell)

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Ceil(10 ´ P) ´ measCyclePSCell | Ceil(5 ´ P) ´ measCyclePSCell |
| DRX cycle≤ 320ms | Ceil(15 ´ P) ´ Max(TDRX, measCyclePSCell) | Ceil(7.5 ´ P) ´ Max(TDRX, measCyclePSCell) |
| DRX cycle> 320ms | Ceil(10 ´ P) ´ Max(TDRX, measCyclePSCell) | Ceil(5 ´ P) ´ Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

Table 8.1.2.2-5: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2(deactivated PSCell)

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Ceil(10 ´ P´ N) ´ measCyclePSCell | Ceil(5 ´ P´ N) ´ measCyclePSCell |
| DRX cycle≤ 320 ms | Ceil(15 ´ P´ N) ´ Max(TDRX, measCyclePSCell) | Ceil(7.5 ´ P´ N) ´ Max(TDRX, measCyclePSCell) |
| DRX cycle> 320 ms | Ceil(10 ´ P´ N) ´ Max(TDRX, measCyclePSCell) | Ceil(5 ´ P´ N) ´ Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

SSB based beam failure detection

According to clause 8.5.2.2 for SSB based beam failure detection requirements, the evaluation period TEvaluate_BFD_SSB depends on periodicity of SSB in the set , configured for BFD, except for DRX cycles > 320ms, as shown below for FR1 and FR2 for serving cell. For the deactivated PSCellthe evaluation periods depend on the configured measCyclePSCell parameter independent of DRX enabled/disabled and o the DRX cycle length. However, the measCyclePSCell period is configured based on the SSB periodicity, i.e. it cannot be lesser.

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 ´ P) ´ TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 ´ P) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.2.2-2: Evaluation period TEvaluate_BFD_SSB for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 ´ P ´ N) ´ TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 ´ P ´ N) ´ Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 ´ P ´ N) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.2.2-4: Evaluation period TEvaluate_BFD_SSB for deactivated PSCell in FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Ceil(5 ´ P) ´ measCyclePscell |
| DRX cycle ≤ 320ms | Ceil(7.5 ´ P) ´ Max(measCyclePscell, TDRX) |
| DRX cycle > 320ms | Ceil(5 ´ P) ´ Max(measCyclePscell, TDRX) |
| Note: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

Table 8.5.2.2-5: Evaluation period TEvaluate_BFD_SSB for deactivated PSCell in FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Ceil(5 ´ P ´ N) ´ measCyclePscell |
| DRX cycle ≤ 320ms | Ceil(7.5 ´ P ´ N) ´ Max(measCyclePscell, TDRX) |
| DRX cycle > 320ms | Ceil(5 ´ P ´ N) ´ Max(measCyclePscell, TDRX) |
| Note: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

For relaxed measurement criteria, clause 8.5.2.4 specifies the evaluation period TEvaluate_BFD_SSB depending on periodicity of SSB in the set , configured for BFD, as shown below.

Table 8.5.2.4-1: Evaluation period TEvaluate_BFD_SSB_Relax for FR1

| Configuration | TEvaluate_BFD_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX, TSSB) ≤ 80 ms | Max(50 ´ K3, Ceil(7.5 ´ K1 ´ P) ´ Max(TDRX,TSSB)) |
| 80ms＜Max(TDRX, TSSB) ≤ 160 ms | Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K1 is the relaxation factor. K1 = 2 for 40ms＜Max(TDRX, TSSB) ≤ 80 ms, K1 = 4 for Max(TDRX, TSSB) ≤ 40 ms Note 3: K3 is the relaxation factor for the lower bound. K3 = K1, if 1 < K1 ≤ 2; K3 = 1 otherwise. |  |

Table 8.5.2.4-2: Evaluation period TEvaluate_BFD_SSB_Relax for FR2

| Configuration | TEvaluate_BFD_SSB_Relax (ms) |
| --- | --- |
| Mas(TDRX, TSSB) ≤ 80 ms | Max(50 ´ K4, Ceil(7.5 ´ K2 ´ P ´ N) ´ Max(TDRX,TSSB)) |
| 80ms＜Max(TDRX, TSSB) ≤ 160 ms | Max(50, Ceil(7.5 ´ P´ N) ´ Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K2 is the relaxation factor. K2 = 2. Note 3: K4 is the relaxation factor for the lower bound. K4 = K2, if 1 < K2 ≤ 2; K4 = 1 otherwise. |  |

Similar requirements exist for TRP specific SSB beam failure detection in clause 8.18.2.

SSB based candidate beam detection

According to clause 8.5.5.2 for SSB based candidate beam detection requirements, the evaluation period TEvaluate_CBD_SSB depends on periodicity of SSB in the set , configured for CBD, except for DRX cycles > 320ms, as shown below for FR1 and FR2 for serving cell.

Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 ´ P ´ PCBD) ´ TSSB) |
| DRX cycle > 320ms | Ceil(3 ´ P ´ PCBD) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.5.2-2: Evaluation period TEvaluate_CBD_SSB for FR2

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 ´ P ´ N ´ PCBD) ´ TSSB) |
| DRX cycle > 320ms | Ceil(3 ´ P ´ N ´ PCBD) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Similar requirements exist for TRP specific SSB based candidate beam detection in clause 8.18.5.

Other L1-RSRP dependent procedures are:

- Measurement restrictions in case of overlap of SSB and CSI-RS reference symbols

- Active downlink TCI state switching delay for unified TCI for single/multi-DCI mTRP

- Active uplink TCI state switching delay for unified TCI for single/multi-DCI mTRP

L1-SINR

For SSB based L1-SINR, no evaluation period requirements exist in TS 38.133, but requirements related to measurement restrictions in case of overlap of SSB and CSI-RS reference symbols. This impact needs to be further analyzed.

Proposal 8: 	RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for RLM, BFD and CBD, to TCI state switching delay and to L1-RSRP measurement restrictions as well as to L1-SINR for measurement restrictions.

  - Transition requirements

In addition, RAN4 needs to investigate whether a transition period is needed when changing SSB burst periodicity, where no or relaxed RRM requirements are specified. Assuming such transition period can be a starting point for defining the RRM impact.

Proposal 9: 	RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact.

- Conclusion

In this contribution, as part of the objective on “adaptation of common signal/channel transmissions” of the Rel-19 WID [1] to further enhance network energy savings for NR, we discussed scenarios for investigating the RRM impact from SSB periodicity adaptation in time domain based on based on RAN1/2 progress and RAN4#112 agreements and presented our views on the potential RRM impact.

Following observations are made.

Observation #1: Options 1 and 2 of the RAN1#117 agreement are discussing the alternating between different SSB periodicities and the operation of up to two SSB configurations, respectively.

Observation #2: For Option 1, only one SSB pattern is active at a time, and this may have different SSB burst periodicities.

Observation #3: Option 4 of the RAN1#116 agreement is intended for Case-2 with always-ON periodic SSB transmission in SCell, where the NW triggers on-demand denser SSB transmission with shorter SSB periodicity from time instance A, i.e. when NW sends SCell activation command, and later NW starts sending SSB with longer periodicity after time instance B, i.e. when NW has received UE measurement report.

And following proposals are made.

Proposal 1: 	RAN4 to start working on the impact from SSB periodicity adaptation in time domain to RRM requirements.

Proposal 2: 	RAN4 to study the RRM impact for the scenario of SSB adaptation in the PCell for NCD-SSB, regardless of whether on sync-raster or not.

Proposal 3: 	RAN4 to study the RRM impact for the scenario of SSB adaptation in the SCell for NCD-SSB, regardless of whether on sync-raster or not.

Proposal 4: 	RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single active SSB pattern, due to usage of different SSB burst periodicities. Further scenarios can be considered based on further RAN1 agreements.

Proposal 5: 	RAN4 to maintain the specified range for SSB burst periodicity i.e. 5ms to 160ms.

Proposal 6: 	RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity.

Proposal 7: 	RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra- / inter-frequency cell detection delay and to NR intra- / inter-frequency measurement period, but not to inter-RAT NR cell detection delay or inter-RAT NR cell measurement delay when UE camps on E-UTRAN.

Proposal 8: 	RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for RLM, BFD and CBD, to TCI state switching delay and to L1-RSRP measurement restrictions as well as to L1-SINR for measurement restrictions.

Proposal 9:	RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact.

- References

[1] 	RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

[2]	R4-2413885, “WF on Netw_Energy_NR_enh”, Ericsson

[3]	R4-2411817, “Topic summary for [112][222] Netw_Energy_NR_enh”, Ericsson

[4]	TR 38.864, “Study on network energy savings for NR“, V18.1.0