3GPP TSG-RAN WG4 Meeting #112bis	R4-2416441

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On Testability and Interoperability Issues for AIML Mobility

Agenda item:	6.18.3

Document for:	Discussion

Introduction

RAN4 discussion on the AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR (FS_NR_AIML_Mob study item) starts from RAN4#112bis. The recent RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

It is expected the following aspects to be discussed during this meeting.

- Work Plan for RAN4 Discussions on AI/ML for Mobility in NR

- AI/ML Mobility Scenarios Evaluation and RRM Requirements

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution we provide our views and discuss different aspects of Testability and Interoperability issues in RAN4 for AI/ML Mobility based on the RAN2 framework.

Discussion

Interoperability and testability within the 3GPP framework are crucial for the success deployment of AI/ML solutions. Specially, when the part of AI/ML functionality are at UE side, the NW needs to properly validate the functionality. Interoperability allows AI/ML based mobility-driven solutions such as measurement prediction, measurement event prediction, and RLF prediction operate within diverse set of networks environments, including devices from different vendors. This enables improved network optimization, radio resource management, and predictive maintenance that results in seamless, consistent, and enhanced user experience.

Based on the current RAN2 agreements, following scenarios/use cases have been potentially considered for AI/ML mobility.

- Measurement Prediction

- Handover and Radio Link Failure Prediction, and

- Measurement Event Prediction

However, for better understanding and designing of testability and interoperability aspects, RAN4 needs to wait for further RAN2 progress as well as RAN4 agreements on work plan and final agreed scenarios and corresponding RRM requirements.

In below, for measurement prediction use case, we provide some our views on issues related to testability and interoperability aspects, as listed below:

- Testability aspects of measurement prediction use case

- Comparison with AI/ML use case

- Discussion on channel models

- Consistency between different Carriers

RRM core requirements related topics are discussed in our accompanying paper [2].

Testability aspects for measurement prediction use case

RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model)

At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In the draft of TR 38.744 [3], the two main identified study goals are:

- First Goal; Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency, reduces the required signalling, and network overhead of transmitting reference signals (RSs).

- Second Gaol; Mobility optimization. Predict future UE radio measurements and use them to optimize HO performance and mobility decisions.

Three domains (or a combination them) have been identified for mobility predictions:

- Spatial: when UE is not measuring every cell and/or beam

- Frequency domain: The UE is not measuring every frequency carrier/bands

- Temporal: when UE may skip and predict some of the measurement time steps

Correspondingly, the following scenarios for RRM measurement prediction use case were captured and prioritized in RAN2#126:

Table 1: Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Based on the RAN4 objectives, the Interoperability and testing aspects of AI/ML Mobility should be based on the RAN2 framework. RAN4 should also study the requirements and testing frameworks to validate Measurement prediction use case considering the existing mobility requirements and test framework as the starting point.

Study the requirements and testing framework for AI/ML based Measurement prediction use case considering the existing AI/ML based BM test framework as the starting point for FR2 scenarios.

Considering high priority scenarios listed in the above table, we can observe that the output of the AI/ML functionality may be different for different scenarios. For instance, for scenario 2, the output of the AI/ML functionality may be L3 cell-level measurements for the same cell as input measurements while in scenario 3, the output of the AI/ML functionality may be L3 cell-level measurements of a different cell than the cell of the input measurements to the AI/ML functionality.

Therefore, accuracy of L3 cell-level measurements of source and target cells may be candidate performance metrics/KPIs in RAN4.

Furthermore, candidate cell id prediction accuracy may also be another candidate performance metric/KPI for above listed scenarios. Since, the agreement on the selected performance metric/KPI will impact the testability and testing setup related discussions, in our view, it is better to first agree on performance metric/KPI for different scenarios listed in Table1 before discussing testing setup for these kpis.

Different sub use cases and scenarios for measurement prediction use case may have different performance metrics/KPIs.

Discussion on performance metric/KPI will impact the study of aspects related to testability and interoperability of AI/ML mobility use cases.

RAN4 first should start discussions on performance metrics/KPIs for different sub-use cases/scenarios of measurement prediction use case.

In addition of performance metric, aspects related to life cycle management of Mobility use cases, including measurement prediction use case, will have certain impacts that need to be covered in testability and interoperability discussions. The framework developed for LCM related aspects in rel. 19 WI (RP-234039, AI/ML for NR Air Interface) should be used as the baseline.

RAN4 should study any additional impact on the life cycle management related issues coming from AI/ML mobility use cases. The framework developed for rel. 19 WI (AI/ML for Air Interface) should be used as the baseline.

Comparison with AI/ML BM use case

Testing setup for AI/ML based Beam management use case is currently an active topic of discussion in RAN4. Therefore, in this study, the proposed approach is to use testing framework in BM use case as baseline and complement if for the additional requirements related to mobility use case.

One of the main challenges in the performance evaluation for the mobility use case is the availability of ground truth i.e. ‘true’ reference values, which allow to compare the predicted measurements to these values and assess accordingly a performance KPI.

In AI/ML mobility use case, different scenarios are identified for both FR1 and FR2 and use cases are relative to L1 and L3 measurements. This is different from Beam Management use case focusing only on FR2 and L1 measurements.

Furthermore, in AI/ML mobility SI, the aspects of inter-cell inference also need to be considered. In AI/ML BM, only single cell inference was considered.

In AI/ML mobility SI, RAN4 also need to consider inter-cell aspects in the testability aspects and testing setup.

In legacy, the requirements on measurement accuracy are defined for both L1 and L3 measurements and defined in TS 38.133 clause 10 for both FR1 and FR2 as follows:

- 10.1.2 Intra-frequency RSRP accuracy requirements for FR1

  - 10.1.2.1 Intra-frequency SS-RSRP accuracy requirements

  - 10.1.2.3 Intra-frequency CSI-RSRP accuracy requirements

- 10.1.3 Intra-frequency RSRP accuracy requirements for FR2

10.1.3.1 Intra-frequency SS-RSRP accuracy requirements

10.1.3.3 Intra-frequency CSI-RSRP accuracy requirements

- 10.1.4 Inter-frequency RSRP accuracy requirements for FR1

  - 10.1.4.1 Intra-frequency SS-RSRP accuracy requirements

  - 10.1.4.3 Intra-frequency CSI-RSRP accuracy requirements

- 10.1.5 Inter-frequency RSRP accuracy requirements for FR2

10.1.5.1 Intra-frequency SS-RSRP accuracy requirements

10.1.5.3 Intra-frequency CSI-RSRP accuracy requirements

- 10.1.19 L1-RSRP accuracy requirements for FR1

  - 10.1.19.1. SSB based L1-RSRP accuracy requirements

  - 10.1.19.1 CSI-RS based L1-RSRP accuracy requirements

- 10.1.20 L1-RSRP accuracy requirements for FR2

  - 10.1.20.1 SSB based L1-RSRP accuracy requirements

  - 10.1.20.2 CSI-RS based L1-RSRP accuracy requirements

The measurement accuracy requirements are followed during the measurement tests, also defined in RAN4, e.g.,

- A.17.6.3.1 SSB based L1-RSRP measurement when DRX is not used

- A.17.6.3.3 CSI-RS based L1-RSRP measurement when DRX is not used

The current approach is based on comparison of the measurement value reported by the UE with the known/reference/ground truth value that is defined by the testing setup parameters.

More insightful details on test tolerance topics for radiated test cases can be found in the TR 38.903 [4], Clause A.2.6.2 for Absolute RSRP.

In FR2, for a UE power class 3 in band n257, the range of valid RSRP values is huge (on the level of several 10s of dB) due to test tolerances and uncertainties. Hence, the requirements that can be defined based on legacy principles, i.e., based on the comparison to the specified value in the test setup, are not very meaningful for testing the performance/accuracy of the predictions.

In FR2, existing principles of testing UE-reported RSRP accuracy based on setup parameters and measurement uncertainties allow for very wide range of valid RSRP values. This devaluates such tests for verification of AI/ML-enabled RSRP/best beam prediction.

Ground truth extraction should be handled differently for FR1 and FR2 as it involves different testing frameworks (conducted, OTA).

For scenarios involving FR2 (Scenarios 4, 5 and 6), reported RSRP measurements are used as an approximate of the ground truth, as agreed in AI/ML Beam Management use case

For scenarios involving FR1 (Scenario 1, 2 and 3), actual ground truth measurements are considered since it can be realized in conducted mode similar to legacy approach.

Discussion on Channel models

- In the scope of AI/ML BM use case of rel. 19 WI, there is an active discussion ongoing on the selection of channel models for BM testing setup. Here is an agreement from RAN4#112 meeting on this issue:

Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

As it can be seen from above agreement that the options of both TDL model and simplified CDL model are open at this stage. In our view, it will be judicious to first let the BM use case conclude on the channel models before starting channel models discussion in Mobility use case.

For OTA testing in FR2, AI/ML mobility SI should follow the channel model selected for AI/ML BM use case in rel. 19 WI (RP-234039, AI/ML for NR Air Interface).

Consistency between different carriers

In AI/ML Mobility, in order to train and infer, AI/ML functionalities need a dataset/environment that guarantees certain consistency between primary and secondary carrier components. This constraint is not present in the AI/ML BM use case and therefore is not considered in the scope of ongoing WI. Therefore, in the scope of mobility SI, RAN4 should consider the methods to guarantee the consistency between different carriers.

RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to testability and interoperability of AI/ML Mobility use cases. Specifically, we cover following aspects for measurement prediction use case:

- Testability aspects of measurement prediction use case

- Comparison with AI/ML use case

- Discussion on channel models

- Consistency between different Carriers

In the paper, the following Observations and Proposals were made:

Proposal 1: Study the requirements and testing framework for AI/ML based Measurement prediction use case considering the existing AI/ML based BM test framework as the starting point for FR2 scenarios.

Observation 1: Different sub use cases and scenarios for measurement prediction use case may have different performance metrics/KPIs.

Observation 2: Discussion on performance metric/KPI will impact the study of aspects related to testability and interoperability of AI/ML mobility use cases.

Proposal 2: RAN4 first should start discussions on performance metrics/KPIs for different sub-use cases/scenarios of measurement prediction use case.

Proposal 3: RAN4 should study any additional impact on the life cycle management related issues coming from AI/ML mobility use cases. The framework developed for rel. 19 WI (AI/ML for Air Interface) should be used as the baseline.

Observation 3: In AI/ML mobility SI, RAN4 also need to consider inter-cell aspects in the testability aspects and testing setup.

Observation 4: In FR2, existing principles of testing UE-reported RSRP accuracy based on setup parameters and measurement uncertainties allow for very wide range of valid RSRP values. This devaluates such tests for verification of AI/ML-enabled RSRP/best beam prediction.

Observation 5: Ground truth extraction should be handled differently for FR1 and FR2 as it involves different testing frameworks (conducted, OTA).

Proposal 4: For scenarios involving FR2 (Scenarios 4, 5 and 6), reported RSRP measurements are used as an approximate of the ground truth, as agreed in AI/ML Beam Management use case

Proposal 5: For scenarios involving FR1 (Scenario 1, 2 and 3), actual ground truth measurements are considered since it can be realized in conducted mode similar to legacy approach.

Proposal 6: For OTA testing in FR2, AI/ML mobility SI should follow the channel model selected for AI/ML BM use case in rel. 19 WI (RP-234039, AI/ML for NR Air Interface).

Proposal 7: RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI.

- References

- RP-242393, Revised SID on AIML for mobility in NR , OPPO, 3GPP TSG RAN Plenary #105, Melbourne, Australia, September 9-12, 2024.

- R4- 2416449, On AI/ML Mobility Scenarios Evaluation and RRM Requirements Impacts, Nokia, RAN4#112-bis, Hefei, China, 14th –18th October 2024.

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Release 19)

- TR 38.903 V18.4.0, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases (Release 18)