3GPP TSG-RAN WG4 Meeting #112bis	R4-2416442

Hefei, China, October 14 – 18, 2024

Source:	Nokia

Title:	NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC

Agenda item:	6.16.3

Document for:	Discussion

Introduction

RAN4#112 was the first meeting to discuss Rel-19 NR demodulation performance Phase 5. The way forward (WF) from the discussion is documented in [1]. In particular, the discussion on BS demodulation performance requirements has led to few agreements as captured in [1] and summarized below:

MMSE-IRC Receiver assumption - Reuse the same receiver model for MMSE-IRC in TR36.884 Baseline receiver assumption (for performance gain study only) - MMSE receiver as defined in TR36.884: Signalling model - Reuse the same signalling model in TR36.884 with agnostic to the waveform Network type - Only synchronous scenario

Apart from the agreements above, the WF [1] also documented several open issues to be addressed in RAN4#112bis meeting. In the following sections, we provide our views on the remaining open issues.

Discussion

The WF [1] has classified the remaining open issues into 3 subtopics, namely, General, Interference modelling, and Test parameters and simulation. Our views on the remaining open issues are provided in each of the respective subsections.

General

The remaining open issue in this subtopic is about the test scenarios:

Test scenarios - Proposals: - Proposal 1: Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104 - Proposal 2: Consider the scenario with different TDD patterns between BS and discuss how to model the corresponding interference profile

The objective of the performance part of BS MMSE-IRC in the Work Item Descriptions (WID) [2] is as highlighted below:

4.2 Objective of Performance part WI - Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

Referring to the objective above, one can note that this WI considers homogeneous and heterogeneous networks scenarios. Furthermore, only synchronous deployments (for both TDD and FDD) are considered, with aligned UL:DL configurations among cells in the case of TDD.

The WID consider homogeneous and heterogeneous networks.

Only synchronous networks are considered, with aligned UL:DL configuration among cells in case of TDD deployments.

Introducing a scenario with different TDD patterns between BSs into the work scope would require a revision of the WID, which needs to be approved in RAN plenary meeting.

RAN4 to work according to the WID and consider homogenous and heterogeneous networks with synchronous deployment. Furthermore, in the case of TDD deployment, only aligned UL:DL configurations among cells is considered.

Interference modelling

The first open issue is about the interference modelling criteria:

Interference modelling criteria - Candidate options on interference modelling criteria: - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value - Option 2: DIP-based modelling - Candidate options on DIP definition: - Option 1: Reuse definition of DIP in sub-clause B.6.1 of TS 36.104 - Candidate options on SINR definition: - Option 1: For BS type 1-C and BS type 1-H: For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as: Where: is the total signal energy of one intra-cell UE in the subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interferer relative to is defined by its associated DIP value. For BS type 1-O: For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as: Where: is the total signal energy of one intra-cell UE in the subframe on a RIB. is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a RIB. The respective energy of each inter-cell interferer relative to is defined by its associated DIP value..

On interference modelling criteria and Dominant Interference Proportion (DIP) definition: In 36.884 [3], the DIP obtained from system-level simulation was used to provide interference profile for the link level simulation used to define BS demodulation performance requirement in LTE. The definition of DIP itself is given in 36.884 [3] and in sub-clause B.6.1 of TS 36.104 [5]. Nonetheless, referring to TS 38.101-4 [7] for UE demodulation performance requirement in NR, Interference-to-Noise Ratio (INR) was used instead of DIP.

During offline discussion in RAN4#112, there was a question on how to change the DIP into INR. In the following we provide the derivation of INR from DIP based on the definition of DIP in [3] and [5]:

Referring to 36.884, there are two DIP values, namely, DIP1 and DIP2, in which they are given in dB. Let in the following, DIP1(dB), DIP2(dB) and No(dB) denote the DIP1, DIP2 and Noise power in dB, respectively. Furthermore, Dip1, Dip2 and N, denote the linear values of DIP1(dB), DIP2(dB) and No(dB), which are given by:

Dip1=10DIP1dB10,  Dip2=10DIP2dB10 and N=10NodB10, respectively.

The Dip1 and Dip2 are ratios of the corresponding dominant interferer power to the sum of interferer powers plus noise power:

Dip1=I1I1+I2+N#1

and

Dip2=I2I1+I2+N.#2

Rewriting (1) and (2) into vector and matrix form, the interference powers I1 and I2 can then be obtained from:

I1I2=Dip1-1Dip1Dip2Dip2-1-1-NDip1-NDip2. #3

Hence, we have INR1=I1N and INR2=I2N and the INR is given by INR=I1+I2N.

RAN4 to use INR instead of DIP in defining the BS MMSE-IRC demodulation requirements.

The INR can be derived from the DIP definition in 36.884 and 36.104 according to the following: I1I2=Dip1-1Dip1Dip2Dip2-1-1-NDip1-NDip2, which leads to INR1=I1N and INR2=I2N. Consequently, INR=I1+I2N.

On SINR definition:

In 38.104 [6], the definition of SNR has been given for BS type 1-C, 1-H and 1-O. For BS type 1-C and 1-H the definition of SNR is as follows:

### 8.1.1 Scope and definitions ... The SNR used in this clause is specified based on a single carrier and defined as: SNR = S / N Where: S is the total signal energy in the slot on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). N is the noise energy in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H).

For BS type 1-O the definition of SNR in 38.104 is as below:

### 11.1.1 Scope and definitions … The SNR used in this clause is specified based on a single carrier and defined as: SNR = S / N Where: S is the total signal energy in a slot on a RIB. N is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration where signal energy exists on a RIB.

The addition of SINR definition into 38.104 would simply be an adaptation of the definition of the SNR in [6] by adapting the noise (N) definition to interference plus noise (N').

Concerning the SINR definition in Option 1, it seemed to be directly adapted from the definition of SINR in 36.104 for LTE instead of adapting the SNR definition of NR in 38.104. We note here that the SINR definition in LTE in 36.104 was an adaptation of the SNR definition in LTE. Hence, it is better to have the SINR definition in 38.104 for NR to be an adaptation of the SNR definition in NR.

The SNR definition in 38.104 can be adapted to SINR definition by changing the definition of noise (N) to interference plus noise (N') appropriately.

The SINR definition in Option 1 seemed to refer to the SINR definition in 36.104 instead of the SNR definition in 38.104.

On the SINR definition, RAN4 to adapt the SNR definition in 38.104 to SINR accordingly.

For BS type 1-C and BS type 1-H, the following SINR definition shall be used:

The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in the slot on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.

For BS type 1-O, the following SINR definition shall be used:

The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in a slot on a RIB. N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a RIB. The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.

The next open issue is about interference profile:

Interference profile - Candidate options on interference modelling: - Option 1: Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH - Other options not precluded - On interference power level: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - FFS how to account for HPUE in existing LTE profiles.

The proper way to work on this WI is to conduct system-level simulation for deriving the proper DIP in NR due to the HPUEs, which at the same time could be used to study the performance gain of MMSE-IRC versus the MMSE. While using either system-level or link-level simulation one could study the performance gain of MMSE-IRC over MMSE, nonetheless, system-level simulation provides more insights in inter-cell interference scenario as it includes the interactions among many cells. Hence, it brings more realistic and holistic observation on what could be expected in real deployment.

System-level simulation could provide a more realistic DIP for NR by taking into account interference due to HPUEs, which is one of the justifications of this WI.

Furthermore, to assess the performance gain of MMSE-IRC over MMSE in inter-cell scenario, a system-level simulation is more appropriate to be used than link-level simulation.

RAN4 to discuss possibilities of using system-level simulation to generate proper DIP for NR due to HPUEs and at the same time to study the performance gain of MMSE-IRC.

In case system-level simulation is not possible, RAN4 to reuse the DIP from LTE (TR 36.884) and consider the study on the performance gain of MMSE-IRC versus MMSE in LTE as reference. Hence, there is no need to study it again in NR by conducting additional link-level simulation.

The last open issue in this subsection is about the number of interferers:

Number of interferers - Candidate options: - Option 1: Consider 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna - Other options not precluded

The justification of this WI is that the introduction of HPUEs in NR would make the interference in NR to be more severe than in LTE. Hence, with such a justification, in link-level simulation there should be one interference signal that represent a HPUE and another interference signal that represent normal UE.

RAN4 to consider only 2 interference signals, in which one signal represent HPUE and one signal represent a normal UE.

As stated in 36.884 [3], there is no significant gain of MMSE-IRC to the baseline MMSE receiver in the case of 2RX when the explicit interferer number is increased from 1 to 2. Hence, 2 explicit interferers are modeled for only 4Rx and 8Rx cases.

In 36.766 [4], 8Rx BS is not covered in the evaluation, and one of the reasons is due to a large number of channel faders are needed for potential 8Rx IC (interference cancellation) test. It is noted here that in [4], the study is on intracell interference cancellation, in addition to the intercell interference.

In TR 36.884, with 2 explicit interferers, only 4Rx and 8Rx are considered.

In TR 36.766, only 2Rx and 4Rx are considered.

Based on previous studies documented in TR 36.884 and TR 36.766, RAN4 shall prioritize 4Rx case with 2 explicit interferers.

Test parameters and simulation assumptions

The remaining open issues in this subtopic are:

Test scope - Candidate options: - Option 1: Cover the following SCS and TDD patterns - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U - 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U - Other options not precluded Channel bandwidth - Candidate options for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: 10MHz/15kHz and 40MHz/30kHz for initial simulation purpose - Option 3: Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz - Option 4: Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz - Candidate options for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz Antenna configuration - Candidate options: - Option 1: Cover 2/4/8 Rx with 1Tx - Option 2: Consider only 4RX and 8RX gNB - Option 3: Cover 2/4/8 Rx with 1/2/4 Tx - Option 4: Prioritize 4Rx case especially in the case of 2 explicit interferers - Option 5: Prioritize 1Tx Propagation condition - Candidate options: - Option 1: - For the serving PUSCH, use TDLA30-10 and TDLC300-100 propagation channel model for HetNet and HomNet respectively - Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000. - Option 2: Select among legacy TDLA30, TDLB100, TDLC300 - Option 2A: - Cover TDLA30-10, TDLB100-400 and TDLC300-100 - Same propagation condition for serving and interference PUSCH - Option 2B: - For the serving PUSCH, use TDLA30-10 for HetNet and TDLB100-100 for HomNet - For the interference PUSCH, use TDLC300-100 - Option 2C: - Cover TDLB100-400, TDLC300-100 and TDLA30-10 low for study purpose, select one for requirement definition - For the interference PUSCH, use TDLC300-100 MCS - Candidate options: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20 Waveform - Candidate options: - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only DMRS configuration - Candidate options: - Option 1: DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS position; pos1. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port(s); 0. DM-RS sequence generation; NID0=0, group hopping and sequence hopping are disabled. - Other options not precluded PUSCH mapping type - Candidate options: - Option 1: Cover both PUSCH mapping Type A and Type B - Other options not precluded Test metric - Candidate options: - Option 1: Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation - Other options not precluded

38.104 [6] has already captured test parameters and FRCs for different scenarios. Such test parameters and FRCs can be reused as well for defining requirements for BS MMSE-IRC. Reusing as many as possible the test parameters and FRCs already available in 38.104 will also bring advantages such as easier to maintain and the compactness of the specification.

The test parameters and FRCs in 38.104 for the corresponding bandwidth sizes can be reused for the intended signals.

As synchronous deployments (with same UL:DL configurations for TDD) among cells are considered, the interference signals could also use the same FRC as the intended signal’s FRC.

Reusing test parameters and FRCs currently available in 38.104 as many as possible would make the specification to be compact and easier to maintain.

The bandwidth sizes considered in 38.104 are 5/10/20MHz for 15KHz subcarrier spacing and 10/20/40/100MHz for 30KHz.

RAN4 to reuse the test parameters and FRCs available in 38.104 for the considered bandwidth sizes.

RAN4 to prioritize 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS to cover both extremes and have all DUTs testable regardless of BW support declaration. If RAN4 reuse FRCs from 38.104, 5/20MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz shall be prioritized.

RAN4 to consider only CP-OFDM.

RAN4 to consider the same MCS for both the intended signal and the interference signals. Furthermore, as the DIP in LTE is generated with 16QAM, the intended signal is also using 16QAM.

Conclusion

In this contribution, we provide our views on Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC. The following Observations and Proposals were made:

Observation 1: The WID consider homogeneous and heterogeneous networks.

Observation 2: Only synchronous networks are considered, with aligned UL:DL configuration among cells in case of TDD deployments.

Observation 3: Introducing a scenario with different TDD patterns between BSs into the work scope would require a revision of the WID, which needs to be approved in RAN plenary meeting.

Proposal 1: RAN4 to work according to the WID and consider homogenous and heterogeneous networks with synchronous deployment. Furthermore, in the case of TDD deployment, only aligned UL:DL configurations among cells is considered.

Proposal 2: RAN4 to use INR instead of DIP in defining the BS MMSE-IRC demodulation requirements.

Proposal 3: The INR can be derived from the DIP definition in 36.884 and 36.104 according to the following: I1I2=Dip1-1Dip1Dip2Dip2-1-1-NDip1-NDip2, which leads to INR1=I1N and INR2=I2N. Consequently, INR=I1+I2N.

Observation 4: The SNR definition in 38.104 can be adapted to SINR definition by changing the definition of noise (N) to interference plus noise (N') appropriately.

Observation 5: The SINR definition in Option 1 seemed to refer to the SINR definition in 36.104 instead of the SNR definition in 38.104.

Proposal 4: On the SINR definition, RAN4 to adapt the SNR definition in 38.104 to SINR accordingly.

Proposal 5: For BS type 1-C and BS type 1-H, the following SINR definition shall be used:

The SINR used in this clause is specified based on a single carrier and defined as:

SINR = S / N’

Where:

S	is the total signal energy of one intra-cell UE in the slot on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H).

N’	is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.

Proposal 6: For BS type 1-O, the following SINR definition shall be used:

The SINR used in this clause is specified based on a single carrier and defined as:

SINR = S / N’

Where:

S	is the total signal energy of one intra-cell UE in a slot on a RIB.

N’	is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a RIB. The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.

Observation 6: System-level simulation could provide a more realistic DIP for NR by taking into account interference due to HPUEs, which is one of the justifications of this WI.

Observation 7: Furthermore, to assess the performance gain of MMSE-IRC over MMSE in inter-cell scenario, a system-level simulation is more appropriate to be used than link-level simulation.

Proposal 7: RAN4 to discuss possibilities of using system-level simulation to generate proper DIP for NR due to HPUEs and at the same time to study the performance gain of MMSE-IRC.

Proposal 8: In case system-level simulation is not possible, RAN4 to reuse the DIP from LTE (TR 36.884) and consider the study on the performance gain of MMSE-IRC versus MMSE in LTE as reference. Hence, there is no need to study it again in NR by conducting additional link-level simulation.

Proposal 9: RAN4 to consider only 2 interference signals, in which one signal represent HPUE and one signal represent a normal UE.

Observation 8: In TR 36.884, with 2 explicit interferers, only 4Rx and 8Rx are considered.

Observation 9: In TR 36.766, only 2Rx and 4Rx are considered.

Proposal 10: Based on previous studies documented in TR 36.884 and TR 36.766, RAN4 shall prioritize 4Rx case with 2 explicit interferers.

Observation 10: The test parameters and FRCs in 38.104 for the corresponding bandwidth sizes can be reused for the intended signals.

Observation 11: As synchronous deployments (with same UL:DL configurations for TDD) among cells are considered, the interference signals could also use the same FRC as the intended signal’s FRC.

Observation 12: Reusing test parameters and FRCs currently available in 38.104 as many as possible would make the specification to be compact and easier to maintain.

Observation 13: The bandwidth sizes considered in 38.104 are 5/10/20MHz for 15KHz subcarrier spacing and 10/20/40/100MHz for 30KHz.

Proposal 11: RAN4 to reuse the test parameters and FRCs available in 38.104 for the considered bandwidth sizes.

Proposal 12: RAN4 to prioritize 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS to cover both extremes and have all DUTs testable regardless of BW support declaration. If RAN4 reuse FRCs from 38.104, 5/20MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz shall be prioritized.

Proposal 13: RAN4 to consider only CP-OFDM.

Proposal 14: RAN4 to consider the same MCS for both the intended signal and the interference signals. Furthermore, as the DIP in LTE is generated with 16QAM, the intended signal is also using 16QAM.

- References

- R4-2413570, WF on NR demodulation performance: Phase 5, 3GPP RAN4#112, China Telcom, Maastricht, Netherlands, August 19-23, 2024

- RP-241297, Revised WID on NR demodulation performance: Phase 5, 3GPP RAN#104, China Telecom, NTT DOCOMO, Shanghai, China, June 17-20, 2024

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, v13.1.0, 09-2016

- 3GPP TR 36.766, Study on interference cancellation receiver for LTE BS, v15.0.0, 07-2017

- 3GPP TS 36.104, Evolved Universal Terrestrial Radio Access (E-UTRA) - Base Station (BS) radio transmission and reception, v18.5.0, 03-2024

- 3GPP TS 38.104, NR - Base Station (BS) radio transmission and reception, v18.6.0, 06-2024

- 3GPP TS 38.101-4, NR - User Equipment (UE) radio transmission and reception, v18.5.0, 09-2024