3GPP TSG-RAN WG4 Meeting #112bis	R4-2416445

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Nokia

Title:	RRM Performance Requirements for RedCap Positioning

Agenda item:	4.2.2.4

Document for:	Discussion

Introduction

The Rel-18 WID [1] for NR positioning includes the enhancements for RedCap positioning. The WID depicts the RAN4 work related to this accuracy improvement technology. The RAN4#112 meeting [2] has made the following agreements on RRM performance requirements for RedCap positioning.

### Issue 5-3: Group delay calibration margin for RedCap positioning Agreement: - RAN4 to use the following assumptions for group delay calibration margin for RedCap - Non-FH: existing values for non-RedCap for the applicable BW. - FH: existing values for non-RedCap for per-hop BW [FFS: plus an extra margin (value TBD)]. ### Issue 5-4: PRS RMC for RedCap positioning TCs Agreement: - [For PRS BW for FH TCs, consider the BW in test configuration as UE BW and allow cell BW to be larger.] – double check with TE vendors - For FH TCs, introduce a new PRS RMC with large BW and repetitions. - A CR revision is needed to include the new PRS RMC for 15 kHz (FR1) and 120 kHz (FR2) – Qualcomm to revise a submitted CR Discussion in AH2: - TE(R&S) input on the earlier tentative agreement from AH1 [For PRS BW for FH TCs, consider the BW in test configuration as UE BW and allow cell BW to be larger.]: - Different BWs for cell and UE have never been used - There will be confusion between OCNG and UE BW, even if we go that way - Challenge for TE: interpret OCNG (and other RMCs) as the UE BW - Two steps could be considered: - Step 1: in the configuration, separately cell BW and UE channel BW are mentioned - Step 2: resolve the OCNG issue - FFS: UE total BW should match a cell BWP (added off-line) - Recommendation: consider the above input in the work on TCs

This contribution further investigates impacts to RRM performance requirements for NR positioning enhancements.

Discussion

The impacts to RRM performance requirements for some of the above enhancements are discussed in this section.

Group delay calibration margin for RedCap positioning

For the group delay calibration margin, the open issue is how to define the value of the extra margin in a FH case. To define the extra margin, the case of RSTD can be used as a baseline. Therefore, the margin can be initially set as Table 10.1.23.2-5 ‘Margin for RSTD measurement accuracy in FR1’ and Table 10.1.23.2-6 ‘Margin for RSTD measurement accuracy in FR2.’ Then, the exact value can be found by reducing the initial extra margin.

RAN4 can initially define the margin as the same as the margin in the RSTD case.

PRS RMC for RedCap positioning TCs

PRS RMC for RedCap positioning TCs are discussed in RAN4#112. For PRS BW for FH TCs, it is discussed to consider the BW in test configuration as the same as UE BW. The issue is the relationship between the UE’s total BW and the cell BWP. When defining the configuration in TCs, a typical case can be configured with the same UE’s total BW and the cell BWP for a positioning purpose. Hence, TCs should assume that UE total BW should be matched to a cell BWP.

UE total BW should be matched to a cell BWP in PRS RMC in RedCap positioning TCs.

Conclusion

The following proposals are made in this contribution.

- RAN4 can initially define the margin as the same as the margin in the RSTD case.

- UE total BW should be matched to a cell BWP in PRS RMC in RedCap positioning TCs.

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning,” Intel Corporation, RAN#99.

- R4-2414032, “WF on RedCap positioning and PRS/SRS bandwidth aggregation,” Ericsson, RAN4#112.