3GPP TSG-RAN WG4 Meeting #112bis	R4-2416447

# Hefei, China, October 14th – 18th, 2024

Source:	Nokia

Title:	RRM core requirements for AI/ML positioning accuracy enhancement

Agenda item:	6.17.5.1

Document for:	Discussion

Introduction

In RAN#102, the Release-18 study on artificial intelligence (AI)/machine learning (ML) for NR air interface (FS_NR_AIML_Air) [1] was completed [2] and the findings (with agreements and open issues) were documented in the outcome technical report 3GPP TR 38.843 V18.0.0 [3]. Furthermore, a new Release-19 work item on artificial intelligence (AI)/machine learning (ML) for NR air interface (NR_AIML_Air) [4] was also approved in RAN#102 to start the normative work for the general AI/ML framework for air interface and to enable the recommended use cases in the preceding study.

During the SI, as mentioned in the extract from 3GPP TR 38.843 V18.0.0 [3] below, following sub use cases for positioning accuracy enhancement are considered:

The following are selected as representative sub-use cases: - Direct AI/ML positioning: - AI/ML model output: UE location - e.g., fingerprinting based on channel observation as the input of AI/ML model - AI/ML assisted positioning: - AI/ML model output: new measurement and/or enhancement of existing measurement - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement More specifically, the following Cases are considered for the study: - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning One-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

During SI, RAN4 studied different test metrics for performance requirements for AI/ML enabled positioning use case.  Here is an extract from 3GPP TR 38.843 V18.0.0 [3] covering these aspects:

Both direct AI/ML positioning and AI/ML assisted positioning are considered. For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above The feasibility and testability of different options should be further justified in WI.

In RAN4#110, it has been agreed on the testability and interoperability issue for AIML positioning [5].

2.3 Testability and interoperability issues for positioning accuracy enhancement 2.3.1 Agreements in ad-hoc session (R4-240xxxx) Issue 3-2: Requirements for case 3a/3b RAN4 will not define positioning accuracy requirements for case 3a/3b Issue 3-6: Requirements for case 2a/2b RAN4 to come back to case 2a/2b based on progress in the other working groups

In RAN4#110bis, it has been agreed on the testability and interoperability issue for AIML positioning [6].

Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme(if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced

In RAN4#111, it has been agreed on the testability and interoperability issue for AIML positioning [7].

Issue 3-2: Requirements for case 2a Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups Issue 3-3: Requirements for case 2b Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups

In RAN4#112, it has been agreed on the testability and interoperability issue for AIML positioning [8].

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. Issue 3-6: Reported metrics for case 3a/3b Agreement: postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

The issues on AI/ML based positioning require further discussion on selected sub use cases. In this paper, the performance requirements for case 1 and case 2b are discussed in the aspects of performance test metric, measurement accuracy requirements, testability, and LCM-related latency requirements. This paper also discusses the requirements for LMF-side model in Case 2b.

Discussion

Requirements for UE-side model: Case1, Case 2a

This section discusses the requirements in the use cases with a UE-side model.

Measurement accuracy requirements

Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

In RAN1#117, it is confirmed that the path-based measurement in the context of AIML-based positioning is the existing specifications’ measurements including their measurement reporting. When the measurements in AIML positioning are the same as the measurements defined in the legacy positioning, the various target positioning requirement types for NR positioning in Rel-18 summarized in TR38.859 can be considered as the candidate measurements in AIML positioning. In particular, the functionality input may be one of the existing measurements such as PRS-RSRP, PRS-RSRPP, etc. Also, if any new measurements such as CIR, DP, and/or PDP are agreed in RAN1 as functionality input, there are impacts in RAN4 measurement accuracy requirements for these new measurements.

In our view, the input to the AI/ML functionality should have and satisfy any measurement accuracy requirements to ensure the AI/ML functionality performs as expected. Hence, RAN4 should define the requirements on the measurement types supported in the legacy positioning (e.g., RSTD, RSRP, and RSRPP). Moreover, RAN4 should study how to define the requirements on new measurement is supported (e.g., CIR, DP, and PDP).

For AIML positioning, RAN4 to define the requirements on the measurements supported in legacy positioning (e.g., RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress.

Requirements for consistency between training and inference

In NR positioning, DL-PRS configuration is provided in a hierarchy. There can be at most 4 positioning frequency layers (PFL) and each positioning frequency layer has at most 64 TRPs. Each TRP per positioning frequency layer can have 2 DL-PRS Resource sets, thus resulting in a total of 8 DL-PRS resource sets per TRP and each DL-PRS resource set can have up to 64 DL-PRS resources. Each DL-PRS resource corresponds to a beam. Having 2 different DL-PRS resource sets per positioning frequency layer per TRP allows gNB to configure one DL-PRS resource set of wide beams and another DL-PRS resource set of narrow beams for each positioning frequency layer.

Figure 1 DL-PRS configuration hierarchy considering PFL, DL-PRS resource sets and DL-PRS resources.

Existing measurement requirements for NR positioning in TS 38.133 for RSTD, RTOA, UE/gNB Rx-Tx time difference and for PRS-RSRPP / SRS-RSRPP are based on paths. However, accuracy requirements are only defined for first path RSTD, RTOA, UE Rx-Tx, gNB Rx-Tx and PRS-RSRPP / SRS-RSRPP, although additional paths can be reported but for them, no accuracy requirements exist.

Single PFL and dual PFL requirements are specified in TS 38.133 for RSTD, PRS-RSRP and UE Rx-Tx time difference measurements, but accuracy requirements are distinguished only for RSTD. For dual PFL, accuracy for RSTD is decreased from Y=32 Tc to Y=256 Tc, see clause 10.1.23.2 and furthermore the RF margin is increased as defined in Table 10.1.23.2-5a for FR1 and Table 10.1.23.2-6a for FR2. This accuracy degradation and additional uncertainty for dual PFL will result in more difficult (and longer) training and inherence procedures for AI/ML positioning. Thus, single PFL should be the starting point for AI/ML positioning.

RAN4 requirements (e.g., in Cases 1, 2a) should be firstly defined with respect to single PFL.

When collecting data for training UE-side model cases, the measurements (such as channel samples in the form of CIR, PDP, DP) serve as the training input. Consequently, the data collection process that creates the dataset for training models is dependent on the positioning configuration used to obtain these measurements. When a trained model is used in the inference stage, if the current positioning configuration (e.g., PFL) is different from the positioning configuration used in the training stage, it is not effective to use such a trained model for inference.

Therefore, in both training and inference stages, it is essential to remain in the same positioning configuration (i.e., to maintain consistent configuration). To maintain the consistent positioning configuration in training and inference stages, the consistency needs to be checked to verify the same positioning configuration. In particular, the UE and network (e.g., LMF) must ensure the positioning configuration during inference phase matches with the training phase’s positioning configuration. Specifically, such a consistent positioning configuration can be achieved by validating the PF) across training and inference. Hence, the requirements on the measurements can be defined to ensure that same PFL is configured in training and inference.

The consistency between training and inference should be achieved by having a requirement on consistent configuration (e.g., same PFL) when UE-sided models are used.

Requirements on ground truth in data collection

In RAN1#116-bis meeting, training data collection of AI/ML based positioning has been discussed to identify the collect ed data samples as follows.

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

The data collection with ground-truth dataset is essential for training. In particular, the training data needs to include both the ground-truth UE location and measurements associated with the UE’s location. To enhance the accuracy of AIML positioning, it is important to have an accurate ground-truth information for positioning labelling. When collecting the ground-truth information from the UE side, a high accuracy positioning information needs to be acquired by the UE. If UE is unable to acquire a high accuracy ground-truth information, the network/testing equipment may assist UE to acquire a high accuracy ground-truth positioning label. For instance, UE may obtain positioning label from multiple sources e.g., non-NR and/or NR RAT-dependent positioning methods. Therefore, the degree of accuracy on ground-truth positioning label can be defined as a quality indicator. The quality indicator on the ground-truth accuracy can be subject to a new requirement.

RAN4 to study the requirements on a quality indicator for the ground-truth accuracy on positioning label in data collection.

Positioning accuracy requirements in Case 1

In Case 1 for direct AI/ML positioning, the output of the AI/ML functionality is UE position. As a metric for positioning requirements and tests, we propose the positioning accuracy metric as follows over the selected test positions:

- At each test position, the following steps will be followed:

  - Get the output from AI/ML functionality inference with the input of radio measurements,

  - Compute a positioning error, e.g., absolute error value between estimated position and true position, and

  - Confirm that the positioning error is within the acceptable range (i.e., positioning accuracy requirements).

For instance, positioning accuracy requirements are already specified for NR A-GNSS where the minimum performance requirements are defined in TS 37.571-1 Clause 13.3. Therefore, a similar approach should be envisaged in the context of AI/ML-enabled direct positioning.

Positioning accuracy should be considered as the performance metric in Case 1 (UE-based positioning with UE-side model, direct AI/ML positioning).

One of the main challenges in introducing Case 1 requirements on positioning accuracy is the availability of accurate enough UE position that could be used as a ground truth for AI/ML based position. As discussed above, when such ground truth is known to the UE or test equipment (TE) then the AI/ML-based positioning can be compared to it, i.e., positioning error can be evaluated. However, the AI/ML based Case 1 positioning functionality should not be left without any requirements.

Even when ground truth, i.e., accurately known position of the UE, for Case 1 AI/ML-based positioning is not available in the testing setup for absolute positioning accuracy calculation, the new functionality should not be left without any requirements or tests.

One approach to test UE-based positioning (Case 1) could be to compare the position inferred by the UE not to the ground truth but to the position provided with the same functionality in a different moment of time or conditions. For example, it should be insured that UE is not reporting very different locations after insignificant change in the radio conditions. In this case, the relative positioning accuracy metric can be considered instead of absolute accuracy relevant to the ground truth.

RAN4 to consider relative positioning accuracy metric for Case 1, i.e., requirement on the possible change in AI/ML-based position due to a change/state of the environment/configuration.

LCM related aspects: latency requirements for LCM actions in Cases 1 and 2a

WID RP-234039 [4] also discusses RAN4-related aspects to be considered during WI phase for Beam Management and Positioning use cases. The WID covers core requirements aspects as follows.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

The WID also covers the aspects related to performance part.

- For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

Also, the agreement related LCM procedures in RAN4#111 are as follows:

Agreement: RAN 4 to consider defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning) and candidate are: - Identification - Selection - Activation - Deactivation - Switching - Fallback to non-AI operation - Performance monitoring - Others are not precluded

UE can be capable of performing different positioning techniques, e.g., capabilities of the different AIML models/functionalities and legacy positioning techniques while initially having different requirements for each model/functionality and technique/algorithm. If a performance monitoring process detects a performance degradation to a certain point, it is possible to switch the currently running model / functionality with another model / functionality. In the same case, UE can be also enabled to perform a fallback operation to a legacy/default positioning technique and algorithm. For instance, a fallback operation is needed when the measurements can be more suitable for a legacy/default positioning technique than the current model/functionality. It is also possible to utilize AIML feature to determine a measurement requirement feature. If UE’s measurement on the upcoming RS indicates a potential satisfactory of positioning measurements, the UE may be required to process positioning measurements. By avoiding unnecessary processing of positioning measurements with a detected performance degradation, UE can reduce the power consumption. Therefore, when the AI/ML functionality is degrading the system performance lower than any required level associated with the current model/functionality, the continuous operation of such a functionality with the detected performance degradation may have a catastrophic impact on the overall system performance.

Therefore, it is crucial to detect the performance degradation and stop this model/functionality, either by falling back to legacy method or by switching to another model/functionality, within a specified time. The specified time allowed to switch/disable the model/functionality should guarantee that the system performance remains within the acceptable levels.

If an LCM action is required and it is not taken in a timely manner, the performance degradation for AI/ML enabled Positioning use case may be degraded to undesirable level.

RAN4 to define the time latency limit on UE’s LCM actions indicated by network in Cases 1 and 2a.

In RAN4#111 meeting, various candidates for were considered for defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning). Since RRM requirements may not be necessary for all listed candidates, RAN4 should prioritize the following candidates with a high impact on RRM requirements:

- Activation

- Deactivation

- Switching

- Fallback to non-AI operation, and

- Performance monitoring.

Activation, Deactivation, Switching, Fallback to non-AI operation, and Performance monitoring should be prioritized for defining RRM requirements for LCM AI/ML positioning use case.

Performance test metric for Case 2a

Agreement For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

RAN1#116 meeting agreed that LOS/NLOS indicator and/or timing information are supported for reporting in AI/ML assisted positioning Case 2a. LOS/ NLOS indicator can be considered as an intermediate performance metric for case 2a - UE-assisted/LMF-based positioning with UE-side model, AI/ML-assisted positioning.

LOS/ NLOS reporting from UE to LMF is already supported for NR positioning in Rel-17.  However, when the AI/ML model/ functionality inference is the LOS/NLOS indicator, it should be treated as an intermediate performance metric and specify the minimum performance requirements. Otherwise, LOS/ NLOS indicator used as one of the inputs to the positioning algorithm at LMF will have impact on the positioning accuracy.

LOS/ NLOS indicator should be considered as an intermediate performance metric for case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML-assisted positioning).

Requirements for LMF-side model: Case 2b and Case 3b

Reporting requirements

Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

The agreements in RAN1#118 are made to use DL PRS-RSRPP and UL SRS-RSRPP as an input to the AIML model. Therefore, it is needed for RAN4 to start a discussion on how to define the reporting requirements on both PRS- and SRS-RSRPP. RAN1 has agreed to use the existing measurement report mapping table for PRS-RSRPP and SRS-RSRPP as a starting point. Thus, in RAN4, the existing report mapping and report resolution can be considered as a starting point to define the reporting requirements for AIML-based positioning.

For the report mapping of DL PRS-RSRPP (or UL SRS-RSRPP), the existing mapping of measured quantity is defined in Table 10.1.38.3.1-1 (or Table 13.6.1-1) can be reused/used as a baseline.

When defining report mapping in the legacy positioning, the measured quantity value, e.g., RSRPP, is directly used as the input of a positioning algorithm, e.g., triangulation, to calculate the distance between the UE and gNB. Therefore, it is possible to analyze and estimate the impact of the reporting resolution on the positioning accuracy when a certain positioning algorithm is used. Unlike the legacy positioning, the AIML-based positioning will generate a dataset based on the reported measurements. Then, the measurement dataset is used as the input of the model. In this case, the composition of the input measurement dataset, such as a granularity and quality, has a significant impact on the performance of the AI/ML models. To improve the accuracy of AIML positioning, a larger and more diverse dataset can be generated for training and testing while using a higher reporting resolution of these measurements. In Rel-19, AIML positioning could adopt a flexible/adaptive reporting method with a possible modification of the reporting mapping designed for the legacy positioning. Hence, it is needed to study the use cases where UE sends a report on the path-based measurement of the positioning reference signals and how to define the reporting requirements for AIML positioning (e.g., report mapping, reporting resolution, etc.).

RAN4 to study the reporting requirements of PRS-RSRPP / SRS-RSRPP for AIML positioning (e.g., report mapping, reporting resolution, etc.).

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to AI/ML based positioning. In the paper, the following observations and proposals were made:

- For AIML positioning, RAN4 to define the requirements on the measurements supported in legacy positioning (e.g., RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress.

- RAN4 requirements (e.g., in Cases 1, 2a) should be firstly defined with respect to single PFL.

- The consistency between training and inference should be achieved by having a requirement on consistent configuration (e.g., same PFL) when UE-sided models are used.

- RAN4 to study the requirements on a quality indicator for the ground-truth accuracy on positioning label in data collection.

- Positioning accuracy should be considered as the performance metric in Case 1 (UE-based positioning with UE-side model, direct AI/ML positioning).

- Even when ground truth, i.e., accurately known position of the UE, for Case 1 AI/ML-based positioning is not available in the testing setup for absolute positioning accuracy calculation, the new functionality should not be left without any requirements or tests.

- RAN4 to consider relative positioning accuracy metric for Case 1, i.e., requirement on the possible change in AI/ML-based position due to a change/state of the environment/configuration.

If an LCM action is required and it is not taken in a timely manner, the performance degradation for AI/ML enabled Positioning use case may be degraded to undesirable level.

RAN4 to define the time latency limit on UE’s LCM actions indicated by network in Cases 1 and 2a.

Activation, Deactivation, Switching, Fallback to non-AI operation, and Performance monitoring should be prioritized for defining RRM requirements for LCM AI/ML positioning use case.

LOS/ NLOS indicator should be considered as an intermediate performance metric for case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML-assisted positioning).

For the report mapping of DL PRS-RSRPP (or UL SRS-RSRPP), the existing mapping of measured quantity is defined in Table 10.1.38.3.1-1 (or Table 13.6.1-1) can be reused/used as a baseline.

RAN4 to study the reporting requirements of PRS-RSRPP / SRS-RSRPP for AIML positioning (e.g., report mapping, reporting resolution, etc.).

- References

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#96, Budapest, Hungary, June 06 – June 09, 2022.

- RP-233132, Status report for SI on AI/ML for NR air interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- R4-2406617, WF on AI/ML, Qualcomm, RAN4#110bis, Changsha, China, April 15 - 19, 2024.

- R4-2403712, WF on AI/ML, Qualcomm, RAN4#110, Athens, Greece, February 26 – March 1, 2024.

- R4-2410570, WF on NR ALML air interface, Qualcomm, RAN4#111, Fukuoka, Japan, May 20 – 24, 2024.

- R4-2404323, WF on NR ALML air interface, Qualcomm, RAN4#112, Maastricht, Netherlands, Aug. 19 – 23, 2024.

- Annex

This section captures the agreements on AIML for NR air interface support for positioning accuracy enhancement from RAN1#118, RAN1#117, and RAN1#116bis meetings.

RAN1#118

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB. Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

RAN1#117

Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

RAN1#116bis

Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Conclusion - It is out of RAN1 scope to decide whether/how synthetic data (i.e., not direct physical data) and related entities are used in AI/ML based positioning. In RAN1 discussion, data (e.g., measurement data, label data) refer to physical data, not synthetic data.

Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 2a, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.