3GPP TSG-RAN WG4 Meeting #112bis                               						     R4-2416450

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	Apple

Title: 	On definition of UE types for NonCol_intraB

Agenda Item:	6.5.2.3

Document for:	Discussion

# Introduction

In the last RAN4 meeting, a WF on introducing type 1 and type 2 UE definitions [1] was agreed. The motivation of such proposal is to simplify the specification description. This contribution will give our view on this issue and present our views.

# Discussion

The issue on introducing definition for different type of UEs has ever been discussed in Changsha meeting but not agreed. Actually, the existing way of using “UE indicating [signalling name]” was achieved after a lot of discussion in RF and RRM room and was deemed as the preferred way at that time. But we are open to have such discussion if the group feels better to introduce such a definition. While it would be better to consider align the definition for all the potential UE types together since in Rel-19 type 4 and 8Rx type 1 is also being discussed.

With the introduction of type 2 in Rel-18 and type 4 in Rel-19, 4 UE types need to be considered,

- Type 2

- Type 4

- 4Rx type 1

- 8Rx type 1

For the type 2 and type 4 the common ground is that they can support 33us MRTD and 25dBm power imbalance. While the difference is the number of maximum supported MIMO layers. To effectively represent and differentiate type 2 and type 4, 3 factors need to be addressed in the definition, e.g. MRTD, power imbalance and maximum supported MIMO layers.

Fot 8Rx type 1 and 4Rx type 1, the common ground is that they can support 6us MRTD and 6dBm PSD imbalance. While the difference is also the number of maximum supported MIMO layers. To effectively represent and differentiate 4Rx type 1 and 8Rx type 1, 3 factors need to be addressed in the definition, e.g. MRTD, power imbalance and maximum supported MIMO layers.

Proposal 1:  MRTD, Power imbalance and maximum supported MIMO layers need to be included in the definition for different type of UE.

The next issue is where to put the definition for such UE. Since type 1 UE is not relevant to section 7.10A, we think it’s not appropriate to put it in section 7.10A. The other candidate places could be section 3 or section 7.1 in 38.101-1/-3 or 36.306 (by adding type clarification in square bracket) depending on how we define the UE type.

Proposal 2: candidate place for capturing UE type definition could consider section 3 or section 7.1 in 38.101-1/-3 or 36.306 (by adding type clarification in square bracket) depending on how we define the UE type

# Summary

This contribution presented our consideration on UE type definition. The following proposals are presented for discussion.

Proposal 1:  MRTD, Power imbalance and maximum supported MIMO layers need to be included in the definition for different type of UE.

Proposal 2: candidate place for capturing UE type definition could consider section 3 or section 7.1 in 38.101-1/-3 or 36.306 (by adding type clarification in square bracket) depending on how we define the UE type

# References

[1]	R4-2414454, WF on Rel-18 clarifications of RF requirements for different UE types, Huawei.