3GPP TSG-RAN WG4 Meeting #112-bis 	R4-2416453

Hefei, China, October 14th – 18th, 2024

Agenda Item: 6.20.3

Source: MediaTek inc.

Title: Discussion on SBFD

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2]. In this contribution, we provide further discussion on the issues captured in the WF.

# Discussion

The discussion on the RRM impact for the evolution of NR duplex operation for SBFD is provided in this section. This section provides high level discussion for both agendas: (i) CLI handling; and (ii) SBFD operation.

# Discussion on the RRM impact for CLI handling mechanisms

In the previous meeting, RAN4 RRM captured the following issues in the WF:

### Sub-topic 2-1: RRM impacts of UE-to-UE CLI handling #### Issue 2-1-1: Scope of requirements for UE-to-UE CLI handling Agreements (from online session) - RAN4 to define RRM requirements for L1 based UE-to-UE CLI measurement and reporting. #### Issue 2-1-2: Measurement quantities Recommended WF - RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. This can be revisited based on further RAN1 agreement. - FFS if other measurement quantities to be considered in the core requirements based on further RAN1 agreement. #### Issue 2-1-3: Baseline for defining requirements Recommended WF - Both R16 CLI measurement requirements and L1-RSRP measurement requirements are to be considered for defining requirements for L1 based UE-to-UE CLI measurement. - RAN4 can directly discuss the requirements for L1 based UE-to-UE CLI measurement in next meeting. #### Issue 2-1-4: Measurement methods Recommended WF - RAN4 to wait for RAN1 conclusion on the supported methods of L1 based UE-to-UE CLI measurement. - Meanwhile, RAN4 to discuss the impacts of different measurement methods on the requirements. #### Issue 2-1-5: Rx beam Recommended WF - RAN4 to wait for RAN1 conclusion on the Rx beam for L1 based UE-to-UE CLI measurement. - Meanwhile, RAN4 to discuss the impacts of Rx beam configuration/determination on the requirements. #### Issue 2-1-6: Measurement resources Recommended WF - RAN4 to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements. #### Issue 2-1-7: Side condition Agreements (from online session) - RAN4 to define side conditions for L1 based UE-to-UE CLI measurement requirements. - RAN4 to discuss at least time offset between DL timing and SRS arrival timing, SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP. - The RAN1 progress will be taken into account when discussing the values for the side conditions. Options for further discussion and down-selection (for information purpose): - Time offset between DL timing and SRS arrival timing - Option 1: Rel-16 CLI SRS RSRP assumption - Option 2: Remove cell phase error from option 1 - Other options are not precluded, pending on RAN1 progress. - SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP - Option 1: Rel-16 CLI SRS RSRP assumption - Other options are not precluded. #### Issue 2-1-8: Measurement period Agreements (from online session) - RAN4 to define measurement periods for L1 based UE-to-UE CLI measurement requirements, - RAN4 to discuss the number of shots for the measurement, which can be based on RAN4 simulation results and RAN1 design. - Number of shots for simulation: 1, 2, 3. - Companies to bring proposals on other simulation parameters in the next meeting. #### Issue 2-1-9: Measurement reporting Recommended WF: - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting - FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement - FFS whether L1-RSRP measurement reporting requirement can be re-used. #### Issue 2-1-10: Measurement accuracy Recommended WF: - RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period. #### Issue 2-1-11: Scheduling and measurement restriction Recommended WF - RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed. - FFS the impact of measurement methods and Rx beam assumption #### Issue 2-1-12: Measurement capability Recommended WF - FFS whether RAN4 needs to discuss measurement capability for L1 based UE-to-UE CLI measurement in terms of number of resources UE shall be able to monitor. #### Issue 2-1-13: Report mapping Recommended WF - FFS whether RAN4 needs to discuss report mapping for L1 based UE-to-UE CLI measurement. - FFS whether R16 report mapping can be re-used.

In legacy 5G NR TDD, the RRM specification cover the CLI measurements delay and accuracy for SRS-RSRP and CLI-RSSI. The UE reports these measurements to the gNB to help the dynamic allocation of uplink/downlink in time. Nevertheless, these measurements are based on L3 measurements, i.e. it requires L3 filtering, which has longer measurement delay compared to L1 measurements. Therefore, in SBFD Rel-19 WID objective, new CLI measurements and reporting shall be introduced for L1 based UE-to-UE CLI with new measurement quantities, such as L1-SRS-RSRP, L1-CLI-RSSI.

Issue 2-1-2: Measurement quantities: 
In the previous meeting, RAN4 agreed to define RRM requirements for L1 based UE-to-UE CLI measurements and reporting, also, RAN4 captured the issue of whether to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. As mentioned earlier that the main motivation for defining the new requirements is introduce faster reporting based on shorter measurement delay compared to that of R16 CLI measurements. Therefore, RAN4 should consider to define L1 CLI with shorter measurements delay compared to that of L3 CLI defined in R16. Nevertheless, the number of measurement delay samples for R16 CLI-RSRP and R16 CLI-RSSI are equal to three and one sample(s). Therefore, RAN4 can define the new measurement quantity of L1-CLI-RSRP with number of measurements samples less than three. However, given that CLI-RSSI measurement is based on one sample then the motivation to define L1-CLI-RSSI is not clear.

Observation 1: The measurement delay for the new measurement quantities, such as L1-SRS-RSRP, L1-CLI-RSSI, should be faster compared to the existing measurement delay for SRS-RSRP.

Observation 2: The number of samples for the existing measurements of R16 CLI-RSSI is just one sample, while R16 CLI-RSRP has three samples.

- RAN4 RRM shall define core requirements for L1-CLI-RSRP, and to further assess the necessity of defining L1-CLI-RSSI.

Issue 2-1-3: Baseline for defining requirements: 
Provided that RRM spec in 38.133 includes requirements for both R16 CLI measurement and L1-RSRP, then RAN4 can take these measurements as baseline defining the new CLI measurements based L1 reporting.

- RAN4 RRM shall consider both R16 CLI measurement requirements and L1-RSRP measurement requirements as a baseline to define requirements for L1 based UE-to-UE CLI measurement.

Issue 2-1-4/2-1-5/2-1-6: Measurement methods, Rx beam and measurement resources: 
Provided that the measurements methods, resources, Rx beam and framework should be defined by RAN1 WG while measurements accuracy and delay requirements should be defined by RAN4 RRM. Hence, RAN4 might wait for RAN1 to complete the framework design for these new measurements.

- RAN4 to wait for RAN1 conclusion on the supported methods, measurement resources and the Rx beam for L1 based UE-to-UE CLI measurement.

Issue 2-1-7: Side condition: 
In the previous meeting, RAN4 discussed the issue briefly and two main side conditions were mentioned, which are (i) time offset and SRS Ed/IoT. Furthermore, the discussion on time offset between DL timing and SRS arrival timing included two options, where the first option is use R16 requirements as baseline, while Option 2 is based on removing the cell phase error from option 1, resulting in better accuracy requirements. It should be mentioned that Option 2 is only applicable for intra-cell CLI measurements, also, it is worth highlighting that the UE is not aware of whether this measurement is intra-cell CLI or inter-cell CLI. Therefore, in order to enable the benefits of Option 2, then the framework and requirements of intra-cell and inter-cell CLI measurements should be differentiated. Thus, we suggest to RAN4 to further discuss the usefulness of this new option.

Regarding the second side condition of SRS Es/IoT, reusing R16 CLI SRS RSRP assumption seems to be sufficient.

Observation 3: In order to remove the cell phase error from R16 time offset side condition, the measurement has to be restricted to intra-cell CLI.

Observation 4: The UE can’t differentiate between intra-cell and inter-cell CLI measurements and hence there is no point of defining two different requirements.

- RAN4 RRM should follow R16 CLI SRS RSRP assumption for L1 CLI measurement requirements.

Issue 2-1-8: Measurement period and simulation assumptions: 
In the previous meeting, RAN4 agreed to bring proposals on simulation assumptions and parameters to assess the number of shots needed for L1 based UE-to-UE CLI measurements requirements. In R16, RAN4 agreed on simulation assumptions for CLI-RSSI and CLI-RSRP to define the requirements. Therefore, we suggest to reuse these simulation assumptions to run the new simulations for the new L1 based UE-to-UE CLI measurements. The simulations assumptions are provided in Table 1, mainly considered from R16 in Meeting R4#91 [3] with timing error updated from later meetings, R4#92Bis [4]. To evaluate SRS-RSRP measurement accuracy, only UE0 and UE1 are considered in Table 1, and to evaluate cyclic shift impact, UE0, UE1, and UE2 are considered in Table 1.

Table 1 Simulation assumptions

| Parameters |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | UE0 (cell 0) [victim UE] | UE1 (cell 1) [aggressor UE] | UE2 (cell 1) [aggressor UE] |
| Carrier frequency |  | 4GHz for FR1, 30GHz for FR2 |  |  |
| Subcarrier spacing |  | 15,30,60kHz FR1 60, 120kHz FR2 | same as UE0 | same as UE0 |
| SRS configuration | Number of port | N/A | 1 | 1 |
|  | TransmissionComb |  | 2 | 2 |
|  | combOffset |  | 0 | 0 |
|  | Cyclic shift |  | 0 | 0 |
|  | StartPosition (SRS symbol start position) |  | 0 | 0 |
|  | Number of symbol |  | 1 | 1 |
|  | Number of RB Note1 |  | 24 or 48 | same as UE1 |
|  | Frequency domain position and shift |  | 0 | 0 |
|  | Periodicity [slot] |  | 5 | same as UE1 |
| Timing errorNote2 for UE 1 and UE2 |  | 4.67usec FR1, 3.67usec FR2 | - | - |
| CP Length |  | Normal |  |  |
| Number of measurement samples |  | 3 or 5 | N/A | N/A |
| Measurement bandwidth [RB] |  | Same as SRS transmission RB |  |  |
| Measurement period [slot] |  | 5 | N/A | N/A |
| Number of UE receive antennas |  | 2 Rx | N/A | N/A |
| Propagation conditions |  | AWGN, TDL-A10, TDL-C100 for FR1, AWGN, TDL-A75, TDL-C300 for FR2 |  |  |
| SINR |  | -3dB or -1dB | N/A | N/A |  |
| Note 1: Allocated RBs are within active BWP for measurement UE. Note 2: Timing error without constant offset is (NTA_offset + NTA)A + Dpropagation_V – Dpropagtion_V2A . (NTA_offset + NTA)A is timing advance for aggressor(A) UE. Dpropagation_V is downlink propagation delay for victim(V) UE from the gNB of the victim UE. Dpropagtion_V2A is propagation delay from aggressor(A) UE to victim(V) UE. Note 3: No frequency hopping is assumed. |  |  |  |  |  |

- RAN4 to define the simulation assumptions as in Table 1 in this contribution paper.

Issue 2-1-9: Measurement reporting: 
The current recommended WF is agreeable, which is: (i) RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting; (ii) FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement; and (iii) FFS whether L1-RSRP measurement reporting requirement can be re-used.

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting and FFS whether L1-RSRP measurement reporting requirement can be re-used pending on RAN1 agreement.

Issue 2-1-10: Measurement accuracy: 
The current recommended WF is agreeable, i.e. RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period.

- RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period.

Issue 2-1-11: Scheduling and measurement restriction: 
The current recommended WF is agreeable, i.e. RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed and FFS the impact of measurement methods and Rx beam assumption.

- RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed and FFS the impact of measurement methods and Rx beam assumption.

Issue 2-1-12/2-1-13: Measurement capability and reporting mapping:

Provided that measurement capability and mapping is based on the RAN1 design and agreements, hence, RAN4 might wait for RAN1 to complete the framework design for these new measurements.

- RAN4 RRM should wait for RAN1 WG to define the new measurement resources and framework before defining measurements capability and reporting mapping.

# Discussion on the RRM impact due SBFD operation

In SBFD operation UE remains a half-duplex node. This implies that in SBFD symbols, the UE can perform either DL reception (on DL subbands) or UL transmission (on UL subband), but not both. To avoid collisions between DL receptions and UL transmissions, the network must provide a way for UE to determine the link direction on SBFD symbols, i.e., whether the UE transmit or receive in a SBFD symbol. Therefore, several issues were brought up to the RRM discussion by multiple companies, which are captured in the WF.

#### Issue 2-3-2: Requirements for SSB based measurement Agreements (from online session) - For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4. - Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell. Recommended WF - RAN4 further discuss impact on SSB based neighbour cell measurement in RAN4.

Before assessing which measurements are impacted with the SBFD operation, RAN4 RRM should take into consideration the agreements made in RAN1. Selected RAN1 agreements related to the measurements are provided below:

In RAN1#117, the following agreements were made. SBFD TX/RX/measurement procedures Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

Figure 1: Serving cell (L1/L3) and neighbouring cell SSB based measurements

Based on the agreements from RAN1 and RAN4, L1/RLM/BFD SSB based measurements are prioritised to the UL transmission. Yet, RAN4 highlighted that SSB based neighbouring cell measurements are not part of the RAN1 discussion and the issue should be discussed and resolved in RAN4. Therefore, we provide our analysis regarding this matter.

For the neighbouring cell measurement scenarios that the SSB falls within the active BWP, the neighbouring cells and serving cell are measured at the same time within the SMTC. Now, given that serving cell measurements are prioritised to UL transmission, therefore, other neighbouring cell with SSB within the active BWP shall be prioritised over UL transmission and existing requirements shall apply. In addition, when the neighbouring cell SSB is outside the active BWP then MG is needed, and given that MG interrupt UL/DL, thus, the existing requirements can apply.

Observation 5: The serving SSB resources are prioritized over UL transmission and hence there RLM/BFD/RRM measurements based SSB are not impacted by the SBFD operation.

Observation 6: The neighbouring SSB resources within the active BWP are measured with serving cell measurements, hence, SSB based measurements are not impacted by the SBFD operation.

- For SBFD-aware UE, SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission for SSB based neighbouring cell measurement.

- For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement.

Issue 2-3-3: Requirements for CSI-RS based measurement:

In RAN1#117, the following agreements were made. SBFD TX/RX/measurement procedures Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

RAN4 agreed to discuss the impact of SBFD operation on the CSI-RS measurement requirements. Based on the agreement made in RAN1 that the CSI-RS can be overlapped with SBFD subbands and the CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. Hence, CSI-RS based measurements are impacted with the SBFD operation. Besides, as stated in the scope of the WID, for SBFD operation in a slot defined as downlink and/or flexible by tdd-UL-DL-ConfigurationCommon, uplink transmission is within uplink subband only whereas downlink reception is within downlink subband(s) only. Therefore, the CSI-RS based measurements are based on the CSI-RS frequency resources within DL usable PRBs. Figure 2 depicts the configured CSI-RS resources, which are overlapped with SBFD subbands. The figure also depicts the CSI-RS resources mapped within DL usable PRBs.

Figure 2: depicts the configured CSI-RS resources overlapped with SBFD subbands.

Based on the above, RAN4 should pay attention to two aspects, which are: (i) CSI-RS resources configuration and (ii) whether existing RRM requirements are applicable. Firstly, in order for the UE to be able to do CSI-RS based measurements the UE should be aware of which CSI-RS resources are overlapped with SBFD subband. To ensure this, RAN4 can highlight that to RAN2 to ensure that the signalling design cover the issue. Second, the existing RRM requirements for CSI-RS based measurements are defined by the minimum number of PRBs of CSI-RS resources, otherwise, requirements are not applicable. Therefore, in order to ensure the existing requirements are applicable, the minimum number of configured CSI-RS resources are equal or larger than that specified in the RRM requirements.

Observation 7: The configured CSI-RS resources can be overlapped with SBFD subbands and hence the CSI-RS based measurements might be impacted.

Observation 8: The RRM requirements for CSI-RS based measurements are defined by the minimum number of PRBs of CSI-RS resources used for the measurements.

- For SBFD-aware UE, NW shall configure the UE with the punctured CSI-RS resources by the SBFD subband operation.

- RAN4 shall check whether there is a need to update the CSI-RS based measurements based on the minimum number of CSI-RS resources after being punctured by the SBFD subband.

Issue 2-3-4: Requirements for scheduling restriction:

In R16, the scheduling restrictions requirements are defined for the CLI-RSSI and CLI-RSRP. Now, to judge whether there is a need to update these requirements or define a new one is based on the progress in RAN1. Therefore, RAN4 shall wait for RAN1 to complete the framework design for these UE-to-UE CLI measurements.

- RAN4 RRM should wait for RAN1 WG to complete the design for UE-to-UE CLI measurement quantities and framework before defining the scheduling restrictions RRM requirements.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 RRM shall define core requirements for L1-CLI-RSRP, and to further assess the necessity of defining L1-CLI-RSSI.

Proposal 2: RAN4 RRM shall consider both R16 CLI measurement requirements and L1-RSRP measurement requirements as a baseline to define requirements for L1 based UE-to-UE CLI measurement.

Proposal 3: RAN4 to wait for RAN1 conclusion on the supported methods, measurement resources and the Rx beam for L1 based UE-to-UE CLI measurement.

Proposal 4: RAN4 RRM should follow R16 CLI SRS RSRP assumption for L1 CLI measurement requirements.

Proposal 5: RAN4 to define the simulation assumptions as in Table 1 in this contribution paper.

Proposal 6: RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting and FFS whether L1-RSRP measurement reporting requirement can be re-used pending on RAN1 agreement.

Proposal 7: RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period.

Proposal 8: RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed and FFS the impact of measurement methods and Rx beam assumption.

Proposal 9: RAN4 RRM should wait for RAN1 WG to define the new measurement resources and framework before defining measurements capability and reporting mapping.

Proposal 10: For SBFD-aware UE, SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission for SSB based neighbouring cell measurement.

Proposal 11: For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement.

Proposal 12: For SBFD-aware UE, NW shall configure the UE with the punctured CSI-RS resources by the SBFD subband operation.

Proposal 13: RAN4 shall check whether there is a need to update the CSI-RS based measurements based on the minimum number of CSI-RS resources after being punctured by the SBFD subband.

Proposal 14: RAN4 RRM should wait for RAN1 WG to complete the design for UE-to-UE CLI measurement quantities and framework before defining the scheduling restrictions RRM requirements.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2413900, “WF on SBFD RRM”, Huawei, Maastricht, NL, August 2024.

- R4-1907392, “Simulation assumption for SRS-RSRP measurement requirements”, LGE, Reno, USA, May 2019.

- R4-1912667, “Way forward on CLI remaining RRM open issues”, LGE, Chongqing, China, Oct., 2019.