3GPP TSG-RAN WG4 Meeting #112bis	R4-2416467

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On NR MIMO Phase 5 asymmetric DL sTRP/UL mTRP

Agenda item:	6.19.3.2

Document for:	Discussion

Introduction

MIMO is one of the key technologies in NR systems, and Rel-19 will bring important enhancements to both FR1 and FR2. Among the enhancements to be studied, the following is listed in the WID [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this document, we present Nokia’s views regarding the core requirements that RAN4 need to discuss in relation to the WID, in particular about:

- Objective 5: Asymmetric DL sTRP/UL mTRP deployment scenarios.

Discussion on TCI switching requirements

Regarding asymmetric DL sTRP/UL mTRP TCI switching, the following was agreed and discussed in the last RAN4#112 meeting [2]:

Issue 2-5-4: RRM core impacts of Active uplink TCI state switching delay for unified TCI? <Way forward> - Option 1: The legacy requirements can be reused. - Option 2: The legacy requirements cannot be reused. - Option 3: RAN4 to discussion how to update R17/R18 requirement for asymmetric DL sTRP/UL mTRP scenarios Issue 2-5-5: Whether to define RRM core requirements of pathloss offset update? Agreement: Whether to define RRM core requirements of pathloss offset update? - Not define RRM requirement for MAC CE decoding delay. - The agreement can be revisited depending on further RAN1 progress.

An asymmetric DL sTRP/UL mTRP deployment scenario is a s-DCI scenario where the symmetric DL/UL TRP will transmit and receive the PDCCH and the PUCCH respectively. In Rel-18 MIMO evo, RAN4 has defined downlink as well as uplink TCI state switching delay for unified TCI state switch for single-DCI mTRP.

In case of downlink TCI state switching delay requirements, we see no difference in the downlink TCI state switching behavior and hence see no need to define new set of downlink TCI state switch delay requirements.

Similar to the downlink TCI state switching delay requirements, there is no difference in the uplink TCI state switching behavior either when the UE is indicated to switch to a target uplink TCI state associated to the uplink only TRP. Hence, the legacy RAN4 unified TCI state switching delay requirements for UL TCI state can be reused.

Since existing Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework is to be reused for asymmetric DL sTRP/UL mTRP, there is no change in DL and UL TCI state switch delay requirements.

For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI s-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

Although the delay for UL TCI state switch does not need to be changed from the legacy, the requirements still need some updates for the UL-only TRP TCI state switch case.

The uplink TCI state switching delay requirements specified up to Rel-18 have been defined for an uplink TCI state which was associated with a downlink reference signal. For an UL-only TRP’s TCI state, the reference signal configured as QCL-D is an SRS, which is an uplink signal. The existing UL TCI state switching requirements consider only the case of a TRP which transmits downlink reference signal. In the case of asymmetric DL sTRP/UL mTRP, since the uplink only TRP does not transmit any downlink reference signal, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements. Hence, the applicability of the existing uplink TCI state switch delay requirements should be extended to include uplink TCI states which are associated to an uplink reference signal (SRS).

Existing Rel-17 and Rel-18 unified TCI switching requirements are not applicable for target TCI state without DL reference signal.

Unified Uplink TCI state switching delay requirements to be applicable when the target uplink TCI state is associated with an uplink reference signal.

The legacy TCI state switch delay requirements have been defined based on a TCI state being known or unknown. For a TCI state to be considered as known there are set of conditions defined which are primarily centered around a downlink reference signal being transmitted by the TRP. The known/unknown conditions are used to determine if the UE has reported a valid recent measurement from the target TCI state before making the switch. In case the target TCI state is unknown the UE is expected to perform measurements on the target TCI state, in order to determine transmission parameters such as spatial filter of the target TCI state.

However, in case of an asymmetric DL sTRP/UL mTRP, since the uplink only TRP does not transmit any reference signal, this implies that the conditions defined up to Rel-18 for an uplink TCI state to be known cannot be used in the context of an asymmetric DL sTRP/UL mTRP.

Legacy uplink TCI state switching delay requirements are based on a TCI state switch been known or unknown.

Conditions defined for a TCI state to be considered as known or unknown are based on a downlink reference signal

For an uplink only TRP, since there is no downlink reference signal being transmitted, the legacy known/unknown conditions cannot be reused.

RAN4 to define UL TCI state switch delay requirements for an uplink only TRP with no distinction between known and unknown TCI state.

Discussion on two TAs

Regarding asymmetric DL sTRP/UL mTRP with two TAs enhancement, the following was discussed in the last RAN4#112 meeting [2]:

Issue 2-5-3: Whether RRM impacts exist by 2TA enhancement? <Way forward> - FFS on whether there are RRM impacts by 2TA enhancement.

The revised MIMO Phase 5 WID [1] has updated the objective 5 of the initial WID [3], by explicitly introducing the two TAs enhancement for the asymmetric DL sTRP/UL mTRP, and RRM requirements should be defined for that. When considering the RRM requirements for single TA, we understand that the existing requirements also apply for asymmetric DL sTRP/UL mTRP deployment.

Existing RRM requirements with single TA apply for asymmetric DL sTRP/UL mTRP deployment.

Define RRM requirements for asymmetric DL sTRP/UL mTRP deployment with two TAs enhancement.

It is worth recalling that Rel-18 introduced the two TAs enhancement for multi-DCI multi-TRP scenario allowing UL transmissions from a UE toward two TRPs whose links may be characterized by a large difference in the propagation delays with respect to the CP length. The most relevant enablers that were introduced were the following ones:

- Two TAGs can be configured for a serving cell configuration, each corresponding to a TRP.

  - To allow the UE to know which beam/TCI state is associated to which TAG, the TAG ID has been introduced as a field in the TCI state IE.

  - That allows a UE to receive a TA command TA1 for TRP1 and a TA command TA2 for TRP2, for both intra-cell and inter-cell multi-TRP.

- The UE is required to monitor two DL reference timings, i.e., one DL reference per TAG.

  - As with legacy single TA requirements, it is up to the UE to decide which RS, among the RSs transmitted by each TRP, to use as DL reference timing for each TAG.

The Rel-19 asymmetric DL sTRP / UL mTRP scenario can be identified as a single-DCI multi-TRP scenario, where the UL transmissions are scheduled by the DL/UL capable TRP, which is the only TRP sending data and control information to the UE.

From the RAN4 perspective, the Rel-18 and Rel-19 two TAs enhancements are very similar and in the RAN4 context the main difference may be in the number of DL reference timings that the UE shall monitor:

- If the TRP, which is not sending data or control to the UE, is a TRP that also transmits some DL signals, for example transmits SSBs, then the UE can measure DL RSs for each of the two configured TAGs, and the UE shall monitor two DL reference timings, one per TAG, and apply each TA command to the respective DL reference timing to determine the UL transmission timing toward that TAG.

- If the TRP, which is not sending data or control to the UE, is an UL-only TRP, then there simply is no DL RS associated to the second configured TAG (assuming the first configured TAG is associated to the DL/UL capable TRP), and the UE shall monitor just one DL reference timing, and apply both TA commands to that same DL reference timing to determine the UL transmission timings toward the two TAGs.

Specify the conditions under which a UE shall monitor one or two DL reference timings for asymmetric DL sTRP/UL mTRP with two TAs enhancement.

Conclusion

In this paper, the following Observations and Proposals were made:

Observation 1: Since existing Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework is to be reused for asymmetric DL sTRP/UL mTRP, there is no change in DL and UL TCI state switch delay requirements.

Proposal 1: For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI s-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

Observation 2: Existing Rel-17 and Rel-18 unified TCI switching requirements are not applicable for target TCI state without DL reference signal.

Proposal 2: Unified Uplink TCI state switching delay requirements to be applicable when the target uplink TCI state is associated with an uplink reference signal.

Observation 3: Legacy uplink TCI state switching delay requirements are based on a TCI state switch been known or unknown.

Observation 4: Conditions defined for a TCI state to be considered as known or unknown are based on a downlink reference signal

Observation 5: For an uplink only TRP, since there is no downlink reference signal being transmitted, the legacy known/unknown conditions cannot be reused.

Proposal 3: RAN4 to define UL TCI state switch delay requirements for an uplink only TRP with no distinction between known and unknown TCI state.

Proposal 4: Existing RRM requirements with single TA apply for asymmetric DL sTRP/UL mTRP deployment.

Proposal 5: Define RRM requirements for asymmetric DL sTRP/UL mTRP deployment with two TAs enhancement.

Proposal 6: Specify the conditions under which a UE shall monitor one or two DL reference timings for asymmetric DL sTRP/UL mTRP with two TAs enhancement.

- References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105, Melbourne, Australia, September 9-12, 2024.

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, RAN4#112, Maastricht, Netherlands, 19th ‒ 23rd August, 2024.

- RP-232915, Proposal for Rel-19 MIMO Objectives, RAN#102, Edinburgh, Scotland, December 11-15, 2023.