3GPP TSG-RAN WG4 Meeting #112bis	R4-2416468

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Discussion on XR ph3 RRM requirements

Agenda item:	6.25.2

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on RRM requirements for XR enhancements. As part of this discussion we highlight the deployment scenarios that we expect to be important for XR enhancements, which types of measurements to be considered, the potential benefits of UAI, and the discussion on reply LS to RAN1.

Discussion

Deployment scenarios

In the RAN4 #112 meeting, the following issue was left for discussion in the agreed WF:

## Issue 1-2: Deployment scenarios <Way forward>: - Discuss if there are aspects related to deployment scenarios to be considered.

Firstly, we would like to focus the attention to which scenarios the enhancements of scheduling during gaps or scheduling restrictions would apply.

With respect to frequency ranges, RAN1 has provided simulation results only considering FR1. The enhancements proposed with measurement skipping could be translated in extension of measurement delays. Therefore we need to consider the baseline measurement delay requirements before deciding which FRs to apply the enhancements. As an example, the PSS/SSS detection minimum delay is 600 ms, and for 20 ms MGRP TPSS/SSS_sync_inter=800 ms for FR2-1. On the other hand, for FR2-2, the beam sweeping scaling factor is extended to 12 instead of 8, and there is an additional scaling factor of KFR=2 and KFR=3 for 480 kHz and 960 kHz respectively. As a result the measurement delay in FR2-2 is 1.5x, 3x and 4.5x times larger than the delay for FR2-1 for 120 kHz, 480 kHz and 960 kHz SCS respectively. Additionally, FR2-2 has shorter slot durations and larger bandwidth, and as a result, measurement skipping feature is not as needed as other scenarios. Considering the long measurement delays for FR2-2 we believe it is best to avoid further extensions that would be resulting from measurement skipping.

FR2-2 measurement delay requirements are 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS.

Measurement skipping apply for FR1 and FR2-1 measurements

Types of measurement gaps

In the last RAN4 meeting, the types of measurement gaps (MG) were discussed, where companies proposed for RAN4 to decide which types of gaps are applicable for measurement skipping. As part of this discussion, the following way forward was agreed:

## Issue 1-3: Types of measurement gaps to consider In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. <Way forward>: Please discuss the priorities for RAN4 to work based on the following table: Cap types; First prio; Second prio; Don’t include. Type 1; Nokia, MTK, QC, Vivo, ZTE, Apple, CMCC, HW, Ericsson, OPPO, CATT, IDC. Type 2; QC, vivo, Apple, CMCC, HW, OPPO, CATT; Nokia; ZTE. NCSG; Apple, CMCC, HW, OPPO, MTK; QC, Nokia, CATT, IDC; vivo. MUSIM; HW; Vivo, Apple, Nokia, Ericsson, CATT. Positioning; QC, vivo, HW; ZTE, Apple, Nokia, Ericsson, CATT, IDC. Rel 18 concurrent gap; QC, Apple, CMCC, HW, CATT, MTK; Vivo, ZTE, Nokia, Ericsson. PreConfigured; Apple, QC, CATT, CMCC, HW, OPPO, MTK.

As part of the RAN4 #112 discussion it was clear that for many companies, Type 1 MG can be treated with higher priority on this work item.

From our perspective, RAN4 can start by agreeing on studying the requirement impact for Type 1 MG, i.e. Rel-16 MGs. This is the clearest and most general case for the measurement skipping scenario. Additionally, once the requirement impact for Type 1 MG is identified, we can discuss how simple it is to extend and apply the same principles for other types of measurement gaps.

RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Application of the requirements to other types of gap should be dealt with in a second stage.

As for the second priority, we believe that Type 2 MG, NCSG and MG types used for positioning should be considered in this work item. As gNB is not controlling the positioning request and positioning may require longer MG, the case of measurement gap types for positioning should be addressed.

Once the impact on requirements for measurement skipping for Type 1 MG has been agreed, Type 2 MG, NCSG and MG for positioning should be considered as 2nd priority.

When MUSIM gaps are considered, we think there is little benefit on defining requirements for this type of gaps. From Network A (the network configuring MUSIM gaps) point of view, there are no requirements impacting Network A performance due to MUSIM gaps. Therefore, it is unclear how the UE behaviour should be impacted when a MUSIM gap is skipped. In our view it is UE’s responsibility to avoid the need for MUSIM gaps, and send UAI requesting no MUSIM gaps while operating with XR.

MUSIM,  Preconfigured, and Rel-18 concurrent gaps are not considered for Rel 19 XR ph3 enhancements.

In case skipping of multiple types of gaps will be supported, it should be up to the network to configure the UE if the DCI-based skipping indication is applicable to all of those, or only a subset. This could be configured from the NW to the UE by means for RRC signalling.

Proposal 4: If multiple types of gaps will be supported for potential skipping, the NW should be able to configure the UE which gap types to skip upon reception of DCI-based skipping indication.

Finaly, it is important that we decide on the meaning of “Don’t include” column in last meeting’s discussion. When RAN1 discussed measurement skipping, they also made a decision that which types of gaps and scheduling restrictions can be skipped it up to RAN4 decision. Therefore, it is within the scope of RAN4 the decision of which types of gaps and measurements can be skipped using the DCI skipping indication.

Measurement skipping is not applicable for MUSIM, Preconfigured, and Rel-18 Concurrent gaps measurement gap based on the Rel 19 XR DCI skipping indication.

Types of measurements

In the RAN4 #112 meeting, the following issue was left for discussion in the agreed WF regarding types of measurements:

## Issue 1-1: Workscope < Agreement>: - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed. ## … ## Issue 1-4: Types of measurements <Way forward>: - Discuss types of measurements that may potentially be impacted by the XR operation. - FFS: measurements without gaps

With respect with types of measurements, RAN4 needs to discuss and conclude on which cases to include as part of this WID. As discussed in Section 3, RAN1 has an agreement stating that RAN1 conclusions are agnostic to the types of measurement gaps for the enhancements in XR_ph3. Therefore, it must be clarified which types of gaps and scheduling restrictions apply for measurement skipping enhancements.

RAN1 is defining enhancements for skipping measurements that cause scheduling restrictions which are agnostic to the types of measurement gaps or scheduling restrictions.

Legacy measurements with gaps are natural candidates for the measurement skipping enhancements. Here we refer to legacy measurement gaps as excluding pre-configured, NCSG, MUSIM, Positioning, NTN gaps. When the UE is configured with a measurement gap, the latency of any data transmission that is arriving shortly before the gap will be extended by at least the measurement duration.

As for measurements without gaps, they may cause scheduling restrictions in some situations. One typical situation for scheduling restrictions of measurements without gaps is when performing measurements in FR2. In that case the UE is expected to beam sweep during L3 measurements, and the beams will in most cases not coincide with beams for data reception. For this reason, scheduling restrictions are needed for FR2 measurements without gaps.

Intra-frequency measurements always cause scheduling restrictions in FR2, around SSB symbols to measure.

For measurements without gaps, another aspect to consider is whether interruptions are needed or not. Inter-frequency measurements without gaps can typically be performed using a spare RF chain present in the UE. When this RF chain is turned on, or retuned, it may cause some internal UE interference that causes data loss in the main RF chains used for data reception/transmission. This is referred to as interruptions in RAN4 RRM requirements.

Interruptions will cause the UE to lose some data/control before and after SMTC occasions to be measured. This will typically be more problematic for XR traffic than for normal eMBB traffic. The first reason for interruptions being more damaging to XR traffic is that losing data/DCI scheduling data will cause extra delay which can be large in relation to the total delay budget available for the UE.

Another reason why interruptions need to be avoided for XR traffic is that they may cause the UE to miss commands for measurement skipping. If DCI command containing that command is missed, that network could still think that the UE is available during the SMTC occasions, and many data transmissions during that time would be missed. That would further increase the XR data reception latency by the duration of the SMTC occasion. In particular, only the missing HARQ feedback can help the network in detecting the DCI command loss due to interruption. In TDD configuration, the detection may happen beyond the delay budget for the XR data transmission.

Interruptions will cause scheduling problems for latency sensitive traffic.

RAN4 has defined RRM requirements for interruptions for measurements without gaps in Rel-18 NR_MG_enh. During this work item, it was defined that interruptions are not restricted to a location. That means that although interruptions are expected to happen close to a SMTC to measure, there is no requirement of how close they should be. During this discussion, some companies pointed out that it was not clear whether Rel-16 UEs supporting NeedForGaps-r16 are causing interruptions.

If measurement skipping requirements are to be defined for measurements without gaps with interruptions, it is important that some method is defined in order to limit the interruptions over XR traffic. As an example a window where interruption is avoided could be defined. This would avoid the problems described above.

Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location in time.

Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements and restrictions.

Define requirements for measurement skipping only for measurement with gaps.

Impact of measurement skipping on RRM requirements

As part of the Rel 19 discussion, RAN4 has to conclude on the impact that measurement skipping might have on RRM requirements. In order to analyze the impact of measurement skipping we are going to compare it with what RAN4 has already specified in previous releases.

The effect of measurement skipping can be compared to the effect of CCA failures in Rel-16 NR_unlic. In unlicensed bands all devices transmitting need to perform a listen-before-talk (LBT) procedure, or clear channel assessment (CCA). That is also applicable for gNB transmitting SSB. As a result, it is possible that the channel is not available when an SSB occasion is to be transmitted. The impact of the gNB not transmitting SSB was captured in NR_unlic by extending the measurement delay, i.e. the UE is expected to identify that the SSB sample is missing and update the L3 filter only when the next valid SSB sample is available. In the case of NR_unlic the CCA failure is out of network control, since it is caused by the environment. On the other hand, the XR measurement skipping is under network control, and known to both UE and network by the DCI signaling. However, from RRM requirement perspective there is little difference on the measurement impact due to CCA failure or DCI-based skipping. Therefore, we propose that this is captured as an extension of the measurement delay, and that no changes in existing accuracy requirements is needed.

In Rel-16, RAN4 defined requirements for NR_unlic, where RRM measurement delay is extended due to CCA/LBT failures without impact to measurement accuracy.

Measurement skipping is reflected on RRM measurements via extension of measurement delay.

Reuse existing measurement accuracy requirements.

Additionally, we also have to consider the case where multiple frequency layers are measured. When multiple layers are configured, the current RAN4 assumption is that the UE is using a single searcher for measurements with gaps, and a single frequency layer is measured at every given measurement occasion. In some network deployments, the importance of certain frequencies is no equal in relation to others. Therefore, it is desirable that the measurement skipping is not affecting equally the measurement delay for all the frequency layers. This can be achieved by the UE prioritizing higher priority frequency layers to be measured first in case on measurement gap is skipped, while low priority gaps may be measured on remaining gap occasions and experience longer measurement delays.

RAN4 to define requirements such that the network can configure which frequency layers are measured with higher priority

Timeline for dynamic skipping of RRM measurement

As mentioned, RAN1 has agreed to adopt Alt 1-1 solution (i.e., explicit dynamic skipping indication using DCI signalling) where the gNB sent command(s) to the UE to skip RRM measurements occasions for the sake of having the UE prioritize PDCCH/PDSCH decoding and/or PUCCH/PUSCH transmissions. Alt 1-1 relies on the gNB sending a skipping indication command with DCI on PDCCH. For this solution, the UE will naturally need time for decoding the DCI with skipping indication command, and secondly to act on the command to actually skip the RRM measurement. The current 3GPP specs already include requirements for a UE to decode a DCI in 3GPP TS 38.214. For cases where the DCI include a scheduling grant (as is also being the default assumption in RAN1 for carrying skipping indication), the current specs define processing times that include both DCI and decoding PDCSH, or PUSCH preparation. We suggest relying on these existing processing values also for a UE to decode a DCI that carries skipping indication commands, i.e., the UE processing time for decoding a DCI with RRM measurement skipping indication should be the same as the currently defined processing requirements in 3GPP TS 38.214 for decoding of DCI and PDSCH decoding, or PUSCH preparation.

Once the UE has realized that it has received a RRM measurement skipping indication command, it needs time to react on this. This time could be rather short as the UE in principle only needs to continue normal operation (i.e. listening to DCI and receiving PDSCH / preparing PUSCH transmissions as per the received scheduling commands). Thus, it does, for instance, not involve any RF retuning or alike actions. In setting the UE requirements for acting on reception of a RRM measurement skipping indication command, we could get inspiration from alike UE behaviours of other commands carried by DCI that instruct the UE to take actions. Among others, those include a power control command received on DCI, a command for a UE to cancel an ongoing PUSCH transmission (aka as cancellation indication). Common for those cases is that the UE react rather fast and implement the action received on the DCI in less than a slot time.

Once the UE has realized that it has received a RRM measurement skipping indication command, the UE only needs to continue normal operation without executing complex RF retuning and gap activation procedures.

Additionally, we have provided simulations on annex A that show the benefits of reduced time between DCI indication and gap to be skipped. When comparing the results of Tsrt=1 ms and Tsrt=5 ms maximum interval between DCI and gap to be skipped, the results with Tsrt=1 ms could achieve improved XR capacity. In one example when comparing (Tp= 6ms;Tsrt+ 5ms) and (Tp=4 ms; Tsrt=1 ms) the same skipping ratio is observed. However, even though the skipping ratio is the same for those examples, the configuration with Tsrt=1 ms provided 1 additional user in relation to Tsrt=5 ms. That indicates that a shorter timeline will enable better decisions on which gaps to skip, which will translate in overall better user experience. This also shows that smaller Tsrt enables reduced skipping ratio for the same XR capacity.

Shorter time between DCI skipping command to measurement gap enables better XR capacity for the same skipping ratio.

We therefore suggest the following:

RAN4 shall define processing requirements for a UE to act on a DCI with RRM measurement skipping indication as a fraction of a slot.

Need/feasibility of UAI

In the RAN4 #112 meeting, the following issue was left for discussion in the agreed WF regarding UE assistance information:

# Topic #2: Need/feasibility of UE assistance information ## Issue 2-1: General on UAI <Way forward>: - Discuss if UAI is needed and which aspects to consider.

RAN1 sent LS to RAN4, requesting RAN4 to consider whether or not to introduce UE assistance information related to measurement occasions [4]. As part of this LS, the RAN1 agreement included FFS points related to measurement occasions, channel conditions, traffic, and UE mobility.

From RAN1 LS several other FFS points are included related to UE assistance information related to measurement occasions. Those include:

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

Additionally, it is also included in the RAN1 agreement the following related to gNB behaviour:

Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information.

The benefit of UAI related to measurement occasions can be potentially useful provided that measurement skipping can be performed without extension of measurement delay. As an example, if the UE indicates that X SSBs are required in  T ms, and the gNB sends skipping commands leaving at least X SSBs in the window of T ms, we would expect that this operation would not cause any extension of the measurement delay.

UAI only makes sense if UAI information is used to provide skipping implies on no extension of measurement delay and no impact on measurement accuracy.

As part of the FFS points, one many bullets relate to the number of SSBs that need to be available in a time window. The existing measurement delay and accuracy requirements are defined considering a minimum SNR. From that SNR a minimum number of samples for averaging is derived, which result on the accuracy and measurement delay. The measurement requirements also include delay regarding detection of new cells. The UE might know the SNR of cells that were previously measured, and assume a smaller number of samples is needed for those based on previous information. But to cells that are still not known it is impossible for the UE to estimate the SNR for cells that are not yet detected.

Therefore, for the UAI information related to measurement occasions, RAN4 has to provide analysis on the following questions:

- Can the UE provide information that guarantee that measurement delay and measurement accuracy is not impacted by measurement skipping?

- Are UEs capable of determining how many measurement occasions are needed (for RRM measurements) to achieve a given accuracy beforehand and for unknown cells?

- How is the XR capacity improved if the UE is reporting UAI related to measurement occasions?

- How is the measurement performance improved if the UE is reporting UAI related to measurement occasions?

Discuss the feasibility and accuracy of UE reporting UAI in relation to XR capacity;

Discuss the feasibility and accuracy of UE reporting UAI in relation to measurement performance.

Discuss the benefits of UE reporting UAI in relation to XR capacity;

Discuss the benefits of UE reporting UAI in relation to measurement performance.

When considering the feasibility of UAI related to measurement occasions, it should be ensured that measurement delay and measurement accuracy are not impacted when measurement skipping is influenced by UAI.

Any proposals for UAI related to measurement occasions should be justified by clear performance benefit either in terms of measurement accuracy and delay or XR capacity.

In relation to channel conditions, the RAN1 agreement includes an FFS point on s-MeasureConfig. The network can configure the UE with a search threshold (s-MeasureConfig, see below text from TS 38.331) for a UE in connected mode to enable reduction of the measurement effort.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

If the network has configured the UE with the s-MeasureConfig condition for conducting RRM measurements, it is important that the network knows if this condition has been fulfilled. This is needed for the network to know if the UE perform RRM measurements (and hence is subject to corresponding scheduling restrictions), or not. Once the s-MeasureConfig condition is fulfilled for a UE, it is suggested that the UE informs the network as it would then need to take corresponding scheduling restrictions into account.

For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network when the condition is met (i.e. defined RSRP threshold is exceeded).

Similarly, for a UE that no longer has its s-MeasureConfig condition fulfilled, and as a consequence may stop conducting RRM measurements, we also suggest that the UE informs the network. This is important for the network to know as it no longer needs to take scheduling restrictions from RRM measurements into accounts (as the UE has stopped such measurements). This leads to:

For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network that it is stopping doing RRM measurements because the s-MeasureConfig condition is no longer fulfilled.

The basic principle of the solution is as follows and shown in Figure 1. The UE is first configured with the s-MeasureConfig conditions, and asked to report back to the gNB when this condition is fulfilled (e.g. SS-RSRP< s-MeasureConfig) and it starts performing RRM measurements that causes scheduling restrictions. Similarly, when the condition is no longer fulfilled, and the UE is not required to do RRM measurements on neighbouring cells, it shall also report that to the gNB. Thereby, the gNB always knows when the UE is conducting RRM measurements, and hence knows when it can freely schedule the UE, and when it needs to respect certain scheduling restrictions.

Figure 1 Illustration of case where the UE is configured to report when it starts, or stops, conducting RRM measurements as per the s-MeasureConfig condition that results in scheduling restrictions.

We note that the proposals related to UE to network reporting of s-MeasureConfig conditions, has some relations to the already defined RRC Events A1 (Serving cell becomes better than threshold) and A2 (Serving cell becomes worse than threshold). However, while A1/A2 events solely links to serving cell RSRP power levels as compared to a threshold, what we suggest in the listed Proposals links also to UE behaviour of performing RRM measurements (and is related to scheduling restrictions).

LS from RAN1

In RAN1#118 it was decided that DCI-based indication of measurement skipping would be considered as working assumption by RAN1. As part of the conclusion, RAN1 sent an LS to RAN4 and tasked RAN4 to inform RAN1 if there are issues related to that decision [4].

From RAN4 point of view, we can cite at least few cases where similar requirements were made, where UEs had to adapt measurements in a dynamic manner.

In Rel 16 RAN4 has defined requirements for NR_unlicensed. In unlicensed bands all transmissions are subject to CCA failures, which means that some SMTC occasions were not available to be measured by the UEs. The availability of the SMTC occasions was considered as part of the measurement delay requirements.

In Rel 17 RAN4 has defined requirements for Pre-configured measurement gaps. In one example, PreMGs can be activated/deactivated for example by DCI containing BWP switch command. In that case, one measurement gap patterns may be activated or deactivated dynamically, and the UE has to adapt measurement procedure accordingly.

Therefore, the important aspects of Alt 1-1 from RAN4 perspective are

- the definition of the measurement delay requirements, and

- the timeline for DCI-based measurement skipping

RAN4 already defined requirements for features where dynamic adaptation of measurements were necessary, e.g. NR-U and Pre-configured gaps.

Alt 1-1 is feasible from RAN4 perspective and RAN4 is working on the related requirements on timeline.

Conclusion

In this paper we have presented Nokia’s views on the RAN4 RRM requirements related XR ph3. As part of this discussion, the following Observations and Proposals were made:

Observation 1: FR2-2 measurement delay requirements are 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS.

Proposal 1: Measurement skipping apply for FR1 and FR2-1 measurements

Proposal 2: RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Application of the requirements to other types of gap should be dealt with in a second stage.

Proposal 3: Once the impact on requirements for measurement skipping for Type 1 MG has been agreed, Type 2 MG, NCSG and MG for positioning should be considered as 2nd priority.

Proposal 4: MUSIM,  Preconfigured, and Rel-18 concurrent gaps are not considered for Rel 19 XR ph3 enhancements.

Proposal 5: Measurement skipping is not applicable for MUSIM, Preconfigured, and Rel-18 Concurrent gaps measurement gap based on the Rel 19 XR DCI skipping indication.

Observation 2: RAN1 is defining enhancements for skipping measurements that cause scheduling restrictions which are agnostic to the types of measurement gaps or scheduling restrictions.

Observation 3: Intra-frequency measurements always cause scheduling restrictions in FR2, around SSB symbols to measure.

Observation 4: Interruptions will cause scheduling problems for latency sensitive traffic.

Observation 5: Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location in time.

Observation 6: Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements and restrictions.

Proposal 6: Define requirements for measurement skipping only for measurement with gaps.

Observation 7: In Rel-16, RAN4 defined requirements for NR_unlic, where RRM measurement delay is extended due to CCA/LBT failures without impact to measurement accuracy.

Proposal 7: Measurement skipping is reflected on RRM measurements via extension of measurement delay.

Proposal 8: Reuse existing measurement accuracy requirements.

Proposal 9: RAN4 to define requirements such that the network can configure which frequency layers are measured with higher priority

Observation 8: Once the UE has realized that it has received a RRM measurement skipping indication command, the UE only needs to continue normal operation without executing complex RF retuning and gap activation procedures.

Observation 9: Shorter time between DCI skipping command to measurement gap enables better XR capacity for the same skipping ratio.

Proposal 10: RAN4 shall define processing requirements for a UE to act on a DCI with RRM measurement skipping indication as a fraction of a slot.

Observation 10: UAI only makes sense if UAI information is used to provide skipping implies on no extension of measurement delay and no impact on measurement accuracy.

Observation 11: Discuss the feasibility and accuracy of UE reporting UAI in relation to XR capacity;

Observation 12: Discuss the feasibility and accuracy of UE reporting UAI in relation to measurement performance.

Observation 13: Discuss the benefits of UE reporting UAI in relation to XR capacity;

Observation 14: Discuss the benefits of UE reporting UAI in relation to measurement performance.

Proposal 11: When considering the feasibility of UAI related to measurement occasions, it should be ensured that measurement delay and measurement accuracy are not impacted when measurement skipping is influenced by UAI.

Proposal 12: Any proposals for UAI related to measurement occasions should be justified by clear performance benefit either in terms of measurement accuracy and delay or XR capacity.

Proposal 13: For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network when the condition is met (i.e. defined RSRP threshold is exceeded).

Proposal 14: For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network that it is stopping doing RRM measurements because the s-MeasureConfig condition is no longer fulfilled.

Observation 15: RAN4 already defined requirements for features where dynamic adaptation of measurements were necessary, e.g. NR-U and Pre-configured gaps.

Proposal 15: Alt 1-1 is feasible from RAN4 perspective and RAN4 is working on the related requirements on timeline.

- References

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia, RAN4 #112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024

- RP-241771, Revised WID: XR (eXtended Reality) for NR Phase 3,

- R1-2405736, LS on UE assistance information, RAN4 #112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024

- R4-2414910, LS on gaps/restrictions that are caused by RRM measurements, RAN4 #112-bis, Hefei, Anhui, China, 14th – 18th October, 2024

- R4-2413314, Discussion on RRM requirements for XR phase 3, Nokia, RAN4 #112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024

,

- Annex A: System-level simulation results

## Evaluation assumptions

In this subsection, we present the results of system-level simulations (SLS) of the skipping policies discussed in RAN1. Simulations have been performed in the Dense Urban FR1 scenario with parameters configured according to TR38.835. On top of the parameters defined in TR38.835, we configured measurement gaps considering two configurations: (40,6), where the interval is 40ms and the gap length is 6ms. In our evaluation, we consider the following KPIs:

XR capacity: system load CP of policy P in terms of number of UEs/cell (N) when the ratio of satisfied UEs/cell RP is above or equal to 90% (CP=maxN:RP≥90%. A UE is marked as satisfied if 99% of XR frames are delivered within the packet delay budget (PDB) of 10ms (for VR/AR) or 15ms (for CG). The KPI is reported by computing the average across all UEs, all cells, and all SLS drops.

Skipping ratio: fraction of skipped measurement opportunities with respect to the total number of measurement opportunities defined by the measurement gaps configuration. The KPI is reported by computing the average across all UEs, all cells, and all SLS drops.

XR capacity loss: loss of XR capacity measured considering the system load when MG is disabled.

XR capacity gain: gain of XR capacity measured considering the system load when MG and skipping policies are enabled.

We evaluate the following skipping policies (alternatives):

Explicit DCI indication (Alt. 1-1): the logic of the skipping decision is implemented by the network and the skipping decision is dynamically delivered with dedicated DCI signaling to the UE.

Pattern-based (Alt 3): the network communicates the pattern of the XR traffic to the UE e.g. via RRC signalling. Both the UE and the network determine whether to skip using both the pattern of XR traffic and the pattern of measurement occasions communicated by the network.

Additionally, we compare the skipping policies against the two following baseline configurations:

No MGs: measurement gaps are disabled. This corresponds to the ideal case where scheduling restrictions are not delaying scheduling decisions. This configuration provides an upper bound in terms of XR capacity.

MG enabled: measurement gaps and corresponding scheduling restrictions are enabled. This configuration provides a lower bound in terms of XR capacity.

Genie: scheduling restrictions are skipped every time skipping is needed to deliver a XR frame. This policy assumes perfect knowledge of traffic and channel status and skipping is triggered even when a transmission or retransmission happens during a measurement occasion. The skipping decision is delivered to the UE using Alt-1-1. While being ideal (traffic might arrives when measurement occasion has already started and prediction missed this case), this policy provides an upper bound in terms of skipping ratio.

## Skipping policy based on explicit DCI indication (Alt-1-1)

Figure 2 and Figure 3 illustrate the timeline of the “Explicit DCI indication” (Alt 1-1) policy to trigger the skipping of the next measurement occasion. In the figure, tMG represents the beginning of the measurement occasion, when the UE becomes unavailable for scheduling, TMG is the gap length of the measurement occasion, TSRT is the UE timeline (i.e., the retuning time of the RF interface). Our Alt-1-1 skipping policy combines two skipping strategies: buffer-based and prediction-based, respectively. As illustrated in the figures, these two policies make use of two windows to decide whether to skip the next measurement occasion: TB for buffer-based and TP for prediction-based. More specifically, the buffer-based policy uses the time window TB as follows: if the scheduler buffer for a UE is not empty during the time window TB, the network indicates to skip the next measurement occasion to the UE. Skipping is communicated using a dedicated indication (e.g., a bit in the DCI format). This flexibility proves essential in the presence of the UE timeline constraint (i.e., the latency due to the re-tuning of RF interface) especially when the XR frames get fragmented, and the fragments enter the buffer at different instants. For example, as illustrated in Figure 2, the buffer-based policy can use the first DCI command to indicate the skipping of the next measurement occasion and transmit the first TB as soon as resources are available.

If the XR frame (or PDU Set) enters the buffer outside the time window TB, the network uses the prediction-based policy to decide whether to skip the next measurement occasion as follows: if the next XR frame for a UE is predicted/expected to enter the scheduler buffer during the time window TP, the network indicates to skip the next measurement occasion to the UE. As illustrated in Figure 3, when the XR frame k-1 enters the buffer of the gNB scheduler, the next measurement occasion is still far ahead in the future. However, gNB can estimate the next XR frame k is expected to collide with the time window TP, which includes the next measurement occasion, and trigger the skipping indication. In the figure, we illustrate the skipping command as a separate message for the sake of explanation to show that the skipping can be delivered to the UE anytime between the arrival of frame k-1 and the UE re-tuning deadline. However, the skipping indication can be delivered with the first or second scheduling DCI in the example of Figure 3.

We observe that setting the time window TP for triggering skipping equal to the jitter of the traffic may be unnecessary. As illustrated in Figure 3, the XR frame k is expected close to the end of the MG and avoiding skipping in this case would slightly delay the transmission of the XR frame. Therefore, the window of the prediction-based skipping policy can be optimized to minimize the skipping ratio.

Figure 2 Explicit DCI indication (Alt 1-1) based on buffer status. Skipping is triggered by the status of the buffer during the time window TB and communicated to the UE by a dedicated skipping indication.

Figure 3 Explicit DCI indication (Alt 1-1) based on traffic prediction. Skipping is triggered by a prediction of the next frame colliding with the measurement occasion and communicated by an explicit “skip indication” in the DCI format.

## Skipping policy based on pattern (Alt-3)

Pattern-based policy is based on three parameters as we discussed in Section 2.2. In particular, the pattern is defined using the following parameters, as depicted in Figure 4:

PS: Period of the skipping pattern/cycle.

LS: Skipping duration/length within a cycle where scheduling is always prioritized.

OS: Offset of the skipping pattern indicating when skipping duration starts (i.e., when scheduling is always prioritized within a cycle).

The skipping pattern defines a cycle that repeats every PS time units where OS and LS indicate when and for how long scheduling is always prioritized over RRM measurements within a cycle. This means that MG is skipped every time the gap length (TMG) overlaps with the skipping duration (LS).

As it was observed for Alt 1-1, setting a skipping duration/length equal to the jitter of the traffic may be unnecessary when the traffic is expected after the beginning of the MG. As illustrated in Figure 4, both depicted MGs (on the bottom of the figure) are skipped since they overlap with the skip pattern (on the top of the figure). However, the first XR frame (first skip length) is expected after the beginning of first MG, whereas the third XR frame (third skip length) is expected before the second MG. Only this second case is critical from scheduling perspective, whereas in the first case skipping can be avoided resulting in a small extra delay. However, the skip pattern is static and must be configured to cover both cases. Therefore, the optimization prevents the optimization of the skipping duration/length LS to minimize the skipping ratio.

Observation 6: With configured pattern the skipping duration/length cannot be dynamically adjusted. This prevents the optimization of the skipping duration/length LS to minimize the skipping ratio.

The three parameters of the skipping pattern are indicated as follows {PS, LS, OS} in plots and tables.

Figure 4 Pattern (Alt 3) alternative for skipping MG. Skipping is triggered if skipping length LS and MG length TMG overlaps.

## Main parameters and results

In our SLS, we evaluate several configurations including the retuning time TSRT and parameters specific to the alternative solution. Table 2 in Appendix B summarizes the values of the parameters used in our analysis.

Figure 5 shows the XR capacity versus the skipping ratio when TSRT=1 or 5 ms, while Figure 6 shows the ratio of satisfied UEs, which is measured when the cell load is equal to 7 UEs/cell (i.e., the capacity when MGs are not configured), versus the skipping ratio when TSRT=1 or 5 ms. It can be observed that the larger the skipping ratio is, the larger the capacity is. For example, with TSRT=1ms the Alt 1-1 skipping policy can be configured to achieve the same number of satisfied XR UEs as without scheduling restrictions (black point w/o MG), which corresponds to skipping all gaps. It can be further observed that the parameters of the Alt 1-1 skipping policy can be optimized to maximize the number of satisfied XR UEs and minimize the skipping ratio. For example, Alt 1-1 with TSRT=1 (magenta cross) can be configured to have the same skipping ratio of Alt 1-1 when TSRT=5ms (green square), namely 44%. However, a smaller retuning time TSRT  allows higher flexibility to Alt 1-1 since skip decision can be communicated close to the beginning of the MG. This additional flexibility can be used to minimize the time window TP used by the prediction-based skipping policy and, hence reducing the skipping ratio.

Figure 5 – XR capacity vs skipping ratio for Alt-1-1 when TSRT=1 or 5 ms.

Figure 6 – Ratio of satisfied UEs vs skipping ratio for Alt-1-1 when TSRT=1 or 5 ms. Ratio of satisfied UEs has been measured when cell load is equal to 7 UEs/cell (i.e., the XR capacity of ‘w/o MG’ when PDB=10ms).

Table 1 below illustrates the capacity for different PDBs (10ms and 15ms as per TR38.835) and the skipping ratio of the alternatives we have considered in our SLSs. We also evaluated the same KPIs with legacy MGs (label “MGs enabled”) and without any MG configured (label “No MGs”).

In the presence of scheduling restrictions, results show that the system should skip approximately 70% of measurement occasions to fully recover the same XR capacity as when scheduling restrictions are disabled. When using Explicit DCI indication (Alt 1-1) the system experiences the largest capacity gain (in some configurations up to 75%). Instead, pattern-based alternatives (Alt. 3) result in lower capacity gain (around 25%) than Explicit DCI (Alt 1-1). Skipping ratio for Alt. 1-1 can be reduced down to 26% (for Alt. 3 it is around 43%) while still maintaining higher capacity gain than Alt. 3 (50% for Alt. 1-1 vs 25% for Alt. 3).

More specifically, in our SLS evaluation summarized in Table 1 for measurement gap configuration (40, 6) ms, we show that Explicit DCI indication (Alt 1-1) achieves the best performance among all alternatives. For example, Explicit DCI indication with TB=3ms and TP=4ms achieves XR capacity of 7 UEs/cell and capacity gain of 43% with PDB=10ms. Even with large UE skipping retuning time (TSRT=5 or 6 ms) the capacity gain can be kept around 50% and the skipping ratio can be minimized down to 26%. This is obtained by combining the prediction-based and the buffer-based skipping strategies as well as the flexibility provided by the dedicated DCI skipping indication.

Instead, pattern-based policy achieves smaller capacity and lower capacity gain than Explicit DCI even with comparable skipping ratio. For example, Pattern-based policies with {PS,LS,OS}={17,8,12}ms {PS,LS,OS}={17,8,RRC}ms archive XR capacity of 5 UEs/cell with PDB=10ms even if skipping ratio is 43% and comparable to Explicit DCI. Additionally, reconfiguring the offset using RRC reconfiguration helps in reducing variability of capacity and skipping ratio: as it can be observed in the table both {16,8,RRC} and {17,8,RRC} achieve very similar performance. However, Alt 3 (pattern) still underperforms in capacity compared to Alt 1-1 (Explicit DCI) in the worst case and with comparable skipping ratio.

Based on the above, we proposed in afore sections that RAN1 should agree to support Alt. 1-1 explicit indication via DCI.

Table 1 – Results of SLS in DU deployment with (MG interval, MG gap length) equal to (40,6) ms. N/A means Not Applicable.

| Scheme | Alternative | Retune Time TSRT [ms] | Capacity (PDB: 10ms) [#UEs/cell] | Capacity (PDB: 15ms) [#UEs/cell] | Capacity Loss (PDB: 10ms) | Capacity Loss (PDB: 15ms) | Capacity Gain (PDB: 10ms) | Capacity Gain (PDB: 15ms) | Skipping ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No MGs | N/A | N/A | 7 | 9 | N/A | N/A | N/A | N/A | N/A |
| MGs enabled (40,6) | N/A | N/A | 4 | 7 | 43% | 22% | N/A | N/A | N/A |
| Genie | N/A | N/A | 7 | 9 | N/A | N/A | 75% | 29% | 72% |
| Explicit DCI TB=3ms, TP=6ms | Alt 1-1 | 3 | 7 | 8 | 0% | 11% | 75% | 14% | 55% |
| Explicit DCI TB=2ms, TP=6ms | Alt 1-1 | 4 | 6 | 8 | 14% | 11% | 50% | 14% | 50% |
| Explicit DCI TB=1ms, TP=6ms | Alt 1-1 | 5 | 6 | 8 | 14% | 11% | 50% | 14% | 44% |
| Explicit DCI TB=0ms, TP=6ms | Alt 1-1 | {1;6} | 6 | 8 | 14% | 11% | 50% | 14% | 38% |
| Explicit DCI TB=3ms, TP=4ms | Alt 1-1 | 1 | 7 | 8 | 0% | 11% | 50% | 14% | 44% |
| Explicit DCI TB=2ms, TP=4ms | Alt 1-1 | 2 | 6 | 8 | 14% | 11% | 50% | 14% | 38% |
| Explicit DCI TB=1ms, TP=4ms | Alt 1-1 | 3 | 6 | 8 | 14% | 11% | 50% | 14% | 32% |
| Explicit DCI TB=0ms, TP=4ms | Alt 1-1 | {1-6} | 6 | 8 | 14% | 11% | 50% | 14% | 26% |
| Pattern {16,8,12}ms | Alt 3 | N/A | 5 | 7 | 29% | 22% | 25% | 0% | 50% |
| Pattern {17,8,12}ms | Alt 3 | N/A | 5 | 7 | 29% | 22% | 25% | 0% | 43% |
| Pattern {16,8, 1st PDU} | Alt 3 | N/A | 5 | 7 | 29% | 22% | 25% | 0% | 45% |
| Pattern {17,8,1st PDU} | Alt 3 | N/A | 5 | 7 | 29% | 22% | 25% | 0% | 43% |

- Annex B: System-level simulation parameters

This appendix illustrates the results obtained considering all configurations discussed in Section 3. Results are clustered by alternative as follows:

Baseline: it includes the results of legacy with MG disabled, legacy with MG enabled, and Genie. Results are illustrated in Table 3.

Alt 1-1: it includes the results of Explicit DCI indication solution where the network uses both buffer-based and prediction-based policies to decide whether to skip next measurement opportunity. Curves with the same value of prediction-based window TP and different values of UE timeline constraint (skipping retuning time) TSRT are depicted in the same plot. The buffer-based window is equal to the difference between the prediction-based window and the UE timeline constraint: TB=TP-TSRT. Results are illustrated in Table 4.

Alt 3: it includes the results of Pattern-based solution indication where the patter is configured semi-statically by the network. Two set of plots are illustrated: with fixed offset and with offset set as first PDU transmission. Results are illustrated in Table 5.

Table 2 below summarizes the values of the parameters used in our analysis.

Table 2 – Parameters used in SLS. Note that {X}+ means max{0, X}.

| Parameter | Description | Value(s) |
| --- | --- | --- |
| TMGI | Measurement gap interval | 40 ms |
| TMG | Measurement gap length | 6 ms |
| TSRT | Skipping retuning time | {1, 2, 3, 4, 5, 6} ms |
| TB | Buffer-based window for Explicit DCI policy (Alt 1-1) | {6 – TSRT, 4– TSRT}+ ms: {5 – TSRT}+ ={4,3,2,1} ms {4 – TSRT}+ ={2,1,0} ms |
| TP | Prediction-based window for Explicit DCI policy (Alt 1-1) | {6, 5, 4} ms |
| PS | Period of the skipping cycle for pattern-based policy (Alt 3) | {16, 17} ms |
| LS | Duration of skipping within the cycle for pattern-based policy (Alt 3) | 8 ms |
| OS | Start of skipping within the cycle for pattern-based policy (Alt 3) | 12 ms RRC (learned from PDU transmission) |

We observe that the traffic pattern of video stream for XR applications is defined in TR38.835 as follows:

tk=k∙1fps+J

Where k≥1 is the index of the XR frame, fps is the frame rate (frames per second), and J is the jitter distributed as a Truncated Gaussian distribution in [-4; 4]ms. With this model and assuming no offset in frame generation, the first frame (k=1) is expected to enter the gNB buffer in the time interval [12.67; 20.67]ms. Therefore, for pattern-based policy (Alt 3) we evaluate as offsets the following options:

OS=12: it matches the first frame arrival (labels {16,8,12} and {17,8,12});

OS=RRC reconfiguration: offset is reconfigured once after first packet transmission (labels {16,8,1st PDU} and {17,8,1st PDU}).

Table 3 – Results of baseline schemes (legacy without MG, legacy with MG, and Genie). Note that for these schemes the UE timeline constraint does not affect performance.

Table 4 –Results of Alt 1-1 considering different configurations of the skipping policy (i.e., TB and TP) and UE timeline constraints. The parameter TB is equal to TB=TP-TSRT. From top to bottom: TP from 4 to 6 ms.

Table 5 – Results of Alt 3 (Pattern) considering different configurations of the skipping pattern (i.e., PS , LS, and OS). The duration/length of the window LS where transmission/reception is prioritized over RRM measurement is LS = 8 ms in all simulations (i.e., it is equal to the jitter). From top to bottom: PS from 16 to 17 ms.