3GPP TSG-RAN WG4 Meeting #112bis	R4-2416471

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.20.3

Source: 	Qualcomm Incorporated

Title: 		RRM core requirements for SBFD

Document for:	Discussion

# Introduction

For release 19, a new work item on Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been agreed where the main objective is to specify the sub-band full duplex operation. In addition to that, enhancements to CLI handling scheme is another WID [1] objective that is expected to have impact on RRM requirements:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# L1 based UE-to-UE CLI measurement requirements

During last RAN4 meeting, it was agreed to define RRM requirements for L1 based UE-to-UE CLI measurement and reporting [2]. RAN1 has agreed to define L1-SRS-RSRP and L1-CLI-RSSI as two new report quantities and is currently discussing the following methods to specify measurement resources:

During the last meeting, RAN1 agreed to support Method#1 and Method#2 for L1-CLI-RSSI and L1-SRS-RSRP respectively, while Method#3 is still under discussion. So, RAN4 can start the initial discussion on RRM requirements specification based on Method#1 and Method#2.

Observation 1: RAN1 agreement: For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- FFS: Method#3: UE measures RSSI within UL subband

Proposal 1: RAN4 to specify RRM requirements for L1-CLI-RSSI and L1-SRS-RSRP based on Method#1 (UE measures RSSI within DL subband) and Method#2 (UE measures RSRP of aggressor UE within UL subband), respectively. Additional methods can be considered based on further RAN1 agreements.

RAN4 can consider specifying one or more of the following requirements for new report quantities:

- Measurement capability

- Measurement side conditions

- Measurement period

- Measurement accuracies

- Measurement restrictions

- Scheduling restrictions

For measurement capability, similar to R16 CLI requirements, the L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE, so that the UE is not required to perform any RF retuning for such measurements.

Proposal 2: The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

Another aspect that was brought up during the last meeting was related to the time offset between DL timing and SRS arrival timing. The CLI measurement requirements should ideally be based on UL transmit timing, however, the interfering and the victim UE may have different UL timings because of the propagation delay to corresponding gNbs and UE transmit timing errors. This was extensively discussed during Rel-16 and it was agreed to specify a minimum timing offset value and is up-to UE implementation.

TS 38.133, Clause 9.7.1 - “When the UE measures SRS-RSRP and CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset”.

Since the underlying timing related challenges are still the same, it’s best to re-use Rel-16 assumptions for L1-CLI-RSSI and L1-SRS-RSRP measurements as well.

Proposal 3: For timing offset between L1 CLI measurement resources and DL timing, reuse the same assumptions from Rel-16, and specify the following:

- When the UE measures L1-SRS-RSRP and L1-CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset

Scheduling restrictions requirements need to be defined but they depend on the timing offset between the DL timing and SRS arrival timing. So, we think RAN4 can focus first on finalizing the timing offset requirements and then discuss the scheduling restrictions for better use of the RAN4 meeting time.

Proposal 4: RAN4 to discuss scheduling restriction requirements after the timing offset between DL timing and SRS arrival timing is finalized.

Some companies also brought up whether we need to define measurement restrictions for L1 CLI measurements. In our understanding, Rel-16 CLI measurements were considered as best effort measurements and UE mobility should not be affected with such measurements. With that assumption, RAN4 didn’t define any measurement restrictions or rules to handle the scenario when the CLI measurement resources partially or fully overlap with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. We think similar principles can be followed for L1 CLI measurements as well. Furthermore, there would be a huge specification impact of considering all collision scenarios and it’ll be hard to complete it with the limited WI TUs allocated during this release.

Proposal 5: If the L1-SRS-RSRP and/or L1-CLI-RSSI resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, corresponding measurement delay requirements are not specified.

For QCL assumptions, Rx beam assumptions and the number of L1 CLI resources to be monitored, it’s best to wait for more agreements from RAN1.

Moving on to the side conditions and corresponding measurement delays and accuracy requirements, we think the R16 side conditions such as SRS Es/Iot, SRS configuration etc., can be assumed as SBFD scenario is not much different from R16 scenario from CLI measurement perspective.

Proposal 6: Reuse R16 assumptions on side conditions such as SRS Es/Iot, SRS configuration etc., for L1 CLI measurements.

For measurement delays, we would like to highlight that one of the main motivation of introducing L1 CLI measurements is to shorten the overall delay in measurement reporting. So, RAN4 requirements should strive to not increase the measurement delays. Now, R16 CLI RSSI measurements are reported as single RSSI sample for each measurement resource configured for CLI-RSSI measurement within the configured measurement periodicity. We think this could directly apply to L1-CLI-RSSI measurements as well and no need to consider multiple samples for L1 RSSI measurements.

Proposal 7: Reuse the R16 CLI-RSSI measurement delay requirements for L1-CLI-RSSI.

R16 SRS-RSRP, on the other hand, is defined with 3 samples periodicity. Since, RAN1 has agreed to use CSI-RS based reporting framework for L1 CLI measurements, and to facilitate faster reporting of CLI measurements using the L1 reporting framework, we think RAN4 can define the measurement delay for aperiodic reporting, at-least, based on a single sample.

Proposal 8: RAN4 to define the measurement delay for aperiodic reporting of L1-SRS-RSRP atleast based on a single sample.

# SBFD operation aspects

From SBFD operation perspective, RAN4 can start identifying impact to existing requirements and if any new requirements are needed to support RRM operation. Since RAN1 agreed that SSBs shall be preserved, we don’t see much impact to SSB based measurements. However, CSI-RS based measurements may be impacted because of reduced number of PRBs either in a single sub-band or across two sub-bands. Filtering CSI-RS measurements across sub-bands may be challenging, so performing measurements and reporting per sub-band is desirable.

Proposal 9: RAN4 to start identifying impact of SBFD operation to RRM requirements such as CSI-RS based measurements.

Another issue that was brought up during the last RAN4 meeting was to discuss whether to clarify the term “UL slot” in MG and BWP switch requirements when SBFD operation is enabled. We think it’s a valid issue as the UE should be able to transmit in the SBFD slot, if the slot after the BWP switching is an SBFD slot. So, we support to clarify this in the specification

Proposal 10: Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification.

# Conclusion

Proposal 1: RAN4 to specify RRM requirements for L1-CLI-RSSI and L1-SRS-RSRP based on Method#1 (UE measures RSSI within DL subband) and Method#2 (UE measures RSRP of aggressor UE within UL subband), respectively. Additional methods can be considered based on further RAN1 agreements.

Proposal 2: The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

Proposal 3: For timing offset between L1 CLI measurement resources and DL timing, reuse the same assumptions from Rel-16, and specify the following:

- When the UE measures L1-SRS-RSRP and L1-CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset

Proposal 4: RAN4 to discuss scheduling restriction requirements after the timing offset between DL timing and SRS arrival timing is finalized.

Proposal 5: If the L1-SRS-RSRP and/or L1-CLI-RSSI resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, corresponding measurement delay requirements are not specified.

Proposal 6: Reuse R16 assumptions on side conditions such as SRS Es/Iot, SRS configuration etc., for L1 CLI measurements.

Proposal 7: Reuse the R16 CLI-RSSI measurement delay requirements for L1-CLI-RSSI.

Proposal 8: RAN4 to define the measurement delay for aperiodic reporting of L1-SRS-RSRP atleast based on a single sample.

Proposal 9: RAN4 to start identifying impact of SBFD operation to RRM requirements such as CSI-RS based measurements.

Proposal 10: Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification.

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104

[2] R4-2413900, “WF on SBFD RRM”, Huawei, RAN4#112