3GPP TSG-RAN WG4 Meeting #112-bis									                    R4-2416474

Hefei, China, October 14 – 18, 2024

Agenda item:			6.1.1.3.3

Source:	Ericsson

Title:	On remaining DeltaT_RxSRS requirement for 6Rx UE RF

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” WI [1]. One of the three main parts of the WI is to specify 6Rx UE RF requirements for both handheld and FWA UEs, with the following objectives:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this contribution, we focus on the second-to-last bullet point, which is to specify the requirements to support SRS antenna switching. As we can see in the objectives, ‘t1r6’, ‘t2r6’ and ‘t3r6’ SRS antenna switching (AS) capabilities are included in the WID. Currently, ‘t3r6’ AS capability is not specified in TS38.306, so in the revised WID it was agreed for “RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability”, where the work in RAN1 will not start before RAN1#118bis meeting (Oct. 2024). Hence, in this contribution we will still only focus on ‘t1r6’, ‘t2r6’ as well as on ‘t1r6-t2r6’ SRS AS capabilities and propose the corresponding ΔTRxSRS requirements.

# Discussion

In the approved WF from RAN4#112 [2], the following was captured on the SRS AS requirements topic:

Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Way forward: Further discuss the following options.

Option 1: Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Option 2: Consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Issue 2-1-2: Whether to consider separate ∆TRxSRS values when the device is power class 2 in the band and ΔPPowerClass = 0dB and not indicating Tx diversity capability

Agreement: Have additional 3dB for power class 2 when ΔPPowerClass = 0dB and not indicating Tx diversity capability.

Issue 2-2-1: Proposed ∆TRxSRS values

Way Forward: RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers.

-	Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n41, n77, n78 | 4.0 | 3.5 | 4.5 |

-	Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79, n104 | 5.4 | 4.7 | 5.9 |

-	Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79 | 5.4 | 4.7 | 5.9 |
|  | Band n104 | 5.7 | 5.0 | 6.3 |

ΔTRxSRS is a maximum allowed relaxation applied to the lower bound of the maximum configured transmitted power PCMAX,f,c due to the additional insertion loss of a given diversity branch relative to the main branch. The main “contributors” to the insertion loss are the antenna switches, filters, as well as PCB trace/RF cable routing located between the PA which sounds the given SRS port and the antenna port/connector.

ΔTRxSRS is measured at the antenna connector, and it is important not to have too loose margins in order to limit degradation of DL-MIMO performance, i.e. to have an adequate reciprocal CSI-RS estimation, especially since the gNB measures the resulting radiated SRS power per SRS port(s) which include different antenna element gains (in a typical implementation, antenna elements on diversity paths have a poorer quality compared with the antenna elements on the main path).

As it can be seen from the approved WF, ΔTRxSRS requirement has been agreed for ‘t1r6’, ‘t2r6’ and ‘t1r6-t2r6’ AS capabilities for bands n41, n77 and n78 (for PC3). The values are 4.0dB, 3.5dB and 4.5dB, respectively. The remaining work to be done on this topic is to specify ΔTRxSRS requirement for the same AS capabilities for bands n79 and n104.

## ‘t1r6’, ‘t2r6’ and ‘t1r6-t2r6’ SRS AS capabilities for bands n79 and n104

For bands whose FUL_high is higher than the FUL_low of n79, up to Rel-18 the practice has been to adopt 1.5dB higher value than for bands whose FUL_high is lower than the FUL_low of n79. This includes both bands n79 and n104, and applies for all xTyR AS capabilities which include 2Rx, 4Rx and 8Rx, as well as for all UE types.

Observation 1: For bands whose FUL_high is higher than the FUL_low of n79, up to Rel-18 the practice has been to adopt 1.5dB higher value than for bands whose FUL_high is lower than the FUL_low of n79, which includes both bands n79 and n104 and applies for 2Rx, 4Rx and 8Rx AS capabilities.

According to the analysis made by different companies and the averaging of the submitted results to the previous meeting [2], 1.5dB is too relaxed for band n79 as the average difference between the two groups of frequency bands is around 1.4dB for ‘t1r6’ and ‘t1r6-t2r6’ and 1.2dB for ‘t2r6’ AS capability. On the other hand, for band n104 the average difference between the two groups is in the range 1.5dB – 1.8dB for the same AS capabilities.

Observation 2: According to the analysis made by different companies and the averaging of the submitted results to the previous meeting, 1.5dB is slightly too relaxed for band n79 and slightly too tight for band n104.

We recognize that the insertion loss of different components increases with the frequency, but given the averaged results by companies and that (in our understanding) the currently specified ΔTRxSRS values already have a certain margin, it would be meaningful to adopt the same value for both n79 and n104 bands, which is, for the given AS capability, 1.5dB larger than the agreed value for bands whose FUL_high is lower than the FUL_low of n79. In addition, as explained above, it would be consistent with the previous Releases, which cover AS capabilities for 2Rx, 4Rx and 8Rx, and it would be more convenient from the specification’s simplicity perspective.

Otherwise, for an outside reader not familiar with the discussions which took place for the specification of ΔTRxSRS values for 6Rx AS capabilities (‘t1r6’, ‘t2r6’ and ‘t1r6-t2r6’), it would be impossible to figure out why the difference between the bands whose FUL_high is higher than the FUL_low of n79 and the bands whose FUL_high is lower than the FUL_low of n79 has one value for 2Rx, 4Rx and 8Rx AS capabilities and another value for 6Rx AS capabilities.

Observation 3: If a value other than “1.5dB larger” would be specified for n79 and n104, for an outside reader not familiar with the ongoing discussion it would be impossible to figure out why there is a difference between 2Rx, 4Rx and 8Rx AS capabilities on side and 6Rx AS capabilities on another.

Observation 4: Given more relevant and important issues, such as the lack of SRS conformance testing and the lack of reporting of the SRS IL imbalance, in our view the differences of the order of 0.2 – 0.3dB are not worth making the important inconsistencies in the specifications for the AS requirements and their (even further) increased complexity.

Proposal 1: For bands n79 and n104, and for ‘t1r6’, ‘t2r6’ and ‘t1r6-t2r6’ AS capabilities, specify ΔTRxSRS to be 1.5dB larger than the agreed values for bands n41, n77 and n78, i.e. 5.5dB, 5.0dB and 6.0dB, respectively, which is roughly in line with the submitted results by the companies and consistent with previous Releases and 2Rx, 4Rx and 8Rx AS capabilities.

# Conclusion

Based on the discussion in the previous sections we have made the following observations and the proposal:

Observation 1: For bands whose FUL_high is higher than the FUL_low of n79, up to Rel-18 the practice has been to adopt 1.5dB higher value than for bands whose FUL_high is lower than the FUL_low of n79, which includes both bands n79 and n104 and applies for 2Rx, 4Rx and 8Rx AS capabilities.

Observation 2: According to the analysis made by different companies and the averaging of the submitted results to the previous meeting, 1.5dB is slightly too relaxed for band n79 and slightly too tight for band n104.

Observation 3: If a value other than “1.5dB larger” would be specified for n79 and n104, for an outside reader not familiar with the ongoing discussion it would be impossible to figure out why there is a difference between 2Rx, 4Rx and 8Rx AS capabilities on side and 6Rx AS capabilities on another.

Observation 4: Given more relevant and important issues, such as the lack of SRS conformance testing and the lack of reporting of the SRS IL imbalance, in our view the differences of the order of 0.2 – 0.3dB are not worth making the important inconsistencies in the specifications for the AS requirements and their (even further) increased complexity.

Proposal 1: For bands n79 and n104, and for ‘t1r6’, ‘t2r6’ and ‘t1r6-t2r6’ AS capabilities, specify ΔTRxSRS to be 1.5dB larger than the agreed values for bands n41, n77 and n78, i.e. 5.5dB, 5.0dB and 6.0dB, respectively, which is roughly in line with the submitted results by the companies and consistent with previous Releases and 2Rx, 4Rx and 8Rx AS capabilities.

# References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#104.

- R4-2414285, “WF on 6Rx UE requirements”, AT&T, RAN4#112.