3GPP TSG-RAN4 Meeting #112bis	R4-2416479

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.12.2.3

Source:	Keysight Technologies UK Ltd, CAICT

Title:	Preliminary Results from Dynamic MIMO OTA Measurement Campaign

Document for:	Approval

# Introduction

This contribution presents preliminary measurement data to compare Dynamic MIMO OTA results for the UE noise-limited environment (3GPP baseline) with the SIR-based environment (CTIA baseline).

# Discussion

In RAN#103, a new WID [1] was approved which suggests the study of a new and more realistic FR1 MIMO OTA methodology

In Rel-17, NR MIMO OTA WI specified the test methodology for FR1 and corresponding requirements were defined for bands n41 and n78. As we move to Rel-18, the NR MIMO OTA WI enhancement aims to specify requirements for more FR1 bands, specifically n1, n5, and n28 with the test methodology defined in Rel-17. However, the current test methodology only considers stationary test scenarios, limiting its ability to verify UE MIMO performance under static channel models. For static MIMO OTA testing, factors such as UE orientation, MCS (Modulation and Coding Scheme), and Angle of Arrival (AoA) remain fixed. Consequently, the existing FR1 MIMO OTA test system cannot adequately verify UE performance in realistic environments. Therefore, it is essential to study dynamic channel models and develop corresponding channel validation solution in Rel-19 to create more realistic FR1 MIMO OTA test scenarios.

The following objectives were identified for this ‘FR1 dynamic MIMO OTA methodology’

- Study and develop FR1 dynamic MIMO OTA test methodology with the following aspects - Reusing Rel-17 16-probes FR1 MPAC system layout - Study and define dynamic channel model parameters. CDL channel models defined in TR 38.901 and TR 38.827 should be considered as the starting point - Specify channel model validation procedures and pass/fail limits - Decide environmental conditions - Noise-limited environmental condition is the baseline - Study and define appropriate performance metric under dynamic channel model - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system (RAN5)

Especially the selection of the UE noise-limited environment as baseline in the WID triggered multiple discussions and whether the SIR-based environment, selected by CTIA [2], should be adopted in 3GPP instead. It was agreed to keep the UE noise-limited environmental condition and study the differences based on measurements, e.g., as captured in [3]

Issue 4-2-4: Environmental condition for RAN4 FR1 dynamic MIMO OTA Agreements - RAN4 adopt the full path loss model from TR 38.901 for the UE Noise-limited environment condition. - With understanding of noise-limited is baseline for dynamic MIMO OTA in RAN4, the study of Interference-limited environment condition for RAN4 FR1 Dynamic MIMO OTA is not precluded.

and [4]

Issue 4-3-1: UE measurement results collection Agreements: - Collect measurement throughput data from different UEs during the measurement campaign to study FoM. Benchmarking Throughput vs. Noise-limited and Throughput vs. Interference-limited environmental conditions. - Presentation of channel model validation is encouraged.

Details about the UE noise-limited environment were presented in [2] which were consequently adopted by 3GPP [5]. The signal power/SIR profiles for the two different environmental conditions are shown in Figure 1.

Figure 1: Top: Signal Power Profile for UE Noise-Limited Environment using UMa Route, Bottom: SIR Profiles for UMa Route

This remainder of the contribution presents findings about the differences in environmental conditions based on measurements.

# Channel Model Validation Results

Detailed channel model validations for the UMa route based on the (draft) CTIA test plan using dynamic channel models and link adaptation, i.e., utilizing the SIR-based environmental condition, were presented by Keysight and CTTL, CAICT in [6] which used the Keysight PROPSIM fading/channel emulator. As the same system from CAICT (SW & HW, chamber) was used and good agreement was observed previously, the CM validation was not repeated again. However, the validation of the dynamic pathloss model adopted by 3GPP for the UE noise-limited environment [2], [5] was validated using measurements and are shown in Figure 2.

Figure 2: Pathloss validation results for the UMa route (UE noise-limited environment)

Observation 1: Pathloss validation results are in good agreement with the target curves.

# UE Measurement Campaign

In [6], [7], Keysight and CTTL, CAICT presented results from a measurement campaign based on the (draft) CTIA test plan using dynamic channel models and link adaptation, i.e., utilizing the SIR-based environmental condition. The results in this contribution are based on measurements performed by CTTL, CAICT with five different commercial smartphones in the n41 band (2496 – 2690 MHz). The measurement setup consisted of Keysight UXM 5G Test Application (TA), Keysight UXM BS emulator, and Keysight PROPSIM fading/channel emulator. The test case implementation utilizes proof-of-concept implementation of Keysight test automation and UXM TA parameterization.

The test conditions, i.e., device orientations (FS DMP, FS DML-left tilt, FS DML-right tilt, FS DMSU) and the four device rotations (0°, 90°, 180°, 270°), used for these measurements, match those agreed in 3GPP [5]. It should be noted that only the UMa route was evaluated in these measurements.

The first set of measurements was to assess the repeatability. For these measurements, UE#2 was tested in the DMP orientation (with 0° rotation) multiple times in a row. In between each set of TP measurements, the BS emulator, the CE, and the device were power cycled, i.e., there was ~20min in between measurements. The results for the throughput (TP) and the CDF of the TP are shown in Figure 3.

Figure 3: Results for the repeatability assessment of UE#2 in DMP orientation. Top: TP vs time, Bottom: CDF vs TP.

Overall, especially given the fact that the entire system and device was power cycled between measurements, the results show good repeatability.

Observation 2: Measurements show good repeatability

Figure 4 shows just a single (#1) measurement result from Figure 3 to compare the overall TP curves between the UE noise-limited (3GPP) and the SIR-based (CTIA) environmental condition. On top of the figure, the different Signal Power Profile/SIR Profiles from Figure 1 are shown. The following general observations can be made:

- In high-SIR/Signal Power regions, e.g., 0-20s, 90-100s, 110-123s, TP is better for the 3GPP model (UE noise-limited). In these regions, the scenario is LOS where the signal comes from one direction and for the 3GPP model noise is non-directional system noise, i.e., the UE can suppress the noise by directional filtering. On the other hand, the CTIA model (SIR-based) has the intra-site interference from the same direction as the LOS signal and thus, the noise cannot be filtered out which results in lower TP.

- In Low SIR/Signal Power regions, e.g., 20-90sec, the CTIA model performs better. In this region, the scenario is NLOS where the signal comes from multiple directions at the same time. In the CTIA model, the interference can be filtered out by beamforming to the NLOS path directions. In the 3GPP model, the spatial filtering cannot be performed. Also, the signal level is low (RSSI~-80 dBm) and over 20 dB closer to the UE noise floor compared to the CTIA model.

Figure 4: Comparison of 3GPP (UE Noise-Limited) and CTIA (SIR Based) measurement results for UE#2 and UMa route.

Observation 3: In high-SIR/Signal Power regions, the TP is better for the 3GPP model while in low-SIR/Signal Power regions, the TP is better for the CTIA model.

The comparison of TP and CDF of TP of the five different UEs in the DMP orientation with 0° rotation is shown in Figure 5.

Figure 5: Results for the five UEs in the DMP orientation with 0° rotation for UMa route. Top: TP vs time, Bottom: CDF vs TP.

These results show an interesting trend in that the UE noise-limited CDF curves shows a plateau (flat CDF at around 60%-65%) while the CDF curves for the SIR based environment shows a rather steep monotonic increase of TP. When looking at the TP results as a function of time, it can be observed that the TP for the UE noise-limited environment is generally lower than for the SIR-based environment between 18s and 85s of the route for all UEs.

The variation in TP performance for different device orientations is shown in Figure 6 for UE#2 and for different device rotations in Figure 7 for UE#5 in the DMSU orientation.

Figure 6: CDF of TP measurement results for the four different orientations (DMP, DML-L, DML-R, DMSU) for UE#2 with 0° rotation.

Figure 7: CDF of TP measurement results for the four different angles (0°, 90°, 180°, 270°) for UE#5 in DMSU orientation.

We observed from other measurements not presented here that different angles do not have a very large impact on TP performance with the UE noise-limited environment while there is a more noticeable TP impact with the SIR-based environment (with directional noise). A side-by-side comparison for UE#2 is shown in Figure 8.

Figure 8: CDF of TP measurement results for the four different angles (0°, 90°, 180°, 270°) for UE#2 in DMP orientation. Left: UE noise-limited, right: SIR-based environment.

Observation 4: The SIR-based environment seems to have a stronger impact on TP performance for different device angles when compared to the UE noise-limited environment

Aside from the MIMO OTA performance assessment, the TRS performance was measured for each of the 5 devices in Band n41 (100MHz CBW, 30 kHz SCS). The results are shown in Table 1.

Table 1: Measured TRS performance for the 5 UEs

| UE | FS TRS [dBm] |
| --- | --- |
| UE#1 | -85.8 |
| UE#2 | -87.8 |
| UE#3 | -84 |
| UE#4 | -87.2 |
| UE#5 | -84.1 |

The three different percentile CDF values (10%, 50%, 90%) from Figure 5 including the post-processed means and medians are tabulated in Table 2. The relative performance ranking of the non-post-processed results is shown with colour coding (red: poor, yellow: medium, green: good) and the corresponding best and worst results are shown with bolded font.

Table 2: Comparison of device performance for TRS and Dynamic MIMO OTA assessment in DMP with 0° rotation

| UE | TRS [dBm] | TP per 3GPP (UE Noise-Limited) [Mbps] |  |  |  |  | TP per CTIA (SIR-based) [Mbps] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CDF10% | CDF50% | CDF90% | Mean | Median | CDF10% | CDF50% | CDF90% | Mean | Median |
| UE#1 | -85.8 | 49.1 | 125.1 | 431.2 | 201.8 | 125.1 | 212.3 | 282.7 | 376.5 | 290.5 | 282.7 |
| UE#2 | -87.8 | 34.3 | 110.1 | 427.5 | 190.6 | 110.1 | 158.1 | 257.9 | 363.4 | 259.8 | 257.9 |
| UE#3 | -84.0 | 93.2 | 161.8 | 433.8 | 229.6 | 161.8 | 172.6 | 256.1 | 378.0 | 268.9 | 256.1 |
| UE#4 | -87.2 | 85.6 | 159.8 | 439.5 | 228.3 | 159.8 | 160.8 | 291.9 | 433.4 | 295.4 | 291.9 |
| UE#5 | -84.1 | 62.1 | 138.1 | 374.8 | 191.7 | 138.1 | 175.3 | 246.4 | 372.0 | 264.5 | 246.4 |

When comparing the DMP MIMO OTA results from Figure 5 with the TRS results from Table 1 (summarized in Table 2), the following comparison in terms of absolute ranking can be made in Table 3.

Table 3: Comparison of device ranking from TRS and Dynamic MIMO OTA assessment in DMP with 0° rotation

| UE | Ranking TRS | Ranking of TP per 3GPP (UE Noise Limited) |  |  |  |  | Ranking of TP per CTIA (SIR-based) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CDF10% | CDF50% | CDF90% | Mean | Median | CDF10% | CDF50% | CDF90% | Mean | Median |
| UE#1 | 3 | 4 | 4 | 3 | 3 | 4 | 1 | 2 | 3 | 2 | 2 |
| UE#2 | 1 | 5 | 5 | 4 | 5 | 5 | 5 | 3 | 5 | 5 | 3 |
| UE#3 | 5 | 1 | 1 | 2 | 1 | 1 | 3 | 4 | 2 | 3 | 4 |
| UE#4 | 2 | 2 | 2 | 1 | 2 | 2 | 4 | 1 | 1 | 1 | 1 |
| UE#5 | 4 | 3 | 3 | 5 | 4 | 3 | 2 | 5 | 4 | 4 | 5 |

The following observations could be drawn from the available data:

- UE#1: device with good TRS performance has rather mixed dynamic MIMO OTA performance (poor ≤50%-ile CDF performance but good 90%-ile CDF performance with UE noise-limited environment; on the other hand, for the SIR-based environment, the device has good ≤50%-ile CDF performance but poor 90%-ile CDF performance)

- UE#2: device with the best TRS performance generally performs poorly in the dynamic MIMO OTA tests

- UE#3: device with very poor TRS performance has very different dynamic MIMO OTA performance (overall best CDF performance with UE noise-limited environment but very poor CDF performance with SIR-based environment)

- UE#4: device consistently demonstrates one of the best performing TRS, 3GPP CDF and CTIA CDF performance (other than the 10%-tile CDF performance with SIR-based environment)

- UE#5: worst performing device generally demonstrates one of the worst performing TRS, poor 3GPP CDF and worst CTIA CDF performance

Overall, the following high-level observations can be drawn:

Observation 5: The UE noise-limited environment in Dynamic MIMO OTA tests does not rank devices in a similar fashion as TRS

Observation 6: The UE noise-limited environment in Dynamic MIMO OTA tests does not necessarily rank devices in a similar fashion as the SIR-based environment.

Based on the preliminary results presented in this contribution, it is proposed to continue with the UE noise-limited environment in 3GPP.

Proposal 1: Given the lack of similarities in ranking between TRS, dynamic MIMO OTA with UE noise-limited and SIR-based environments, continue with the UE noise-limited environment in 3GPP for dynamic MIMO OTA.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: Pathloss validation results are in good agreement with the target curves.

Observation 2: Measurements show good repeatability

Observation 3: In high-SIR/Signal Power regions, the TP is better for the 3GPP model while in low-SIR/Signal Power regions, the TP is better for the CTIA model.

Observation 4: The SIR-based environment seems to have a stronger impact on TP performance for different device angles when compared to the UE noise-limited environment

Observation 5: The UE noise-limited environment in Dynamic MIMO OTA tests does not rank devices in a similar fashion as TRS

Observation 6: The UE noise-limited environment in Dynamic MIMO OTA tests does not necessarily rank devices in a similar fashion as the SIR-based environment.

Proposal 1: Given the lack of similarities in ranking between TRS, dynamic MIMO OTA with UE noise-limited and SIR-based environments, continue with the UE noise-limited environment in 3GPP for dynamic MIMO OTA.

# References

- RP-240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, 3GPP TSG RAN Meeting #103, March 2024

- R4-2409769, On FR1 dynamic MIMO OTA methodology, Keysight Technologies, 3GPP TSG-RAN4 Meeting #111, May 2024

- R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA, vivo, 3GPP TSG-RAN WG4 Meeting #111, May 2024

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo, 3GPP TSG-RAN WG4 Meeting #112, August 2024

- R4-2413536, TP on Dynamic MIMO Aspects, Keysight Technologies, Spirent Communications, 3GPP TSG-RAN4 Meeting #112, August 2024

- OTAMIMO_2023_003_004 V2, Preliminary validation measurement results, Keysight Technologies, CTTL
This is a CTIA Certification Program Working Group Document available to CTIA Certification Program Working Group members

- R4-2404142, On FR1 dynamic MIMO OTA methodology, Keysight Technologies, 3GPP TSG-RAN4 Meeting #110bis, April 2024