3GPP TSG-RAN WG4 Meeting #112-bis									                    R4-2416484

Hefei, China, October 14 – 18, 2024

Agenda item:			6.8.2.2.2

Source:	Ericsson

Title:	On UE RF Rx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Tx requirements for NR-NTN HPUE from the approved WF in RAN4#112 [2].

# Discussion

In the approved WF [2], the following has been agreed:

Issue 4-1: General considerations

- Agreement

  - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class.

The only remaining open issue in this part is the following:

Issue 4-2: RSD for NR-NTN

- Agreement

  - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE.

    - Further discuss the following aspects:

      - Use similar way in TN spec by introducing a new table of RSD compared to the PC3

      - Whether default Tx-Rx frequency separation is used for test point

For FR1-NTN, the only identified potential impact on Rx requirements by introducing PC2 is on a reference sensitivity. In TS38.101-1[3], in clause 7.3.2 there is a Table 7.3.2-1c on “Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity”, where the allowed reference sensitivity degradation is specified for FDD bands. Since in FR1-NTN all specified bands are FDD (n256, n255 and n254), the similar table should be studied to be introduced to clause 7.3.2 on “Reference sensitivity power level” of TS38.101-5 specifications.

Proposal 1: It should be studied case by case whether	reference sensitivity degradation (RSD) requirements are needed for NR-NTN HPUE. If so, similar way as in TN spec should be followed, i.e. introducing a new table of RSD compared to the PC3.

# Conclusion

Based on the discussion in the previous sections we have the following proposals:

Proposal 1: It should be studied case by case whether	reference sensitivity degradation (RSD) requirements are needed for NR-NTN HPUE. If so, similar way as in TN spec should be followed, i.e. introducing a new table of RSD compared to the PC3.

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2414452, “WF on UE RF requirements for NTN HPUE”, Samsung, RAN4#112.

- 3GPP TS 38.101-1 V18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”