3GPP TSG-RAN WG4 Meeting #112-bis									                    R4-2416489

Hefei, China, October 14 – 18, 2024

Agenda item:			6.8.3.3

Source:	Ericsson

Title:	On SAN RF requirements for less than 5 MHz for NTN

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In RAN4#112, the WF has been approved in [2]. In this contribution, we focus on the SAN RF requirements for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands and we provide our view on the open issues.

# Discussion

## NR SAN RF conducted Tx/Rx requirements

In the approved WF [2], the following has been agreed on the NR SAN RF conducted Tx/Rx requirements:

| NR SAN RF conducted Tx/Rx requirement | Proposal for 3MHz channel bandwidth in FR1-NTN bands |
| --- | --- |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | Minimum requirement calculated according to transmission bandwidth configuration in Table 6.3.3.2-1. |
| 6.4 Transmit ON/OFF power | No specification impact. |
| 6.5 Transmitted signal quality | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 6.6.3.2-1 and Table 6.6.3.2-2. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact |
| 6.7 Transmitter intermodulation | No specification impact. |
| 7.2 Reference sensitivity level | Add minimum requirements for 3 MHz in Table 7.2.2-1 and Table 7.2.2-2. Scaling from 5 MHz can be used. Use FRC G-FR1-A1-7, same as in TN. |
| 7.3 Dynamic range | Add minimum requirements for 3 MHz in Table 7.3.2-1. Scaling from 5 MHz can be used. Reuse TN FRC G-FR1-A2-15 (to be introduced to A.2). |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Define ACS requirement similar to TN for 3MHz in Table 7.4.1.2-1, and assume same interferer frequency offset values as TN 3 MHz in Table 7.4.1.2-2. |
| 7.4.2 In-band blocking | No specification impact. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | No specification impact. |
| 7.8 In-channel selectivity | Add minimum requirements for 3 MHz in Table 7.8.2-1 and Table 7.8.2-2. Reuse TN FRC G-FR1-A1-20 (to be introduced to A.1). |
| A.1 Fixed Reference Channels for RF Rx requirements (QPSK, R=1/3) | FRC for 3MHz in-channel selectivity need to be added in Table A.1-1. Reuse TN FRC G-FR1-A1-20. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |
| A.2 Fixed Reference Channels for dynamic range (16QAM, R=2/3) | FRC for 3MHz needs to be added in Table A.2-1. Reuse TN FRC G-FR1-A2-15. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |

Regarding the total power dynamic range requirement, we agree with WF that the minimum requirement should be calculated according to transmission bandwidth configuration, where the same value could be adopted as for 3MHz CBW for TN: 11.7dB.

Proposal 1: Regarding the total power dynamic range requirement, the minimum requirement should be calculated according to the transmission bandwidth configuration: specify 11.7dB in Table 6.3.3.2-1, which is the same value as for 3MHz CBW for TN.

When it comes to the ACLR requirement, 3 MHz CBW should be added to Tables 6.6.3.2-1 and 6.6.3.2-2, where the ACLR limit should remain the same and the assumed adjacent channel carrier should be an NR of same BW.

Proposal 2: For the ACLR requirement, 3 MHz CBW should be added to Tables 6.6.3.2-1 and 6.6.3.2-2, where the ACLR limit should remain the same and the assumed adjacent channel carrier should be an NR of same BW.

For the ACS requirement, we propose the following:

Proposal 3: For the ACS requirement, 3 MHz CBW shall be added. The same PREFSENS + 8dB wanted signal mean power as for TN 3 MHz CBW should be specified in Table 7.4.1.2-1. In Table 7.4.1.2-2, the same type of interfering signal and the interfering signal center frequency offset could be reused from 3 MHz BS CBW for TN: ±1.5075 MHz frequency offset for 3 MHz DFT-s-OFDM NR signal with 15 kHz SCS and 15 RBs.

## NR SAN RF radiated Tx/Rx requirements

In the approved WF [2], the following has been agreed on the NR SAN RF radiated Tx/Rx requirements:

| NR SAN RF radiated Tx/Rx requirement | Proposal for 3MHz channel bandwidth in FR1-NTN bands |
| --- | --- |
| 9.2 Radiated transmit power | No specification impact. |
| 9.3 OTA SAN Output power dynamics | No specification impact. |
| 9.4 OTA output power dynamic range | 1-O follow same requirement as 1-H, no update on specification. |
| 9.5 OTA transmit ON/OFF Power | No specification impact. |
| 9.6 OTA transmitted signal quality | No specification impact. |
| 9.7 OTA unwanted emissions | 1-O follow same requirement as 1-H, no update on specification. |
| 9.8 OTA Transmitter intermodulation | No specification impact. |
| 10.2 OTA sensitivity | No specification impact. |
| 10.3 OTA reference sensitivity level | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.4 OTA dynamic range | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.5 OTA In-band selectivity and blocking | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.6 OTA Out-of-band blocking | No specification impact. |
| 10.7 OTA receiver spurious emissions | No specification impact. |
| 10.8 OTA receiver intermodulation | No specification impact. |
| 10.9 OTA In-channel selectivity | Similar update as conductive requirements to add 3MHz channel bandwidth. |

After a further check, we would like to confirm that we agree with the initial analysis proposed in the approved WF [2].

Proposal 4: Agree with the tentative analysis on the impact on NR SAN RF radiated Tx/Rx requirements. The requirements to be defined for 3 MHz CBW are the following:

- OTA reference sensitivity level

- OTA dynamic range

- OTA adjacent channel selectivity

- OTA in-band blocking

- OTA in-channel selectivity

# Conclusion

Based on the discussion in the previous sections we have the following proposals:

Proposal 1: Regarding the total power dynamic range requirement, the minimum requirement should be calculated according to the transmission bandwidth configuration: specify 11.7dB in Table 6.3.3.2-1, which is the same value as for 3MHz CBW for TN.

Proposal 2: For the ACLR requirement, 3 MHz CBW should be added to Tables 6.6.3.2-1 and 6.6.3.2-2, where the ACLR limit should remain the same and the assumed adjacent channel carrier should be an NR of same BW.

Proposal 3: For the ACS requirement, 3 MHz CBW shall be added. The same PREFSENS + 8dB wanted signal mean power as for TN 3 MHz CBW should be specified in Table 7.4.1.2-1. In Table 7.4.1.2-2, the same type of interfering signal and the interfering signal center frequency offset could be reused from 3 MHz BS CBW for TN: ±1.5075 MHz frequency offset for 3 MHz DFT-s-OFDM NR signal with 15 kHz SCS and 15 RBs.

Proposal 4: Agree with the tentative analysis on the impact on NR SAN RF radiated Tx/Rx requirements. The requirements to be defined for 3 MHz CBW are the following:

- OTA reference sensitivity level

- OTA dynamic range

- OTA adjacent channel selectivity

- OTA in-band blocking

- OTA in-channel selectivity

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2413518, “Way Forward for [112][313] NR_IoT_NTN_less_than_5MHz_BSRF”, Nokia, RAN4#112.