3GPP TSG-RAN WG4 Meeting #112bis                                                          R4-2416493

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:	Further discussion on testability and interoperability issues for beam management

Agenda item:			6.17.4.2

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two confirmed use cases (i.e., beam management and positioning accuracy enhancements) [2].

Particularly for the use case of beam management, it is required to provide specification support for DL TX beam prediction for both UE-sided and NW-sided model, and correspondingly necessary RAN4 core requirements for this use case and LCM procedure including performance monitoring [2]. Since AI-BM involves more complicated test procedure compared with Rel-15 FR2 beam management related test, the testability issue are highlighted in previous RAN4 discussion.

In RAN4#111 and RAN4#112, for reported measurements and ground truth, test setup (including channel model) and datasets for training/testing, the following agreements are achieved:

Agreement from RAN4#111 Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded Issue 2-5: Test setup needs Agreement: - maximum number of set B Tx beams that test system should be able to emulate: [8-16] - maximum number of set A Tx beams that test system should be able to emulate: [64-128] - FFS on AoAs - UE rotation during the test: FFS - UE rotation/repositioning between different tests: Yes Issue 2-6: Datasets for training/testing Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment - RAN4 to strive to make the test conditions similar to field deployment conditions Agreement from RAN4#112 Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy. Issue 2-7: UE reporting for network side models Agreement: postpone discussion until other WGs define any reporting mechanisms

Based on the outcome of the study objectives captured in TR 38.843 [1] and other RAN1 progress, we would like to discuss on core requirement and testability issues for beam management in this contribution.

# 2. Discussion

## 2.1 Testing target

Issue 1: Which RAN4 requirement to be focused for testability study?

In the recent RAN4 meeting (RAN4#111 and RAN4#112), there was an extensive discussion regarding the test environment setup for AI-BM use case. However, it was observed that the objectives of these tests were not clearly defined, leading to a lack of clarity on the necessary procedures to be followed. This ambiguity has resulted in a stagnation of progress. Consequently, it is imperative that we establish a clear understanding that any test setup discussed in the context of testability must minimally encompass the assessment of AI/ML model inference for UE-side model. This is crucial to ensure that the following two key performance indicators (KPIs) are met:

- AI-BM model inference precision, which shall meet the predefined KPIs to ensure that the AI/ML models are providing accurate predictions.

- Reporting delay of AI-BM inference results (including model inference latency), which shall be within the acceptable timeframes to guarantee inference results can be timely utilized in gNB.

Moreover, if the test setup is intended to evaluate the generalization capabilities of the AI/ML models, it is essential that the test environment is flexible and robust enough to accommodate such assessments. This would involve testing the models' ability to perform well on unseen data, which is a critical aspect of their real-world applicability. This would not only streamline the testing process but also ensure that the models are thoroughly evaluated for their practical use in various scenarios.

On the other hand, in our accompanying contribution on AI-BM core requirement, we've identified that the new RAN4 requirements is required for L1-RSRP reporting for data collection to support AI/ML model inference and training on the network side (AI-BM Case1 and 2). Fortunately, the existing FR2 IFF test setup (used for FR2 RRM testing) should be adequate for verifying the L1-RSRP reporting requirements. However, it's crucial to confirm its suitability through a detailed analysis to ensure it meets the specific needs of these cases, guaranteeing effective data collection for AI/ML model development.

Proposal 1: AI-BM testability study shall be performed to test UE-side model inference requirement as 1st priority, including (1) testing the accuracy of inference results; (2) testing the delay of reporting inference results; (3) generalization performance testing if introduced.

Issue 2: The test environment related to produce appropriate AI-BM model input?

For AI-BM model inference, no matter using the information of Tx beam ID and/or Rx beam ID explicitly or implicitly, there are two kinds of beam prediction assumed in RAN1, i.e., DL Tx beam prediction with the measurements from (1) the best Rx beam or (2) Tx-Rx beam pair. Although from RAN1 perspective, both methods can be viewed as different AI/ML model implementation, it may have impact on testability discussion especially for determining the applicable channel model. Particularly, for the model inference, the following figure is provided [1] to illustrate the inference procedure:

Observation 1: For UE-sided model, DL Tx beam prediction, testability discussion could be different for AI/ML model input by using the measurements from (1) the best Rx beam or (2) Tx-Rx beam pair.

However, it has been recognized that relying solely on Set B of beams for the test setup might not suffice. Based on previous RAN4 agreement, the ground truth shall be “the ideal measurement of RSRP on the predicted Tx beam”, which in the conformance test is “the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam”. Our interpretation is that the ideal RSRP measurement on the predicted transmission beam should be executed by producing Set A of beams.

Agreement from RAN4#111 Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

Proposal 2: AI-BM test setup shall generate the not only Set B of beams, but also Set A of beams for the ideal measurement of RSRP on the predicted Tx beam.

The proposal 2 could be essential in the testability discussion: In the vendor analysis outlined in document [R4-2411254], it's intriguing to note that only the four most powerful TX beams are emulated using an 8-probe configuration in an enhanced MPAC OTA chamber. However, this approach might fall short if the emulation of Set A beams (with the number of Set A beams much larger than 4) is also necessary to achieve the optimal measurement of RSRP on the predicted transmission beam.

## 2.2 Test procedure for AI-BM (UE-side model)

Among two AI-BM use case (Case1 & Case2), we would like to take Case 1 as example to elaborate the test procedure for AI-BM conformance testing.

Proposal 3: The conformance testing procedure for AI-BM UE-side model, using AI-BM Case1 as a reference, is detailed as follows:

Preparation Phase:

- Step-1: TE configures resource sets for both Set A and Set B beams in CSI-ReportConfig, and associated ID as side condition

Model Inference Phase:

- Step-2: TE configures the test environment to transmit Set B beams

- Step-3: DUT (a.k.a, UE) predicts Set A beams based on the measurement from Set B beams, and reports the prediction results back to TE

Ideal Measurement Phase:

- Step-4: TE adjusts the test environment to transmit Set A beams, ensuring a certain level of SNR for the ideal measurement of RSRP

- Step-5: DUT measures the Set A beams, and report measurement results to TE

TE Evaluation Phase:

- Step-6: TE evaluates the success of the prediction by comparing the predicted results from Step 3 with the actual measurement results from Step 5.

## 2.3 Beam prediction KPI

In previous RAN4 discussion, For the metrics/KPI discussion, the following options are given in the Rel-18 study item and captured in TR [1]:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

Here, RAN4 requirement shall be established by considering the expected behavior and performance from the typical and exemplary UE implementation. When it comes to beam prediction accuracy, it is clear that the network will primarily rely on the UE's top prediction by TCI switching to the Top-1 predicted beam by UE, rendering the second and subsequent strongest beams less relevant. To harness the strengths of both Option 2 and Option 3, we suggest a balanced approach that integrates aspects of Option 2 (Top-1/K) and Option 3:

Proposal 4: For metrics for UE-sided AI-BM requirements/tests, RAN4 adopt the combined Option 2 and 3 for compromise:

- Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

  - FFS: K = 2 and x = 3 as baseline

## 2.4 Channel model for UE-sided AI-BM testing

As discussed over previous RAN 4 meetings, the group agrees to further discuss and down-select the following options considering TE feasibility:

- Option 2: TDL

- Option 5: simplified CDL

  - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

Here we would like to firstly identify the key difference by using TDL and simplified CDL models from the perspective of testing target (based on the analysis in section 2.1):

- DUT RX beam performance and relevant procedure can only be verified with CDL models, in which the spatial differentiated AoAs can be emulated:

  - For AI/ML model with TX-RX beam pair measurement as model input, CDL model with the spatial differentiated AoAs is required.

  - For AI/ML models by TX beam measurement from the best RX beam as model input, if an ideal spherical coverage (uniform in all directions) is assumed, to test this AI/ML models, the spatial AoA characteristics could be not that important.

Figure 1: AI/ML models by using the measurements from the best Rx beam (left) and Tx-Rx beam pair (right)

From technical aspects, for DFF chamber, RAN4 has studied the method to use one probe to simulate multi-path fading condition, as provided in Section 8.2 in TR 38.810, in which two options are given. However, it should be noted that in the corresponding single probe channel modelling, the UE RX beamforming can be included in the channel model, while since there is only one AoA, it is impossible to test actual RX beam management procedure.

#### 8.2.1.1 Channel model Option 1 Channel model option 1 is based on the TDL methodology in the TR 38.901 [10]: - The multi-path propagation conditions model consists of several parts: - A power delay profile in the form of a "tapped delay-line" (TDL), characterized by a number of taps with certain power at fixed positions on a sampling grid. - The channel model parameters include the Delay spread scaling factor and the maximum Doppler frequency. The test system shall allow flexible control of the respective parameters. - Each tap is modeled based on the Jakes fading model. - Generation of TDL channel models and power delay profiles from CDL channel models by including spatial filters to capture Tx and Rx antenna patterns is not precluded and based on the procedure described in the TR 38.901 [10].

Observation 2: The necessity of introducing spatial differentiated AoAs in AI-BM testing could depends on different AI/ML model inputs:

- For AI/ML model with TX-RX beam pair measurement as model input, CDL model with the spatial differentiated AoAs is required.

- For AI/ML models by TX beam measurement from the best RX beam as model input, if an ideal spherical coverage (uniform in all directions) is assumed, to test this AI/ML models, the spatial AoA characteristics could be not that important.

Another aspect is how many TRPs to be simulated for AI-BM. Based on our understanding, the relevant AI-BM prediction are primarily used for single TRP case. At least for Rel-19 AI-BM, RAN4 discussion shall focus only on single TRP case.

Proposal 5: At least for Rel-19 AI-BM, RAN4 discussion shall focus on single TRP case, which is considered as baseline assumption for testability study.

To determine the channel model to be used in OTA chamber, we propose to follow the following steps:

Proposal 6: The following steps can be followed to emulate the required channel model in OTA chamber for UE-sided AI-BM model inference requirement testing:

Step-1: Determine the required evaluation scenario:

- RAN1 SLS assumption can be used as baseline:

Certain channel model, e.g., UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 can be used as baseline.

FFS UE mobility included in this evaluation scenario

BS TX Beambook assumed

For DFF/IFF chamber, FFS how UE RX antenna modeling; For 3D MPAC OTA chamber, RX antenna modeling is precluded.

Step-2: SLS for channel model for all TX beams:

- Based on the targeted evaluation scenario, run SLS to get channel model for training/testing

To obtain the data for different UE location, multiple drops will be simulated in SLS.

For each drop, the resultant channel models are obtained for all beams in Set-A/Set-B

Step-3: Test signal is mapped over probe(s) in OTA chamber

- The signals from tester (to be over certain TX beam-i) + corresponding channel model for TX beam-i (obtained in Step-2), to be mapped over the probe(s) in OTA chamber.

- Depends on DFF or 3D MPAC OTA chamber is decided to be used, the mapping can be different.

Figure 2: Procedure of emulating the required channel model in OTA chamber

Proposal 7: RAN4 shall use UMa model as used in RAN1 evaluation to perform SLS for generating the received signals from different TX beams, in order to obtain the dataset for AI model training/evaluation.

Specifically, the evaluation assumption from Table 6.3.1-1 in TR38.843 could be used as the baseline for RAN4 to perform SLS for generating the UE received signals from different beams. Here we try to simplify the parameters based on the baseline SLS assumptions for AI/ML in beam management evaluations from Table 6.3.1-1 in TR38.843:

Table 1: Simplified Baseline System Level Simulation assumptions for RAN4 to generate the UE received signals from different beams

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | For spatial domain beam prediction: 3km/h For time domain beam prediction: 30km/h (baseline), 60km/h (optional) 90km/h (optional), 120km/h (optional) Other values are not precluded |
| UE distribution | 10 UEs per sector/cell for system performance related KPI (if supported) [e.g., throughput] for full buffer traffic (if supported) evaluation (model inference). X UEs per sector/cell for system performance related KPI for FTP traffic (if supported) evaluation (model inference). Other values are not precluded. Number of UEs per sector/cell during data collection (training/testing) is reported by companies if relevant. For spatial domain beam prediction (optional to compare different UE distributions assumptions): - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Number of BS beams: 32 or 64 downlink Tx beams (max number of available beams) at NW side. Other values, e.g., 256 not precluded. |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Number of UE beams: 4 or 8 downlink Rx beams (max number of available beams) per UE panel at UE side. Other values, e.g., 16 not precluded. |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | For system performance related KPI (if supported) evaluation (model inference), companies report either of the following traffic model: Option 1: Full buffer Option 2: FTP model with detail assumptions (e.g., FTP model 1, FTP model 3) |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies to explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200 m |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |
| UE measurements/reports | At least for Temporal Downlink beam prediction: - Periodicity of time instance for each measurement/report in T1: 20ms, 40ms, 80ms, [100ms], 160ms, [960ms]. Other values can be reported. - Number of time instances for measurement/report in T1 can be reported. Time instance(s) for prediction can be reported. |
| Scenario | Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation. Other scenarios are not precluded. |
| Spatial consistency | At least for BM-Case1, companies report the one of spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| UE trajectory model | UE trajectory model is defined at least for temporal beam prediction in initial phase of the evaluation. Further details below. UE trajectory model is not necessarily to be defined at least for spatial-domain beam prediction in initial phase of the evaluation. |
| UE rotation | UE speed to be reported. Note: UE rotation speed = 0, i.e., no UE rotation, is not precluded |
| Baseline for performance evaluation | For temporal beam prediction: - Option 1: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2 - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1 - Companies to explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1. where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams. T1 and T2 are aligned with those for AI/ML based methods. Whether Set A and Set B are the same or different depend on the sub-use case. Other options are not precluded. For spatial-domain beam prediction: - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping) - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams - Other options are not precluded. |

Proposal 8: For FR2 TX beambook, the reference model used in TR 38.827 can be used as baseline for further discussion:

-	For FR2: A code book of 128 fixed beams is constructed to a grid of eight elevation angles from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size.

## 2.5 OTA testing chamber for AI-BM model inference

For the selection of OTA chamber, we see the difference for the required probes for MPAC OTA chamber. For instance, the analysis provided in [R4-2411254] requires 8 probes (2 additional one compared with existing 3D MPAC chamber) to simulate 4 strongest TX beams, with acceptable PSP (PAS similarity percentage) for InO and UMi scenarios. On the other hand, in [R4-2411587], upscaling the 3D MPAC chamber to emulate CDL-C UMi requires a test system with >100 probes. The difference may come from how many TX beams and how many clusters to be emulated with accepted accuracy.

From our understanding, the feasibility of enhanced 3D MPAC chamber shall consider the following factors:

- FFS how many beams out of Set A (e.g., 128 beams containing Set B beams) are relevant for AI-BM inference testing;

- All Tx beams from a single TRP;

- Acceptable PAS similarity percentage based on a certain configuration of probes

Proposal 9: The feasibility of enhanced 3D MPAC chamber shall consider the following factors:

- FFS how many beams out of Set A (e.g., 128 beams containing Set B beams) are relevant for AI-BM inference testing;

- All Tx beams from a single TRP;

- Acceptable PAS similarity percentage based on a certain configuration of probes

# 3. Conclusion

In this contribution, we provided our viewpoints on testability and interoperability issues for beam management in this contribution, accordingly the following observations and proposals are obtained:

Testing target

Proposal 1: AI-BM testability study shall be performed to test UE-side model inference requirement as 1st priority, including (1) testing the accuracy of inference results; (2) testing the delay of reporting inference results; (3) generalization performance testing if introduced.

Observation 1: For UE-sided model, DL Tx beam prediction, testability discussion could be different for AI/ML model input by using the measurements from (1) the best Rx beam or (2) Tx-Rx beam pair.

Proposal 2: AI-BM test setup shall generate the not only Set B of beams, but also Set A of beams for the ideal measurement of RSRP on the predicted Tx beam.

Test procedure for AI-BM (UE-side model)

Proposal 3: The conformance testing procedure for AI-BM UE-side model, using AI-BM Case1 as a reference, is detailed as follows:

Preparation Phase:

- Step-1: TE configures resource sets for both Set A and Set B beams in CSI-ReportConfig, and associated ID as side condition

Model Inference Phase:

- Step-2: TE configures the test environment to transmit Set B beams

- Step-3: DUT (a.k.a, UE) predicts Set A beams based on the measurement from Set B beams, and reports the prediction results back to TE

Ideal Measurement Phase:

- Step-4: TE adjusts the test environment to transmit Set A beams, ensuring a certain level of SNR for the ideal measurement of RSRP

- Step-5: DUT measures the Set A beams, and report measurement results to TE

TE Evaluation Phase:

- Step-6: TE evaluates the success of the prediction by comparing the predicted results from Step 3 with the actual measurement results from Step 5.

Beam prediction KPI

Proposal 4: For metrics for UE-sided AI-BM requirements/tests, RAN4 adopt the combined Option 2 and 3 for compromise:

- Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

  - FFS: K = 2 and x = 3 as baseline

Channel model for UE-sided AI-BM testing

Observation 2: The necessity of introducing spatial differentiated AoAs in AI-BM testing could depends on different AI/ML model inputs:

- For AI/ML model with TX-RX beam pair measurement as model input, CDL model with the spatial differentiated AoAs is required.

- For AI/ML models by TX beam measurement from the best RX beam as model input, if an ideal spherical coverage (uniform in all directions) is assumed, to test this AI/ML models, the spatial AoA characteristics could be not that important.

Proposal 5: At least for Rel-19 AI-BM, RAN4 discussion shall focus on single TRP case, which is considered as baseline assumption for testability study.

Proposal 6: The following steps can be followed to emulate the required channel model in OTA chamber for UE-sided AI-BM model inference requirement testing:

Step-1: Determine the required evaluation scenario:

- RAN1 SLS assumption can be used as baseline:

Certain channel model, e.g., UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 can be used as baseline.

FFS UE mobility included in this evaluation scenario

BS TX Beambook assumed

For DFF/IFF chamber, FFS how UE RX antenna modeling; For 3D MPAC OTA chamber, RX antenna modeling is precluded.

Step-2: SLS for channel model for all TX beams:

- Based on the targeted evaluation scenario, run SLS to get channel model for training/testing

To obtain the data for different UE location, multiple drops will be simulated in SLS.

For each drop, the resultant channel models are obtained for all beams in Set-A/Set-B

Step-3: Test signal is mapped over probe(s) in OTA chamber

- The signals from tester (to be over certain TX beam-i) + corresponding channel model for TX beam-i (obtained in Step-2), to be mapped over the probe(s) in OTA chamber.

- Depends on DFF or 3D MPAC OTA chamber is decided to be used, the mapping can be different.

Proposal 7: RAN4 shall use UMa model as used in RAN1 evaluation to perform SLS for generating the received signals from different TX beams, in order to obtain the dataset for AI model training/evaluation.

Proposal 8: For FR2 TX beambook, the reference model used in TR 38.827 can be used as baseline for further discussion:

-	For FR2: A code book of 128 fixed beams is constructed to a grid of eight elevation angles from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size.

OTA testing chamber for AI-BM model inference

Proposal 9: The feasibility of enhanced 3D MPAC chamber shall consider the following factors:

- FFS how many beams out of Set A (e.g., 128 beams containing Set B beams) are relevant for AI-BM inference testing;

- All Tx beams from a single TRP;

- Acceptable PAS similarity percentage based on a certain configuration of probes

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402389, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] 3GPP TR 38.810, v16.7.0, “NR; Study on test methods”.

[6] 3GPP TR 38.827, v16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[7] R4-2405652, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110bis, April 2024.

[8] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[9] R4-2409761, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#111, May 2024.

[10] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[11] R4-2411635, “Further discussion on testability and interoperability issues for beam management”, Samsung, RAN#112, Aug. 2024.

[12] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.