3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2416504

# Hefei, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Discussion on MPR applicability for FR1 intra-band UL CA

Agenda item:	6.1.1.2.2

Document for:	Discussion

Introduction

In 3GPP RAN #112, WF[1] was approved. In this paper, we provide our view on Power domain enhancements for single carrier.

### Sub-topic 2-1: Intra-band contiguous UL CA - Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell - Proposals - Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. (Samsung) - Proposal 2: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied (Samsung, CATT,, Nokia, Huawei, LGE) - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C; - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated; - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - Proposal 3: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements (Qualcomm). - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4 - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4 - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4 - WF - FFS in next meeting - Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE - Proposals - Proposal 1: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW. (Samsung) - Proposal 2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR. (Qualcomm, Huawei) - WF - FFS in next meeting - Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell - Proposals - Proposal 1: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements. (Skyworks) - Proposal 2: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. (CATT, Samsung, ZTE, vivo) - Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated/configured CBW. (Huawei, Ericsson) - WF - FFS in next meeting - Issue 2-1-4: Draft Rel-19 CR on MPR applicability for intra-band contiguous CA with single CC with activated cell - Proposals - Proposal 1: Add the following description into clause 6.2A.2.1: - “For intra-band contiguous carrier aggregation with single CC with activated cell, MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.” - WF - FFS the details of draft CR - Sub-topic 2-2: Intra-band non-contiguous CA Sub-topic description Open issues and candidate options before meeting: - Issue 2-2-1: General considerations - Proposals - Proposal 1: The work scope for FR1 NC CA should be limited to only study MPR applicability. (Samsung) - WF - Agree with the proposal as it is aligned with the WID - Issue 2-2-2: Applicable MPR for FR1 intra-band non-contiguous UL CA - Proposals - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. (Qualcomm, ZTE, Huawei) - Proposal 2: When DualPA is not signalled or TxD or UL MIMO is signalled, transmit interruption may be needed to allow LO switching and the single carrier MPR can be sued when transmitted RBs are allocated in only one of the CC. (Skyworks) - Proposal 3: for non-contiguous UL CA configurations with sub-blocks consisting of one cell and supported by dual PA architecture, the non-CA (single CC) MPR applies for one cell active among the configured uplink serving cells, the other cell deactivated. (Ericsson) - Proposal 4: for non-contiguous UL CA configurations with sub-blocks consisting of one cell not supported by a dual PA architecture, applicability of the non-CA (single CC) MPR for one active cell among the configured uplink serving cells, the other cell deactivated, is subject to UE capability, e.g. indication of [mpr-singleCC-activated-FR1] for the band combination. (Ericsson) - WF - FFS in next meeting

Discussion

Applicable MPR for intra-band contiguous CA with single activated cell

All of the above issues seem to stem from the question of whether it is necessary to follow the CA MPR table when using only 1CC in a structure that is both PC3 and PC2 contiguous UL CA. Since the existing defined MPR table considers the MPR table assuming 2CC as currently configured, We think this is a validation that a structure that is a contiguous UL CA can fall back to a structure for single carrier when assuming 1CC. there are three structures for MPR table defined by contiguous UL CA. UL CA have 1PA+1LO structure that does not support TxD or dualPA-Architecture or both. In this case, the above issue is a structure that does not include the following two structures, TxD or dualPA-Architecture, and has only 1 LO and 1PA. In this case, it is possible to modify the structure to 1CC. So MPR defined in Table 6.2.2-1 in 38.101-1 could applies for UE power class 3 CA bandwidth classes B and C. The TxD structure is currently characterized by having a 2PA+1LO structure. The way to fall back to the structure of single carrier operation from that structure is to shift the LO frequency. If it shift the LO frequency from the center of each CC to one side, it can fall back to the structure of single carrier operation. So MPR defined in Table 6.2D.2-1 also could applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. MPR defined in Table 6.2.2-2 in 38.101-1 could applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

- 2. Conclusion

In this contribution, we want to share some views on MPR applicability for FR1 intra-band UL CA and proposal is made as following:

Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

- 3. Reference

[1] R4-2414278, “WF on Power domain enhancements”