3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416507

# Hefei, China, October 14 – 18, 2024

Source:			T-Mobile USA, AT&T, US Cellular

Title:			NTN S-Band UE-UE coexistence with TN

Agenda item:		5.17.2

Document for:		Discussion

# Introduction

At the RAN4#112, a Way Forward was approved for the NTN S-Band Work Item. The Way Forward included the following:

#### Issue 1-2-6: TN-NTN UE-to-UE Coexistence

Offline Agreement:

- Further study the UE-to-UE coexistence between NTN S-band UL and B2/n2, B25/n25 DL, considering:

  - Realistic usage scenarios

  - consider UE antenna gain for smartphone of -5.5dBi

  - UE-to-UE separations distance

  - probability of the UE-to-UE coexistence scenarios occurring

- Companies are encouraged to examine the previous work in TR 38.863 for UE-to-UE coexistence noting that the solution might need to be re-evaluated for this work.

- Companies are encouraged to investigate the background scenario for applicability of the -50dBm/MHz requirement to assess its applicability to TN-NTN coexistence (e.g. reference R4-080710)

- Companies are also encouraged to investigate other prior work in UE-to-UE coexistence where different values from -50 dBm/MHz have been adopted

This contribution discusses the views of T-Mobile on USA TN-NTN UE-to-UE Coexistence.

# Discussion

## Further study the UE-to-UE coexistence between NTN S-band UL and B2/n2, B25/n25 DL

### Realistic Usage scenarios

We feel the following are valid usage scenarios:

- B252/n252 with terrestrial B2/n2/B25/n25 terrestrial coverage and potential B2/n2/B25/n25 victims nearby.

- B252/n252 with no terrestrial coverage but B25/n25 Supplemental Coverage from Space and potential B25/n25 victims nearby.

- B252/n252 with no terrestrial coverage but or B25/n25 Supplemental Coverage from Space.

We believe all three of these are valid usage scenarios, but unless the B252n252 network knows that there is not terrestrial or SCS B2/n2/B25/n25 coverage, then there could be B2/n2/B25/n25 victims that need protection.

Proposal 1: There are realistic usage scenarios with both B252/n252 aggressors and B2/n2/B25/n25 victims in close proximity.

### Consider UE antenna gain for smartphone of -5.5dBi

In [2] Motorola stated that 0 dBi antenna loss would be extremely pessimistic. For frequencies >1GHz they proposed 8 dB loss per UE for “antenna, head and hand losses”

Figure 3.2-1 Hand and Head loss coupling losses

However, in the analysis, it looks like they used 10 + 10 = 20 dB for < 1 GHz, but 9+9 = 18 dB for >1 GHz for “Ant+Body”:

Figure 4.2-1 UE to UE interference analysis on proposed spurious limits (LTE)

Rather than discuss the antenna gain and head and body loss separately, we propose using the same “antenna plus body loss” value from [2]

Proposal 2: Rather than choose a specific antenna gain value, RAN4 should re-use the 9 dB per UE antenna plus body loss used in R4-080710 [2] for frequencies above 1 GHz.

### UE-to-UE separations distance

1m separation distance was used in [2]. One of the actions from the Way Forward is to consider what UE-UE distances should be used for UE-UE coexistence for the S-Band

Scenario 1: NTN aggressor, TN B2/n2/B25/n25 victim

Scenario 2: NTN aggressor, SCS B25/n25 victim

There have been some claims that 1m separation may be inappropriate for some S-Band NTN scenarios. However, there are several factors that could make 1m likely or even optimistic even in remote areas:

- While some people venture into remote areas alone, many people travel into remote areas with others. When traveling with others 1m is a reasonable minimum separation distance.

Observation 1: When traveling in remote areas, it is quite common to travel with two or more people.

- In 1999 when the 1m for indoor and 5m for outdoor were agree to in TS 25.942, it was unusual for individuals to have a single mobile device, much less more than one. In 2024 many people have a work phone and a personal phone or other device on them at any given time.

A lot has changed in the world since 1999. In 1999 there were roughly 30 mobile subscriptions per 100 people in the USA. In 2022 the number was roughly 110 subscriptions per 100 people.

Observation 2: The number of mobile subscriptions per 100 people in the USA has more than tripled since 1999.

- Looking around at breakfast at the Melbourne Hilton Little Queen Street in September, many people had two phone right next to or on top of each other.

Observation 3: 1 meter may even be optimistic for UE-UIE separation distances. However, [2] advises against using UE-UE separation distances of <1m due to near-field conditions.

- We have all seen taxi or rideshare drivers with multiple phones on their dashboard:

Observation 4: In 2024, nearly everyone in the USA has at least one mobile device and it is quite common for people, including first responders and government employees and even corporate employees, to carry a personal phone and a work phone.

- There is also the scenario where multiple people in the same car/van/bus/train are in close proximity, and one could be trying to access B252/n252 while others are accessing B2/n2/B25/n25. In this scenario, it may happen that the B252/n252 uplink link budget won’t close due to window losses, but that doesn’t mean the UE won’t try. If the B252/n252 UE can receive the downlink signal, it could repeatedly attempt to RACH at full power.

Observation 5: The scenario of multiple people in a car/van/bus/train can lead to 1 meter separation between a b252/n252 aggressor and B2/n2/B25/n25 victim.

To follow the example set in [2] we followed a similar an analytical approach to determining the protection level based on the minimum separation distance. We created a spreadsheet to recreate the calculations in to ensure we had the correct equations:

Figure 1 Recreation of Figure 4.2-1 from R4-080710

Then we redid the calculations with -50 dBm/MHz which was finally chosen for LTE rather than -60 dBm/100 kHz which was in [2].

Figure 2 Recreation of Figure 1 with -50 dBm/MHz rather than -60 dBm/100 kHz

Finally, we ran the calculations for B252/n252 into B25/n25:

This analysis shows that 1 meter is an appropriate separation distance for UE-UE coexistence, and that it is achievable with -50 dBm/MHz spurious emission limit into B2/B25/n2/n25.

Proposal 3: Use 1m separation distance for UE-UE coexistence. For both NTN->TN and NTN-> SCS/NTN.

In the NTN UE specifications TS 36.102 and 38.101-5, the spurious emissions for UE co-existence levels are -50 dBm/MHz across the board, with the exception of B256/n256 which overlap with B2/n2/B25/n25 which overlap, making this impossible.

Observation 6: For existing NTN bands, no one has questioned the assumption of 1 m separation between NTN and TN and/or the -50 dBm/MHz spurious emissions for UE co-existence levels.

### Probability of the UE-to-UE coexistence scenarios occurring

B2/n2/B25/n25 are used for terrestrial networks, and there may be areas where one or more operators have h B2/n2/B25/n25 terrestrial coverage, but B252/n252 has NTN satellite coverage. Some have claimed that the network will always choose the terrestrial network if it is available, but we do not believe this is not true. The NTN operator may prefer to have their customers use the NTN network to avoid roaming charges.

Observation 7: NTN->TN UE-UE coexistence is a likely scenario where there is B2/n2/B25/n25 terrestrial coverage, but not B70/n70 terrestrial coverage, but B252/n252 NTN coverage.

In addition, the FCC is studying the potential use of terrestrial bands for Supplemental Coverage from Space (SCS). LTE Band 25 has been proposed for use for SCS. So, even in areas where there is no B2/n2/B25/n25 terrestrial coverage, UE-UE coexistence is still a valid scenario.

Observation 8: NTN->NTN UE-UE coexistence is a likely scenario where B2/n2/B25/n25 is used for satellite coverage.

Proposal 4: UE-UE coexistence is an issue everywhere because B2/n2/B25/n25 can be used for terrestrial coverage, as well as supplemental coverage from space.

## Examine the previous work in TR 38.863 for UE-to-UE coexistence noting that the solution might need to be re-evaluated for this work

The following coexistence scenarios were studied in the NTN Study Item and documented in 38.863.

Table 6.1-1: Scenarios for NTN-NTN/TN co-existence

| FR1: 2GHz |  | NTN1,4,5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  | Set 22 |  |  | HAPS |
|  |  | GEO3 | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |  |
| NR / NB-IoT | Rural | X | X | X | X | X | X | X |
|  | Urban macro | X | X | X | X | X | X | X |
|  | Dense Urban6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| HAPS |  | N/A | N/A | N/A | N/A | N/A | N/A | X |
| NOTE 1: Earth Fixed beam has been considered for co-existence studies. NOTE 2: Use Set 1 satellite antenna has been used as the starting point for co-existence studies. NOTE 3: GEO and LEO only operate at adjacent channel. NOTE 4: Use GEO and LEO@600km when TN is victim. NOTE 5: The satellite-to-satellite coexistence scenarios are not in the scope of this study considering this is already addressed by ITU (ITU RR Article 9 etc.) and regional regulations (e.g. FCC rules). NOTE 6: Dense Urban is not considered as it is expected NTN UE will connect to terrestrial networks rather than satellite networks in such scenario. |  |  |  |  |  |  |  |  |

Proposal 5: The Rural and Urban macro scenarios are applicable for S-Band NTN coexistence.

- Table 6.1-2: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | NTN with NTN | NTN DL | NTN DL | HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | HAPS-HAPS | Phase 2 |

RAN4 has already agreed to the following:

Agreement:

  - Focus on coexistence of S-band UL with B2/n2 and B25/n25 DL.

  - Capture the clarifications on the co-existence issues with B70/n70 and B66/n66 and that there is no 3GPP solution for them in the TR

- Table 6.1-2a: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with B2/n2/B25/n25. | Phase 1 |
| 2 | SCS/NTN with NTN | NTN UL | SCS/NTN DL | Applicable for satellite operating in S-band, for e.g. coexistence with SCS/NTN B2/n2/B25/n25. | Phase 1 |

Proposal 6: Consider Scenario 1 and Scenario 2 in Table 6.1-2a.

## Investigate the background scenario for the applicability of the -50 dBm/MHz requirements to assess applicability to TN-NTN coexistence

Prior to the introduction of LTE, the UMTS requirement for spurious emissions into other bands was -60 dBm/3.84 MHz which is equivalent to -65.8 dBm/MHz. [2] proposed -60 dBm/100 kHz, which eventually became -50 dBm/MHz. The LTE spurious emission for UE coexistence level is nearly 16 dB higher than the UMTS protection level.

Observation 9: The LTE spurious emission for UE coexistence level is 15.8 dB higher than the UMTS protection level.

Rather than simply re-use the UMTS protection level, [2] proposed an analytical approach to determining an appropriate protection level.

Observation 10: Rather than blindly re-using the UMTS protection level, LTE use an analytical approach proposed in R4-080710 [2].

[2] mentioned 5 meter separation for outdoor and 1 meter for indoor, which was from TR25.942 version 1.0.0.

Observation 11: 5m for outdoor and 1m for indoor UE-UE (actually MS-MS at the time) was from TS 25.942 v1.0.0 which was published in June of 1999 when there were fewer than 1/3 as many mobile phones per person in the USA.

Proposal 7: In 2024, the -50 dBm/MHz coexistence requirement is applicable for UE-UE coexistence between B252/n25 and B25/n25.

## Investigation of other prior work in UE-to-UE coexistence where different values from -50 dBm/MHz have been adopted

When LTE band 23 was created there were contentious discussions between the proponents of new Band 23 and new Band 25. Since both were new bands, it was impossible to complete the work on either band without consensus on the protection levels. -40 dBm/MHz was accepted for protection of 1990-1995 MHz because B25 was a new band, but legacy Band 2 below 1990 MHz received -50 dBm/MHz protection.

Figure 3 From Table 6.6.3.2-1 of 36.101 v10.12.0

Later this was replaced by NS_20:

Table 6.6.3.3.14-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 1990 ≤ f < 1999 | -40 | 1 MHz |
| 1999 ≤ f ≤ 2000 | -40 | NOTE 1 |
| NOTE 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |

Observation 12: When LTE Band 23 and LTE Band 25 were created as new bands, there was a compromise of -40 dBm/MHz protection for 1990-1995 MHz, while legacy Band 2 received -50 dBm/MHz protection.

Since Band 25 and n25 are now widely deployed legacy bands, including proposed use for Supplemental Coverage from Space, and because Band 23 had been replaced by Band 70/n70 for terrestrial use in the US, it would be appropriate for Band 252/n252 to provide -50 dBm/MHz protection for Band 25/n25.

Proposal 8: Band 252/n252 should meet -50 dBm/MHz spurious emission limit into the legacy Band 25/n25 downlink, including “NOTE 15.”

-	Companies are encouraged to examine the previous work in TR 38.863 for UE-to-UE coexistence noting that the solution might need to be re-evaluated for this work.

-	Companies are encouraged to investigate the background scenario for applicability of the -50dBm/MHz requirement to assess its applicability to TN-NTN coexistence (e.g. reference R4-080710)

-	Companies are also encouraged to investigate other prior work in UE-to-UE coexistence where different values from -50 dBm/MHz have been adopted

## An alternative to A-MPR for protecting other bands

In R4-2413305 [4], T-Mobile USA proposed the use of restricting channel uplink RB allocations. Another alternative would be to restrict uplink channel bandwidths.

Proposal 9: RAN4 should consider restricting RB uplink allocations and/or uplink channel bandwidths to enable UE-UE coexistence while minimizing the impact on the uplink link budget compared to the use of A-MPR.

## An alternative to the proposed B256/n256 Band Plan

In R4-2413305 [4], T-Mobile USA proposed the use of an the B70/n70 bandplan as an alternative to the proposed B252/n252 bandplan. Since Band 70/n70 is being used in the use for terrestrial deployments, the use of the B70/n70 bandplan for NTN would greatly improve coexistence both between B252/n252 and B70/n70 as well as between B252/n252 and B2/n2/B25/n25. With 2000-2020 MHz used for terrestrial downlink and for satellite uplink, even with downtilted antennas, it is likely that reflections off metal surfaces including cars and trucks will provide significant interference into the satellite uplink, even if the terrestrial and satellite coverage don’t overlap. We will note that B70/n70 is not currently included in the FCC SCS initiative.

Proposal 10: Consider the use of the B70/n70 bandplan to enhance coexistence between satellite and terrestrial networks.

# Summary

Observation 1: When traveling in remote areas, it is quite common to travel with two or more people.

Observation 2: The number of mobile subscriptions per 100 people in the USA has more than tripled since 1999.

Observation 3: 1 meter may even be optimistic for UE-UIE separation distances. However, [2] advises against using UE-UE separation distances of <1m due to near-field conditions.

Observation 4: In 2024, nearly everyone in the USA has at least one mobile device and it is quite common for people, including first responders and government employees and even corporate employees, to carry a personal phone and a work phone.

Observation 5: The scenario of multiple people in a car/van/bus/train can lead to 1 meter separation between a b252/n252 aggressor and B2/n2/B25/n25 victim.

Observation 6: For existing NTN bands, no one has questioned the assumption of 1 m separation between NTN and TN and/or the -50 dBm/MHz spurious emissions for UE co-existence levels.

Observation 7: NTN->TN UE-UE coexistence is a likely scenario where there is B2/n2/B25/n25 terrestrial coverage, but not B70/n70 terrestrial coverage, but B252/n252 NTN coverage.

Observation 8: NTN->NTN UE-UE coexistence is a likely scenario where B2/n2/B25/n25 is used for satellite coverage.

Observation 9: The LTE spurious emission for UE coexistence level is 15.8 dB higher than the UMTS protection level.

Observation 10: Rather than blindly re-using the UMTS protection level, LTE use an analytical approach proposed in R4-080710 [2].

Observation 11: 5m for outdoor and 1m for indoor UE-UE (actually MS-MS at the time) was from TS 25.942 v1.0.0 which was published in June of 1999 when there were fewer than 1/3 as many mobile phones per person in the USA.

Observation 12: When LTE Band 23 and LTE Band 25 were created as new bands, there was a compromise of -40 dBm/MHz protection for 1990-1995 MHz, while legacy Band 2 received -50 dBm/MHz protection.

Proposal 1: There are realistic usage scenarios with both B252/n252 aggressors and B2/n2/B25/n25 victims in close proximity.

Proposal 2: Rather than choose a specific antenna gain value, RAN4 should re-use the 9 dB per UE antenna plus body loss used in R4-080710 [2] for frequencies above 1 GHz.

Proposal 3: Use 1m separation distance for UE-UE coexistence. For both NTN->TN and NTN-> SCS/NTN.

Proposal 4: UE-UE coexistence is an issue everywhere because B2/n2/B25/n25 can be used for terrestrial coverage, as well as supplemental coverage from space.

Proposal 5: The Rural and Urban macro scenarios are applicable for S-Band NTN coexistence.

Proposal 6: Consider Scenario 1 and Scenario 2 in Table 6.1-2a.

Proposal 7: In 2024, the -50 dBm/MHz coexistence requirement is applicable for UE-UE coexistence between B252/n25 and B25/n25.

Proposal 8: Band 252/n252 should meet -50 dBm/MHz spurious emission limit into the legacy Band 25/n25 downlink, including “NOTE 15.”

Proposal 9: RAN4 should consider restricting RB uplink allocations and/or uplink channel bandwidths to enable UE-UE coexistence while minimizing the impact on the uplink link budget compared to the use of A-MPR.

Proposal 10: Consider the use of the B70/n70 bandplan to enhance coexistence between satellite and terrestrial networks.

# References

- R4-2414287, “WF on Rel-19 FR1 NTN bands,” Moderator (Inmarsat)

- R4-080710, “TS36.101: TP for UE Spurious emission limits,” Motorola

- TS 25.942 v1.0.0, “RF System Scenarios”

- R4-2413305, “NTN S-Band coexistence,” T-Mobile USA