3GPP TSG-RAN WG4 Meeting #112-bis	R4-2416509

Hefei, China, October 14 – October 18, 2024

Agenda Item:	6.23.4.1

Source:	MediaTek Inc.

Title:	Simulation assumptions for R19 LP-WUS

Document for:	Discussion

# Introduction

In the last meeting, RAN4 discussed the simulation assumptions for LP-WUS/WUR in FR1, and some agreements were achieved as captured in the WF [1]. In this contribution, we further discuss the remaining aspects in simulation assumption for RRM performance evaluation for LP-WUS/WUR UE, including the general parameters, cell specific parameters and UE specific parameters. In addition, we provide some initial simulation results based on the simulation assumption provided.

# Simulation assumptions and results

One of the issues discussed in RAN4 was the definition of the target SNR for LP-WUR RRM measurements as captured in the WF [1].

Issue 2-1-1: SINR setting Agreement: - As starting point for RAN4 RRM simulation purpose only, -3dB, 0.5dB, 2dB dB Ês/Iot value is used for serving cell for both OOK based and OFDM based LP-WUR. - The SINR value can be updated if any update on the noise figure value in RF session. - FFS the side condition for RAN4 requirement.

For the RRM simulation purpose only, it was agreed to use -3dB, 0.5dB, 2dB dB Ês/Iot value for serving cell measurement for both OOK based and OFDM based LP-WUR. These values were selected based on the previous agreement made in RAN1 (RAN1#117). On the other hand, the side condition requirement is still FFS and RAN4 needs to discuss the value considering the measurement inaccuracy of LP-WUR UE. It is also noted that RAN1 have made further agreement in the last meeting (RAN1#118) on the RRM measurement accuracy as captured below:

Agreement (RAN1) - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report.

Therefore, RAN4 can discuss the side condition requirements taking into account the current agreement in RAN1. Considering the two values of SNR from RAN1 (-3dB and -6dB), RAN4 to discuss the worst case scenario of SNR=-6dB based on RAN1 agreement to define the side condition for RRM measurement accuracy.

Proposal 1: RAN4 to consider the worst-case scenario of SNR=-6dB based on RAN1 agreement to define the side condition for RRM measurement accuracy for LP-WUS capable UE.

In addition, the timing error in the simulation assumption can be defined considering the impact of both the residual timing error of the OOK-based LP-WUR and the timing drift error of measurement. In our view, a maximum residual timing error of 1us for OOK-4 (M=2) and 0.5us for OOK-4 (M=4) can be assumed. Therefore, together with the timing drift error, the total timing error can be defined using 2us for OOK-4 (M=2) and 1us for OOK-4 (M=4), which also align with RAN1 agreement on the synchronization accuracy as captured below. On the other hand, for the number of symbols used by OOK-based LP-WUR, we propose to use 16 symbols for OOK-4 (M=2) and 20 symbols for OOK-4 (M=4). Note that, the symbol number is selected to achieve the target timing sync accuracy agreed by RAN1.

Agreement (RAN1) To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified

Proposal 2: Define the timing error: 2us for OOK-4 (M=2) and 1us for OOK-4 (M=4), which also align with RAN1 agreement on the synchronization accuracy.

Proposal 3: For OOK-based LP-WUR, use 16 symbols for OOK-4 (M=2) and 20 symbols for OOK-4 (M=4). The symbol number is selected to meet the target timing sync accuracy agreed by RAN1.

Furthermore, the overall simulation assumption list for RRM performance evaluation for LP-WUS/WUR UE is provided below including the general parameters, cell specific parameters and UE specific parameters.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30KHz |
| Measurement period (in number of measurement samples) | 5 |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-4 with M = 2,4 | when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 |
| LP-SS pattern |  | M sequence with circular extension to match the LPSS length | M sequence with circular extension to match the LPSS length Note: Cell 2 utilizes a different LP-SS pattern than Cell 1 |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OOK based receiver [5] ppm To be updated based on RAN1 agreement | N/A |
| Timing error |  | Residual timing error: 2us for OOK-4 (M=2) and 1us for OOK-4(M=4) |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
| Ês/Iot | dB | -3dB | N/A |
| Propagation conditions | - | FR1: TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | 3rd Order Butterworth with 3.84MHz bandwidth |
| --- | --- |
| [Receiver ADC bit width] | 4-bit ADC |
| [Receiver Sampling Rate for LP-SS only] | 7.68MHz |
| Receiver architecture | I-branch only for OOK-WUR |

Based on the above simulation assumption, we also provide some preliminary simulation results as shown in  Table 4. The simulation results are demonstrated in terms of delta RSRP and are achieved at 90 percentiles of the CDF for the measured LP-SS.

Table 4: RSRP error (dB) in TDL-C channel

| RSRP error in dB (90%) |  |  |  |  |
| --- | --- | --- | --- | --- |
| SINR= -3dB SNR (Cell 1) = -2.4 SNR (Cell 2) = -8.4 | Number of samples | 1 sample | 3 samples | 5 samples |
|  | OOK-4 (M=2) | 1.52 | 0.87 | 0.69 |
|  | OOK-4 (M=4) | 1.5 | 0.83 | 0.67 |

The simulation results above demonstrate the RSRP error using different number of samples in TDL-C channel. It can be noticed that even with 1 sample measurement LP-WUS capable UE satisfactory results can be achieved for the given simulation assumption in Table 1,2, and 3. However, to select number of samples for RRM performance evaluation for LP-WUS capable UE, we propose to use at least 3 or 5 samples to guarantee achieving the required performance (as 1 sample might be impacted due to deep fading and might not be detected by the LP-WUS capable UE).

Proposal 4: For OOK-based LP-WUR, use 3 or 5 samples for RSRP measurements.

In addition, for the simulation results obtained for OOK-based LP-WUR, it is our understanding that the ideal RSRP and the measured RSRP are derived based on the assumption that the OOK-based LP-WUR is equipped with I-branch only (even though the transmitted LP-SS is an I/Q signal). Therefore, RAN4 needs to align on how the RSRP measurement results are obtained for OOK-based LP-WUR, otherwise results in RAN4 might not be comparable.

Proposal 5: The ideal RSRP and the measured RSRP are derived based on the assumption that the OOK-based LP-WUR is equipped with I-branch only, and RAN4 needs to align the understanding for the obtained results.

# Summary

In this contribution, the following proposals are concluded:

Proposal 1: RAN4 to consider the worst-case scenario of SNR=-6dB based on RAN1 agreement to define the side condition for RRM measurement accuracy for LP-WUS capable UE.

Proposal 2: Define the timing error by using the residual timing error: 2us for OOK-4 (M=2) and 1us for OOK-4 (M=4), that is to align with RAN1 agreement on the synchronization accuracy.

Proposal 3: For OOK-based LP-WUR, use 16 symbols for OOK-4 (M=2) and 20 symbols for OOK-4 (M=4). The symbol number is selected to meet the target timing sync accuracy agreed by RAN1.

Proposal 4: For OOK-based LP-WUR, use 3 or 5 samples for RSRP measurements.

Proposal 5: The ideal RSRP and the measured RSRP are derived based on the assumption that the OOK-based LP-WUR is equipped with I-branch only, and RAN4 needs to align the understanding for the obtained results.

# References

- R4-2413899, WF for RRM core requirements for LP-WUSWUR, vivo.