3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2416513

Hefei City, CN, Oct 14 – Oct 18, 2024

Agenda Item: 6.19.3.1

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for UE-initiated/event-driven beam management

Document for: Discussion

# Introduction

According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WID [1] is divided into two items to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “UE-initiated/event-driven beam management”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- RRM requirement on UE-initiated/event-driven beam management

- Capability on UE-initiated/event-driven beam management

## 2.1 RRM requirement on UE-initiated/event-driven beam management

Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period? Agreement: - RAN4 will define Event triggered measurement reporting delay requirement. - Further discuss: - Whether to support both mode A and mode B - Whether to differentiate mode A and mode B - Other related aspects RAN1 Agreement: On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details. RAN1 Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. Issue 2-1-11: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward> Option 1: No new accuracy requirements for existing L1 measurements are required. Option 2: RAN4 need to discuss whether to update legacy accuracy requirements. TS 38.133 #### 9.5.3.3 Aperiodic Reporting Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. The UE shall only send aperiodic L1-RSRP measurement reports, if a DCI trigger has been received. After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

In our understanding, the main difference for Mode A and Mode B is the UL resource for UE beam reporting is dynamic scheduling or pre-configure by NW. Regardless of the details of UE beam reporting, we discussed as below into two phases:

- Phase 1: Before UE initiate event beam reporting.

- Phase 2: After UE initiate event beam reporting.

On Phase 1, there’s no difference on L1-RSRP measurement for both Mode A and Mode B.

On Phase 2, the reporting delay for Mode A may be longer than Mode B since UE should wait for the scheduled UL resource for beam reporting. The details will be specified in RAN1. Therefore, the following proposal is suggested.

Proposal 1: RAN4 to discuss two phases of UE-initiated/event-driven beam reporting:

- Phase 1: Before UE initiate event beam reporting.

  - The measurement delay for L1-RSRP is the same for both Mode A and Mode B.

- Phase 2: After UE initiate event beam reporting.

  - The reporting delay could be differentiated based on RAN1 conclusion.

Furthermore, we could reuse the requirement for aperiodic L1-RSRP reporting in legacy since it’s quite similar as UE initiated event beam reporting.

Proposal 2: RAN4 to reuse L1-RSRP measurement requirement as a baseline for UE-initiated/event-driven beam management on the following aspect:

- Requirement applicability.

- Measurement requirement for aperiodic reporting.

The RS measurement type for UE initiated beam reporting could be either CSI-RS or SSB based on the scheme 1 or scheme 2 selected by NW from RAN1 agreement. In our understanding, the typical case for CSI-RS measurement is periodic CSI-RS for L1-RSRP. Since UE should measure and evaluate the L1 beams during evaluation window. Periodic CSI-RS is more stable to select a better beam for UE initiated beam reporting than semi-persistent and aperiodic CSI-RS. Therefore, the following proposal is suggested.

Proposal 3: RAN4 to consider only periodic CSI-RS based L1-RSRP measurement for UE initiated beam reporting.

For the accuracy requirement, RAN4 to reuse legacy requirement since no new measurement metrics except for L1-RSRP introduced in RAN1.

Proposal 4: RAN4 to reuse the legacy L1-RSRP accuracy requirement for UE-initiated/event-driven beam management.

## 2.2 Capability on UE-initiated/event-driven beam management

There’re similar issues discussed about the capabilities for event triggered L1 measurement reporting in R19 LTM. In legacy, capabilities for support of event triggered L3 measurement reporting had been specified in section 9.1.4. For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1.

In our understanding, we could follow the similar principle to define the number of events supported for UE-initiated beam reporting in MIMO. Since we’re targeting FR2 and sTRP with intra- and inter-cell beam management, the number of events for UE-initiated beam reporting should be specified for intra-frequency.

Proposal 5: Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

Refer to the discussion on R19 LTM, the evaluation method of events configured in mobility and MIMO may be different, so the events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell.

Proposal 6: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

We prefer to specify a unified capability that indicates the total number of L1 events supported for mobility and MIMO. This allows UE with more flexibility to utilize the whole capability of L1 events compared to share the total number of L1 events to separate UE capability.

Proposal 7: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

R19 LTM discussion on the capabilities for support of event triggered L1 measurement reporting, the number of events supported shall be defined. Please refer to LTM discussion for more details. 38.133 Table 9.1.4.2-1: Requirements for reporting criteria per measurement category Measurement category; Ecat; Note. Intra-frequency Note 1,2,3,4,5; 9; Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells. Inter-frequency Note 2,3,4,5; 10; Events for any one or a combination of inter-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells. Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5; 10; Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5; 1; Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5; 1; Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3; 1; One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA.. Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3; 1; One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA.. Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5; 1; Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP.. Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5; 1; Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16.. Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5; 1; Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP.. Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5; 1; Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16.. DL RSTD Note 2,4,5; 1; DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34].. UE Rx-Tx Note 2,4,5; 1; UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34].. DL PRS-RSRP Note 2,4,5; 1; DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34].. DL PRS-RSRPP Note 2,4,5; 1; DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34].. SRS-RSRP Note 2,3,4,5; 1; SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16.. CLI-RSSI Note 2,3,4,5; 1; CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16.. NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode..

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: RAN4 to discuss two phases of UE-initiated/event-driven beam reporting:

- Phase 1: Before UE initiate event beam reporting.

  - The measurement delay for L1-RSRP is the same for both Mode A and Mode B.

- Phase 2: After UE initiate event beam reporting.

  - The reporting delay could be differentiated based on RAN1 conclusion.

Proposal 2: RAN4 to reuse L1-RSRP measurement requirement as a baseline for UE-initiated/event-driven beam management on the following aspect:

- Requirement applicability.

- Measurement requirement for aperiodic reporting.

Proposal 3: RAN4 to consider only periodic CSI-RS based L1-RSRP measurement for UE initiated reporting.

Proposal 4: RAN4 to reuse the legacy L1-RSRP accuracy requirement for UE-initiated/event-driven beam management.

Proposal 5: Define a capability for supported number of UE-initiated/event-driven beam reporting on intra-frequency.

Proposal 6: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

Proposal 7: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

# Reference

[1] RP-233090 New WID - Rel-19 MIMO

[2] R4-2414035, WF on RRM requirements for NR_MIMO_Ph5 (Samsung)