3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2416514

Hefei City, CN, Oct 14 – Oct 18, 2024

Agenda Item: 6.19.3.2

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for other RRM requirements

Document for: Discussion

# Introduction

RAN#105 agreed on the revised WID “NR_MIMO_Ph5” [1]. According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WID [1] is divided into two items to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “Other RRM requirements”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- CSI support for up to 128 CSI-RS ports.

- UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul.

- Non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions.

- Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios.

## 2.1 CSI support for up to 128 CSI-RS ports

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung The detailed objectives are as follows: RAN1: - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator

In our understanding, there’s no RRM impact on the revised part of objective for the CSI enhancement. This is mainly focus on SRS port grouping for 6/8 RX low complexity receiver. Therefore, the following proposal is suggested:

Proposal 1: No RRM impact for CSI support for up to 128 CSI-RS ports.

## 2.2 UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung The detailed objectives are as follows: RAN1: - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

RAN1 has already specified some mechanism of UE reporting enhancement for CJT deployments including timing misalignment, frequency offset and phase offset reporting. In our understanding, these are new feature for reporting in terms of the compensation of delay, frequency, and phase offset. Based on RAN1 agreement for CJT calibration reporting, RAN4 to discuss whether there’s any RRM impact based on feasibility study.

Proposal 2: Whether to define RRM requirement for CJT calibration reporting based on RAN4 feasibility study.

## 2.3 Non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung The detailed objectives are as follows: RAN1: - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. RAN1 Agreement Update the agreement made in RAN1 #116bis as the following. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port.

For the revised part of objective to enable 3T3R antenna switching, it could reuse single 4-port SRS resource to support antenna switching. For 3T6R antenna switching, it could reuse two 4-port SRS resource to support antenna switching.

In our understanding, the legacy interruption requirement of SRS antenna switching can be reused for either single or two 4-port SRS resource(s) based on NW configuration. Therefore, there’s no RRM impact for 3-antenna-port codebook-based transmissions.

Proposal 3: No RRM impact for 3TX CB-based Uplink (3-antenna-port codebook-based transmissions) since legacy interruption requirement of SRS antenna switching can be reused.

## 2.4 Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung The detailed objectives are as follows: RAN1: 5. Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources [3] R1-2403415 Summary #2 on Rel-19 asymmetric DL sTRP/UL mTRP Moderator (OPPO) From Tuesday session Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification. - Above is subject to a separate UE capability signaling. [4] R1-2405346 Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP Moderator (OPPO) Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting: - Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS: the details of DCI field design. Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

Based on RAN1 agreement, NW should indicate PL offset for UE to calibrate UL only TRP. The DL reference timing definition for PDCCH-order PRACH is based on current RAN4 specification. The unified TCI state indication for the asymmetric DL sTRP/UL mTRP can totally reuse Rel-17 and Rel-18 unified TCI framework. Therefore, the following proposal is suggested.

Proposal 4: No RRM impact at least for unified TCI state switching since RAN1 agreed to reuse Rel-17/Rel-18 unified TCI framework to indicate TCI states of asymmetric DL sTRP/UL mTRP deployment scenarios.

For the revised part of objective on two TAs for asymmetric DL sTRP/UL mTRP deployment scenarios. This principle is to reuse R18 two TAs for mDCI based mTRP without the association to two coresetPoolIndex. Since there’s only one coresetPoolIndex for asymmetric DL sTRP/UL mTRP. The restriction related to coresetPoolIndex should be removed for two TAs. Whether or how we specify two TAs for asymmetric DL sTRP/UL mTRP based on further RAN1 conclusion.

Proposal 5: Wait for RAN1 conclusion for the two TAs on asymmetric DL sTRP/UL mTRP deployment.

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: No RRM impact for CSI support for up to 128 CSI-RS ports.

Proposal 2: Whether to define RRM requirement for CJT calibration reporting based on RAN4 feasibility study.

Proposal 3: No RRM impact for 3TX CB-based Uplink (3-antenna-port codebook-based transmissions) since legacy interruption requirement of SRS antenna switching can be reused

Proposal 4: No RRM impact at least for unified TCI state switching since RAN1 agreed to reuse Rel-17/Rel-18 unified TCI framework to indicate TCI states of asymmetric DL sTRP/UL mTRP deployment scenarios.

Proposal 5: Wait for RAN1 conclusion for the two TAs on asymmetric DL sTRP/UL mTRP deployment.

# Reference

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung

[2] R4-2414035, WF on RRM requirements for NR_MIMO_Ph5 (Samsung)

[3] R1-2403415 Summary #2 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

From Tuesday session

[4] R1-2405346 Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)