# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416518

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	Huawei, HiSilicon

Title: 		Initial analysis of the NB-IoT requirements for n87 and n88

Agenda Item:	5.15.1

Document for:	Discussion

# Introduction

In this contribution we provide initial analysis of the NB-IoT requirements for n87 and n88, as per WF in R4-2414272 which has captured the following:

- Analyse annex 4 in ECC/Dec/(19)02 for the identification of any applicable requirements for in-band/guardband/standalone NB-IoT operation in bands n87 and n88

# Discussion

Referring to ECC/Dec/(19)02, it captures NB-IoT requirements (applicable to both E-UTRA and NR) which were not addressed so far. Below, initial analysis is provided for further discussion:

- Inband NB-IoT

According to ECC/Dec/(19)02 clause A4.1, there is no impact of the NB-IoT inband operation.

Therefore, it would be straightforward to capture inband NB-IoT operation in bands n87 and n88 while there are is explicit objective captured in the WID.

- Guardband NB-IoT / Standalone NB-IoT

According to ECC/Dec/(19)02 clause A4.2, for the guardband and standalone NB-IoT operation, the following applies:

There are no technical issues identified with Transmissions parameters and Frequency offset as in tables 16 and 17.

For the BS OBUE in table 18, it was identified that the mask seems (at least partially) aligned with some existing OBUE requirement, e.g,

- MSR BS spec: E-UTRA WA BS OBUE requirement, e.g. see TS 37.105 Table 6.6.5.2.3-2:

Table 6.6.5.2.3-2: WA BS OBUE in BC2 bands applicable for: BS operating with E-UTRA 1.4 or 3 MHz carriers adjacent to the Base Station RF Bandwidth edge or the sub-block edge

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic Limit (NOTE 5, 6) | Measurement bandwidth (NOTE 10) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| NOTE 4: The limits in this table only apply for operation with an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge or the sub-block edge. NOTE 5: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub-block gap. NOTE 6: For a MSR multi-band TAB connector with Inter RF Bandwidth gap < 2×ΔfOBUE the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. |  |  |  |

- E-UTRA BS spec: Regional WA BS OBUE

Table 6.6.3.2.2-3: Regional Wide Area BS operating band unwanted emission limits in band 3, 8 or 65 for 1.4 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0. 065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.2 MHz | 0.165MHz f_offset < 0.215MHz | -14 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215MHz f_offset < 1.015MHz | (Note 3) | 30 kHz |
| (Note 9) | 1.015MHz f_offset < 1.5 MHz | -26 dBm (Note 3) | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | -13 dBm (Note 3) | 1 MHz |
| 2.8 MHz f fmax | 3.3 MHz f_offset < f_offsetmax | -15 dBm (Note 3) | 1 MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -15dBm/1MHz. For BS supporting multi-band operation, either this limit or -16dBm/100kHz (f_offset adjusted according to the measurement bandwidth), whichever is less stringent, shall apply at f ≥ 10MHz for operating bands <1GHz. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: For BS supporting multi-band operation, either this limit or -16dBm/100kHz (f_offset adjusted according to the measurement bandwidth), whichever is less stringent, shall apply for operating bands <1GHz. |  |  |  |

Observation 1: With the above, there seem to be no technical showstopper to easily incorporate Guardband and Standalone NB-IoT support for bands n87 and n88.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Introduce NB-IoT support for bands n87 and n88, following ECC/Dec/(19)02.