3GPP TSG-RAN WG4 Meeting #112-bis					R4-2416521

14th Oct, 2024 – 18th Oct, 2024

Source: 	Qualcomm Incorporated

Title: 	Views on 8Rx UE demodulation and CSI requirements with inter-cell and intra-cell interference

Agenda item:	 6.16.2

Document for:	Discussion

1. Introduction

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

In Rel-18 topic NR_ENDC_ RF_FR1_enh2 WI, 8Rx performance requirements for SU-MIMO (single user) were discussed. In Rel-19, 8Rx performance requirements with inter-cell, and intra-cell inter-user interference to be defined for below objectives [1].

# In [2], RAN4 agreed on some of the scope related issues, and encouraged companies to run simulations with R17 interference settings. In this paper, we share some of our observations, and share our proposals.

# 2. PDSCH Demodulation Requirements intra-user inter-cell interference

## 2.1 Rank choice

From our initial simulation results, the gap in SNR (@ 70% of maximum throughput) required between baseline and simplified receivers is proportional to the rank of serving cell as expected due to better interference rejection achievable with higher number of Rx antennas.

Simulation settings:

- TDD

- TDLC300-100, ULA

- Rel-17 interference settings with rank1 or rank2

- Antenna configuration: Rank2: 2 x 8 ; Rank4: 4 x 8

Table 2.1 Gap between baseline and simplified receiver

| Scenario | Gains with baseline w.r.t simplified receiver (dB) |
| --- | --- |
| Rank 2 | 1.5 |
| Rank 4 | 4 |

Observation1: Performance gap between the baseline and simplified receivers increases as rank increases.

Table 2.2 Gap between simplified and 4Rx (Rel-17) receiver

| Scenario | Gains with simplified w.r.t 4Rx (Rel-17) receiver |
| --- | --- |
| Rank 1 | ~ 2 dB |
| Rank 2 | > 3 dB |

Observation2: Performance gap between simplified receiver and 4Rx (Rel-17) receiver is quite high for Rank2.

Observation3: To show performance gains with 8Rx simplified receiver too, Rank2 is a more meaningful choice thank Rank1.

From above observations, Rank2, and Rank4 are suitable rank choices to further study performance requirements for both simplified and baseline receivers. These rank choices also are likely to meet the SINR requirement > -7dB. Other settings such as the right MCS can be further discussed based on simulation results.

Proposal1: RAN4 to use Rank2, and Rank4 for further inter-cell interference PDSCH demodulation performance requirements.

Proposal 2: RAN4 to use MCS13 (Table1) for inter-cell interference simulation studies as a starting point.

Proposal 3: RAN4 to use TDLA30-10 for further inter-cell interference simulation studies.

4. Interference model

Rel-17 interference models in terms of number of interferers, power levels etc. can be used as a starting point for simulation studies.

Proposal 4: RAN4 to use Rel-17 interference models for simulation studies.

# 5. Release independence

Since interference requirements are being discussed in Rel-19 feature, release independence shall be considered from Rel-18 for

Proposal 5: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# 6. Conclusions

Following is the summary of the proposals.

Proposal1: RAN4 to use Rank2, and Rank4 for further inter-cell interference PDSCH demodulation performance requirements.

Proposal 2: RAN4 to use MCS13 (Table1) for inter-cell interference simulation studies as a starting point.

Proposal 3: RAN4 to use TDLA30-10 for further inter-cell interference simulation studies.

Proposal 4: RAN4 to use Rel-17 interference models for simulation studies.

Proposal 5: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# References

[1] RP-241297, “Revised WID on NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #104, June 17– June 20, 2024, Shanghai, China.

[2] R4-2413570, “Way Forward for [112][328] NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #112, Aug 19– Aug 23, 2024, Maastricht, Netherlands.