3GPP TSG-RAN WG4 Meeting #112bis                                                          R4-2416552

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.20.4

Source:	Samsung

Title:	Ad-hoc meeting minutes for [112bis][306] NR_duplex_evo_General

Document for:	Approval

# Introduction

This document captures the meeting minutes for the ad-hoc meeting on Monday, on the topic of [112bis][306] NR_duplex_evo_General

# Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

## Open issues summary

### Sub-topic 1-1: SBFD frequency domain issues

##### Issue 1-1-1: Minimum channel bandwidth for SBFD (X MHz)

- [Moderator] Based on the discussion in RAN4#111, SBFD was recognized as “a feature with requirements of which can potentially be applied to all TDD band”, and the declaration-based method for the SBFD-support in a particular TDD band was agreed in RAN4#112. Particularly, the following agreements was provided:

<From RAN4#112> - Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators.

- [Moderator] It is agreed the supported channel bandwidth shall be no less than X MHz, while X = [20] is the baseline for further study:

- The value of X

  - Option 1 (CMCC, Samsung, Tejas, Qualcomm, ZTE): X = 20 is confirmed

    - Proposal 1-1 (Samsung): By following the principle of “The supported channel bandwidth shall be no less than X=20 MHz”, TDD bands n34, n51, n53, n54 and n101 shall be precluded from SBFD deployment.

  - Option 1a (vivo): X = 20 as starting point, and further check if RSIC can be achievable for 20 MHz channel bandwidth especially for FR1 WA and MR BS

  - Option 2 (Nokia, Ericsson): X = 50

    - Proposal 2-1 (Ericsson): A general principle should be applied when specifying SBFD: The channel bandwidth should be no less than 50 MHz.

    - Observation 2-1 (Nokia): SBFD operation below 50MHz BW would result in a very narrow UL subband, and it introduces some challenges in terms of accomodating reference signals and channels such as SSB and CORESET 0 within the DL subband.

    - Observation 2-2 (Nokia): Analog filters require large guardband which makes the operation of SBFD in narrow channel bandwidths either unfeasible or not attractive from performance point of view.

  - Option 3 (CableLabs, Charter): X = 50MHz for DU and X = 100MHz for DUD

    - Proposal 3-1 (CableLabs): The total bandwidth of the guard band(s) reduces the transmission bandwidth, and such a reduction percentage should be considered as a reference when deciding the minimum channel bandwidth for SBFD operating bands. The transmission bandwidth reduction, R, is suggested between 2% and 5%, and the value of R could be further discussed.

    - Proposal 3-2 (CableLabs): Based on proposal 3-1 and an assumed 1.8 MHz inter-subband guard bandwidth (five RBs), to maintain the spectrum efficiency, SBFD should operate in bands with the minimum channel bandwidth of 50 MHz for the DU configuration and 100 MHz for the DUD configuration.

    - Proposal 3-3 (Charter): The channel bandwidth for a Subband Full Duplex carrier should be no less than 100 MHz.  (100 MHz becomes 1st priority relative to 50 MHz)

  - Option 4 (Huawei): X = 60

    - Proposal 4-1 (Huawei): For single carrier operation, the supported BS CBW for SBFD should be no less than 60MHz.

    - Proposal 4-2 (Huawei): For multi-carriers operation, the SBFD can be supported on only one of the carriers, and the supported CBW for this carrier should be no less than 60MHz and applying 100MHz as maximum.

      - For instance, 100MHz CC for SBFD + N*100MHz CC, where N could be e.g. 1, 2, and 3

- Moderator Recommendation:

  - Further discussion on the above options.

- Ad-Hoc Discussion:

  - Ericsson: 50MHz is proposed for spectrum efficiency by considering the guardband. If channel BW is smaller than 50, guardband cost the overhead which makes dynamic TDD more reasonable.

  - CMCC: 20MHz is preferred.

  - Samsung: Guardband size depends on gNB implementation. 20MHz is the low bound, which is not restricting to channel BW with larger than this.

  - Verizon: How about some value in between 20MHz and 50MHz?

  - ZTE: For MR and LA, the guardband could be smaller, which could not need analog filters. Guardband depends on operators’ requirements.

  - CATT: 20MHz is confirmed even we can’t have the spectrum efficiency but still latency improvement by SBFD.

  - Nokia: 5RB is assumed for guardband in the SI, by changing the value, we need to reconsider the feasibility study.

- Tentative Agreement:

  - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle:

    - The supported channel bandwidth(s) is declared by gNB vendors

    - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

    - The supported channel bandwidth shall be no less than X MHz

    - Option-1:

      - X = 20 is the default value

        - For a specific band, if larger X is applicable depends on operator’s input

    - Option-2:

      - X = 40 as the default value

        - For specific bands, like n38 and n39, X= 20MHz

##### Issue 1-1-2: SBFD as a Band-specific feature

- [Moderator] In RAN4 #112, it is agreed that SBFD is possible to be used on any TDD band, in other words, SBFD is not a band-specific feature. On the other hand, Charter has the following observations and proposals, specifically for US band n48 and n77:

- Observations/Proposals for SBFD as a band-specific feature:

  - Observation 1 (Charter): Figure 2 shows that there are no guard bands between the AMBIT band and the CBRS band, as well as there are no guard bands between the CBRS band and the C-band.

  - Observation 2 (Charter): Figure 2 shows that both the AMBIT band C-band can transmit at 75 dBm /10MHz EIRP, but the CBRS band is limited to 30 dBm/10MHz only. This means that the CBRS band EIRP is 45 dB smaller than band n77. Therefore, legacy TDD operations in the USA CBRS band will suffer much higher risk from adjacent-band SBFD deployments.

Figure 2: Band n48 and n77 spectrum allocation in the USA and EIRP requirements

  - Proposal 1 (Charter): To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n50, n78, n79, n90, n96, n102, n104 for FR1, excluding n48 and n77.

  - Proposal 2 (Compromised Proposal from Charter): To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: 
n38, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104 for FR1, 
except for the following bands in the USA:  n48 and n77.

- Moderator Recommendation:

  - It should be noted that:

    - It is agreed that “is agreed that SBFD is possible to be used on any TDD band”, and in general, we shall discuss a general principle to handle this, rather than discussing band by band.

    - Even with the restriction of “channel BW no less than X=20MHz”, some of TDD bands are precluded, which is not far from Charter’s proposal

- Ad-Hoc Discussion:

  - Charter: FCC may not require the coordination between operators for TDD sync.

  - QC: confused about the proposal, which is not aligned with the co-existence study results.

  - Samsung: During SI, already clarified SBFD only on DL slots/symbols. The problem here also exists for unsync TDD. The WID already clarify this, and this problem here is not relevant to SBFD.

  - CableLabs: Support the proposal because the channel bandwidth is 10MHz, which means the guardband between UL and DL subband will limits the benefits for SBFD.

    - Samsung: SBFD is already considered to be deployed in a band with channel BW larger than X MHz.

  - Verizon: Have spectrum in CBRS and C-bands, we don’t prefer to exclude SBFD from these bands, which will make SBFD not usable.

  - CATT: No need to introduce such generic principle which also restrict the usage of SBFD in other regions.

  - Huawei: SBFD is already on DL slots, which already solves the issue.

  - CMCC: We don’t prefer the restriction in C-bands.

- Agreement:

  - N/A