# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416556

Hefei, China, 14th – 18th Oct, 2024

Agenda item:			6.8.5

Source:	Samsung

Title:	Way Forward for [112bis][321] NTN_testing_NGSO_channel_model

Document for:	Approval

Introduction

This t-doc provides WF for [112bis][321] NTN_testing_NGSO_channel_model.

WF

Sub-topic 1-1 TE emulated channel model for satellite mobility

Issue 1-1-1 Methodology for Time varying Doppler Shift and propagation delay modelling

- WF

- RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay model matching a satellite motion with following option:

  - Satellite motion emulated using the two-body Keplerian propagator model, [i.e., spherical gravity model]

    - RAN4 further discuss how to derive the Doppler shift and propagation delay variation based on Keplerian model with the necessary parameters and details procedure

    - Companies are encouraged to provide the two-body Keplerian propagator model to be implemented in the TE, to derive the Doppler shift and propagation delay.

Issue 1-1-2-1: Assumption of Satellite altitude

- WF

- Prioritize the LEO-600 orbit for TE emulated channel modelling

- FFS on the LEO-1200 orbit for TE emulated channel modelling pending on the test purpose

Issue 1-1-2-2: Assumption of UE motion & UE position

- WF

- Same UE location information should be set for both DUT and TE before the test start with the following initial position

- Assuming UE is stationary during the demodulation and RF requirement testing

- FFS on the assuming UE is in motion during the RRM requirement testing

Issue 1-1-2-3: Assumption of Elevation angle range

- WF

- Option 1

  - In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE.

      - This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator.

      - The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test.

- Option 2: To align with Rel-17 NR NTN channel model discussion, the elevation angle with 30 degrees and 90 degrees can be considered for testing. Large testing period is required for IoT NTN testing

- Other options are not precluded.

Issue 1-1-2-4: Assumption of feeder link delay

- Agreement

- Do not consider feeder link delay

Issue 1-1-2-5: Assumption of updated granularity for Satellite ephemeris information

- WF

- Companies are encouraged to provide the view for RAN4#113

  - Satellite ephemeris information will be updated at least every 10.24 seconds as the worst case according 38.331

  - NOTE: other (smaller) value may be considered depending on the test purpose

Issue 1-1-2-6: Assumption of TE updated granularity for Doppler and timing offset

- WF

- The Doppler shift and propagation delay should be calculated and updated based on agreed TE emulated channel model

  - TE vendors are encouraged to provide the feedback about the implementation feasibility to update the Doppler shift and propagation delay per slot

Issue 1-1-2-7: Assumption of Multiple satellite scenario

- WF

- RAN4 focuses TE emulated channel modeling for one satellite

- FFS on the TE emulated channel modelling for multiple satellite scenario, considering the serving satellite changes.

Issue 1-1-3 Time varying of doppler shift and propagation delay generation procedure

- WF

  - RAN4 further discuss on general test principle and how to generate channel model to match with satellite motion trajectory with the following options

    - Option 1: The entire N orbits within a test time of X seconds are generated as follow

      - The initial ephemeris for each orbit is generated by either GMAT or any numerical method.

      - The initial ephemeris for each orbit should be chosen such that the elevation angle from the UE perspective is not smaller than Z degrees. The value of Z may vary for different test cases. And if multiple satellites (i.e. N > 1) are needed in RRM mobility/measurement test cases, the value of Z may be different for different satellites.

      - The ephemeris data after the initial data is generated based on the two-body Keplerian model at every Y sec. The value of Y may differ for NR NTN and IoT NTN.

      - The propagation delay and Doppler shift at every Y sec are derived based on the projected ephemeris data and given UE location

      - In case Y is larger than the slot length of interest in the test case due to, e.g., test equipment/channel emulator complexity, the propagation delay and Doppler shift at the beginning of each slot can be generated by applying, e.g., linear interpolation between the propagation delays and Doppler shifts at Y sec intervals.

    - Other options are not precluded

Sub-topic 1-2 RAN4 requirements and tests

Issue 1-2-1 UE assumption on propagator model for satellite motion for RAN4 simulation purpose

- WF

  - Option 1: Consider Keplerian model for UE assumption on the propagator model

  - Option 2: Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span.

Issue 1-2-2 RF requirements

- Agreement

  - Once sufficient progress made for channel model introduction, RAN4 can start the discussion on corresponding requirements and tests according to WI objective

Sub-topic 1-3 Inclusion in Specfication

Issue 1-3-1 Specification of Channel Model

- WF

  - RAN4 further discuss how to incorporate the channel model into specification pending on channel model discussion progress.

- Reference

[1] R4-2416542, “Topic summary for [112bis][321] NTN_testing_NGSO_channel_model “, Moderator (Samsung)

- Appendix (for information)

  - SIB19 NTN Ephemeris Information for zero Doppler conditions specified in TS 38.508

Table 5.6.2.1-1: SIB19- NR NTN Ephemeris information for GSO satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

SIB19 NTN Ephemeris Information for zero Doppler conditions for UL frequency error test specified in TS 38.521-5

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 |  |  |
| positionY-r17 | 4550419 |  |  |
| positionZ-r17 | 852799 |  |  |
| velocityVX-r17 | 6164 |  |  |
| velocityVY-r17 | -19424 |  |  |
| velocityVZ-r17 | 124281 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 |  |  |
| positionY-r17 | 3404229 |  |  |
| positionZ-r17 | 3535794 |  |  |
| velocityVX-r17 | 35394 |  |  |
| velocityVY-r17 | -74414 |  |  |
| velocityVZ-r17 | 94682 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

Table 6.4.1_2.4.3-3: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler/2)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2592823 |  |  |
| positionY-r17 | 4245650 |  |  |
| positionZ-r17 | 2024520 |  |  |
| velocityVX-r17 | 19359 |  |  |
| velocityVY-r17 | -43278 |  |  |
| velocityVZ-r17 | 116553 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 60.25 degrees, one-way delay equal to 2.30 ms and Doppler equal to 11.29 ppm. |  |  |  |

UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0