3GPP TSG-RAN WG4 Meeting#112bis                              R4-2416573

Hefei, China, 14th – 18th Oct, 2024

Agenda item:	6.11.5

Source: 	ZTE Corporation, Sanechips

Title: 	         TP to TR 38.908 clause 6 – Conformance testing

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In this contribution, we provides some draft TP for TR 38.908.

TP

6.1	General

In this section, it mainly provides some technical background for EEIRP conformance testing from the following issues:

- The calculation for EEIRP mask

- Testing beams including both the number of test beams and also weighting factor

- Spatial sampling grid

- The method of test including both testing conditions and also test procedure

- Measurement uncertainty

- Confidence interval

- Other

## 6.2	The calculation for EEIRP mask [Nokia]

    - 6.2.1. Angle definition

The angles in the spherical coordinate system used in this section is as shown in figure 6.2.1-1, and the arrow indicates the direction of increasing angle from the base station perspective:

Figure 6.2.1-1: Angle definition for EEIRP

    - 6.2.1 Averaging over horizontal and vertical angles

One of the average calculations for the Expected EIRP is averaging the EIRP over horizontal and vertical angles. There are seven elevation angular ranges with different elevation bin sizes. Each bin is affectively a spherical strip (or spherical cap for the last range) bounded by θHL and θHH as shown in Figure 6.2.1-1.

Figure 6.2.1-1: Spherical strip (blue shade)

The average EIRP for each elevation bin is defined as:

EIRPθL , θH=12πsinθH−sinθLθLθH−ππEIRPθ,φ cosθ dφ dθ        ,                1

The EIRP EIRPθ,φ is be measured in discrete steps and equation (1) needs to be written in discrete form, and the next section describes how this is considered.

It should be noted that all the averaging is done on linear units.

    - 6.2.1.1 Approximating the double integral in the e.i.r.p. averaging over an elevation angle range

The EEIRP evaluated over a spherical strip involves taking EIRP measurements, EIRPθ,φ, around the ‘point source’ (i.e. the Base Station) bounded by the elevation range θL and θH. Figure 5.1.1.1-1 is an illustration of the measured e.i.r.p. points and the area of the spherical strip for one of the elevation bin. Note that the measured EIRP points are over two discrete elevation angles with equal number of points in azimuth.

Figure 6.2.1.1-1: Measured EIRP points and the spherical strip

The double integral in equation (1) can be approximated in discrete form, and Figure 6.2.1.1-2 illustrates some of the relevant discrete parameters used in the approximation. It is important to note that this approximation is based on equal grid method.

Figure 6.2.1.1-2: Illustration of the parameters involved

For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [0 to π/2]) and azimuth φ (over the range of [-π to π]), and with N and M points respectively, this average e.i.r.p. can be approximated to:

EIRPθL , θH=12πsinθH−sinθLθLθH−ππEIRPθ,φ cosθ ∆φ ∆θ                      (2)

By substituting the following:

dφ≈∆φ=2π÷M

dθ≈∆θ=π2÷N

the overall equation can be written as:

EIRPθL , θH=π2MNsinθH−sinθLn=nLnHm=1MEIRPθn,φm cosθn    ,          (3)

where

θn	is the midpoint between the elevation intervals

φm	is the midpoint between the azimuth intervals

nL	is the lowest elevation point within the θL , θH bounding range

nH	is the highest elevation point within the θL , θH bounding range

m	is the azimuth point within the −π , π bounding range

Note: the equation is assuming the angles in radians and the elevation angle is from the geographical horizon rather than from the antenna panel as used in 3GPP technical specifications previously. Anyhow, these different angle definitions are related and can be easily mapped.

    - 6.2.1.2 Example illustration of average e.i.r.p. for an elevation angular range

This example uses the ‘elevation angular range’ of 30° ≤ θ < 60° (bin number 6) as defined at WRC-23 for ITU-R Resolution 220 [x]:

| Elevation angular range [θL ≤ θ < θH] (Degrees) | Bin number |
| --- | --- |
| 0<<5 | 1 |
| 5<<10 | 2 |
| 10<<15 | 3 |
| 15<<20 | 4 |
| 20<<30 | 5 |
| 30<<60 | 6 |
| 60<<90 | 7 |

Table 6.2.1.2-1: The elevation angular ranges defined in ITU-R Resolution 220 [x]

- Let’s say we have a 10° interval for both azimuth and elevation. This will result to M = 36 (i.e. 360°/10°), and N = 9 (i.e. 90°/10°) points.

- Let’s consider the spherical strip between θL = 30° and θH = 60° (bin number 6)

- This will result to:

  - θn = 35°, 45° and 55°

  - φm = -175°, -165°, -155°, …, 155°, 165° and 175°

  - nL = 35°

  - nH = 55°

  - m = 1, 2, 3, …, 35, 36.

- Note: equation is in radians

- Figure 6.2.1.2-1 is the illustration of this Example

Figure 6.2.1.2-1: Illustration of the Example

It is worth noting that this Example shows that we have 3 vertical sets of e.i.r.p. points (i.e. θn = 35°, 45° and 55°) for 1 of the elevation angle range, i.e. bin number 6.

6.3	Test beams [Ericsson]

6.4	Spatial sampling grid [ZTE]

6.5	Method of test [Huawei]

6.5.1	Initial conditions

The test environment: Normal, see TS 38.141-2, Annex B.2.

RF channels to be tested for single carrier: B and T; see TS 38.141-2, subclause 4.9.1. It is sufficient to test only one carrier, the one characterised by all of requirements below:

- widest supported carrier bandwidth

- largest supported Power Spectral Density (PSD) level.

Beams to be tested: For each declared CAR test specified test beams, as described in TS 38.141-2, Annex N.

The test object shall be placed on the positioner with mechanical tilt set to 0 degrees.

Since this requirement is targeting the power transmitted above the horizon for a specific site installation, it is required that the BS is mounted on the positioner such that blocking effects due to the positioner are minimized.

6.5.2	Test procedure

- Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3.

- Place the test object on the positioner, such that blocking effect in the vertical domain shall be minimized.

- Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

- Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

- Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1..N.

- Enable test beam  k for k=1..K.

- Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

- Repeat step 6 and 7 for all K test beams.

- Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

- Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in TS 38.141-2, Annex N.

6.6	Measurement uncertainty [Ericsson, Huawei]

For the measurement uncertainty of EEIRP mask requirement, considering the number of test beams and also spatial sampling grid for the EEIRP calculation, additional EEIRP summation error 1dB is assumed on top of the legacy TRP measurement uncertainty estimation. The detailed MU budget for EEIRP calculation could be found in the following table 6.6-1.

Table 6.6-1. MU budget for CATR MU value derivation [tr37.941]

| Table 11.2.3.3-1: CATR MU value derivation for OTA BS output power measurement, FR1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤7.125 GHz |  |  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤7.125 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) | 0.30 | 0.30 | 0.30 | Rectangular | 1.73 | 1 | 0.173 | 0.173 | 0.173 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.140 | 0.260 | 0.260 |
| A2-2a | Standing wave between BS and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | 1.41 | 1 | 0.148 | 0.148 | 0.148 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.001 | 0.001 | 0.001 |
| A2-4a | QZ ripple experienced by BS | 0.09 | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.093 | 0.093 | 0.093 |
| A2-12 | Frequency flatness of test system | 0.25 | 0.25 | 0.25 | Gaussian | 1.00 | 1 | 0.250 | 0.250 | 0.250 |
| A2-17 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | 0.51 | Gaussian | 1 | 1 | 0.51 | 0.51 | 0.51 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 | 0.20 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 0.13 | 0.33 | 0.33 | U-shaped | 1.41 | 1 | 0.09 | 0.23 | 0.23 |
| A2-6 | Insertion loss of receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | 1.73 | 1 | 0.10 | 0.10 | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.02 | 0.02 | 0.02 | U-shaped | 1.41 | 1 | 0.02 | 0.02 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) | 0.50 | 0.50 | 0.50 | Exp. normal | 2.00 | 1 | 0.25 | 0.25 | 0.25 |
| A2-9 | Rotary joints | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.03 | 0.03 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 | 0.26 | 0.26 | Rectangular | 1.73 | 1 | 0.15 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.78 | 0.84 | 0.84 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 1.53 | 1.65 | 1.65 |
| EEIRP Summation error |  |  |  |  |  |  |  | NA | NA | [1.0] |
| Total MU |  |  |  |  |  |  |  | NA | NA | 2.10 |

* note there may be an error in A2-5a, the value in the SEM TRP budget is larger – check this value

6.7	Confidence intervals [Spark]

TBD

6.8	Testing requirements

Since the EEIRP mask requirement defined in WRC-23 is the regulatory requirement from RAN4 standardization perspective, the test tolerance requirement should be zero without any further relaxation on top of regulatory limits.

6.9	Others