# 3GPP TSG-RAN WG4 Meeting #112bis	                 R4-2416580

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.13.3

Source:	Qualcomm Incorporated

Title:	WF on [112bis][325] NR_FR2_OTA

Document for:	Approval

# Topic #1: Test method for STxMP

Issue 1-1-1: Default AoA separation and UE orientation for EIRP PUMAX,f,c,k testing

- Agreements:

  - RAN4 confirms the baseline multi-RX/STxMP system is suitable to guarantee the same relative angular separations between specific UE test orientations and probes.

  - The following test procedure can be considered as the solution to select default AoA separation and UE orientation EIRP PUMAX,f,c,k testing:

    - AoA1 direction for STxMP testing should re-use the legacy beam peak direction (from single carrier test) for the declared UE orientation (selected from any of the approved STxMP device orientations

      - For AoA1, no new BP search is necessary for device orientations used for STxMP test that have not been used for single carrier test

    - AoA2 direction is offset by AoA1 by the declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system

Issue 1-1-2: Whether a test direction other than the beam peak direction of single CC operation without STxMP can be used for first AoA

- Agreements:

  - The test direction other than the beam peak direction of single CC operation without STxMP can be used for the first AoA if the test results based on the default AoA separation fail the limits in the spec. The following options can be considered:

    - Option 1: A local scan in xz plane is conducted. As long as test results do not pass the spec yet during scan, the local scan does not stop until AoA1 EIRP fail the legacy beam peak EIRP limit in both directions. FFS on the measurement grid of local scan.

    - Option 2: Alternative test directions on top of the default AoA separation can be declared by UE. FFS on the allowed offset of alternative test directions to the default test directions.

  - Option 1 and Option 2 only apply for EIRP PUMAX,f,c,k testing and will not have the impact on the EIRPmax and TRPmax test directions selection.

  - RAN4 to make down selection from Option 1 and Option 2 in next meeting

Issue 1-2-1: AoA separation and UE orientation EIRPmax testing

- Agreements:

  - The following two sets of AoA separation and UE orientation needs to be tested for EIRPmax compliance

    - The default AoA separation and UE orientation declared by UE for EIRP PUMAX,f,c,k testing.

    - For EIRPmax test, one of AoA is at the beam peak direction of single CC operation without STxMP enabled, the other AoA is at the direction decided by UE declared orientation (listed in TS38.101-2) and 30deg AoA separation.

Issue 1-2-2: Additional margin for skipping rule for EIRPmax testing

- Agreements:

  - 0.5dB as an additional margin for EIRPmax test skipping rule.

Issue 1-3-1: AoA separation and UE orientation TRPmax testing

- Agreements:

  - Default AoA separation and UE orientation for EIRP PUMAX,f,c,k testing should be reused for TRPmax testing.

Issue 1-3-2: Additional margin for skipping rule for TRPmax testing

- Agreements:

  - 0.5dB as an additional margin for TRPmax test skipping rule.

Issue 1-3-3: Measurement grid for TRPmax testing

- Agreements:

  - Reuse the legacy TRP grid for TRPmax testing of STxMP

Issue 1-4-1: Link polarization combinations

- Agreements:

  - Further discuss the link polarization combinations for EIRP PUMAX,f,c,k, EIRPmax and TRPmax testing.

  - The following options can be considered:

    - Option 1: Limit the link polarization combinations for the 2AoA, and crossed link polarization (‘’ and ‘’) can be considered for the sake of lower interference

    - Other options are not precluded.

Work split for TR 38.771

| Section | Title | Company |
| --- | --- | --- |
| 5.1 | General | Qualcomm |
| 5.2 | Measurement setup | Keysight |
| 5.3 | Test system aspects | Keysight |
| 5.4 | Test procedure | vivo, Samsung, Qualcomm |
| Annex A | Measurement uncertainty budget for STxMP UE RF testing methodology | Keysight |