3GPP TSG-RAN WG4 Meeting #112bis                                                          R4-2416584

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.20.4

Source:	Samsung

Title:	Way forward for [112bis][306] NR_duplex_evo_General

Document for:	Approval

# Introduction

This document is provided for capturing agreements and way forward on the thread [306] NR_duplex_evo_general based on the discussion in RAN4#112-bis.

# Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

### Sub-topic 1-1: SBFD frequency domain issues

##### Issue 1-1-1: Minimum channel bandwidth for SBFD (X MHz)

Agreement:

- Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle:

  - The supported channel bandwidth(s) is declared by gNB vendors

  - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

  - The supported channel bandwidth shall be no less than X MHz

    - X = 40 as the default value

      - For band n39, X = 20MHz is used instead.

##### Issue 1-1-3: Specification implementation for band/channel bandwidth support for SBFD

Agreement:

- To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD.

  - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD

  - FFS the band for NR-U shall be supported

##### Issue 1-1-4: Transmission bandwidth configuration for SBFD

Way Forward:

- Illustrative figures from Ericsson will be used as the baseline for further study:

  - FFS the restriction of SCS for DL and UL subbands

  - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL

  - FFS other potential issues.

##### Issue 1-1-5: DL/UL subband configurations support for SBFD

Way Forward:

- RAN4 to further study Option 2b by considering the below additional restrictions such as:

  - For DUD case, the UL subband should be placed in the middle of channel bandwidth

  - FFS the number of DL subband sizes to be declared and tested

    - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed.

      - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS.

    - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested

      - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS.

  - Restriction on UL subband size based on the specific channel bandwidth

    - FFS to specify a range of ratio for UL subband size over channel bandwidth.

    - FFS the restriction is different for DU and DUD

    - FFS the restriction is applied for FR2 case.

### Sub-topic 1-2: Other general issues for SBFD requirements

##### Issue 1-2-1: BS RF Specification structure for SBFD requirements

Way Forward:

- FFS options of how to introduce BS RF new requirements for SBFD-capable BS:

  - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD

  - Option-3: A new specification for SBFD BS RF requirements

# Topic #2: Potentially new requirements for SBFD

## Open issues summary

### Sub-topic 2-1: Requirements for Transient period between SBFD and non-SBFD

##### Issue 2-1-1: Transient period requirement applicability on frequency/antenna

Way Forward:

- FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498.

##### Issue 2-1-2: Location of transient period between SBFD and non-SBFD

Way Forward:

- FFS location of transient period between SBFD and non-SBFD:

  - Option 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not.

  - Option 2: Place the transient period outside of both SBFD and non-SBFD slots.

  - Option 3: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol).

### Sub-topic 2-2: In-channel adjacent subband leakage ratio requirements

##### Issue 2-2-1: Necessity of in-channel adjacent subband leakage ratio requirements

[Background] In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

Way Forward:

- FFS based on the way forward approved in RAN4#112.

### Sub-topic 2-3: In-channel adjacent subband selectivity/blocking requirements

##### Issue 2-3-1/2-3-2: Necessity of in-channel adjacent subband selectivity/blocking requirements

[Background] In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline. - Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

Way Forward:

- FFS based on the way forward approved in RAN4#112.

  - Consider the possibility of specifying RX requirement to cover both in-channel adjacent subband selectivity/blocking requirements