3GPP TSG-RAN WG4 Meeting #112bis		R4-2416847

Hefei, China, 14 – 18, 2024

Agenda Item:	6.24.2.1

Source: 	Apple

Title:	Discussion on measurements related enhancements for LTM

Document for:	Approval

# Introduction

Measurement related enhancement for LTM has been widely discussed in previous RAN4 meeting. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

## Event triggered L1 reporting

Issue 2-2-1: RRM scope of Event triggered L1 measurement reporting

- Candidate solutions:

  - Option 1: RAN4 need to discuss the absolute threshold for beam of serving cell and candidate cell in L1 LTM measurement events, including the measurement matrix and the value. RAN4 can wait for more RAN2 input. (CATT)

  - Option 2: (Apple)

    - Requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting. Details are FFS.

    - Existing framework in L3 event triggered reporting RRM requirements can be reused to cover L1 event triggered reporting.

  - Option 3: define measurement reporting requirements for event triggered L1 measurement reporting, e.g. the L1 measurement reporting delay. (CMCC, Apple, Samsung, HW, ZTE, Nokia, E///, vivo,CTC)

    - Option 3a: “Event Triggered Reporting” for LTM L1 measurement is to be introduced in section 9.14.3 and section 9.15.3.

    - Option 3b: RAN4 can start the discussion based on SSB firstly. Further RAN1/RAN2 progress are needed. (Samsung)

    - Option 3c: RAN4 to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements. (Nokia, E///, vivo, CMCC)

  - Option 4: RAN4 should discuss the accuracy requirement for event triggered L1 measurement reporting. (ZTE)

  - Option 5: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements, for example (Nokia)

    - Known TCI state condition for early TCI state activation and cell switch

    - Applicability of cell switch delay requirements

    - early DL sync (Tfirst-RS = 0 conditions)

    - early ASN.1 decoding and validity check

    - LTM L1 measurement requirements

  - Option 6: For the topic of ‘Measurements related enhancements for the purpose of supporting LTM,’ RAN4 to wait for further progress to be made in other working groups under this WI . (QC, vivo)

Regarding option 1, we don’t think RAN4 need to discuss absolute threshold for beam of serving cell and candidate cell in L1 LTM measurement events. It is our understanding that threshold shall be configured by network.

Option 2 is from our proposal. In RAN4#112 we encouraged companies to discuss whether it is necessary to update the requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133 section 9.1.4 to cover event triggered L1 measurement reporting. If needed, we would like to propose the following update on table 9.1.4.2-1:

Table 9.1.4.2-1: Requirements for reporting criteria per measurement category

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5 | 10 | Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5 | 1 | Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5 | 1 | Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| DL RSTD Note 2,4,5 | 1 | DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34]. |
| UE Rx-Tx Note 2,4,5 | 1 | UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34]. |
| DL PRS-RSRP Note 2,4,5 | 1 | DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| DL PRS-RSRPP Note 2,4,5 | 1 | DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| SRS-RSRP Note 2,3,4,5 | 1 | SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16. |
| CLI-RSSI Note 2,3,4,5 | 1 | CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16. |
| NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode. |  |  |

Proposal 1: discuss and confirm in RAN4#112 that whether requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell. If needed, it is proposed to use above table as baseline.

Option 3 is straightforward, RAN4 needs to define measurement reporting requirements for event triggered L1 measurement reporting, e.g. the L1 measurement reporting delay. For intra-frequency, new requirements shall be introduced in clause 9.14.3, while for inter-frequency new requirements shall be introduced in 9.15.3.

Proposal 2: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency.

Regarding option 4, for SSB based L1 RSRP measurement, existing LTM L1-RSRP accuracy requirements can be reused. For CSI-RS based L1 RSRP measurement, it will be discussed in CSI-RS based L1 RSRP agenda. Whatever agreed under that agenda can be reused here in reporting requirements.

Proposal 3: for SSB based L1 RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R18 LTM L1-RSRP accuracy requirements

Proposal 4: for CSI-RS based L1-RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R19 CSI-RS based L1-RSRP accuracy requirements for LTM, which will be discussed under CSI-RS based L1-RSRP measurement agenda.

Issue 2-2-2: measurement result for event evaluation

- Candidate solutions:

  - Option 1: Use beam level measurement result for event evaluation as baseline and FFS for cell level measurement. (CTC)

In RAN2#127, it was agreed that:

Agreements on L1 MR event evaluation

1.     Event LTM1 is not defined.

2.     Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

3.     Any beam in candidate RS configuration can be used for LTM event evaluation.

4.     Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

5.     R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Therefore, RAN4 doesn’t need to discuss this issue.

Proposal 5: RAN4 doesn’t need to discuss whether to consider cell level measurement result for event evaluation, since RAN2 already agreed to use beam level measurement only.

Issue 2-2-3: filtering assumption in event evaluation

- Candidate solutions:

  - Option 1: Define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. Wait for more RAN1/2 progress to discuss the detailed requirements. (MTK)

So far it is premature to conclude that additional SSB based L1 measurement delay requirements are necessary. It highly depends on filtering design. For instance, if it is something similar to measurement filtering coefficient for L3 measurement, then legacy L1 measurement delay requirements can still apply.

Observation 1: it is premature to conclude that additional SSB based L1 measurement delay requirements are necessary. It highly depends on filtering design.

Issue 2-2-4: L1 event triggered reporting delay

- Candidate solutions:

  - Option 1: The event triggered L1 measurement reporting delay, measured without L3 filtering, is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM. (CMCC)

  - Option 2: For the delay of event triggered L1 measurement reporting requirements based on SSB, current LTM L1-RSRP measurement delay can be reused. (Samsung)

  - Option 3: Starting point for the SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap. CSI-RS based delay is pending on Rel-19 work. (Nokia)

  - Option 4: RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (E///)

In event triggered reporting delay requirements, we need to define a starting point and reporting delay. The starting point is proposed as the time when a condition exists at the UE which will trigger the reporting. Note that it is from channel condition point of view. UE may not realize that immediately due to measurement delay. Same as legacy the maximum reporting delay shall be one L1-RSRP measurement period.

Proposal 6: in event triggered L1 reporting requirements, RAN4 needs to define a starting point and reporting delay.

Proposal 7: starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting. Reporting delay is defined as one L1-RSRP measurement period on target cell.

## Preliminary analysis for CSI-RS based measurement

### Pre-requisite for CSI-RS based L1-RSRP

In our view, it is only meaningful to perform CSI-RS based L1-RSRP measurement for neighbour cells that have good enough quality. The cells for L1-RSRP measurement could be selected from those ranked based on L3 measurement. So, the next question is which type of L3 measurement should be used to determine the ranking for target cells for CSI-RS based L1-RSRP measurement. In the current RRM spec, there are SSB based L3 measurement and CSI-RS based L3 measurement. From technical perspective, both can be used as the pre-requisite condition for cell quality ranking. While we all know that CSI-RS based L3 measurement is only specified assuming the existence of associated SSB and it is not a standalone measurement. In this regard, using CSI-RS based L3 measurement as condition will unnecessarily complicate the measurement design. Our view is that it is sufficient to only use SSB based L3 measurement as the prerequisite condition to determine the target cells for CSI-RS based L1-RSRP measurement. In this case it is by default to consider the scenario for CSI-RS based L1 measurement with associated SSB, which is also aligned with L3 CSI-RS measurement principle in terms of timing acquisition.

Proposal 8: It is proposed to consider SSB based L3 measurement as the pre-requisite condition to determine the target cells for CSI-RS based L1-RSRP measurement.

### Definition of intra-frequency and inter-frequency

The following options were put on table after discussion in the last RAN4 meeting.

Issue 2-3-2: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement

- Option 1: following CSI-RS based L3 measurement, (Apple)

  - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

      - It is applied for SCS = 60KHz

    - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

- Option 2: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. (MTK)

- Option 3: For CSI-RS based L1 measurement, RAN4 not to introduce definition of intra-frequency/inter-frequency candidate cell measurement. (vivo)

For SSB based L1-RSRP measurement for LTM, it was decided to follow the definition of L3 measurement.

Following the same logic, for CSI-RS based L1 RSRP measurement, we think it’s reasonable to follow the intra-frequency/inter-frequency definition for CSI-RS based L3 measurement. i.e. a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement.

One concern during the last meeting is that there is no MO for L1 measurement which may impact on how to descript the definition. While we think this is not an issue since anyway the CSI-RS resource will be allocated with clearly frequency location and bandwidth. We don’t see any issue to have a definition even though no MO is defined for L1 measurement.

Option 2 and option 3 based on the motivation to avoid intra-frequency/intra-frequency definition for CSI-RS based L1 measurement. While we doubt it’s a good choice to do so for CSI-RS based L1 measurement only. The past measurement requirements are all classified based on intra/inter-frequency definition. we should avoid changing the specification logic too much. Otherwise, it will be a lot of unnecessary effort on requirement introduction and eventually the specification readability may be unnecessarily impacted.

Proposal 9: it is proposed to follow intra-frequency/inter-frequency definition for CSI-RS based L3 measurement, e.g. a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

Since definitions of intra/inter-frequency measurement have impact on RAN2, we also prepare a draft LS in Annex.

### Scenarios to be considered

#### Frequency range

It seems there is no strong motivation to consider FR2-2 in CSI-RS based LTM and L1-RSRP measurement, and SSB based LTM and L1-RSRP measurement does not consider FR2-2 as well. Hence, we suggest only focusing FR1 and FR2-1.

Proposal 10: It is proposed to only consider FR1 and FR2-1 for CSI-RS based LTM and L1-RSRP measurement.

#### Measurement type

The following was captured in the WF agreed in RAN4#112 meering,

Issue 2-3-3: supported measurement types

- Option 1: (CATT)

  - Intra-frequency

  - Inter-frequency without gap

  - Inter-frequency with gap

- Option 2: (Apple)

  - Intra-frequency without gap

  - Inter-frequency with gap

- Option 3: deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK)

- Option 4: (vivo)

  - For CSI-RS based L1 measurement on candidate cell with periodic reporting, RRM requirements are specified for the following cases:

    - CSI-RS based L1 measurement without gap

L3 based CSI-RS only considered intra-frequency without GAP and inter-frequency with GAP. As for CSI-RS based L1 RSRP measurement, it is suggested to follow the same logic as CSI-RS based L3 measurement to prioritize intra—frequency measurement without GAP and inter-frequency measurement with GAP as starting point. Inter-frequency measurement without GAP seems beneficial from measurement delay perspective. While CSI-RS resource configuration is quite flexible and complicated. Our preference would be to deprioritize inter-frequency measurement without GAP before resource configuration issue is clear.

Also, in the last RAN1 meeting, it was agreed to focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement. RAN4 could follow RAN1 decision on this aspect.

Proposal 11: It is proposed to prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP for CSI-RS based L1 measurement.

Proposal 12: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

#### RTD assumption

It is possible that RTD between serving cell and neighbor cell on the same carrier is larger than CP from deployment perspective. While it will put a lot complexity for UE implementaiton complexity in terms of inreased number of FFTs. We think RTD<CP should be taken as the baseline. It is noted that R18 SSB based L1 RSRP measurement on neighbor cell already addressed the case when RTD is larger than CP by UE capability. UE capabiltiy should also be considerd if RAN4 agrees to consider the case where RTD is larger than CP between serving cell and neighbour cell on the same carrier.

Proposal 13: RTC<CP is taken as baseline. UE capability shall be considered if RAN4 agrees to consider the case where RTD is larger than CP between serving cell and neighbour cell on the same carrier.

#### CSI-RS resource configuration

The following was agreed in the WF from RAN4#112.

Issue 2-3-6: CSI-RS resource configuration

  - Option 1: the same CSI-RS resource configuration {D=3, PRB≧48} as for CSI-RS based L3 measurement is considered as a starting point. FFS on resource location limitation. (Apple)

Option 1a: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirements for Density=3 with (≥) 48 PRBs and not to define tighter requirements for more 48 PRBs. (MTK)

The two options are same in essence, and we also agree to avoid tighter requirements for more PRBs than 48. According to legacy physiscal layer design, CSI-RS can be allocated with different PRB width, e.g. PRB= RB = {24，48，96，192，256} with different RE density within each PRB, D = {0.5，1，3}. RAN2 further limited D = {1，3} configuration for RRM measurement. It is unclear whether RAN1 would have some new restriction for L1 measurement on neighbour cell. It means CSI-RS resource configuration can be very flexible. In Rel-16 CSI-RS based L3 measurement discussion, several confiurations has been discussed. In the end, only {D=3, PRB≧48} was chosen for requirement development based on trade off between measruement accuracy and UE power consumption. We propose to reuse the same resource configuration of {D=3, PRB≧48} as a starting point.

Proposal 14: The same CSI-RS resource configuration {D=3, PRB≧48} as for CSI-RS based L3 measurement is considered as a starting point. FFS on resource location limitation.

#### Know/unknown cell

From LTM delay and measurement delay perspective, it is suggested to consider know cell only since the measurement delay will be quite long for unknown cell. In Rel-18 SSB based L1-RSRP discussion, the cell is considered as known if the following conditions are met:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements in clause 9.2.

When it comes to CSI-RS based L1-RSRP measurement, the question is whether we choose CSI-RS based L3 measurement during the last 5s or SSB based L3 measurement during the last 5s as the decisive condition for known cell. In Rel-16, CSI-RS based L3 measurement was introduced by only assuming associated SSB timing as pre-requisite condition and CSI-RS cannot be used for neighbour cell measurement as a standalone reference signal. Hence, CSI-RS based L3 measurement is not the good choice to determine the known cell condition. We propose that the pre-requisite condition for CSI-RS based L1-RSRP measurement is also based on whether UE has performed SSB based L3 measurement in the last 5s.  For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met,

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements in clause 9.2.

-	The CSI-RS from the target cell configured for L1 measurement remains detectable.

Proposal 15: It is proposed to consider know cell case only.

Proposal 16: The target cell is considered as known provided that:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements in clause 9.2.

-	The CSI-RS from the target cell configured for L1 measurement remains detectable.

#### Accuracy

In legacy CSI-RS L1 measurement accuracy is defined as ±5dB at SINR=-3dB (48 PRBs and the density is 3, one sample). It is suggested to reuse the existing side condition for CSI-RS based L1-RSRP measurement for serving cell as the starting point for CSI-RS based L1-RSRP measurement for neighbour cell.

Proposal 17: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point.

# Summary

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Proposal 1: discuss and confirm in RAN4#112 that whether requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell. If needed, it is proposed to use the following table as baseline.

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5 | 10 | Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5 | 1 | Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5 | 1 | Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| DL RSTD Note 2,4,5 | 1 | DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34]. |
| UE Rx-Tx Note 2,4,5 | 1 | UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34]. |
| DL PRS-RSRP Note 2,4,5 | 1 | DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| DL PRS-RSRPP Note 2,4,5 | 1 | DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| SRS-RSRP Note 2,3,4,5 | 1 | SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16. |
| CLI-RSSI Note 2,3,4,5 | 1 | CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16. |
| NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode. |  |  |

Proposal 2: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency.

Proposal 3: for SSB based L1 RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R18 LTM L1-RSRP accuracy requirements

Proposal 4: for CSI-RS based L1-RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R19 CSI-RS based L1-RSRP accuracy requirements for LTM, which will be discussed under CSI-RS based L1-RSRP measurement agenda.

Proposal 5: RAN4 doesn’t need to discuss whether to consider cell level measurement result for event evaluation, since RAN2 already agreed to use beam level measurement only.

Observation 1: it is premature to conclude that additional SSB based L1 measurement delay requirements are necessary. It highly depends on filtering design.

Proposal 6: in event triggered L1 reporting requirements, RAN4 needs to define a starting point and reporting delay.

Proposal 7: starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting. Reporting delay is defined as one L1-RSRP measurement period on target cell.

Proposal 8: It is proposed to consider SSB based L3 measurement as the pre-requisite condition to determine the target cells for CSI-RS based L1-RSRP measurement.

Proposal 9: it is proposed to follow intra-frequency/inter-frequency definition for CSI-RS based L3 measurement, e.g. a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement

Proposal 10: It is proposed to only consider FR1 and FR2-1 for CSI-RS based LTM and L1-RSRP measurement.

Proposal 11: It is proposed to prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP for CSI-RS based L1 measurement.

Proposal 12: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

Proposal 13: RTC<CP is taken as baseline. UE capability shall be considered if RAN4 agrees to consider the case where RTD is larger than CP between serving cell and neighbour cell on the same carrier.

Proposal 14: The same CSI-RS resource configuration {D=3, PRB≧48} as for CSI-RS based L3 measurement is considered as a starting point. FFS on resource location limitation.

Proposal 15: It is proposed to consider know cell case only.

Proposal 16: The target cell is considered as known provided that:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements in clause 9.2.

-	The CSI-RS from the target cell configured for L1 measurement remains detectable.

Proposal 17: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point.

# References

- R4-2414060, WF on NR mobility enhancements Phase 4, Apple, China Telecom

# Annex

3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415521

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	LS on intra-frequency/inter-frequency definition for CSI-RS based L1 measurement

Response to:

Release:	Rel-19

Work Item:	               NR_Mob_Ph4-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Yuexia Song, Yuexia.Song@Apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 discussed intra-frequency and inter-frequency definition for CSI-RS based L1 measurement and concluded that it is sufficient to follow the similar logic as that for CSI-RS based L3 measurement. i.e.

A measurement is defined as an intra-frequency CSI-RS based L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement.

A measurement is defined as an inter-frequency CSI-RS based L1 measurement if the intra-frequency condition is not met.

2	Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to consider the RAN4 decision on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #113                18th – 22nd Nov 2024			Orlando, US

TSG-RAN WG4 Meeting #114                17th – 21nd Nov 2024			Athens, Greece