3GPP TSG-RAN WG4 Meeting #112bis				                                          R4-2416854

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.15.4

Source:	CATT

Title:	WF on RRM requirements for NR_RRM_Ph5_Part2

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [112bis][210] NR_RRM_Ph5_Part2.

- Topic #1: Fast SCell activation for UE supporting Rel-18 EMR

  - Sub-topic 1-1 General

Issue 1-1-1: Applicability of fast SCell activation delay requirements

Proposals:

- Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK)

  - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

- Option 2: (OPPO, Nokia, Samsung, Ericsson)

  - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

- Option 2a: (OPPO)

  - Consider the following two approaches to support the case:

    - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or

    - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration.

- Option 2b: (Samsung)

    - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured

    - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

Issue 1-1-2: Scope of fast SCell activation for UE supporting Rel-18 eEMR

Agreement:

- On top of Single Direct SCell Activation at SCell addition, MAC CE based Single SCell Activation is also supported.

  - Note: MAC CE based PUCCH SCell activation is included.

Issue 1-1-3: Target scenarios of fast SCell activation

Proposals

- Proposal 1: (CMCC, CTC)

  - it is proposed to reduce SCell activation delay for unknown case in FR1 and FR2-1.

- Proposal 2: (Nokia, Ericsson)

  - RAN4 to study both known and unknown conditions for fast SCell activation with Rel-18 EMR.

- Proposal 3: (Qualcomm, Samsung)

  - RAN4 to not consider the following SCells for the eEMR based fast SCell activation requirements:

    - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA)

    - SCell being in the same band as PCell in FR2 (i.e. intra-band CA)

- Proposal 4: (Nokia)

  - RAN4 to consider all possible FR1 and FR2 scenarios for SCell activation, and the work can be initiated for FR1 Pcell + FR2 Pscell/SCell.

- Proposal 5: (Nokia)

  - RAN4 should specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay).

- Proposal 6: (Samsung)

  - RAN4 to consider the following scenarios as the starting point: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1

  - Sub-topic 1-2 SCell activation delay requirements

Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting

Agreement:

- RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception.

  - For FR1,

    - [Y] equals to [5s] for direct SCell activation

    - FFS: Y for other SCell activation case.

  - For FR2,

    - Reuse the existing value of Y in direct SCell activation

    - FFS: Y for other SCell activation case.

Agreement:

- the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting).

  - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

  - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

For continued discussion in next meeting:

- For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with [2] SSB samples.

  - For FR2:

    - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X]

      - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW)

    - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion.

    - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1

      - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured)

    - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT)

      - K = 0 or 1 if  when the “validityduration” is smaller than X

      - K = 1 or 2 otherwise

    - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung)

      - K = 0 or 1 if  when the “validityduration” is applied with smaller than X

      - K = 1 or 2 otherwise

      - FFS how to test

Other proposals:

- Proposal 6: For a UE supporting measValidationReportEMR or measValidationReportReselectionMeasurements, the legacy known conditions with deactivated SCell L3 measurement still apply.

- Proposal 7:

  - In case the SCell has more than one actually transmitted SSB, the report should be based on beam level measurement result, i.e. MeasIdleConfig > MeasIdleCarrierNR > BeamMeasConfigIdle-NR > includeBeamMeasurements = true.

- Proposal 8: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication

  - The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation

    - [Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation

Issue 1-2-2: Consideration on SINR condition during the whole procedure for known SCell activation

Agreement:

- Not to update the SINR condition (i.e., Ês/Iot≥ -2dB) for SCell activation delay requirements.

Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation

Proposals

- Option 1: (CATT)

  - No need to update the measurement period condition for FR1 known SCell activation.

- Option 2a: (Apple)

  - For fast SCell activation based on eEMR, time for AGC is always needed since measurement period in IDLE/INACTIVE mode is longer than 2400ms in most cases.

- Option 2b: (Qualcomm, Nokia)

  - In any case, the SCell activation requirement should allow UE to receive at least one SSB sample based on [SSB period] or [configured SMTC periodicity].

- Option 3: (Samsung)

  - If the SCell is known based on the eEMR based known condition and belongs to FR1, 2400 ms can be modified to Y ms

    - The basic components to define Y and the subsequent impact on AGC training gate for activation shall be discussed.

Issue 1-2-4: Whether the indication to network is needed

Proposals

- Option 1: (CATT, Apple, Samsung)

  - Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation.

- Option 2: (Ericsson, Nokia)

  - RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

Reference

- R4-2415657 Topic summary for [112bis][210] NR_RRM_Ph5_Part2, CATT, RAN4#112bis