3GPP TSG-RAN WG4 Meeting # 112-bis											R4-2416857

Hefei, China, 14 ‒ 18 October, 2024

Title: 	WF on RRM requirements for NR_MIMO_Ph5 (Part2)

Agenda Item:	6.19.4

Source: 	MediaTek

Document for:	Approval

# <Topic 1: CSI enhancement to support up to 128 ports>

Issue 1-1: Whether RRM impacts exist by the added SRS port grouping for CSI enhancement?

In last RAN plenary meeting RAN#105, another sub-bullet was approved for CSI enhancement (Objective#2):

- SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

Way forward:

- Option 1: Previous RAN4 agreement still apply “No RRM impact by CSI enhancement to support up to 128 ports”.

- Option 2: Wait for RAN1.

# <Topic 2: UE reporting enhancement for CJT calibration>

Issue 2-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration?

Agreement:

- RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later.

- Further discuss the simulation assumptions offline during this meeting.

Way forward:

- Companies are encouraged to bring simulation assumption/results in the next meeting. Simulation assumption in Appendix A can be used as a starting point for information only.

Issue 2-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration?

Way forward:

- Option 1: Define accuracy requirements.

- Option 2: FFS.

Issue 2-3: Whether report mapping tables should be specified by UE reporting enhancement for CJT calibration?

Way forward:

- Option 1: Define mapping tables for frequency/delay/phase offset reporting in performance part.

Issue 2-4: On report configuration for CJT calibration:

Way forward:

- TBA

# <Topic 3: 3TX support>

Issue 3-1: RRM core/performance impacts by introducing 3TX support?

Agreement:

- No new RRM core requirements by introducing 3TX support.

# <Topic 4: Enhancement for asymmetric DL sTRP/UL mTRP scenarios>

Issue 4-1: Whether RRM impacts exist by 2TA enhancement?

Agreement:

- RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP]

- As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID.

- Further discuss the details in the next meeting.

Issue 4-2: RRM core impacts of Active uplink TCI state switching delay for unified TCI?

Way forward:

- Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Option 2: RAN4 to specify UL TCI state switching requirements.

- Option 3: FFS.

Issue 4-3: Clarification on UL TCI state activation of asymmetric DL sTRP/UL mTRP:

Way forward:

- TBA

Issue 4-4: Whether the known/unknown definition for UL TCI state activation is impacted?

Way forward:

- TBA

Issue 4-5: Whether to define RRM core requirements of pathloss offset update?

Agreement:

- Previous RAN4 agreement captured below still apply and close the issue.

Issue 2-5-5: Whether to define RRM core requirements of pathloss offset update? Agreement: Whether to define RRM core requirements of pathloss offset update? - Not define RRM requirement for MAC CE decoding delay. - The agreement can be revisited depending on further RAN1 progress.

# <Appendix A>

Simulation assumption below is for information only.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDL-C |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB, 106 RB |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| UE speed | 3km/h, 30kmph, 72kmph, 108kmph |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm. |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples for measurement | 1, 2, 3, 4, 5, 10 |
| CSI-RS periodicity | 20ms / 40 slots |
| CSI-RS offset | Slot 20, 21 |

Evaluation metric for delay offset:

Delay offset error = estimated delay offset – ideal delay difference between TRP.

Delay offset error for one shot measurement and delay offset error for multiple samples measurement are computed.

Evaluation metric for freq. offset:

Freq. offset error = estimated freq. offset – ideal freq. offset between TRP.

Freq. offset error for one shot measurement and freq. offset error for multiple samples measurement are computed.