3GPP TSG RAN WG4 Meeting#112-bis                                                       R4-2416863

Hefei, CN, Oct 14 - Oct 18, 2024

Agenda item:			6.24.3

Source:	China Telecom

Title:	WF on RRM requirements for NR_Mob_Ph4_Part2

Document for:	Approval

Notes

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other options can be proposed.

Topic #1: RRM discussion

Conditional LTM

Issue 1-1-1: RAN4 scope of Conditional LTM

< Agreement>

  - RAN4 to define cell switch delay requirements for conditional LTM

  - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified.

  - Wait for other WGs input to further discuss detailed RRM requirements.

Issue 1-1-2: The applicable scenarios for conditional LTM

< Agreement >

- Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

- Further discuss the DC case if it will be included in the WID based on further RAN plenary discussion.

Issue 1-1-3: Measurement requirements on conditional LTM

< Agreement >

- On whether L3 measurement based conditional LTM in FR1 can be supported

  - RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached.

- As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified.

Issue 1-1-4: Subsequent conditional LTM

< Way Forward > FFS the following proposals:

- Proposal 1: FFS whether there is any difference in the delay for initial and subsequent CLTM cell switch.

- Proposal 2: RAN4 should further clarify the scenario for conditional LTM including subsequent LTM.

  - Proposal 3: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell.

  - Proposal 4: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Issue 1-1-5: The specification structure of the requirements for conditional LTM

< Way Forward > FFS the following options:

  - Option 1: For specification structure, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM.

  - Other option is not precluded.

Conditional LTM delay

Issue 1-2-1: General aspects on Conditional LTM delay

< Way Forward > FFS the following options:

  - Option 1: CHO requirements framework can be used as baseline.

    - Option 1a: Rel-18 LTM requirements framework can be used as references.

  - Option 2: Revisit conditional LTM delay when other WGs achieve further conclusions.

  - Option 3: Wait for RAN2 progress for defining cell switch delay requirements for conditional LTM.

  - Option 4: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario.

  - Option 5: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM.

Issue 1-2-2: The starting point of conditional LTM delay

< Way Forward > FFS the following options:

  - Option 1: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM.

  - Option 2: The starting point of conditional LTM delay is the time when UE transmits an RRCReconfigurationComplete message.

  - Option 3: The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2.

Issue 1-2-3: The ending point of conditional LTM delay

< Way Forward > FFS the following options:

  - Option 1:

    - The ending point of conditional LTM delay is the time when the UE transmits the first UL message on the target cell.

    - FFS the support of RACH-based and/or RACH-less, which is up to RAN1/2 agreement.

    - It can be revisited if any technical issue will be identified considering the progress in other WGs.

  - Other option is not precluded.

Issue 1-2-4: The components of conditional LTM delay

< Way Forward > FFS the following options:

  - Option 1: DCLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TCLTM_execution + Tinterrupt.

  - Option 2: DLTM_Conditional = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

  - Option 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution.

  - Option 4: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU.

    - need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design.

  - Option 5: At least the delay uncertainty (TEvent_DU), the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) also need to be considered for Rel-19 conditional LTM Cell Switch delay.

Issue 1-2-5: TRRC

< Way Forward > FFS the following options:

  - Option 1: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

  - Other option is not precluded.

Issue 1-2-6: TEvent_DU

< Way Forward > FFS the following options:

  - Option 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM.

  - Option 2: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

  - Option 3: Wait for more progress in RAN2.

Issue 1-2-7: Tmeasure

< Way Forward > FFS the following options:

  - Option 1: Tmeasure is measurement time.

    - Option 1a: Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

    - Option 1b: Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes  LTM cell switch to a target cell and interruption time starts.

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

- If L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

  - Option 2: Wait for more progress in RAN2.

Issue 1-2-8: TCLTM_execution / TLTM_execution

< Way Forward > FFS the following options:

  - Option 1: TCLTM_execution / TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

    - Option 1a: TCLTM_execution can be up to [10ms].

  - Option 2: TCHO_execution may need update in conditional LTM.

Issue 1-2-9: Tinterrupt

< Way Forward > FFS the following options:

  - Option 1: Tinterrupt is the time between when the UE starts to execute the conditional LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

  - Option 2: TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch.

  - Option 3: Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PRACH procedure (if supported in conditional LTM).

  - Option 4: The interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline.

  - Option 5: RAN4 to discuss the starting point of the CLTM interruption, and the components included in TCLTM_interrupt. The goal shall be that CLTM interruption is shorter than CHO interruption.

Issue 1-2-10: TTCI-state activate

< Way Forward > FFS the following options:

  - Option 1: TTCI-state activate is the time for active the TCI state of candidate cell(s).

  - Other option is not precluded.

Issue 1-2-11: The components of Tinterrupt

< Way Forward > FFS the following options:

  - Option 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU.

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Early synchronization and TCI state activation

Issue 1-3-1: Early DL sync

< Way Forward > FFS the following options:

  - Option 1: R18 LTM early DL-sync can be as baseline.

  - Option 2: FFS whether new requirements due to CLTM specific DL/UL sync procedures are needed.

  - Option 3: Wait for RAN2 further agreement on the framework.

Issue 1-3-2: Early UL sync

< Way Forward > FFS the following options:

  - Option 1: For UL-sync, we can consider some situations as follows,

- Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0.

- RAR-based TA acquisition.

- UE based TA measurement.

  - Option 2: How/whether to support early RACH on candidate cells during conditional LTM.

  - Option 3: Whether updates are needed to the Rel-18 PDCCH ordered RACH delay requirements.

  - Option 4:  If DL TODA-based UE autonomous TA estimation is found beneficial for the conditional LTM cell switch, RAN4 to define the requirements for DL TODA-based UE autonomous TA estimation.

  - Option 5: Wait for more progress from other WGs.

Issue 1-3-3: Early TCI state activation

< Way Forward > FFS the following options:

  - Option 1: How/whether to support early TCI activation on candidate cells during conditional LTM.

  - Option 2: FFS whether updates are needed to the Rel-18 early TCI state activation delay requirements.

  - Option 3: R18 LTM early TCI state activation can be as baseline.

  - Option 4: Wait for more progress from other WGs.

Others

Issue 1-4-1: Whether cover both RACH-based and RACH-less conditional LTM

< Way Forward > FFS the following options:

  - Option 1: Cover both RACH-based and RACH-less conditional LTM.

  - Option 2: FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2.

Issue 1-4-2: Whether to cover both known and unknown cell for conditional LTM

< Way Forward > FFS the following options:

  - Option 1: The known cell condition for conditional LTM can be revised as, during the last 5 seconds before the reception of the RRC command that contain conditional LTM configuration…….

  - Option 2: All the cell(s) configured as candidate cell(s) for conditional LTM is/are known.