3GPP TSG-RAN WG4 Meeting # 112bis											R4-2416864

Hefei, China, 14th – 18th October, 2024

Title: 	WF on RRM requirements for XR_Ph3

Agenda Item:	6.25.3

Source: 	Nokia

Document for:	Approval

# Topic #1: Scenarios for measurement skipping

## Issue 1-1: Deployment scenarios

< Agreement>:

- Frequency ranges

  - Agree on FR1 and FR2-1

- Scenarios

  - Start the discussion for

    - NR-SA

    - Carrier aggregation

<Way forward>:

- Further discussion needed for

  - FR2-2

  - NR-DC, EN-DC, NE-DC

## Issue 1-2: Types of measurements to be considered

<Way forward>:

- Option 1: Agree at least on the following and further discuss other scenario

  - Intra-frequency

  - Inter-Frequency

  - LTE inter-RAT

- Option 2: Agree at least on the following and further discuss other scenario

  - Intra-frequency

  - Inter-Frequency

  - LTE inter-RAT

  - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

- Option 3: include all related scenarios

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions

<Way forward>:

- Option 1: Support skipping of measurements without gaps

- Option 2: Measurement skipping does not apply for measurements without gaps

## Issue 1-4: L1 measurements with scheduling restrictions and procedures

<Way forward>:

- Option 1: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

- Option 2: Measurement skipping does not apply for L1 measurements and procedures

## Issue 1-5: L1/L2 triggered mobility measurements and procedures

<Way forward>:

- Option 1: support skipping of L1 measurements and procedures for LTM within and outside gap

- Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

# Topic #2: Measurements with gaps

## Issue 2-1: General approach for defining requirements for measurements with gaps

<Way forward>:

- Option 1: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

- Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

## Issue 2-2: Types of measurement gaps

<Way forward>:

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Companies may bring proposals regarding the priorities of each gap type for RAN4 to work based on the following table

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 |  |  |  |
| Type 2 |  |  |  |
| NCSG |  |  |  |
| MUSIM |  |  |  |
| Positioning |  |  |  |
| Rel 18 concurrent gap |  |  |  |
| PreConfigured |  |  |  |

# Topic #3: XR skipping requirements impact

## Issue 3-1: How to capture impact of measurement skipping

<Way forward>:

- Option 1:

  - As minimum requirements, cell detection, index reading, and measurement period is extended.

    - FFS how to extend

  - Reuse existing measurement accuracy requirements.

- Option 2:

  - For the cases where measurement impact is needed

    - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

      - FFS how to extend.

    - Reuse existing measurement accuracy requirements.

  - For the cases where measurement impact is not needed

    - FFS the feasible conditions and necessary UE assistance information.

## Issue 3-2: Principles for measurement delay calculation

<Way forward>:

- Option 1: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

- Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

- Other options are not precluded

## Issue 3-3: Conditions for measurement skipping

<Way forward>:

- Proposal 1: Define a max number of occasions cancelled during a measurement period.

- Proposal 2: RAN4 to discuss which conditions the measurement gap configurations need to meet to be relevant for the UE XR operation, e.g.,

  - minimum MGL,

  - maximum MGL,

  - maximum MGRP, etc.

- Proposal 3: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

- Proposal 4: Other conditions.

# Topic #4: Time offset for measurement skipping processing

## Issue 4-1: Time offset requirement for measurement gap skipping

<Way forward>:

- Option 1: 7 ms

- Option 2: 5 ms

- Option 3: DCI decoding time defined in TS 38.214 + [5]ms

- Option 4: assume subclause 5.3.1 of TS38.214 as a baseline for X offset.

- Option 5: Fraction of a slot

- Option 6: besides the DCI decoding delay, no additional processing delay is needed.

- Option 7: N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214)

# Topic #5: Need/feasibility of UE assistance information

## Issue 5-1: General on UAI

<Way forward>:

- Companies are tasked to discuss the need and feasibility of UAI, based on the following options

- Option 1: UAI is specified

  - Companies supporting this option are encouraged to bring contribution with exact proposals on what information is to be included in UAI.

- Option 2: UAI is not specified

# Topic #6: Additional skipping schemes

## Issue 6-1: Semi-static schemes

<Way forward>:

- Option 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1.

- Option 2: RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.3) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period.

- Option 3: To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC

- Option 4: Follow RAN1 working assumption and consider DCI only

# Topic #7: Reply LS to RAN1

## Issue 7-1: Response on RAN1 working assumption and time offset

<Way forward>:

- Companies are encouraged to bring analysis on the feasibility of the working assumption from the RAN1 LS in R4-2414910.