3GPP TSG-RAN WG4 Meeting # 112-bis	             R4-2416865

# Hefei, China, 14 – 18 October, 2024

Agenda Item:	6.26.5

Source: 	Moderator (CATT)

Title: 	WF on RRM requirements for NR_NTN_Ph3_Part1

Document for:	Approval

0 Notes:

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other options can be proposed.

- < Tentative Agreement > will be changed to < Agreement > if no comments are received.

Topic #1: Applicability clarification

    - Sub-topic 1-1 Scenario

Issue 1-1-1: The network scenario for (e)RedCap UE with FR1-NTN

Online agreement

<Agreement>:

- RAN4 to only consider NR SA operation mode with single carrier for (e)RedCap UE with FR1-NTN bands.

Issue 1-1-2: SCS for (e)RedCap UE with FR1-NTN

Online agreement

<Agreement>:

- Not define timing requirements for 60kHz SCS for (e)RedCap UE with FR1-NTN bands.

- Note: same as normal UE with FR1-NTN bands.

    - Sub-topic 1-2 Bandwidth

Issue 1-2-1: The bandwidth of (e)RedCap UE with FR1-NTN

Online agreement

<Agreement>:

- Both bandwidth with and without reduction of R18 eRedCap UE should be supported for (e)RedCap UE with FR1-NTN bands.

- RRM requirements do not differentiate two different types of eRedCap UEs, and the same RRM requirements can be applied.

Topic #2: General consideration on RRM requirements

    - Sub-topic 2-1 The general principle for defining the RRM requirements

Issue 2-1-1: The general principle for defining the RRM requirements for (e)RedCap UE with FR1-NTN

- Proposals

  - Proposal 1 (HW):

    - RAN4 to focus on the exact requirements developments rather than generic principle to define requirements.

  - Proposal 2 (Nokia):

    - Define RRM requirements for (e)RedCap UE with FR1-NTN bands based on the existing requirements for FR1-NTN.

<Way Forward>

No need to have an agreement. Suggest focusing on the exact requirements rather than generic principle to define requirements.

    - Sub-topic 2-2 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

- Further discussion on the agreed requirements:

Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN

Online agreement

<Agreement>:

- The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands.

  - Both time-based and location-based cell reselection measurement criteria will be defined.

    - For both earth fixed and earth moving scenario

  - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined.

  - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Issue 2-2-2: Further discussion on L3 measurements for (e)RedCap UE with FR1-NTN

In the last meeting, RAN4 has agreed to defined the requirements of NR intra-frequency L3 measurements and NR inter-frequency L3 measurements, the proposal is a further clarification on the supported types of L3 measurements for (e)RedCap UEs with FR1-NTN.

<Way Forward>:

- Proposal 1 (CATT): RAN4 to define the requirements of the following types of L3 measurements for (e)RedCap UEs with FR1-NTN:

  - Intra-frequency measurements without measurement gaps

  - Intra-frequency measurements with measurement gaps

  - Inter-frequency measurement without measurement gaps

  - Inter-frequency measurement with measurement gaps

- Requirements to be further determined

Issue 2-2-3: The Requirements to be further determined

Online agreement

<Agreement>:

- Define NR Rel-17 and Rel-18 Conditional Handover requirements.

  - Include location-based CHO and time-based CHO.

  - Support earth moving cell scenario

- Define RACH-less Handover requirements.

- Define SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state.

  - FFS TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN.

- FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

    - Sub-topic 2-3 Features to be considered for (e)RedCap UE with FR1-NTN

Issue 2-3-1: RedCap R17 relaxed measurement

Online agreement

<Agreement>:

- FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands.

- FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline.

  - If defined, the requirements apply provided that target cell is GSO

Issue 2-3-2: eDRX configuration

Online agreement

<Agreement>:

- For GSO:

  - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN.

    - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used.

- Further discuss for LEO.

Issue 2-3-3: Multiple SMTCs

<Way Forward>:

- Proposal 1 (HW): RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands.

  - Option 1: make it optional for RedCap UE to support 2 SMTC per MO.

  - Option 2: change the requirements so that measurement for different SMTCs are not parallel.

- Proposal 2 (ZTE): Multiple SMTCs shall be reused for RedCap UEs in NTN scenario.

  - Proposal 2a (Ericsson): RedCap for NTN supports (at the least) 2 SMTC.

- Proposal 3 (OPPO): RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands.

- Proposal 4 (QC): NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE.

Issue 2-3-4: Concurrent gap

<Way Forward>:

- Proposal 1 (CMCC, HW): Consider concurrent gap for Redcap over NTN.

  - Proposal 1a (HW): Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline.

- Proposal 2 (CATT, QC): NTN RRM requirements defined for concurrent measurement gaps are not applicable for RedCap NTN UE.

- Proposal 3 (Ericsson): RedCap for NTN supports (at the least) 1 MG.

Issue 2-3-5: The number of searchers and CSSF

<Way Forward>:

- Proposal 1 (CMCC, CATT): Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Issue 2-3-6: FH in PRS measurement for NW verified location requirements

<Way Forward>:

- Proposal 1 (HW): RAN4 to discuss whether to support FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN bands.

- Proposal 2 (Ericsson, OPPO): Not support FH in PRS measurement for RedCap for NTN.

Issue 2-3-7: NCD-SSB

In the last meeting, RAN4 has the following agreements for NCD-SSB according to the WF:

#### Issue 6-6-1: The impact of NCD-SSB for (e)RedCap UEs with FR1-NTN Online agreement <Agreement> - For (e)RedCap UEs with FR1-NTN, there is no need to define NCD-SSB specific measurement requirements.

Therefore, for this issue, suggest RAN4 to further focus on how to understand ‘no need to define NCD-SSB specific measurement requirements’ agreed in the last meeting and how to draft the CR.

<Way Forward>:

- Option 1: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (i.e, the requirements are only applicable to CD-SSB).

- Option 2: Use the terminology of CD-SSB explicitly for (e)RedCap with FR1-NTN requirements.

- Note: Other options are not precluded.

Issue 2-3-8: Other features

<Way Forward>:

- Proposal 1 (CMCC): For the optional NTN features which have impact on RAN4 requirements and need enhanced UE capability, whether Redcap UE could optionally support it and whether RAN4 need to define requirements should be further discussed.

Topic #3: Reduction in the number of UE Rx branches

    - Sub-topic 3-1 The principle of relaxation for 1Rx (e)RedCap UEs with FR1-NTN

#### Issue 3-1-1: How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN ?

<Way Forward>:

No need to have a high level agreement.

In the last meeting, RAN4 has agreed that RAN4 should follow the same principle for (e)RedCap UE in TN network for 1Rx and 2 Rx (e)Redcap UE as baseline, so it is suggested to focus on how to define the specific requirements.

For the specific relaxation methods, it is suggested to directly discuss different requirements separately in the corresponding sub-topics and issues.

    - Sub-topic 3-2 Mobility requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- RRC_IDLE/RRC_INACTIVE mode mobility

#### Issue 3-2-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (Samsung):

    - For 2Rx, reuse the legacy FR1-NTN requirements

    - For 1Rx, the ranking should be 1dB relaxed.

  - Proposal 2 (CATT):

    - RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx(e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin:

      - -1 dB offset can be reused as a baseline.

    - RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx(e)RedCap UEs with FR1-NTN bands.

      - 6 NR inter-frequency carriers can be reused.

    - The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)Redcap UE.

    - For the measurements of intra- and inter-frequency NR cells,

      - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands.

      - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands.

    - In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN:

      - For 1 Rx RedCap by a margin of at least

- 6 dB in FR1 for reselections based on ranking or

- 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

- 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

  - Proposal 3 (Apple):

    - Relaxation can be in terms of sample number for measurement/evaluation, offset to the threshold for cell-reselection.

  - Proposal 4 (HW):

    - Cell reselection for 1RX: offset and relaxation for accuracy

  - Proposal 5 (ZTE):

    - 1Rx RedCap UE measurement capability in NTN shall be the same as legacy RedCap 1Rx UE.

    - 2Rx RedCap UE measurement capability in NTN shall be same as legacy normal UE in NTN.

    - For inter-frequency measurement, RAN4 shall reuse the extra 1dB offset for 1Rx RedCap UE in NR NTN if the legacy requirements are reused.

    - For inter-RAT measurement, the orginal accuracy margin shall be maintained for 1Rx RedCap UE in NR NTN if the legacy requirements are reused.

    - The cell re-selection intra-frequency and inter-frequency measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs when considering NTN to NTN cell re-selection.

    - RAN4 shall define the concrete accuracy margin for 1Rx and 2Rx UE in NTN-NTN scenario. For 1Rx RedCap UE, additional 1dB is needed based on legacy conditions in NTN. For 2Rx RedCap UE, the legacy conditions in NTN shall be reused.

    - For MDT measurement requirements, the legacy measurement requirements shall be reused for 1Rx and 2Rx RedCap UEs in NTN.

    - For accuracy relative time stamp requirements, RAN4 shall reuse the same drift of relative time stamp for 1Rx and 2Rx RedCap UEs in NTN.

    - CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be:

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)) |
| RSRP2 | (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2 |
| Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |

    - Compared to 2Rx UE, extra 1dB margin shall be needed for 1Rx RedCap UEs.

    - If the random access for 2Rx and 1Rx RedCap requirements are defined, the RA based SDT requirements shall be reused. The extra 1dB accuracy margin for 1Rx can be reused.

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, firstly to discuss the following proposals one by one:

  - Proposal 1: RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx (e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin:

    - -1 dB offset can be reused.

  - Proposal 2: RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx (e)RedCap UEs with FR1-NTN bands.

    - 6 NR inter-frequency carriers can be reused.

  - Proposal 3: The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)Redcap UE, and the requirements will be the same.

  - Proposal 4: For the measurements of intra- and inter-frequency NR cells,

    - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands.

    - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands.

  - Proposal 5: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN:

    - For 1 Rx RedCap by a margin of at least

      - 6 dB in FR1 for reselections based on ranking or

      - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

      - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

  - Further discuss SDT requirements.

  - MDT requirements are related to issue 2-2-3, RAN4 to discuss them based on the conclusions on issue 2-2-3 later.

2. RRC_CONNECTED mode mobility

2.1 Handover

#### Issue 3-2-2: NR HO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (CATT, QC, Samsung, HW, MTK, Nokia, ZTE, CMCC): Handover delays should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Proposal 1a (CATT, Samsung, HW, MTK):

      - Define the relaxation on Tsearch for unknown intra/inter-frequency cell.

      - The same relaxation for 1 Rx RedCap UE in TN network can be reused as baseline.

- Tsearch = 2*Trs for intra-frequency HO

- Tsearch = 5* Trs for inter-frequency HO

  - Proposal 2 (Samsung):

    - For Handover requirements

      - For 2Rx, reuse the legacy FR1-NTN requirements

  - Proposal 3 (Samsung):

    - For HO based RACH, RACH-less HO, Time/location-based CHO with/without L3 measurement and Satellite switching with re-sync requirements

      - For 2Rx, reuse the legacy FR1-NTN requirements

      - For 1Rx, use 1 extra delay in FR1-NTN requirements

  - Proposal 4 (ZTE):

    - Considering hard satellite switching delay, Tsearch will not be changed which is 1*Tfirst_ssb ms for 2Rx RedCap UEs. For 1Rx RedCap UEs, Tsearch will be 2*Tfirst_ssb ms.

    - If RAN4 defines the RACH-based handover requirements for RedCap UEs in NTN, RAN4 shall reuse the same requirements for RACH-less handover requirements for RedCap UEs in NTN.

    - The legacy time period for PSS/SSS detection and time period for time index detection in CHO requirements for 1Rx and 2Rx RedCap UEs shall be considered.

<Way Forward>:

- For NR RACH-based HO requirements,

  - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs.

    - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

      - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

      - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

- Further discuss the requirements for RACH-less HO, Time/location-based CHO with/without L3 measurement and Satellite switching with re-sync.

2.2 RRC Connection Mobility Control

#### Issue 3-2-3: RRC Re-establishment requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (vivo): For RRC Connection Mobility Control requirements for Red-cap NTN, including RRC Re-establishment, Random access and RRC Connection Release with Redirection, RAN4 to agree

    - For 2Rx, the existing RRC Connection Mobility Control requirements for NTN can be reused

    - For 1Rx, the existing RRC Connection Mobility Control requirements for Red-cap can be reused with necessary modification to adapt NTN scenario.

  - Proposal 2 (CATT, MTK, HW, ZTE): UE RRC re-establishment delay (TUE_re-establish_delay) should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell.

    - The same relaxation for 1 Rx RedCap UE in TN network can be reused as baseline.

      - Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by 1 more samples for FR1 known and unknown intra/inter-frequency cell.

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC re-establishment delay requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, UE RRC re-establishment delay (TUE_re-establish_delay) should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

    - Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by 1 more samples for FR1 known and unknown intra/inter-frequency cell.

#### Issue 3-2-4: RRC Connection Release with redirection requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (vivo): For RRC Connection Mobility Control requirements for Red-cap NTN, including RRC Re-establishment, Random access and RRC Connection Release with Redirection, RAN4 to agree

    - For 2Rx, the existing RRC Connection Mobility Control requirements for NTN can be reused

    - For 1Rx, the existing RRC Connection Mobility Control requirements for Red-cap can be reused with necessary modification to adapt NTN scenario

  - Proposal 2 (CATT, HW, MTK, ZTE): RRC connection release with redirection to NR delay (Tconnection_release_redirect_NR) should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Define the relaxation on Tidentify-NR for FR1.

    - The same relaxation for 1 Rx RedCap UE in TN network can be reused as baseline.

      - Relax the Tidentify-NR by 1 more samples for FR1.

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC Connection Release with redirection requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the delay of RRC connection release with redirection to NR (Tconnection_release_redirect_NR) should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tidentify-NR for FR1, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

    - Relax the Tidentify-NR by 1 more samples for FR1.

#### Issue 3-2-5: RA requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (vivo): For RRC Connection Mobility Control requirements for Red-cap NTN, including RRC Re-establishment, Random access and RRC Connection Release with Redirection, RAN4 to agree

    - For 2Rx, the existing RRC Connection Mobility Control requirements for NTN can be reused

    - For 1Rx, the existing RRC Connection Mobility Control requirements for Red-cap can be reused with necessary modification to adapt NTN scenario

  - Proposal 2 (CATT): For random access requirements, RAN4 to consider the offset for the following relative cell-specific RSRP thresholds.

    - +1 dB offset can be reused as a baseline for rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB and msgA-RSRP-Threshold.

    - -1 dB offset can be reused as a baseline for rsrp-ThresholdMsg3.

  - Proposal 3 (ZTE):

    - The legacy requirements in NR NTN shall be reused for 1Rx and 2Rx UE in NTN, the conditions for performing the random access procedure for 1Rx in legacy RedCap TN shall be used for 1Rx UE in NTN.

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy random access requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to consider the offset for the following relative cell-specific RSRP thresholds for random access requirements

  - +1 dB offset can be reused as a baseline for rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB and msgA-RSRP-Threshold.

  - -1 dB offset can be reused as a baseline for rsrp-ThresholdMsg3.

    - Sub-topic 3-3 Timing for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

#### Issue 3-3-1: Timing requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (vivo, CATT, Nokia, ZTE):

    - The existing timing requirements for NTN can be reused.

      - UE transmit timing

      - UE timer accuracy

      - Timing Advance

    - Proposal 1a (CATT):

      - The timing requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

    - Proposal 2 (Nokia):

      - The absolute value for transmit timing error shall be the same for RedCap over NTN as defined for FR1-NTN.

      - Capture the same side condition used in TN Redcap related to the acquisition of NCD-SSB every 160 ms in RedCap over NTN specification

      - Reuse UE timer accuracy and timing advance requirements form FR1-NTN for RedCap over NTN.

<Way Forward>:

- The existing timing requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands.

  - UE transmit timing

  - UE timer accuracy

  - Timing Advance

- The timing requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

    - Sub-topic 3-4 Signaling characteristics for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

#### Issue 3-4-1: RLM/BFD/CBD requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (vivo): For requirement on RLM/Link Recovery Procedures for Red-cap NTN, RAN4 to agree

    - For 2Rx, the existing requirements for NTN can be reused

    - For 1Rx, the existing requirements for Red-cap can be used as the baseline and make necessary modification to adapt NTN scenario

  - Proposal 2 (CATT):

    - For SSB based and CSI-RS based RLM/BFD requirements, some of hypothetical PDCCH parameters should be defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN respectively, including aggregation level (CCE) and bandwidth (PRBs).

      - The same values for 1 Rx RedCap UE in TN network can be reused as baseline.

- Aggregation level (CCE) of 16.

- Bandwidth (PRBs) of 48.

  - Proposal 3 (vivo, CATT):

    - For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN.

  - Proposal 4 (CATT, MTK, ZTE):

    - The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - The same extension for 1 Rx RedCap UE in TN network can be reused as baseline.

      - For RLM evaluation periods,

- RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

- RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

      - For BFD evaluation periods,

- RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

  - Proposal 5 (QC, Nokia):

    - Extended RLM OOS and BFD evaluation periods for 1Rx (e)RedCap

    - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters for 1Rx (e)RedCap

  - Proposal 6 (Samsung, HW): For RLM requirements,

    - For 2Rx, reuse the legacy FR1-NTN requirements

    - For 1Rx, reuse TN 1 Rx UE for hypothetical PDCCH parameters, for the measurement periods, reuse the extended measurement periods as 1Rx in TN.

  - Proposal 7 (ZTE):

    - The evaluation period of CBD in legacy NTN shall be used without any change.

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RLM/BFD/CBD requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands,

  - For SSB based and CSI-RS based RLM/BFD requirements, some of hypothetical PDCCH parameters including aggregation level (CCE) and bandwidth (PRBs) are defined as follows:

    - Aggregation level (CCE) of 16.

    - Bandwidth (PRBs) of 48.

  - For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN.

  - The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - For RLM evaluation periods,

      - RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

      - RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

    - For BFD evaluation periods,

      - RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

#### Issue 3-4-2: Other requirements in signaling characteristics for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (vivo, CATT, ZTE):

    - The existing Active BWP switch delay requirements for NTN can be reused.

    - Proposal 1a (CATT): The Active BWP switch delay requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

  - Proposal 2 (CATT, ZTE):

    - For active TCI state switch delay and UE specific CBW change, RAN4 shall reuse the legacy NTN requirements for RedCap UEs in NTN scenario.

    - Proposal 1a (CATT): The active TCI state switch delay and UE specific CBW change requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

  - Proposal 3 (vivo, ZTE): RAN4 shall reuse the legacy NTN requirments of pathloss reference signal switching delay both for 1Rx and 2Rx RedCap UEs in NTN scenario.

<Way Forward>:

- The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands:

  - Active BWP switch delay,

  - active TCI state switch delay

  - UE specific CBW change requirements

  - Pathloss reference signal switching delay

- The above requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

    - Sub-topic 3-5 Measurement Procedure for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

#### Issue 3-5-1: Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (vivo):

    - For Measurement Procedure requirement for Red-cap NTN, for 2Rx, RAN4 to reuse the existing Measurement Procedure requirements for NTN. For 1Rx, the corresponding 1Rx requirements defined in Red-cap can be reused.

  - Proposal 2 (CATT):

    - The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

    - The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

- Extend the lower boundary from ‘120 ms’ to ‘160ms’.

    - The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

    - The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

    - The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

  - Proposal 3 (QC, Nokia):

    - Extended delays for PSS/SSS detection and SSB index identification (PBCH decoding) for 1Rx (e)RedCap

  - Proposal 4 (Samsung): For Intra-frequency w/o MG and inter-frequency w/o MG measurement requirements,

    - For 2Rx, reuse the legacy FR1-NTN requirements

    - For 1Rx, reuse the additional samples of 1Rx in TN in legacy FR1-NTN requirements.

  - Proposal 5 (HW):

    - L3 and L1 measurement for 1RX: relaxation for delay and accuracy

  - Proposal 6 (MTK):

    - Measurement delay

      - PSS/SSS detection delay is extended by 2-3 samples, based on whether it is intra-f/inter-f and whether it is with MG or not.

      - SSB index identification (PBCH decoding) delay is extended by 2 samples.

  - Proposal 7 (Xiaomi, OPPO):

    - For 1RX (e)RedCap UE with FR1-NTN bands, support to relax measurement requirements by extending the measurement delays with more measurement samples.

  - Proposal 8 (ZTE):

    - RAN4 shall define time period of PSS/SSS detection and time index detection. The requirements for 1Rx and 2Rx RedCap UE in NTN scenario shall reuse the legacy RedCap requirements.

    - The RedCap UEs with 1Rx and 2Rx in NTN shall own the same measurement period requirements as legacy RedCap UE.

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN measurement requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, to discuss the following proposals one by one:

  - The time period for PSS/SSS detection defined for intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Reuse the same extensions for 1Rx RedCap UE in TN network.

      - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

      - Not to extend the lower boundary.

  - The time period for SSB index identification (PBCH decoding) defined in intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Reuse the same extensions for 1Rx RedCap UE in TN network.

      - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

      - Extend the lower boundary from ‘120 ms’ to ‘160ms’.

  - The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Reuse the same extensions for 1Rx RedCap UE in TN network.

      - Extend the lower boundary from ‘200 ms’ to ‘400ms’

      - Keep the same number of samples for T SSB_measurement_period_inter.

  - The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

  - The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

#### Issue 3-5-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (vivo):

    - For Network verified UE location, the measurement period and measurement accuracy requirements for NTN can be reused. Additionally, regarding the measurement accuracy requirements for 1Rx, the measurement accuracy requirements for 1Rx defined in Red-cap positioning can be referred.

    - Proposal 1a (HW):

      - NW verified location for 1RX: relaxation for accuracy

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred, and the same relaxation for accuracy should be reused.

    - Sub-topic 3-6 Measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

#### Issue 3-6-1: Measurement accuracy relaxation for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1 (CATT, Samsung): For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the accuracy requirements defined in NTN.

  - Proposal 2 (Xiaomi, CATT, QC, Apple, Samsung, HW, OPPO, Nokia): For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

    - Proposal 2a (CATT, Samsung, MTK): The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

      - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

      - L1-RSRP: relax by 3dB.

  - Proposal 3 (CATT): RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements defined in NTN.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

  - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

    - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

    - L1-RSRP: relax by 3dB.

- RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

Topic #4: The impact of HD-FDD

    - Sub-topic 4-1 General HD-FDD related impact

Issue 4-1-1: General HD-FDD related impact

<Way Forward>:

- Proposal 1 (Xiaomi, CATT, QC, ZTE, CMCC):

  - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions.

  - Proposal 1a (CATT):

    - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

      - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

      - Handover interruption time

      - Random access

      - SA: RRC Connection Release with Redirection

      - Minimum requirement for L1 indication for RLM and LR

      - MAC-CE/DCI/RRC based uplink spatial relation switch delay

      - Scheduling availability of UE performing intra/inter measurements

      - Scheduling availability of UE during L1-RSRP measurement

  - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects:

    - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions.

    - Sub-topic 4-2 The specific impact of HD-FDD on NR Handover

Issue 4-2-1: The impact on NR Handover due to HD-FDD

<Way Forward>:

- Proposal 1 (vivo): RAN4 to reuse NR SAN FR1 – NR SAN FR1 Handover requirements for Red-cap NTN and additionally add the following applicability rule for HD-FDD case as current HO requirements for Redcap specified,

  - For RedCap NTN UE with HD-FDD, the handover requirements are met provided that

    - SSB is available at the UE once every SMTC period during Tsearch

    - One SSB is available during T∆

    - One SSB is available during TIU.

Issue 4-2-2: The impact on NR Satellite switching with re-synchronization due to HD-FDD

<Way Forward>:

- Proposal 1 (vivo): For requirements on NR Satellite switching with re-synchronization for Red-cap NTN, RAN4 to reuse the existing Satellite switching with re-synchronization requirements for NTN with adding the following applicability rules to adapt HD-FDD case:

  - For RedCap NTN UE with HD-FDD, the Satellite switching with re-synchronization requirements are met provided that

    - SSB is available at the UE once every SMTC period during Tsearch

    - One SSB is available during T∆

    - One SSB is available during TIU.

Topic #5: Others

    - Sub-topic 5-1 Specification structure

Issue 5-1-1: Specification structure

<Way Forward>:

- Proposal 1 (CATT, HW):

  - Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands.

  - Utilize the reference method for the requirements that can be reused.

- Proposal 2 (Nokia):

  - Adopt RedCap subclauses under NTN sections (under Suffix C).

    - Sub-topic 5-2 UE capabilities for (e)RedCap UE with FR1-NTN bands

Issue 5-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>:

- Proposal 1 (Nokia):

  - RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)