3GPP TSG-RAN WG4 Meeting #112bis	R4-2416870

Hefei, China, October 14th ‒ 18th, 2024

Title: 	WF on RRM requirements for NR_pos_enh2_Part2

Agenda Item:	4.1

Source: 	CATT

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [112bis][203] NR_pos_enh2_Part2.

Topic #1: Sidelink Positioning Requirements

Sub-topic 1-1 SL Positioning Core Requirements Maintenance

Issue 1-1-1: RAN1 reply LS (R4-2414905/R1-2407401)

Agreement:

- For the RAN1 LS response, neither reply nor updates to RAN4 specification are needed.

Issue 1-1-2: End point of SL-PRS based RSTD measurement period requirements

Agreement:

- Reflect in TS 38.133 the SL PRS measurement of the reference UE link in the SL RSTD core requirement.

Sub-topic 1-2 SL Positioning Performance Requirements

Issue 1-2-1: Update values of accuracy requirement

Agreement:

- Update the average numbers in R4-2410196 (CATT to update). The latest version can be found in R4-2416905.

Issue 1-2-2: SL-PRS signal level configurations

Agreements:

- Agree to have different Es/Iot and Noc numbers in TCs for SL-PRS and PSCCH.

- Es/Iot levels:

  - SL PRS Es/Iot: -3 dB

  - PSCCH Es/Iot: [3] dB

- Transmitted SL-PRS from different anchor UEs do not overlap in TCs.

- The above agreements can be revisited if the above approach is not supported by RAN1 agreements.

Issue 1-2-3: Comb size configuration

Agreement:

- (symbol num, comb size): (4, 4) and (2, 4)

Issue 1-2-4: Type of resource pool

Agreement:

- Add applicability statement in TS 38.133:

  - UE supporting two pools has to pass accuracy TC with one pool configured and delay TC with another pool configured for the same measurement (SL RSTD, SL Rx-Tx, SL RSRP/RSRPP).

  - For other cases, the UE is tested with the supported pool configuration.

Issue 1-2-5: Multiplexing of TX UEs in TCs

Discuss further:

- For 2 transmitting UEs, the time offset between the two SL-PRS configurations can be, e.g.:

  - MUX of multiple TX UEs: TX UE1: slot n, TX UE2: slot n + 1 and slot n + 160ms.

- Time offset for 3 transmitting anchor UEs needs to be also decided.

Come back next meeting.

Topic #2: Carrier Phase Positioning Requirements

Sub-topic 2-1 Carrier Phase Positioning Core Requirements Maintenance

Issue 2-1-1: CPP measurement period requirements with one-shot time window

Agreement:

    - When one-shot time window is configured,

- the measurement period for RSTD+RSCPD and Rx-Tx+RSCP is ⁡(Lwindow+ Ti), and

- the measurement period for RSTD/Rx-Tx is maxTeffective,i+Tlegacy,

    - the start of the measurement is the start of the one-shot time window,

    - Ti is the processing time for PFL i,

    - Tlegacy is the legacy requirements for RSTD and Rx-Tx.

The above requirements assume scaling factors equal 1 for the PFLs, for which the one-shot time window is configured.

Sub-topic 2-2 Carrier Phase Positioning Performance Requirements

Issue 2-2-1: Applicable scenarios for CPP measurements

Agreement:

RAN4 to only define accuracy for the scenarios where CPP measurements are performed on PRS resources within the same sets of slots and sets of symbols (i.e. the agreed scenario), considering PRS repetitions.

- Set of symbols is based on PRS configuration.

Issue 2-2-2: Margins for accuracy requirements due to carrier phase offset/RSTD uncertainty

Agreement:

- Define accuracy requirements based on the available simulation results for:

  - FR1:

    - BW>24 PRBs: [] can be removed

    - 24 PRBs: keep [] until RAN4#113

  - FR2:

    - 60 kHz

      - BW>24 PRBs: [] can be removed

      - 24 PRBs: keep [] until RAN4#113

    - 120 kHz

      - BW>32 PRBs: [] can be removed

      - 32 PRBs: keep [] until RAN4#113

Issue 2-2-3: Accuracy requirements for additional PRS configurations

Agreement:

For RSCP/RSCPD accuracy requirements:

- CATT to update the simulation results summary, including Nokia’s results. The latest version can be found in R4-2416906.

- Discuss the average numbers and include the agreeable numbers and configurations into CR (revision of R4-2416329)

- The requirements may be updated in the next meeting (up to the discussion in the next meeting)

Interested companies can provide simulation results for deriving the TBDs, e.g. in Table 10.1.43.2 (Measurement Accuracy Requirements for RSCPD).

Reference

- R4-2415605, Topic summary for [112bis][203] NR_pos_enh2_Part2, CATT, RAN4#112bis.

- R4-2416871, Ad-hoc minutes for NR_pos_enh2, Ericsson, RAN4#112bis.

- R4-2416881, Ad-hoc minutes #2 for NR_pos_enh2, Ericsson, RAN4#112bis.

- R4-2416882, Ad-hoc minutes #3 for NR_pos_enh2, Ericsson, RAN4#112bis.