# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416872

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.15.4

Source:	Moderator (Apple)

Title:	Ad-hoc minutes for NR_RRM_Ph5

Document for:	Information

# Discussion and agreements

#### Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting

Agreements in RAN4#112 meeting:

Agreement: - RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command. - FFS how to update the known condition (issue 1-2-1a). - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation

- Recommended WF

  - Taking direct SCell activation at SCell addition as baseline to discuss the known condition updates based on the proposals.

  - Taking draft CR [R4-2415182] as example to discuss the wording.

  - Further discuss whether to use the same updates to MAC CE based single DL SCell activation.

For direct SCell activation at SCell addition in FR1: For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, the SCell in FR1 is also known provided the following conditions are met for the SCell: - During the last [5s] before the reception of the direct SCell configuration command: - the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3 for the SCell being directly activated, and - the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. - the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. For direct SCell activation at SCell addition in FR2: The SCell in FR2 is known provided it meets the corresponding conditions as defined in clause 8.3.2. Otherwise, the SCell is unknown.

- Proposals

  - Proposal 1: RAN4 to update known condition that the valid measurement report is sent within [Y] seconds before SCell activation command reception.

    - Option 1-1:

      - [Y] equals to max(5*measCycleSCell,  5*DRX cycles) for MAC CE based SCell activation and equals to [5s] for direct SCell activation for FR1.

      - [Y] equal to [4s] for UE supporting power class 1/5 and [3s] for UE supporting power class 2/3/4 for FR2.

    - Option 1-2:

      - [Y] equals to [5s] for both MAC CE based SCell activation and direct SCell activation for FR1.

      - [Y] equal to [4s] for UE supporting power class 1/5 and [3s] for UE supporting power class 2/3/4 for FR2.

    - Option 1-3:

      - [Y] equals to the period starting from the point UE transmits RACH preamble to the time UE receives the MAC CE activation command for MAC CE based SCell activation.

      - [Y] equals to [5s] for direct SCell activation for FR1.

    - Other options

- Recommendation for proposal 1:

- Agreement:

  - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception.

    - For FR1,

      - [Y] equals to [5s] for direct SCell activation

      - FFS: Y for other SCell activation case.

    - For FR2,

      - Reuse the existing value of Y in direct SCell activation

      - FFS: Y for other SCell activation case.

  - Proposal 2: RAN4 to update known condition that the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3 during [Y].

- Recommendation for proposal 2:

  - RAN4 to update known condition that the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3 during [Y].

  - Proposal 3: the SSB measured remains detectable within a certain duration [Z]:

    - Option 3-1:

      - the SSB measured remains detectable during [Y] and during the SCell activation delay

    - Option 3-2:

      - the SSB detectability applies from the earliest possible eEMR measurement (i.e. the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting).

    - Option 3-3:

      - FR1:  [the SSB measured during the period equal to [5] seconds also remains detectable after UE has entered Idle/Inactive mode and during Idle/Inactive mode specified in clause 4.4.2.1]

        - Note: FFS Whether to modify the existing 4.4.2.1

      - FR2:  [The reported SSBs with indexes remain detectable after UE has entered Idle/Inactive mode and during Idle/Inactive mode specified in clause 4.4.2.1]

        - Note: FFS Whether to modify the existing 4.4.2.1

    - Other options

  - Proposal 4: the SSB measured remains detectable according to the condition [TBD]

    - Option 4-1:

      - the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

      - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

    - Option 4-2:

      - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

    - Option 4-3:

      - FR1:

        - the SSB measured remains detectable according to the cell identification conditions specified in sections 9.2 and 9.3

        - the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3

      - FR2:

        - During the period from [L3-RSRP reporting via Rel-18 EMR ]to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes

    - Other options

FFS: SSB detectability condition:

Option 1:

-	the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

- the SSB measured during the period equal to [5] seconds also remains detectable after UE has entered Idle/Inactive mode and during Idle/Inactive mode specified in clause 4.4.2.1 for FR1

      - Note: FFS Whether to modify the existing 4.4.2.1

Option 2:

-the SSB detectability applies from the earliest possible eEMR measurement (FFS on the definition of “earliest possible eEMR measureme” ) to the end of the defined SCell activation latency (i.e. a valid CQI reporting).

-FFS: the side condition for each status: IDLE/Inactive, transition between IDLE/Inactive and Connected, Connected mode

Agreement:

-the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting).

-	the SSB measured remains detectable according to the IDLE/Inacrive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

  - Proposal 5: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured with smaller than [X]

    - FFS whether X is a constant value defined in spec or up to UE capability.

- Discussion for proposal 5:

  - Option 1(OPPO(for X<=5s),Apple, MTK,vivo, HW):

    - The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X]

    - Discuss the value of [X].

      - FFS whether X is a constant value defined in spec or up to UE capability.

  - Option 2 (Ericsson, CMCC, Nokia): No need to have such side condition in option 1. Need more discussion.

  - Option 3 (OPPO, Apple, Samsung, QC, ZTE, CATT): use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1

[Discussion]:

Does valid report means UE can do fast Scell activation, in case this measurement was performed 100 secs ago?

- [QC]:As long as UE reports this valid result to network, UE and network are synced, it doesn’t matter when UE performs this measurement. because we already have 1 sample of SSB for T/F tracking for known SCell activation.

- [Apple]: if the measurement performed long time ago, UE will have time drifting and need additional T/F tracking.

  - Proposal 6: For a UE supporting measValidationReportEMR or measValidationReportReselectionMeasurements, the legacy known conditions with deactivated SCell L3 measurement still apply.

- Recommendation for proposal 6:

  - For a UE supporting measValidationReportEMR or measValidationReportReselectionMeasurements, the legacy known conditions with deactivated SCell L3 measurement still apply, i.e., UE can still use the legacy known condition to decide the applied requirements.

  - Proposal 7:

    - In case the SCell has more than one actually transmitted SSB, the report should be based on beam level measurement result, i.e. MeasIdleConfig > MeasIdleCarrierNR > BeamMeasConfigIdle-NR > includeBeamMeasurements = true.

- Recommendation for proposal 7:

  - Further discuss.

  - Proposal 8: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication

    - The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation

      - [Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation

- Recommendation for proposal 8:

  - Discuss later for MAC CE based DL SCell activation.

    - Not to introduce a new dedicated eEMR based fast SCell activation indication.

For MAC CE based single SCell activation in FR1: For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, SCell in FR1 is also known if it has been meeting the following conditions: - During the period equal to [5s] for FR1 before the reception of the SCell activation command: - the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3 for the SCell being activated and - the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. - the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. For MAC CE based single SCell activation in FR2: For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, for the first SCell activation in FR2 bands, the SCell is also known if it has been meeting the following conditions: - During the period equal to [4s] for UE supporting power class 1/5 and [3s] for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3, and - SCell activation command is received after valid EMR reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from valid EMR reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

#### Issue 1-2-4: Whether the indication to network is needed

- Proposals

  - Option 1: (CATT, Apple, Samsung)

    - Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation.

  - Option 2: (Ericsson, Nokia)

    - RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

- Recommended WF

  - Discuss the option(s).

#### Issue 1-2-2: Consideration on SINR condition during the whole procedure for known SCell activation

- Proposals

  - Option 1: (CATT)

    - No need to update the SINR condition for known SCell activation.

  - Option 2: (CTC, OPPO)

    - RAN4 to define a unified SINR side condition for eEMR measurement and known SCell activation procedure.

  - Option 3: Ericsson

    - Scell activation SNR Ês/Iot≥ -2dB can be kept unchanged.

- Recommended WF

  - Not to update the SINR condition (i.e., Ês/Iot≥ -2dB) for SCell activation delay requirements.

#### Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation

- Proposals

  - Option 1: (CATT)

    - No need to update the measurement period condition for FR1 known SCell activation.

  - Option 2a: (Apple)

    - For fast SCell activation based on eEMR, time for AGC is always needed since measurement period in IDLE/INACTIVE mode is longer than 2400ms in most cases.

  - Option 2b: (Qualcomm, Nokia)

    - In any case, the SCell activation requirement should allow UE to receive at least one SSB sample based on [SSB period] or [configured SMTC periodicity].

  - Option 3: (Samsung)

    - If the SCell is known based on the eEMR based known condition and belongs to FR1, 2400 ms can be modified to Y ms

      - The basic components to define Y and the subsequent impact on AGC training gate for activation shall be discussed.

- Recommended WF

  - Not to update the measurement period condition (i.e., 2400ms) for FR1 known SCell activation.

  - Further discuss whether additional clarifications in the specification are needed.

#### Issue 1-1-2: Scope of fast SCell activation for UE supporting Rel-18 eEMR

Agreements in RAN4#112 meeting:

Agreement: - Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR: - Direct SCell Activation at SCell addition - FFS: Single MAC CE based DL SCell Activation - FFS: Single MAC CE based PUCCH SCell Activation - RAN4 to discuss whether and how to apply the solutions to other SCell activation procedures after RAN4 has conclusions for the above cases.

- Proposals

  - Option 1: (vivo)

    - RAN4 to start from the case of Direct SCell Activation at SCell addition for fast SCell activation for UE supporting Rel-18 EMR. (agreed in last meeting)

  - Option 1a: (Qualcomm)

    - RAN4 to consider RRC-based direct SCell activation with a TCI state indication in the RRC based on Rel-18 Beam-level eEMR, i.e.

      - SCellConfig includes sCellState-r16 and tci-ActivatedConfig-r17 based on Rel-18 beam-level eEMR.

    - RAN4 to not consider SCG addition/activation

    - RAN4 to not consider MAC CE based SCell activation

    - RAN to consider non-PUCCH SCells and PUCCH SCells where the SCells belonging to PTAG and STAG

  - Option 2: (CATT, Apple, CTC, Ericsson, Nokia, ZTE)

    - On top of Direct SCell Activation at SCell addition, Single MAC CE based DL SCell Activation is also supported.

  - Option 3: (Nokia)

    - On top of Direct SCell Activation at SCell addition, Single MAC CE based PUCCH SCell Activation is also supported.

- Recommended WF

  - The scope of fast SCell activation for UE supporting Rel-18 EMR includes:

    - Direct SCell Activation at SCell addition

      - Only for DL SCell activation

    - Single MAC CE based DL SCell Activation (discuss in issue 1-2-1a)

    - Further discuss the TCI indication condition in option 1a for direct SCell activation (if not resolved in issue 1-2-1a).

#### Issue 1-1-3: Target scenarios of fast SCell activation

- Proposals

  - Proposal 1: (CMCC, CTC)

    - it is proposed to reduce SCell activation delay for unknown case in FR1 and FR2-1.

  - Proposal 2: (Nokia, Ericsson)-

    - RAN4 to study both known and unknown conditions for fast SCell activation with Rel-18 EMR.

  - Proposal 3: (Qualcomm, Samsung)

    - RAN4 to not consider the following SCells for the eEMR based fast SCell activation requirements:

      - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA)

      - SCell being in the same band as PCell in FR2 (i.e. intra-band CA)

  - Proposal 4: (Nokia)

    - RAN4 to consider all possible FR1 and FR2 scenarios for SCell activation, and the work can be initiated for FR1 Pcell + FR2 Pscell/SCell.

  - Proposal 5: (Nokia)

    - RAN4 should specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay).

  - Proposal 6: (Samsung)

    - RAN4 to consider the following scenarios as the starting point: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1

- Recommended WF

  - For proposal 1 and 2, after the known conditions are updated based on the discussion in 1-2-1a, reuse the existing delay requirements for known and unknown case.

  - Discuss proposal 3, 4 and 6 together. The conditions are already included in the existing requirements, no updates are needed.

For FR1: - However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - the SCell is contiguous to an active serving cell in the same band, and - A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell - its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB; For FR2： If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2: If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is: - 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time. If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is: - max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time. If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: ... If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP). If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: ... If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: …

  - For proposal 5, please proponent clarifies the motivation.

#### Issue 1-1-1: Applicability of fast SCell activation delay requirements

Agreements in RAN4#112 meeting:

Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured - and the UE has reported valid results on the SCell to be activated before SCell activation command.

- Proposals

  - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK)

    - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

  - Option 2: (OPPO, Nokia, Samsung)

    - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

  - Option 2a: (OPPO)

    - Consider the following two approaches to support the case:

      - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or

      - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration.

  - Option 2b: (Samsung)

      - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured

      - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

- Recommended WF

  - Further discuss whether to include the following cases:

    - UE supports measValidationReportEMR-r18 but measIdleValidityDuration-r18 is not configured.

    - UE supports measValidationReportReselectionMeasurements-r18 but measReselectionValidityDuration-r18 is not configured.

    - Note: For both of the cases, according to Rel-18 definition, UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.