3GPP TSG-RAN WG4 Meeting # 112bis											R4-2416873

Hefei, Anhui, China, 14th ‒ 18th, October, 2024

Agenda item:			6.19.4

Source:	Moderator (Samsung)

Title:	Ad-hoc minutes for NR_MIMO_Ph5

Document for:	Approval

Introduction

This Ad-hoc minutes is to cover the discussion in [112bis][212] NR_MIMO_Ph5_Part1 and [112bis][212] NR_MIMO_Ph5_Part2 for the following issues:

Topic #1: RRM requirements for UE-initiated/event-driven beam management

Issue 1-1: How to define Event triggered measurement reporting delay?

- Proposals

  - Option 1: (Xiaomi)

    - Starting point is an event that beam quality of current beam or other beams has changed. From that point on, the UE needs to evaluate for L1 reporting through measurement.

    - Ending point is the time when UE starts to send L1 report.

  - Option 2: (CMCC, ZTE)

    - Reporting delay is defined as the time between an event that will trigger a beam report and the point when the UE starts to transmit the beam report over the air interface.

      - Option 2a: (CMCC)

      - SSB based:

        - Less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM .

  - Option 3: (Qualcomm)

    - RAN4 should use the time when the 1st PUCCH can be sent as the reference point for the definition of the delay requirements

    - Both delays Tgap (time from occurrence of the event till first available resource for measurement) and TRS (time from detection of event in the UE till first available PUCCH resource for event reporting) need to be taken into account for defining the measurement reporting delay

  - Option 4: (Apple)

    - Starting point is when event is triggered at the UE

    - Ending point is when the UE transmits the measurement report on the 2nd UL channel.

    - Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B

  - Option 5: (vivo)

    - RAN4 does not define RRM requirements for Mode-B if it can only be carried by CG-PUSCH. If RAN1 comes to further conclusion that it can be carried by PUCCH or DG-PUSCH, RAN4 may also define RRM requirements for Mode-B.

  - Option 6: (Samsung)

    - RAN4 to specify the Event triggered measurement reporting delay as time between an event that will trigger a measurement report and the point when the UE transmits a first PUCCH. No difference for mode A and mode B.

  - Option 6: (Huawei)

    - Mode A and Mode B has minor impact on RRM requirements (e.g. reporting uncertainty). Both Mode A and Mode B are supported, and no differentiation between Mode A and Mode B unless necessary.

  - Option 7: (Nokia)

    - RAN4 to define event-triggered L1 reporting delay requirements with a maximum delay for the UE transmission of the UL indication in the first PUCCH channel after the event that triggers the beam report is met.

    - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

  - Option 8: (Ericsson)

    - RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on first UL channel from the measurement occasion).

  - Option 9: (MediaTek)

    - RAN4 to discuss two phases of UE-initiated/event-driven beam reporting:

      - Phase 1: Before UE initiate event beam reporting.

        - The measurement delay for L1-RSRP is the same for both Mode A and Mode B.

      - Phase 2: After UE initiate event beam reporting.

        - The reporting delay could be differentiated based on RAN1 conclusion.

- Recommended WF

  - RAN4 to discuss and specify what is Event triggered measurement reporting delay?

  - Start point of Event triggered measurement reporting delay

  - End point of Event triggered measurement reporting delay

Moderator: From the contributions, it can be observed that companies have different views of how to counted the event triggered measurement reporting delay. And for mode A and B, except the measurement time typically in RAN4, how to reflect the different time for UL resource in the total measurement reporting delay is related to Issue 1-5

Discuss:

MediaTek: In our Proposal there are two phases, before the event beam reporting, there is L1 measurement/evaluation.

After the condition is met, UE report is initiated.

Start Point: similar as L3 event triggered.

End point: UE transmit beam report.

Apple: UE performs the L1 measurement in L1 resources. After the beam measurement, UE can observe the condition is met. Then UE decide to report the measurement. The delay depends on DCI scheduling info.

Vivo: share similar view as MTK. The start point is the event happened. The L1 measurement is counted as part of reporting delay. For end points, we need further RAN1 progress.

Nokia: The L1 measurement is counted as part of reporting delay. Similar view of L3 can be reused. Further check.

Xiaomi: If the start point is only from the condition is met, the delay is only related to the scheduling info. Suggest to reuse the principle as legacy L3 event trigger reporting.

ZTE: we should separate the L1 event-trigger delay requirement. Follow legacy L3 event trigger delay.

QC: start point, can use L3

Ericsson: UE can perform some L1 measurement is some occasions. Then “the event” is met.

HW: The legacy L3 event trigger reporting delay: “The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface.”

MTK: In the current L3 measurement: one is for serving, one is neighbour cell. The event is just the condition of it can trigger a measurement report.

Apple: if the event is triggered by measurement, it is after the L1 measurement. Then UE can trigger the reporting. UE measure serving beam and candidate beam(s). After the Eventing is triggered is UE, the reporting is need to reporting by Scheduling.

QC: how long does UE to measure L1 measurement.

E///:  the event is met,

MTK: the condition is met in air.

Xiaomi: the start point is the condition is in the air. UE need time to L1 measurement.

Apple: fine with the starting point of L3. Then we should further discussion on the delay.

ZTE: better to align with companies about the starting point of L3 event.

Agreement:

Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay?

- Proposals

  - Option 1: (Xiaomi)

    - If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time.

  - Option 2: (CMCC)

    - It is proposed to take measurement period defend for Rel-18 LTM as baseline

  - Option 3: (Qualcomm)

    - RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management

  - Option 4: (Apple, Samsung, Ericsson, MediaTek)

    - Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based.

    - Option 4a (Apple, Ericsson)

      - Remove Treport or set to 0 in legacy requirements.

    - Option 4b (Apple)

      - Remove considerations for HST enhancements/ scenarios

      - From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1.

      - From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification.

    - Option 4c: (Samsung)

      - For FR2, the T_L1_measurement_period is scaled by N.

- Recommended WF

  - To be discussed

Moderator: For the measurement period: Whether to reuse legacy L1-RSRP measurement period or Rel-18 LTM. Besides that, from the contributions, it can be observed that companies have different views of the relation of measurement period and the time point of “Event-triggered” or “Event-trigger condition is met”. Need further discuss and clarification. In addition, how to handle it multiple beams should be measured?

Discuss:

Agreement:

Issue 1-4: How to consider the time window of triggering event determination in Event triggered measurement reporting?

- Proposals

  - Option 1: (Xiaomi)

    - RAN4 needs to consider the impact of triggering window defined in RAN1.

  - Option 2: (CMCC)

    - It is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M).

    - It is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M)

  - Option 3: (Qualcomm, Apple, Samsung, vivo)

    - No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements

  - Option 5 (ZTE)

    - Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1.

- For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering.

- For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible.

  - Option 6 (Huawei)

    - RAN4 to discuss how to define one Event-2 instance for following Event-2 case:

      - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

        - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

  - Option 7 (Nokia)

    - Wait further RAN1 progress.

- Recommended WF

  - To be discussed

Discuss:

Agreement:

Issue 1-5: How to consider UL resource in Event triggered measurement reporting?

- Proposals

  - Option 1: (Xiaomi)

    - RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting.

  - Option 2: (CMCC, ZTE)

    - The beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

    - No need to differentiate mode A and mode B

  - Option 3: (Apple)

    - Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B

  - Option 4: (vivo)

    - To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not (for Mode-A).

- Recommended WF

  - To be discussed

Discuss:

Agreement:

Topic #2: RRM requirements for other RRM requirements

Issue 1-2-2-4: On report configuration for CJT calibration:

- Proposals

  - Proposal 1: (Ericsson)

    - Propose to RAN1 to add AFO = {20 ppb, 50 ppb} to the set of configurable ranges to frequency offset report. Please find an LS draft in appendix.

    - The required UE Resolution, i.e., more than the current 16 or 32 levels, MPO, for DL/UL phase offset report, from a RAN4 point of view, has to be feed back to RAN1. Please find an LS draft in appendix.

- Recommended WF

    - Moderator: report configuration for delay/frequency/phase offset is up to RAN1, companies to check whether RAN4 need to discuss this issue.

Discuss:

Ericsson: resolution of FO, the measurement accuracy from our recommended is not accuracy enough. RAN4 can inform RAN1, to improve further resolution update. AFO = {20 ppb, 50 ppb}.is provided as further resolution from E/// proposal.

MTK: RAN1 define the resolution. Why the resolution update is not suggested in RAN1?

Ericsson: suggest to do the evaluation in RAN4. If we can observation from RAN4, we can send the suggestion.

Vivo: ran1 agreed the similar as TDCP as one-shot measurement. The others are discussing in RAN1.

Nokia: share similar view with companies of it is in RAN1.

E///: keep it open.

QC: share the similar view with other companies.

Apple: we share the view with other companies. The RAN1 defined and discussing. RAN4 doesn’t need to give some further proposal to RAN1.

Agreement:

Issue 1-2-2-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration?

- Proposals

  - Proposal 1: (Xiaomi, Qualcomm, Apple, Samsung, ZTE)

    - No

  - Proposal 2: (CMCC, Ericsson)

    - Yes

- Proposal 3a: (Ericsson)

      - RAN4 to define the requirements for the Frequency offset (FO), Delay offset (DO), Joint delay offset, and frequency offset (DO-FO), DL/UL phase offset (PO) reporting.

      - RAN4 to define the measurement delay or measurement behavior (number of samples for measurement averaging) for the aperiodic standalone CJT reporting.

  - Proposal 3: (Huawei)

    - RAN4 to discuss whether D/F/P CJTA is one-shot measurement. There is no need to define core requirements if it is one-shot measurement.

  - Proposal 4: (MediaTek)

    - Whether to define RRM requirement for CJT calibration reporting based on RAN4 feasibility study.

- Recommended WF

  - TBA

Discuss:

Whether to define the measurement delay or measurement behaviour for core requirement of CJT reporting?

E///: we define the measurement of the CJT reporting for some condition. For aperiodic L1-RSRP measurement, the time restriction timeRestrictionForChannelMeasurement. To use more samples.

Apple: we don’t have the same understanding of time restriction of L1-RSRP in the CJT-reporting.

In aperiodic reporting, the assumption is the when NW trigger the reporting, UE measure one shot measurement and report.

CMCC: the reporting delay is not related to the report type but related to the resource config. For the aperiodic reporting, the CSI-RS resource can be periodic. It is feasible to define the delay reequipment.

ZTE: based on RAN1. The reporting is aperiodic. Our legacy L1-RSRP is based on the periodic reporting.

Vivo: consider the worst case. Share the similar view is one shot.

Apple:  For aperiodic, the reporting time is defined in RAN1.

E///: UE can measure the L1-measurement if not just before the reporting time. UE can buffer some measurement.

Apple: how can we verify the behaviour as E/// proposed. If the more samples are needed before the receiving the trigger.

E///: we suggest to UE have to [buffer and average] such some UE behaviours for the important CJT reporting.

MTK: share similar view with Apple. UE shall report the reporting based on the one-shot measurement. For accuracy, we can discuss further.

Further discuss:

- Option 1: No need to define the delay requirements if it is based on one shot measurement. (Apple, vivo, MTK, Samsung, ZTE)

- Option 2: Need to define the delay requirements: including more samples of the L1 measurement. (E///, CMCC)

- Option3: need further evaluation (HW, QC, Nokia)

Issue 1-2-2-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration?

- Proposals

  - Proposal 1: (CMCC, Qualcomm, ZTE, Ericsson)

    - Define accuracy requirements.

- Proposal 1a: (Qualcomm)

      - Specification impact of reporting enhancement for CJT deployments should be covered in TS 38.133 under RRM performance and not in 38.101-4 under demodulation performance.

      - The performance part should start in RAN4 #115 in May ’25

  - Proposal 2: (Apple)

    - Study feasibility of defining accuracy requirements for frequency/delay/phase offset for CJT calibration in performance part of WI.

  - Proposal 3: (Samsung)

    - FFS on the accuracy requirements.

- Recommended WF

  - TBA

Discuss:

Agreement:

Issue 1-2-4-1: Whether RRM impacts exist by 2TA enhancement?

- Proposals

  - Proposal 1: (Qualcomm, Apple, Huawei, MediaTek)

    - RAN4 to wait for further progress in RAN1 on two TA support.

  - Proposal 2: (Xiaomi)

    - If TAG ID is configured for UL TCI state, DL reference timing defined in RAN4 Rel-18 for multi-DCI mTRP can be re-used for Rel-19 asymmetric DL sTRP/UL mTRP.

  - Proposal 3: (vivo, CMCC)

    - RAN4 discuss and clarify the DL timing reference of the TA for a UL-only TRP in R19 MIMO.

  - Proposal 4: (Apple)

    - RAN4 to update the reference point definition for 2 TA with asymmetric DL sTRP/ UL mTRP.

  - Proposal 5: (Samsung)

    - RAN4 to specify RRM requirements for two TAs in asymmetric DL sTRP/UL mTRP. Rel-18 uplink timing requirements can be used as start point. In Rel-19 asymmetric DL sTRP/UL mTRP, if all TRPs are from the same serving cell (intra-cell), for both two TAGs, they are corresponding to the same downlink reference point. If all TRPs are from the serving cell and additional PCI cell (inter-cell), for both two TAGs, they can be corresponding to the different downlink reference points. For each TAG, uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point.

  - Proposal 6: (ZTE)

    - 2TA defined in R18 mDCI mTRP scenario is reused in the sDCI case in R19 asymmetric mTRP scenario

    - Regarding how to identify the common DL timing for macro link and micro link, leverage the DL timing derivation identified in R18 MIMO, i.e. the single DL timing is determined as the reception of the first detected path (in time) of the corresponding downlink frame of the reference signal associated with UL/joint TCI state of macro link.

  - Proposal 7: (Nokia)

    - Existing RRM requirements with single TA apply for asymmetric DL sTRP/UL mTRP deployment.

    - Define RRM requirements for asymmetric DL sTRP/UL mTRP deployment with two TAs enhancement.

    - Specify the conditions under which a UE shall monitor one or two DL reference timings for asymmetric DL sTRP/UL mTRP with two TAs enhancement.

- Recommended WF

  - Check whether Proposal 1 is agreeable for RAN4 to wait further progress in RAN1.

Discuss:

Agreement:

Issue 1-2-4-2: RRM core impacts of Active uplink TCI state switching delay for unified TCI?

- Proposals

  - Proposal 1: (Samsung, Huawei (for FR2), MediaTek)

    - No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Proposal 4a: (Samsung)

      - The legacy requirements cannot be applicable.

      - Do not define such requirements if the UL TCI state is associated to SRS.

  - Proposal 2: (Apple)

    - RAN4 to discuss any other impacts to TCI state switching requirements with asymmetric DL sTRP/ UL mTRP based on RAN1 progress.

  - Proposal 3: (Qualcomm)

    - RAN4 should discuss whether new requirements for uplink TCI switching should be introduced involving the UL mTRP.

  - Proposal 4: (Xiaomi, Ericsson, Nokia)

    - RAN4 to specify UL TCI state switching requirements.

- Proposal 4a: (Xiaomi)

      - For UL-only TRP, RAN4 to define UL TCI state activation requirement based on SRS.

      - For anchor DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. RAN4 to discuss whether to also define UL TCI state activation requirement based on SRS for anchor DL-TRP.

- Proposal 4b: (Ericsson)

      - RAN4 to study and specify SRS based beam management requirements.

      - RAN4 to specify UL TCI state switching requirements for UL only TRP.

- Proposal 4c: (Nokia)

      - For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI s-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

      - Unified Uplink TCI state switching delay requirements to be applicable when the target uplink TCI state is associated with an uplink reference signal.

      - RAN4 to define UL TCI state switch delay requirements for an uplink only TRP with no distinction between known and unknown TCI state.

- Recommended WF

  - TBA

Discuss:

Agreement: