3GPP TSG-RAN WG4 Meeting #112bis	R4-22xxxxx

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.22.3

Source:	Ericsson

Title:	Ad-hoc minutes for Netw_Energy_NR_enh_Part1

Document for:	Approval

# Introduction

This is the ad-hoc minutes for Netw_Energy_NR_enh_Part1 WI.

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1)

- Background

  - The deactivated SCell measurement requirement in FR1 is as follow.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

- Proposals

  - When NW indicates the OD-SSB transmission during the deactivated SCell stage, UE is assumed to perform deactivated SCell measurement

    - Option 1: Ericsson, CATT, CMCC, Nokia, Samsung, ZTE, China Telecom, Huawei, Qualcomm

      - Apply with fast measurement

        - Option 1-1: OD-SSB periodicity

        - Option 1-2: SMTC or measurement period in MeasObject for OD-SSB

        - Option 1-3: fast measurement period is FFS

    - Option 2: Apple, , NTT

      - Follow legacy measurement (measCycleSCell)

    - Option 3-1: Apple, vivo, OPPO

      - Define a time window to limit the fast measurement

        - Apply fast measurement (i.e. following OD-SSB periodicity) within a time window after OD-SSB is activated.

        - After the time window, UE fallbacks to legacy measurement (i.e. follow measCycleSCell).

    - Option 3-2: Huawei

      - Define a time interval between two times of deactivated SCell measurement.

      - UE is not required to perform deactivated SCell measurement within the time interval.

- Recommended WF

  - Measurement procedure:

When NW indicates the OD-SSB transmission during the deactivated SCell stage, UE is assumed to perform deactivated SCell measurement in a fast mode based on the following one or more conditions.

    - Option 1: UE performs the OD-SSB meas. based unchanged fast cycle in L3 for deactivated SCell until UE receiving OD-SSB deactivation indication compared to legacy deactivated meas.

      - Option 1a: UE is required to perform the OD-SSB based fast L3 measurement for deactivated SCell until OD-SSB deactivation/or SCell activation.

    - Option 2: UE starts the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can fallback to legacy deactivated SCell measurement.

    - Option 1b: RAN4 to discuss the interval between OD-SSBs for measurement considering following aspects:

- To avoid too frequency measurement which may cause UE power consumption issue

- To maintain the timing/AGC for subsequent measurement/SCell activation

[Moderator understanding]: UE starts the OD-SSB based fast L3 measurement for deactivated SCell. When UE finishes the measurement within the measurement period, UE can fallback to a legacy deactivated SCell measurement or no measurement. After a time interval, UE will resume the OD-SSB based fast L3 measurement for deactivated SCell again.

    - Other option is not precluded

[Comments]

HW: Option 3.

Apple: Not always do the fast measurement. Compromise option 2.

LG: Option 2. RAN1 is discussing OD-SSB deactivation.

Nokia: Option 1 with update

Xiaomi: Option 2.

CMCC: Option 1 not cause power issue in UE since NW understands UE situation. Both option 1 , 2 can be considered.

QC: fast means less than measCycle. Option 2 can be the baseline. Option 1 depends on  the measurement period.

ZTE: It’s NW expected meas. Option 1. How to guarantee the time interval? NW can deactivate OD-SSB

Vivo: OD-SSB can be activated for a long time. Option 2.

MTK: Option 2.

Samsung: Option 2.

CATT: RAN1 may define deactivation timing. OD-SSB is not always transmission.

Apple: OD-SSB can be activated for some other UEs and ask the UE to measure. UE starts the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can fallback to legacy deactivated SCell measurement. Not power issue, and also performance. UE needs to consider the searcher. Option 1 not conflicts the option 2.

CMCC: Different understanding. Two SSB for UE A, two SSB for UE B. After two SSB for UE A, NW can deactivate the OD-SSB for UE A.

Apple: NW can deactivate but not mandatory. No harm NW ask other UE to perform OD-SSB meas.

Xiaomi: Deactivation can be after a long time and UE still waste the time.

QC: Option 2 can be a start point.

OD-SSB based deactivated SCell measurement(Case 1)

Tentative agreements:

Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

- After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 2-1: fallback legacy requirement

  - Option 2-2: stop measurement

  - Option 2-3: No requirement

- Time window means the time duration starting from receiving OD-SSB activation indication

  - FFS the time window configuration or pre-defined

- FFS the fast L3 measurement requirement

- FFS the relation between time window and OD-SSB deactivation command in the time domain

When UE receives OD-SSB deactivation, UE is not required to measurement the deactivated SCell.

No L3 measurement requirement is defined if NW not sends OD-SSB and UE is not received the OD-SSB deactivation command

Intel: Is it a periodic window, like SMTC or one time window?

Apple: One time window after OD-SSB activation

  - Measurement requirement:

OD-SSB based L3 measurement in fast mode is based on

    - Option 1: OD-SSB periodicity

    - Option 2: SMTC or measurement period in MeasObject for OD-SSB if defined

    - Other option is not precluded

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

- Proposals

  - Option 1: Ericsson, Samsung, China Telecom

    - The UE is assumed to perform measurement by combining the always-on SSB and OD-SSB.

  - Option 2: CMCC, Xiaomi, CATT, Samsung, ZTE, China Telecom, Huawei

    - The UE is assumed to perform measurement based on OD-SSB.

  - Option 3: Xiaomi,

    - The UE is assumed to perform measurement based on always-on SSB.

  - Option 4: Qualcomm

    - The UE is assumed to perform measurement based on either always-on SSB or OD-SSB. This implies that no requirements are defined based on combining of always-on SSB and OD-SSB

  - Option 5: Nokia

    - UE is assumed to perform measurement based on at least OD-SSB

  - Option 6: OPPO

    - The UE is assumed to perform measurement based on legacy measCycleSCell.

    - Additional capability is needed to support combining the always-on SSB and OD-SSB

- Recommended WF

  - From moderator’s understanding, this issue can be summarised with the following options.

    - Option 1: The UE is assumed to perform measurement based on OD-SSB only.

    - Option 2: The UE is assumed to perform measurement based on legacy measCycleSCell

    - Option 3:  The UE is assumed to perform measurement by combining the always-on SSB and OD-SSB.

  - Further discussion

[Comments]

Tentative agreements:

OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

  - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE)

  - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO)

  - Option 3:  RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE)

  - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE)

  - Option 5: NW can indicate which SSB will be used (vivo)

Issue 1-4-5: Which type of SSB is used during SCell activation

- Proposals

  - Option 1: Ericsson, Nokia, ZTE

    - After receiving the SCell activation and OD-SSB indication command, UE is required to always use OD-SSB for Scell activation in both Case 1 and Case 2(including scenario #2 or #2A).

  - Option 2: Huawei

    - For pure unknown SCell activation,

      - Case#1: UE shall perform all SCell activation procedures (e.g. AGC/Search/L1-RSRP/timing) based on OD-SSB.

      - Case#2: RAN4 to discuss whether UE shall use OD-SSB only or both OD-SSB and always-on SSB pending on further RAN1 conclusion

  - Option 3: OPPO

    - The UE is assumed to use the SSB with the smaller periodicity in OD-SSB and always-on SSB.

- Recommended WF

  - Based on moderator’s understanding, this issue can be summarised with the following options.

    - In Case 1, UE is required to always use OD-SSB for Scell activation.

    - In Case 2,

      - Option 1: the same as Case 1

      - Option 2: UE needs to use both OD-SSB and always-on SSB for Scell activation.

      - Option 3: UE is assumed to use the SSB with the smaller periodicity.

Case 2

Apple: go to same frequency scenario first. Both SSBs can be used. Option 2.

Vivo: We can use RAN2 L3 meas principle here for SCell activation.

Xiaomi: Requirement side, option 4 longer periodicity

Huawei: QCL relation is uncertain.

CATT: Option 2. Option 1 is our original proposal.

ZTE: Option 2. We need conditions. Same frequency same beam assumption.

Nokia: Option 1.

CMCC: Same as ZTE.

QC: Not support option 2. Up to UE implementation. Option 3. UE assumes OD-SSB is always smaller than A-SSB.

Apple: Option 2 and 3 is the same if we assumes same offset, frequency, beam assumption.

Moderator: Can we assume same spatial relation?

Intel, ZTE: Yes

Tentative agreements:

  - In Case 1, UE is required to always use OD-SSB for Scell activation.

  - In Case 2, when both OD-SSB and always-on SSB are configured are transmitted,

    - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline

    - The agreement can be revisited if different with other WG