3GPP TSG-RAN WG4 Meeting # 112bis											             R4-2416876

Hefei, Anhui, China, 14 – 18, 2024

Agenda item:			8.1.2

Source:	Moderator (Apple)

Title:	Topic summary for [112bis][225] RRM_Spec_Improvement

Document for:	Information

# Introduction

The following agreements were made in RAN4#112:

  - Interested companies are encouraged to collaborate on CR review and drafting before RAN4#112bis

    - CR will be based on TS38.133 v18.6.0

    - The updated CR will be submitted in RAN4#112bis

    - The proposed revision should be strictly limited to the following three issues

      - Issue1: [], “TBD” and “FFS” clean up

      - Issue2: Terminology/style inconsistency, incorrect notation/symbols/abbreviation, undefined abbreviations, redundant information/notes clean up

      - Issue3: Empty test cases clean up

  - CR drafting plan is summarized in Table 1.

    - Volunteer companies are expected to draft the CR, collect the feedbacks and make contribution to RAN4#112bis.

Table 1: CR drafting plan

| - Sections | Page number | # of pages | Volunteer company |
| --- | --- | --- | --- |
| Foreword 1 Scope 2 References 3 Definitions, symbols and abbreviations 4 SA: RRC_IDLE state mobility | 259 | 408 pages | Apple |
| 5 SA: RRC_INACTIVE state mobility | 74 |  |  |
| 6 RRC_CONNECTED state mobility | 75 |  |  |
| 7 Timing | 26 | 330 pages | BeammWave (Joakim Axmon, joakim.axmon@beammwave.com |
| 8 Signalling characteristics | 304 |  |  |
| 9 Measurement Procedure | 444 | 444 pages | Nokia |
| 10 Measurement Performance requirements | 183 | 365 pages | Ericsson (Iana Siomina, iana.siomina@ericsson.com) |
| 11 void |  |  |  |
| 12 V2X Requirements | 45 |  |  |
| A.1 Purpose of annex A.2 Requirement classification for statistical testing A.3 RRM test configurations | 137 |  |  |
| A.4 EN-DC tests with all NR cells in FR1 | 519 | 519 pages | LGE |
| A.5 EN-DC tests with one or more NR cells in FR2 | 410 | 410 pages | Huawei |
| A.6 NR standalone tests with all NR cells in FR1 | 935 | 935 (can be further split if needed) | Company1:Ericsson (Iana Siomina, iana.siomina@ericsson.com) |
|  |  |  | Company2: |
| A.7 NR standalone tests with one or more NR cells in FR2 | 843 | 843(can be further split if needed) | Company1: |
|  |  |  | Company2: |
| A.8 E-UTRA standalone tests for NR RRM | 90 | 395 pages |  |
| A.9 V2X Tests | 70 |  |  |
| A.10 EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1 | 235 |  |  |
| A.11 NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1 | 315 | 315 pages |  |
| A.12 E-UTRA Standalone Tests with at Least One NR Cell under CCA | 150 | 446 pages |  |
| A.14 NR standalone tests for Satellite access | 237 |  |  |
| A.15 NR standalone tests with one or more NR cells in FR2-2 | 59 |  |  |
| A.16 NR standalone tests with all NR cells in FR1 for RedCap | 656 | 656 pages | Mediatek |
| A.17 NR standalone tests with one or more NR cells in FR2 for RedCap | 202 | 470 pages |  |
| A.18 E-UTRA standalone tests for NR RRM for RedCap | 53 |  |  |
| A.19 NR standalone tests for ATG | 120 |  |  |
| Annex B (normative): Conditions for RRM requirements applicability for operating bands | 95 |  |  |

Other agreements in RAN4#112:

- Modal verbs

  - Agreement: deprioritized

- Duplicated requirements

  - Agreement: deprioritized

- Unused test configurations

  - Agreement: deprioritized

- Suffix alignment

  - Agreement: deprioritized

- References or mapping tables in the core part requirements that point to the relevant test cases from Rel-19

  - Agreement: Deprioritized. Meanwhile, interested companies can bring examples on how to implement.

- Abbreviations

  - “PRB” or “RB”:

    - Current status: both are used, choose one of them? None of them is in Abbreviation of TS 38.133.

    - Agreement: “PRB” should be consistently used in all sections instead of “RB” and the abbreviation is to be included in section 3.

  - “BW”:

    - Current status: used, but not in the Abbreviation section in TS 383.133.

    - Agreement: Include the abbreviation in the Abbreviations section 3 of TS 38.133.

  - “TRS”: is used but not defined in Abbreviations section in TS 38.133.

    - Agreement: Include the abbreviation in the Abbreviations section 3 of TS 38.133.

# Companies’ contributions summary

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2415199 | Further discussion on RRM spec quality improvement | vivo | Observation 1: If unified indent format (hierarchy) is used, there is no problem with the existing format. Proposal 1: Use existing indent format, which is also used in RAN1 specifications. Proposal 2: When capturing requirements in the specification, the structure of requirements should be carefully considered and unified indent format should be used, especially when capturing new requirements into existing one. |
| R4-2415438 | Hierarchy of indent | MediaTek inc. | Proposal 1: Put conditions before requirements. Proposal 2: When both new and legacy requirements are to be put in the same hierarchy level, the new requirements start first with ‘if’, while the legacy requirements come after and begin with ‘else if’ or ‘else’. Proposal 3: For requirements with multiple applicable conditions, start the paragraph with UE capability and providing all the remaining conditions in a list. |
| R4-2415546 | Draft CR for RRM Spec Improvement (section 1~6) | Apple | According to the agreements in RAN4#112 meeting, - Remove TBD and square brackets if it’s not technical. - Correct typos. - Correct format. |
| R4-2415606 | A specific example for improvements of Rel-19 RRM specifications | Qualcomm Technologies Ireland | Proposal 1: RAN4 to develop guidelines for drafting requirements with complex logic, including adopting a pseudo-code approach (e.g. similar to the way RAN2 procedures are specified). Observation 1: The hierarchy of indent could be indicated by the same approach as in TS 38.321, i.e., 1>, 2>, 3>, etc. for the indent level. Observation 2: Significant improvement of the readability in TS 38.133 can be achieved by applying proper indentation and reorganizing of the text without the need to rewrite it. Observation 3: Further optimization beyond proper indentation and text reorganization is possible. Proposal 2: RAN4 should take section 8.3.2 as an example and develop rules how to do proper indentation. One potential approach is the methodology of TS 38.321. Proposal 3: RAN4 should develop guidelines how parameters are defined in TS 38.133. |
| R4-2415837 | Draft CR on specification improvement for 38.133 A.5 | Huawei, HiSilicon | Following type of changes are made: Typo correction: E.g. - spelling error - remove spacing - dB -> dBm style inconsistency: E.g. - italic on UE capability - DRx -> DRX - SSB GSCN -> SSB ARFCN - Align font size Remove unnecessary note /Rerange Note index Correct incorrect notation: E.g. - HARQ -> THARQ Remove [] Remove redundant/repeated paragraph |
| R4-2415883 | Discussion on specification quality improvement for TS38.133 | LG Electronics Inc. | - Proposal 1: Use RAN2 pseudo-code for hierarchy of indent - Proposal 2: To align terminology and table description listed above in whole specification, RAN4 consensus is required first. |
| R4-2415884 | Draft CR on specification quality improvement for clause A.4 in TS38.133 | LG Electronics Inc. | TBD and square brackets have been removed., Reference mismatch, style of table, and incorrect notation/symbol/abbreviation have been revised. Typos have been corrected. |
| R4-2416206 | On RRM specification quality improvement – clean up CRs | Ericsson | - Proposal 1: Agree on removing the editor’s note below in clause A.3.15.3 of TS 38.133: A.3.15.3 Setup 3: 2 AoAs There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. The applicable set of relative angular offsets between the 2 active probes is given in Table 3.15.3-1 for each UE power class. - Proposal 2 (RAN5 LS): Inform RAN5 by sending an LS to RAN5 listing the relevant RAN4 CRs agreed during this RAN4 framework on RRM specification quality improvement. - Proposal 3: A big maintenance CR running over a couple of RAN4 meetings after closing the core WI. - Proposal 4: Allocate separate AI/TU for discussing and finalizing the feature CRs after the technical discussions are over. - Depending on the feature and the amount of specification impact, the time allocation can vary and can be up to the entire meeting week in the worst case. - Proposal 5: WI Rapporteurs present a CR implementation plan (e.g., similar to the workplan, but focused on CR handling for the WI), discuss, and get it approved. This applies also when there is no work split. - Proposal 6: For situations where similar text needs to be repeated across multiple sections (or specifications), the general text could firstly be agreed as a reference and then used across different sections/CRs/specifications to improve consistency. - Proposal 7: Creating a checkbox list of key specification editing aspects to remember and check while preparing CRs and/or extend the Forword section of the specification to ensure consistent usage of frequently used terms, notation, abbreviation, CA configuration vocabulary, etc. - Proposal 8: Creating a 3GPP repository of figure templates, editable diagrams, and formulae. The link with templates could be included in the checkbox list described in Proposal 7. |
| R4-2416207 | On RRM specification quality improvement – new issues | Ericsson | - Proposal 1: Agree on the following new text added in clause A.1 in TS 38.133: A.1 Purpose of annex The purpose of annex A is to define test cases and the relevant configurations for verifying various types of RRM requirements with NR cells, including SA NR, EN-DC, and NE-DC deployments as well as standalone E-UTRA deployments. - Proposal 2 (Mapping purpose): It is clarified that the mapping is for information purpose only and presents the information from the test cases description in a more compact and structured way. - Proposal 3 (Mapping): Given the informative purpose of the mapping and its potential length, including such mapping in a new Annex D (informative) of TS 38.133 may be preferred. - Proposal 4 (Mapping format): The mapping can be in a table format and contain, e.g., <requirement clause ID><test case clause IDs> columns. If needed, the column <test case clause IDs> can be further broken into sub-columns, e.g., for SA, EN-DC, and NE-DC. - Proposal 5 (Mapping contents): The mapping can be focus on core requirements and corresponding test cases, but potentially may also be extended to performance requirements and corresponding test cases. - Proposal 6 (Mapping maintenance): Upon each test case introduction, the mapping can be updated with the corresponding reference. |
| R4-2416208 | Draft CR 38133 RRM specification improvement for clauses 10 to A_3 | Ericsson | Section 10: Applied changes: - Removed old [] - “Note“ was removed in some cases outside tables, where the text belongs to the requirements. - Some tabs were added to align formatting. Further comments: - “PRB” or “RB”: both are used, choose one of them? None of them in the Abbreviation section of TS 38.133. - “BW”: used, but not in the Abbreviation section in TS 383.133. - “TRS” is used but not defined in Abbreviations section in TS 38.133. Section 12: Applied changes: - Clarified references. - Changed tabulation in some lists to align with the rest. - EUTRA changed to “E-UTRA”. - Removed the word “Note” in the requirements outside tables. Further comments: - PRB: not defined in Abbreviations section in TS 38.133. Sections A.1-A.3: Applied changes: - A.1 (Purpose of annex): added some simple text, since otherwise the first section of the annex is completely empty. - Duplicate text removed in: Table A.3.21.2-1, Table A.3.32.2-1 - Removed old (Rel-17 or earlier) square brackets - TBDs removed (the referenced clauses are void) - Removed FFS in: A.3.15.2.2 - Added missing references - Removed old (Rel-17 or earlier) editor’s notes in: A.3.12.1.2, A.3.12.2.2, A.3.13.1, A.3.15.3. Further comments: - “PRB” or “RB”: both are used, choose one of them? None of them in the Abbreviation section of TS 38.133. - “BW”: used, but not in the Abbreviations section in TS 383.133. - “TRS”: used, but not in the Abbreviations section in TS 383.133. - “CHO” (Conditional HO): used, but not defined in Abbreviations - “N/A” or “n.a.”: N/A is more common, should “n.a.” be changed to “N.A”? - Old (Rel-17 or earlier) FFS requiring discussion: Table A.3.15.3-1 - Old (Rel-17 or earlier) [] requiring discussion: Table A.3.36.2-1 (replace with the signaling reference?) - Old (Rel-17 or earlier) Editor’s notes requiring technical discussions: A.3.27 - New (Rel-18) []: Table A.3.14.2-3A ([db6]); A.3.15.5, A.3.15.6 - New (Rel-18) FFS: A.3.15.5 - There are many empty columns in test configuration tables – should be removed? - corresponding Band are fulfilled  corresponding band are fulfilled? - Table A.3.13A.5-1: correct references (A.6.3.X…)? |
| R4-2416209 | Draft CR 38133 RRM specification improvement for clause A_6 | Ericsson | (clauses A.6-A.6.5) On received comments after the September review: In general, agree with the comments, except that: - Void sections cannot be removed - Changing sections titles should not be done - The added sentence on the correct events rate of 90% may be not always editorial. - “DRX” is changed to “DRX”, perhaps by mistake - kHz was not always corrected in a correct way Applied changes: - Old [] removed in: A.6.5.1.5.1, A.6.5.3.7.1, A.6.5.7A.1.1, A.6.5.7A.2.1, A.6.5.7C.1.1, A.6.5.7C.2.1, A.6.5.9.1.1, Further comments (the issues which require separate discussions): - Old TBDs and [] requiring discussion: A.6.5.3.7.1 (including [X]) and A.6.5.3.9.1 (including [X]) need to be aligned with the latelyupdated requirements, A.6.5.3.9.2, - New (Rel-18) FFS/editor’s notes requiring discussion in: section A.6.3.4.2, A.6.5.1.13.1, A.6.5.1.14.1, A.6.5.1.15.1, A.6.5.3.15.1, A.6.5.3.16.1, A.6.5.5.8.1, - New (Rel-18) TBDs requiring discussion in: section A.6.1.1.9.1, A.6.3.1.18.2, A.6.5.5.8.1, New (Rel-18) [] requiring discussion in: section A.6.3.2.4.1.1, A.6.3.2.4.1.3, A.6.3.2.4.2.3, A.6.3.2.4.3.1, A.6.3.2.4.3.2, A.6.3.2.4.3.3, A.6.3.4.1.3, A.6.3.4.2.2, A.6.3.4.3.2, A.6.3.4.3.3, A.6.3.4.4.3, A.6.3.5.1.3, A.6.5.1.12.1, A.6.5.1.13.1, A.6.5.1.14.1, A.6.5.1.15.1, A.6.5.3.12.1, A.6.5.3.14.1, A.6.5.3.15.1, A.6.5.3.16.1, A.6.5.5.8.1, A.6.5.5.8.1, A.6.5.7D.3.1, A.6.5.7D.4.1, A,6.5.12.1.1, A,6.5.12.2.1, A.6.5.13.1.1, |
| R4-2416338 | Draft CR for Section 9 specification quality improvement | Nokia | - Draft CR for TS 38.133 according to the work split according to the plenary task: Specification quality improvement (RP-240782). For offline discussion. - Changes are according to the agreement as captured in the WF R4-2414103, agreed in RAN4#112 meeting: - CR will be based on TS38.133 v18.6.0 - The updated CR will be submitted in RAN4#112bis - The proposed revision should be strictly limited to the following three issues - Issue1: [], “TBD” and “FFS” clean up - Issue2: Terminology/style inconsistency, incorrect notation/symbols/abbreviation, undefined abbreviations, redundant information/notes clean up - Issue3: Empty test cases clean up |
| R4-2416339 | Specification improvements in R19 timeframe | Nokia | - Align the phrase used for referring to the ‘SCell’ being addressed in the requirements in section 8.3.2. - Clarify and remove use of ‘and/or’ phrases. - Requirements not conditioned on the SCell known or unknown status, should be defined prior to the requirements conditioned by the SCell known and unknown status. - If a new outline is selected, these need to be supported by clear drafting guidelines and, if needed, template support. - If RAN4 agrees to apply quality improvements and related CRs, a proper review process needs to be agreed. And we observe: - Future CR review time may increase if RAN4 agree to initiate specification quality improvement rewriting in NR timeline. |
| R4-2416452 | Draft CR on NR standalone tests with all NR cells in FR1 for RedCap | MediaTek inc. | Change 1: - Remove pure editorial [], TBD and FFS - Terminology/style consistency - Empty TC clean up Changes based on companies’ comments: - From BeamWave: - For consistency: - Table with upper-case ‘T’, - clause with lower-case ‘c’ - Correct SI prefix for kilo is lower-case ‘k’. - Correct parameter name is: b2-Threshold2EUTRA - PCell is the correct abbreviation. |
| R4-2416472 | DraftCR 38.133 Editorial corrections to sections 7 and 8 | BeammWave AB | Corrections to abbreviations: E-UTRA: clauses 7.6.5, 7.6.5.1, 8.2.1.1, 8.2.3.1, NR-DC: clauses 7.5.1, 7.5.6, 7.6.1, 7.6.6, 8.2.1.1, 8.5.7.4, 8.5A.7.4, 8.18.8.4 PCell: clauses 8.2.4.2.18, 8.10C.1 SCell: clauses 8.2.4.2.18, 8.3.2, 8.3.7, 8.3.12, 8.3.16, 8.3.17, 8.3A.2 pTAG: clause 7.1.2 sTAG: clause 7.1.2 SpCell: clauses 8.3.2, 8.3.3, 8.3.4, 8.3.9, 8.3.14, 8.3.16, 8.3.17, 8.3A.3 RLM-RS: clauses 8.1.7.2, 8.1A.6.2, 8.1B.7.2, 8.1C.7.2, 8.1D.7.2 Consistent use of abbreviations: FR1: clause 8.2.2.2.9 FR2: clauses 8.1.7, 8.2.2.2.9 Corrections to abbreviated units (SI prefix): kHz: clauses 7.7.1, 8.1A.6.3, 8.1B.2.1, 8.1B.3.1, 8.6.2, 8.6.2A.1 µs: clauses 8.2.1.2.12, 8.2.1.2.14, 8.2.2.2.9, 8.2.2.2.19, 8.2.2.2.10A, 8.2.2.2.10D, 8.2.3.2.11, 8.2.4.2.9 Consistent use of plural form for units: milliseconds: clauses 8.3.2, 8.3.7, 8.3.12, 8.3.13, 8.3.16, 8.3.17, 8.3.18, 8.3A.2, 8.13.2, 8.13A.2, 8.13C.2 slots: clauses 8.2.1.2.1, 8.2.1.2.2, 8.2.1.2.3, 8.2.1.2.4, 8.2.1.2.5.2, 8.2.1.2.5.3, 8.2.1.2.5.4, 8.2.1.2.6, 8.2.1.2.7, 8.2.1.2.8.2, 8.2.1.2.11, 8.2.1.2.12, 8.2.1.2.13, 8.2.1.2.19, 8.2.2.2.1, 8.2.2.2.2, 8.2.2.2.5, 8.2.2.2.8, 8.2.2.2.9, 8.2.2.2.17, 8.2.3.2.1, 8.2.3.2.3, 8.2.3.2.6, 8.2.3.2.8.1, 8.2.3.2.8.2, 8.2.3.2.9, 8.2.3.2.11, 8.2.3.2.12, 8.2.3.2.17, 8.2.4.2.6, 8.2.4.2.9, 8.2.4.2.15 (Conventionally, units are in plural form with the only exception when the quantity is exactly one.) Removing units within brackets, and redundant units: [ms], ms clauses 8.1.2.2, 8.1.2.4, 8.1.3.2, 8.1.3.4, 8.1A.2.2, 8.1B.2.2, 8.1B.3.2, 8.1C.2.2, 8.1C.3.2, 8.1D.2.2, 8.1D.3.2, 8.3.2, 8.5.2.2, 8.5.2.4, 8.5.3.2, 8.5.3.4, 8.5.5.3, 8.5.6.2, 8.5A.2.2,8.5A.5.2, 8.5B.2.2, 8.5B.3.2, 8.5B.5.2, 8.5B.6.2, 8.5C.2.2, 8.5C.3.2, 8.5C.5.2, 8.5C.6.2, 8.5D.2.2, 8.5D.3.2, 8.5D.5.2, 8.5D.6.2, 8.18.2.2, 8.18.3.2, 8.18.5.2, 8.18.6.2 (The units are provided in each respective definition and hence not needed in the running text. The use of units within brackets is in conflict with how brackets otherwise are used in TS 38.133.) Consistent style (capitalization): per-FR: clauses 8.2.1.2.12, 8.2.1.2.13, 8.2.2.2.9, 8.2.3.2.11, 8.2.3.2.12, 8.2.4.2.9, Table: clauses 8.1.3.2, 8.1.3.4, 8.18.3.2, 8.1C.3.2, 8.1D.3.2, 8.2.3.2.2, 8.5.5.2, 8.5.6.2, 8.5A.5.2, 8.5B.5.2, 8.5B.6.2, 8.5C.5.2, 8.5C.6.2, 8.5D.5.2, 8.5D.6.2, 8.18.2.2, 8.18.3.2, 8.18.5.2, 8.18.6.2, clause: clauses 7.1.2.3, 8.1.1.1, 8.2.2.2.6, 8.3.17, 8.3.18, 8.5.1.1, 8.5A.7.1, 8.5A.7.2, 8.5A.7.3, 8.5A.7.4, 8.5A.8.1, 8.5A.8.2, 8.5A.8.3, 8.5A.8.4, 8.6.2, 8.6.2A.1, 8.6.2B.1, 8.6.2B.2, 8.6A.2, 8.6C.2, 8.6D.2, 8.9A.2, 8.10.3A, 8.10.5, 8.10A.5, 8.10B.5, 8.10C.5, 8.10D.5, 8.11B.2 Consistent style (formal writing): do not: clause 7.1.2.3 (Avoid contractions in formal writing.) Removing definite article in references: Table: clauses 8.2.2.2.19, 8.3.12, 8.3.13.1 Consistent style of references to specifications: TS xx.xxx [y]: clauses 7.1.1, 7.1C.2, 7.1D.2, 8.1.1.1, 8.1.2.1.12, 8.2.1.2.13, (or [y]) 8.2.2.1, 8.2.2.2.9, 8.2.3.2.11,8.2.4.2.4, 8.2.4.2.9, 8.3.2, 8.3.6, 8.3.9, 8.3.11, 8.3.16, 8.3.17, 8.3.18, 8.3A.2, 8.5.1, 8.5.1.1, 8.5.9.1, 8.5.9.2, 8.10A.5, 8.11, 8.11A, 8.12.5, 8.12A.5, 8.18.7.1, 8.18.7.2 Consistent appearance of parameter names: Lmax: clauses 8.1.1, 8.1B.1, 8.1C.1, 8.1D.1 DRX_cycle_length: clauses 8.1.6, 8.1A.5, 8.18.6, 8.1C.6 NTA offset: clauses 7.1C.2, 7.1C.2.1, 7.1D.2, NTA: clauses 7.1.2, 7.1A.2, 7.1D.2, 7.1C.2, Removing redundant definitions within same clause: TTT: clauses 8.11B.2.1, 8.11B.2.1, 8.11E.2.1 (TTT defined twice in each clause.) Consistent state names: RRC_INACTIVE: clause 7.1.1, 7.1.2, 7.1A.1, 7.1A.2, RRC_CONNECTED: clause 7.1.1, 7.1A.1, 7.1C.1, 7.1D.1, Correcting spelling: aggressor: clauses 8.2.1.2.12, 8.2.2.2.9, 8.2.3.2.11, 8.2.4.2.9 transmitting: clauses 7.1.1, 7.1A.1 requirements: clauses 8.2.4.2.1, 8.2.4.2.2, 8.2.4.2.10, 8.2.4.2.15, 8.2.4.2.16, 8.3.2, 8.3.16, 8.3.17 signalling: clauses 8.1.1.1, 8.1.2.2, 8.1.3.2, 8.1A.2.2, 8.1B.2.2, 8.1B.3.2, 8.3.2, 8.3.7, 8.3.17, 8.3.18, 8.3A.2, 8.5.1.1, 8.5.2.2, 8.5.3.2, 8.5.5.2, 8.5.6.2, 8.5A.2.2, 8.5A.5.2, 8.5B.2.2, 8.5B.3.2, 8.5B.5.2, 8.5B.6.2, 8.18.2.2, 8.18.3.2, 8.18.5.2, 8.18.6.2 respectively: clauses 8.1.2.2, 8.1.3.2, 8.1A.2.2, 8.5.2.2, 8.5.5.2, 8.5.6.2, 8.5A.2.2, 8.5A.5.2 during: clauses 8.2.1.2.1, 8.2.3.2.1 etc… Consistent style (lower-case in headings): requirements: clauses 7.5.2, 7.5.2.1, 7.5.3, 7.5.4, 7.5.5, 7.5.5.1, 7.5.6, 7.5.7, 7.6.2, 7.6.2.1, 7.6.3, 7.6.4, 7.6.5, 7.6.5.1, 7.6.6, 7.6.7, 7.6.8 Adding missing Table or clause in reference: Table: clause 7.6.4 clause: clauses 8.2.4.2.5, 8.3.13, 8.3.18, 8.6.1, 8.6.28, 8.10E.2, 8.15.3, 8.16.3, 8.18.4, 8.22.3, 8.23.4, 8.24.3 Removing brackets: FR2-NTN: clauses 8.1C.1, 8.1C.2.2, 8.1C.2.3, 8.1C.3.2, 8.1C.3.3, 8.1C.7.1, 8.1C.7.2 (FR2-NTN operating bands defined in TS 38.101-5, clause 5.2.) musim-GapPreference-r17: clauses 8.1.2.2, 8.1.3.2, 8.5.2.2, 8.5.3.2, 8.5.5.2, 8.5.6.2, 8.18.2.2, 8.18.3.2, 8.18.5.2, 8.18.6.2 (Capability musim-GapPreference-r17 defined in TS 38.306, clause 4.2.2.) Removing duplicated sentence: If the high layer… clauses 8.1B.2.2, 8.1B.3.2 Removing word with strikethrough: clause 7.1A.2 (first detected path) (Removing detected is consistent with corresponding definition in clause 7.1.2.) Removing Editor’s notes pertaining to Rel-17 features: clauses 8.6C, 8.13C, 8.14C (Notes referring to WID, agreements to be made, and things to look into during the since long completed work on the Rel-17 feature.) Not addressed (for further discussion or to handle in maintenance): - Unnecessary branching of requirements for LTM w.r.t. capability of measurement with RTD>CP. Suggest to be handled in maintenance. - Remaining Editor’s notes containing FFS. Suggest to be handled in maintenance. - Remaining brackets. Suggest to be handled in maintenance. - Footer of Table 8.2.2.2.2-2 with incomplete sentence. Suggest to be handled in maintenance. - Inconsistency in upper/lower-case for mathematical operators. - Correction of duplicate Table 7.1C.2-3. - Additional suggestions to the previous draft (R4-2411047) which may need further discussions during the meeting (change of parameter names etc). |

# Topic #0: time plan

- Formal CR is expected to be approved in Nov. RAN4 meeting.

- CR will be based on TS38.133 v18.7.0

- Work item code to be used: “NR_RRM_enh3-Core” (R18)

- CR cat.

  - “F” should be used in case parameter updates included in the CR

# Topic #1: Identified Issues in RAN4#112

Main technical topic overview. The structure can be done based on sub-agenda basis.

- Generic editing aspects to be addressed

  - Edit Help (i.e. no need to fix in the CR)

    - “table” and “clause” should be in lower case, except in the caption of the table or title of the clause.

    - DRx -> DRX

    - RB->PRB

    - EUTRA(N) -> E-UTRA(N)

    - n.a.-> N/A (edit help)

    - “5ms”->”5 ms”

      - ms, dB, dBm, s, us, kHz, MHz, GHz, Hz,

    - Cell1-> Cell 1

      - TRP, BWP, Note

    - SSB GSCN -> SSB ARFCN in test configurations with clauses start with A.

  - To be addressed in the CR

    - italic on UE capability

    - Update abbreviation section to include the following terms

      - PRB, BW, TRS

    - Missing references should be included.

      - Example: TS 38.101-1 [18]

    - The caption of the table in general should not be changed for sake of simplification, unless error is identified.

- The following aspects may or may not be pure editorial and should be reviewed and decided case by case.

  - Remove [], TBD, FFS

  - Remove/update notes

- Hierarchy of indent (agreement in RAN4#112)

  - RAN4 to discuss the potential improvement of Hierarchy of indent

    - Target release:

      - Step 1: Take 8.3.2 in TS38.133 v18.7.0 as the example to start working with

        - This exercise should not be part of CR drafting and approval in Nov.

      - Step 2: Rel-19 specification for R19 features with independent clause

    - Target section/feature: one specific example, i.e., one sub-section

      - Option 1 (preferred): select one example during the meeting.

      - Option 2: Each interested company can pick one example.

    - Approach

      - Option 1: RAN2 pseudo-code

      - Option 2: RAN1/4 existing approach

      - Interested companies are encouraged to choose their preferred approaches in next meetings and the decision is not expected to be made before next Feb meeting.

# Topic #2: New proposals On CR handling

Moderator: since we have agreed in RAN#111 to continue big CR approach, the proposals beyond this approach will be deprioritized

- E///:

  - Proposal 1: A big maintenance CR running over a couple of RAN4 meetings after closing the core WI.

  - Proposal 2: Allocate separate AI/TU for discussing and finalizing the feature CRs after the technical discussions are over.

    - Depending on the feature and the amount of specification impact, the time allocation can vary and can be up to the entire meeting week in the worst case.

  - Proposal 3: WI Rapporteurs present a CR implementation plan (e.g., similar to the workplan, but focused on CR handling for the WI), discuss, and get it approved. This applies also when there is no work split.

  - Proposal 4: For situations where similar text needs to be repeated across multiple sections (or specifications), the general text could firstly be agreed as a reference and then used across different sections/CRs/specifications to improve consistency.

  - Proposal 5: Creating a checkbox list of key specification editing aspects to remember and check while preparing CRs and/or extend the Forword section of the specification to ensure consistent usage of frequently used terms, notation, abbreviation, CA configuration vocabulary, etc.

  - Proposal 6: Creating a 3GPP repository of figure templates, editable diagrams, and formulae. The link with templates could be included in the checkbox list described in Proposal 7.

# Topic #3: New Proposals For Spec Improvement

Moderator: to maintain the efficiency and reasonable workload, the proposals beyond the list of identified issues can be discussed after the identified ones are resolved. Exceptional case can be considered based on the consensus.

- Vivo:

  - Proposal 1: Use existing indent format, which is also used in RAN1 specifications.

  - Proposal 2: When capturing requirements in the specification, the structure of requirements should be carefully considered and unified indent format should be used, especially when capturing new requirements into existing one.

- MTK:

  - Proposal 1: Put conditions before requirements, as below example:

  - Proposal 2: When both new and legacy requirements are to be put in the same hierarchy level, the new requirements start first with ‘if’, while the legacy requirements come after and begin with ‘else if’ or ‘else’.

  - For requirements with multiple applicable conditions, start the paragraph with UE capability and providing all the remaining conditions in a list. For example:

- QC:

  - Proposal 1: RAN4 to develop guidelines for drafting requirements with complex logic, including adopting a pseudo-code approach (e.g. similar to the way RAN2 procedures are specified).

  - Proposal 2: RAN4 should take section 8.3.2 as an example and develop rules how to do proper indentation. One potential approach is the methodology of TS 38.321

  - Proposal 3: RAN4 should develop guidelines how parameters are defined in TS 38.133.

    - According ot definition of TSMTC_MAX, Trs, and TFirstSSB in section 8.3.2. Three different styles are used to define the parameters. While TSMTC_MAX and TFirstSSB show some clarity, the definition of Trs is difficult to follow since it goes across seven lines. Proper indentation could improve readability here as well. In particular, the phrase There are no requirements if the SSB transmission periodicity is not 5ms in the definition of Trs is unclear since Trs is not a requirement

- LG:

  - Proposal 1: Use RAN2 pseudo-code for hierarchy of indent

  - Proposal 2: To align terminology and table description in whole specification, RAN4 consensus is required first.

Based on offline reviews on draft CRs, some issues have been raised about

terminology alignment such as

- EUTRA  E-UTRA

- EUTRAN E-UTRAN

- MsgA  msgA

- Cell1  Cell 1

- Note1  Note 1

and generic table description (remove redundant description) in test configuration such as

- Table A.4.3.2.2.1.1-1: Supported test configurations for contention based random access test in FR1 for PSCell in EN-DC  Table A.4.3.2.2.1.1-1: Supported test configurations for contention based random access test in FR1 for PSCell in EN-DC

- E///:

  - Proposal 1: Agree on removing the editor’s note below in clause A.3.15.3 of TS 38.133

A.3.15.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. The applicable set of relative angular offsets between the 2 active probes is given in Table 3.15.3-1 for each UE power class.

  - Proposal 2 (RAN5 LS): Inform RAN5 by sending an LS to RAN5 listing the relevant RAN4 CRs agreed during this RAN4 framework on RRM specification quality improvement.

  - Proposal 3: Agree on the following new text added in clause A.1 in TS 38.133:

A.1 Purpose of annex

The purpose of annex A is to define test cases and the relevant configurations for verifying various types of RRM requirements with NR cells, including SA NR, EN-DC, and NE-DC deployments as well as standalone E-UTRA deployments.

  - Proposal 4 (Mapping purpose): It is clarified that the mapping is for information purpose only and presents the information from the test cases description in a more compact and structured way.

  - Proposal 5 (Mapping): Given the informative purpose of the mapping and its potential length, including such mapping in a new Annex D (informative) of TS 38.133 may be preferred.

  - Proposal 6 (Mapping format): The mapping can be in a table format and contain, e.g., <requirement clause ID><test case clause IDs> columns. If needed, the column <test case clause IDs> can be further broken into sub-columns, e.g., for SA, EN-DC, and NE-DC.

  - Proposal 7 (Mapping contents): The mapping can be focus on core requirements and corresponding test cases, but potentially may also be extended to performance requirements and corresponding test cases.

  - Proposal 8 (Mapping maintenance): Upon each test case introduction, the mapping can be updated with the corresponding reference.

- Nokia

  - Proposal 1: Align the phrase used for referring to the ‘SCell’ being addressed in the requirements in section 8.3.2, e.g.

- ‘SCell to be activated’ or

- ‘target SCell’ or

- ‘SCell’.

  - Proposal 2: Clarify and remove use of ‘and/or’ phrases.

  - Proposal 3: Requirements not conditioned on the SCell known or unknown status, should be defined prior to the requirements conditioned by the SCell known and unknown status.

  - Proposal 4: If a new outline is selected, these need to be supported by clear drafting guidelines and, if needed, template support.

  - Proposal 5: If RAN4 agrees to apply quality improvements and related CRs, a proper review process needs to be agreed.

# Appendix: examples of requirement’s revision

## Example from vivo

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in millisecond.

-	If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms.

-	TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

-	If the SCell is unknown and belongs to FR1, and provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

-	If one of the two following conditions is met

-	 ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-	 ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

-	If UE supports shortMeasInterval-r18, then

-	TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

-	TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms, otherwise.

-	Otherwise

-	TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise.

-	Otherwise

-	If UE supports shortMeasInterval-r18, then

-	6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

-	Otherwise

-	6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

-		If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided

-	The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

-	The difference of the reception power with the contiguous active serving cell is <= 6dB, and

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

-	If the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, for a UE supporting scellWithoutSSB-InterBandCA-r18,

-	When the following conditions are fulfilled

-	The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and

-	The [EPRE] difference at the UE is smaller than or equal to [12] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and

-	The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

where the reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Editor notes: FFS whether and how to capture the wording “after the compensation for AGC”.

-	Tactivation_time is

-	Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17

-	Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for Scell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17

-		If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided:

-	The UE is provided with SMTC for the target SCell, and

-	The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

-	The parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell, and

-	SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

-	If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

-	If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the FR2 power class 6 UE supporting scellWithoutSSB and [Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode] is configured with highSpeedMeasFlagFR2-r17, Tactivation_time is 3 ms , provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

-	If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

-	If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

-	If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

-	If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), or

-	6ms + TFirstSSB_MAX,enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced  + TL1-RSRP, enhanced_measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) if UE supports reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, and when SCell activation triggered L3 report is not configured or SCell activation triggered L3 report is configured but not reported.

-	If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

-	If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, or

-	3ms + TFirstSSB_MAX, enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)} if UE supports reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, and when SCell activation triggered L3 report is not configured or SCell activation triggered L3 report is configured but not reported.

-	If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

Where:

TSMTC_MAX:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

TSMTC_MAX, enhanced:

-	In FR1 or FR2-1, a UE supporting shortMeasInterval-r18 if the SMTC for SCell being activated is only configured in measObjectNR, TSMTC_MAX, enhanced is the SSB periodicity of SCell being activated. Otherwise, TSMTC_MAX, enhanced = TSMTC_MAX.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms.

Trs, enhanced is the SSB periodicity of the SCell being activated for a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR. Otherwise, Trs, enhanced = Trs

TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 or in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

TFirstSSB_MAX, enhanced: For a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR, TFirstSSB_MAX, enhanced is the time to the end of the first complete SSB burst indicated by the SSB periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length. Otherwise, TFirstSSB_MAX, enhanced = TFirstSSB_MAX

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

X1 is equal to the reported value in reduceForCellDetection in FR2-1. Otherwise, if reduceForCellDetection is absent, X1 is 8.

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

TL1-RSRP, enhanced_measure is

-	SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0; N is equal to the value reported by the UE in  reduceForSSB-L1-RSRP-Meas. Otherwise, if  reduceForSSB-L1-RSRP-Meas is absent, N= 8. Or,

-	CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

-	If UE supports shortMeasInterval-r18 capability, L1-RSRP measurement for TL1-RSRP, enhanced_measure can be performed based on non-DRX mode even if DRX is configured.

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS38.331 [2].

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

TFirstTRS: is the time to the end of the first complete periodic CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length.

TTRS is the periodicity of periodic CSI-RS burst for SCell activation.

TFirstATRS: is the time to the end of the first complete CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length, where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

TATRS is the CSI-RS burst for SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

Tgap is a gap length between two aperiodic CSI-RS bursts,

-	at least 2 slots for 15kHz and 30kHz

-	at least 3 slots for 60kHz

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3ms;

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes only TFineTiming and no TFirstSSB_MAX;

-	Tfirst_TRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_TRS;

-	Tfirst_ATRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_ATRS.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requriements on interruption due to SCell activation in clause 8.2 apply provided that the SSB of the to-be-activated SCell is within the first active DL BWP of the Scell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

If the SCell is unknown and belongs to FR1, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell

-	its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

When absoluteFrequencySSB is not configured in DownlinkConfigCommon for target SCell but SMTC for target SCell is configured, no requirement would be applied.

## Example from MTK

The revised structure is provided in this Appendix. We added Microsoft WORD’s ‘outline level’ of each hierarchy levels. So that people can easier collapse or expand a certain paragraph. With lower-level detail hidden, it may be helpful to get the idea about the requirement structure. For example,

- The hierarchy expanded down to only the 2nd level would be like

- The hierarchy expanded down to only the 3rd level would be like

Note that this WORD’s ‘outline level’ may not be re-used in final spec.

## Example from QC

### Indentation and Restructuring of Section 8.3.2 – SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation time+TCSI reportingNR slot length. THARQ and TCSI reporting are defined as follows:

- THARQ in ms is the timing between DL data transmission and acknowledgement as specified in TS 38.213

- TCSI reporting is the delay in ms including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331

Tactivation time is the SCell activation delay in ms. Its value depends on the side conditions defined by the pseudo-code below.

- If the SCell to be activated belongs to FR1

  - If the SCell is known

    - If the measurement period of the SCell is equal to or smaller than 2400ms

      - Tactivation time=TFirstSSB+5ms

    - Otherwise

      - Tactivation time=TFirstSSB_Max+Trs+5ms

  - If the SCell is unknown and EsIot≥-2 dB

    - If ssb-PositionsinBurst indicates one SSB only or

    - If ssb-PositionsinBurst indicates multiple SSBs and TCI indication is provided in the same MAC CE as SCell activation

      - The SCell is contiguous to an active serving cell in the same band, and

      - If its ssb-PositionsInBurst is the same as the one of the active serving cell, and

      - If its SMTC offset is the same as the one of the contiguous active serving cell, and

      - If its RTD with the active serving cell is smaller than or equal to 260ns and

      - If its reception power difference with the contiguous active serving cell is smaller than or equal to 6dB

        - Tactivation time=TFirstSSB_Max+TSMTC_MAX+Trs+5ms

      - Otherwise

        - Tactivation time=TFirstSSB_Max+TSMTC_MAX+2Trs+5ms

    - Otherwise

      - If semi-persistent CSI-RS is used for CSI reporting

        - Tactivation time=6ms+TFirstSSB_Max+TSMTC_MAX+Trs+TL1-RSRP,measure+TL1-RSRP,report+THARQ+maxTuncertainty_MAC+TFineTuning+2ms,Tuncertainty_SP

      - If periodic CSI-RS is used for CSI-reporting

        - Tactivation time=3ms+TFirstSSB_Max+TSMTC_MAX+Trs+TL1-RSRP,measure+TL1-RSRP,report+maxTHARQ+Tuncertainty_MAC+TFineTuning+5ms,Tuncertainty_RRC+TRRC_delay

  - If there is at least one active serving cell contiguous in the same band

    - If the UE is not provided a SSB configuration and the UE supports scellwithoutSSB

      - If its RTD with the active serving cell is smaller than or equal to 260ns and

      - If the difference of the reception power with the active serving cell is less than or equal to 6 dB and

      - If the RS(s) of the SCell are QCL-TypeA with its TRS(s) and its TRS(s) are QCL-TypeC with the SSB(s) of the active serving cell

        - Tactivation time=3ms

- If the SCell to be activated belongs to FR2

  - If the SCell is known

    - If semi-persistent CSI-RS is used for CSI reporting

      - Tactivation time=3ms+maxTuncertainty_MAC+TFineTuning+2ms,Tuncertainty_SP

    - If periodic CSI-RS is used for CSI reporting

      - Tactivation time=maxTuncertainty_MAC+TFineTuning+5ms,Tuncertainty_RRC+TRRC_delay-THARQ

  - If the SCell is unknown and EsIot≥-2 dB

    - If semi-persistent CSI-RS is used for CSI reporting

      - Tactivation time=6ms+TFirstSSB_Max+15TSMTC_MAX+8Trs+TL1-RSRP,measure+TL1-RSRP,report+THARQ+maxTuncertainty_MAC+TFineTuning+2ms,Tuncertainty_SP

    - If periodic CSI-RS is used for CSI reporting

      - Tactivation time=3ms+TFirstSSB_Max+15TSMTC_MAX+8Trs+TL1-RSRP,measure+TL1-RSRP,report+maxTHARQ+Tuncertainty_MAC+TFineTuning+5ms,Tuncertainty_RRC+TRRC_delay

  - If there is at least once active serving cell contiguous in the same band

    - If the UE is provided SMTC configuration

      - If the SSB(s) in the SCell and in the active serving cell fulfil the condition defined in clause 3.6.3 and

      - If its ssb-PositionsInBurst is the same as the one of the active serving cell and

      - If SSB is in the same half frame on the SCell and the active serving cell and

        - Tactivation time=TFirstSSB+5ms

    - If the UE is not provided a SMTC configuration and the UE supports scellwithoutSSB

      - If the RS(s) of the SCell are QCL-TypeD with the RS(s) of the active serving cell

        - Tactivation time=3ms

## Example from LG

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in millisecond.

1> If the SCell is known and belongs to FR1, Tactivation_time is:

2>	TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms.

2>	TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

1> If the SCell is unknown and belongs to FR1

2> if one of the following conditions is met

-		‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-		‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

3> 	provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

4>	 If UE supports shortMeasInterval-r18, then

5> 	TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms, if the following conditions are met,

-		the SCell is contiguous to an active serving cell in the same band, and

-		its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-		its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-		its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

5>	 TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms, otherwise.

4>	Otherwise

5>	 TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met,

-		the SCell is contiguous to an active serving cell in the same band, and

-		its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-		its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-		its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

5>	 TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise.

2> otherwise,

3> provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

4>	If UE supports shortMeasInterval-r18, then

5>	6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

5>	3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

4> 	Otherwise

5>	6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

5>	3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

2>	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-		the SCell is contiguous to an active serving cell in the same band, and

-		A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-		its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-		its SMTC offset is same as the one of contiguous FR1 active serving cell

-		its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

1>	For UE supporting scellWithoutSSB if the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band,

2> if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell,

3> Tactivation_time is 3 ms, provided

-	The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

-	The difference of the reception power with the contiguous active serving cell is <= 6dB, and

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

1> For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1

2> if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell,

3> if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled,

-	The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and

-	The [EPRE] difference at the UE is smaller than or equal to [12] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and

-	The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

where the reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Editor notes: FFS whether and how to capture the wording “after the compensation for AGC”.

4> Tactivation_time is

5>	Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17

5>	Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for Scell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17

1>	If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band,

2> if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell,

3> Tactivation_time is 3 ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

2> if the FR2 power class 6 UE supporting scellWithoutSSB and [Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode] is configured with highSpeedMeasFlagFR2-r17,

3> Tactivation_time is 3 ms , provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

2> Otherwise,

3> Tactivation_time is TFirstSSB+ 5ms provided:

-	The UE is provided with SMTC for the target SCell, and

-	The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

-	The parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell, and

-	SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

1>	If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

2> If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

3>	3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

2> If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

3>	max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

1>	If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management,

2> the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

3>	6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), or

3>	6ms + TFirstSSB_MAX,enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced  + TL1-RSRP, enhanced_measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) if UE supports reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, and when SCell activation triggered L3 report is not configured or SCell activation triggered L3 report is configured but not reported.

2> the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

3>	3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, or

3>	3ms + TFirstSSB_MAX, enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)} if UE supports reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, and when SCell activation triggered L3 report is not configured or SCell activation triggered L3 report is configured but not reported.

1>	If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management,

2> the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

3>	6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

2> the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, or

-	3ms + TFirstSSB_MAX, enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)} if UE supports reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, and when SCell activation triggered L3 report is not configured or SCell activation triggered L3 report is configured but not reported.

the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

3>	3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

where,

TSMTC_MAX:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

TSMTC_MAX, enhanced:

-	In FR1 or FR2-1, a UE supporting shortMeasInterval-r18 if the SMTC for SCell being activated is only configured in measObjectNR, TSMTC_MAX, enhanced is the SSB periodicity of SCell being activated. Otherwise, TSMTC_MAX, enhanced = TSMTC_MAX.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms.

Trs, enhanced is the SSB periodicity of the SCell being activated for a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR. Otherwise, Trs, enhanced = Trs

TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 or in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

TFirstSSB_MAX, enhanced: For a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR, TFirstSSB_MAX, enhanced is the time to the end of the first complete SSB burst indicated by the SSB periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length. Otherwise, TFirstSSB_MAX, enhanced = TFirstSSB_MAX

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

X1 is equal to the reported value in reduceForCellDetection in FR2-1. Otherwise, if reduceForCellDetection is absent, X1 is 8.

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

TL1-RSRP, enhanced_measure is

-	SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0; N is equal to the value reported by the UE in  reduceForSSB-L1-RSRP-Meas. Otherwise, if  reduceForSSB-L1-RSRP-Meas is absent, N= 8. Or,

-	CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

-	If UE supports shortMeasInterval-r18 capability, L1-RSRP measurement for TL1-RSRP, enhanced_measure can be performed based on non-DRX mode even if DRX is configured.

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS38.331 [2].

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

When absoluteFrequencySSB is not configured in DownlinkConfigCommon for target SCell but SMTC for target SCell is configured, no requirement would be applied.

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

TFirstTRS: is the time to the end of the first complete periodic CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length.

TTRS is the periodicity of periodic CSI-RS burst for SCell activation.

TFirstATRS: is the time to the end of the first complete CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length, where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

TATRS is the CSI-RS burst for SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

Tgap is a gap length between two aperiodic CSI-RS bursts,

-	at least 2 slots for 15kHz and 30kHz

-	at least 3 slots for 60kHz

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3ms;

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes only TFineTiming and no TFirstSSB_MAX;

-	Tfirst_TRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_TRS;

-	Tfirst_ATRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_ATRS.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requriements on interruption due to SCell activation in clause 8.2 apply provided that the SSB of the to-be-activated SCell is within the first active DL BWP of the Scell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

## Example from Nokia

Following is one TP example how to apply the pseudo-code in section 8.3.2 which is a school example of complex RAN4 specification writing.

<Start of Change #1>

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot:

Tactivation_delay = n+THARQ+Tactivation_time+TCSI_ReportingNR slot length

where:

- THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

- Tactivation_time is the SCell activation delay in millisecond.

- TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

1> If the SCell is known and belongs to FR1:

2> if the measurement period of the SCell being activated is equal to or smaller than 2400ms:

3> Tactivation_time = TFirstSSB+ 5ms.

2> if the measurement period of the SCell being activated is larger than 2400ms:

3> Tactivation_time = TFirstSSB_MAX + Trs + 5ms.

1> If the SCell is unknown and belongs to FR1:

2> if ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

2> if ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

3> if the side condition Ês/Iot ≥ -2dB is fulfilled:

4> if UE supports shortMeasInterval-r18 and:

5> if the SCell is contiguous to an active serving cell in the same band, and

5> if the ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

5> if the SMTC offset of the sCell is same as the one of contiguous FR1 active serving cell, and

5> if the RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB.

6> Tactivation_time = TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms.

5> otherwise,

6> Tactivation_time = TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms.

4> Otherwise:

5> if the SCell is contiguous to an active serving cell in the same band, and

5> if the ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

5> if the SMTC offset is same as the one of contiguous FR1 active serving cell, and

5> if the RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB

6> Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms.

5>  otherwise:

6> Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms.

2> otherwise:

3> if the side condition Ês/Iot ≥ -2dB is fulfilled:

4> If UE supports shortMeasInterval-r18:

5> if semi-persistent CSI-RS is used for CSI reporting:

6> Tactivation_time = 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting:

6> Tactivation_time = 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay).

4> Otherwise:

5> if semi-persistent CSI-RS is used for CSI reporting:

6> Tactivation_time = 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting:

6> Tactivation_time = 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay).

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell

-	its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

1> If the SCell being activated belongs to FR1:

2> if the UE is supporting scellWithoutSSB-InterBandCA-r18:

The reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Editor notes: FFS whether and how to capture the wording “after the compensation for AGC”.

2> if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL); and

2> if the UE is not provided with an SMTC configuration for the target SCell, and

2> if there is one collocated active reference serving cell on different FR1 band, and:

3> if the RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and

3> if the [EPRE] difference at the UE is smaller than or equal to [12] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and

3> if the RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

4> if aperiodic CSI-RS resources are not configured for SCell activation, or

4> if the UE do not support aperiodicCSI-RS-FastScellActivation-r17:

5> Tactivation_time = Tfirst_TRS + TTRS +5 ms.

4> if aperiodic CSI-RS resources are configured for Scell activation, and

4> if the UE supporting aperiodicCSI-RS-FastScellActivation-r17

5> Tactivation_time = Tfirst_ATRS + Tgap + TATRS + 5 ms.

1> else, if the SCell being activated belongs to FR1:

2> if the UE supporting scellWithoutSSB, and:

2> if there is at least one active serving cell contiguous to the SCell on that FR1 band, and

2> if the UE is not provided with SSB configuration (absoluteFrequencySSB), and

2> if the UE is not provided with SMTC configuration for the target SCell, and:

3> if the RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

3> if he difference of the reception power with the contiguous active serving cell is <= 6dB, and

3> if the The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band:

4> Tactivation_time = 3 ms.

1> If the SCell belongs to FR2:

2> if there is at least one active serving cell on that FR2 band:

3> if the UE is provided with SMTC for the target SCell, and

3> if the SSBs in the serving cell(s) and

3> if the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

3> if the parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell, and

3> if the SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

4> Tactivation_time is TFirstSSB+ 5ms.

3> if the UE supporting scellWithoutSSB:

4> if UE is not provided with any SMTC for the target SCell:

5> if the RS(s) of SCell being activated is (are) QCL-TypeD with RS(s) of one active serving cell on that FR2 band:

6> Tactivation_time = 3 ms.

4> if the UE is a FR2 power class 6:

5> if the UE supports [Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode], and

5> if the RS(s) of SCell being activated is (are) QCL-TypeD with RS(s) of one active serving cell on that FR2 band:

6> Tactivation_time = 3 ms.

2> if there is no active serving cell on that FR2 band:

3> if the target SCell is known to UE:

4> if semi-persistent CSI-RS is used for CSI reporting:

5> if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time:

6> Tactivation_time = 3ms + TFineTiming + 2ms.

5> otherwise:

6> Tactivation_time = 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP)

4> if periodic CSI-RS is used for CSI reporting:

5> if UE receives the SCell activation command and TCI state activation commands at the same time:

6> Tactivation_time = max(5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ).

5> otherwise:

6> Tactivation_time = max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ).

3> if the target SCell is unknown to UE:

4> if the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA, or

4> if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management:

5> if semi-persistent CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> if UE supports reduceForCellDetection, and/or

6> if UE supports reduceForSSB-L1-RSRP-Meas, and/or

6> if UE support shortMeasInterval-r18, and

7> if SCell activation triggered L3 report is not configured, or

7> if SCell activation triggered L3 report is configured but not reported:

8> Tactivation_time = 6ms + TFirstSSB_MAX,enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced  + TL1-RSRP, enhanced_measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

6> otherwise:

7> Tactivation_time = 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> if UE supports reduceForCellDetection, and/or

6> if UE supports reduceForSSB-L1-RSRP-Meas, and/or

6> if UE support shortMeasInterval-r18, and

7> if SCell activation triggered L3 report is not configured, or

7> if SCell activation triggered L3 report is configured but not reported:

8> Tactivation_time = 3ms + TFirstSSB_MAX, enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

6> otherwise:

7> Tactivation_time = 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

4> else, if the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA, or

4> if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management:

5> if semi-persistent CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> Tactivation_time = 6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> 3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

Where:

- TSMTC_MAX:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

- TSMTC_MAX, enhanced:

-	In FR1 or FR2-1, a UE supporting shortMeasInterval-r18 if the SMTC for SCell being activated is only configured in measObjectNR, TSMTC_MAX, enhanced is the SSB periodicity of SCell being activated. Otherwise, TSMTC_MAX, enhanced = TSMTC_MAX.

- Trs:

  - is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms.

- Trs, enhanced:

  - is the SSB periodicity of the SCell being activated for a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR. Otherwise, Trs, enhanced = Trs

- TFirstSSB:

  - is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

- TFirstSSB_MAX:

  - Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 or in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

- TFirstSSB_MAX, enhanced:

- For a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR, TFirstSSB_MAX, enhanced is the time to the end of the first complete SSB burst indicated by the SSB periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length. Otherwise, TFirstSSB_MAX, enhanced = TFirstSSB_MAX

- TFineTiming

  - is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

- X1:

  - is equal to the reported value in reduceForCellDetection in FR2-1. Otherwise, if reduceForCellDetection is absent, X1 is 8.

- TL1-RSRP, measure:

  - is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

- TL1-RSRP, enhanced_measure is:

-	SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0; N is equal to the value reported by the UE in  reduceForSSB-L1-RSRP-Meas. Otherwise, if  reduceForSSB-L1-RSRP-Meas is absent, N= 8. Or,

-	CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

-	If UE supports shortMeasInterval-r18 capability, L1-RSRP measurement for TL1-RSRP, enhanced_measure can be performed based on non-DRX mode even if DRX is configured.

- TL1-RSRP, report:

  - is delay of acquiring CSI reporting resources.

- Tuncertainty_MAC:

- is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

- Tuncertainty_RRC:

- is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

- Tuncertainty_SP:

- is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to:

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

- TRRC_delay:

- is the RRC procedure delay as specified in TS38.331 [2].

  - Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

  - When absoluteFrequencySSB is not configured in DownlinkConfigCommon for target SCell but SMTC for target SCell is configured, no requirement would be applied.

- TFirstTRS:

- is the time to the end of the first complete periodic CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length.

- TTRS:

- is the periodicity of periodic CSI-RS burst for SCell activation.

- TFirstATRS:

- is the time to the end of the first complete CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length, where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

- TATRS:

- is the CSI-RS burst for SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

- Tgap:

- is a gap length between two aperiodic CSI-RS bursts,

-	at least 2 slots for 15kHz and 30kHz

-	at least 3 slots for 60kHz

SCell in FR1 is known if it has been meeting the following conditions:

1> if during the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

2> the UE has sent a valid measurement report for the SCell being activated and

2> the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

1> if the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

SCell in FR2 is known if it has been meeting the following conditions:

1> If the SCell is the first SCell to be activated in an FR2 bands, and following conditions:

2> if, during the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

3> if the UE has sent a valid L3-RSRP measurement report with SSB index, and

3> the SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

2> if during the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3ms;

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes only TFineTiming and no TFirstSSB_MAX;

-	Tfirst_TRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_TRS;

-	Tfirst_ATRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_ATRS.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requriements on interruption due to SCell activation in clause 8.2 apply provided that the SSB of the to-be-activated SCell is within the first active DL BWP of the Scell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

<End of Changes>