3GPP TSG-RAN WG4 Meeting # 112bis											R4-2416879

Hefei, China, 14th – 18th October, 2024

Agenda item:			6.25.3

Source:	Moderator (Nokia)

Title:	Coffee break discussion minutes for [112bis][220] NR_XR_Ph3

Document for:	Information

# Ad hoc minutes

#### Issue 1-3-1: How to capture impact of measurement skipping

- Proposals related to measurement delay

  - Proposal 1 (Huawei): RAN4 shall define corresponding RRM behavior/performance when the measurement occasion is skipped/canceled for data transmission/reception.

  - Proposal 2 (Apple): as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

  - Proposal 3 (Mediatek): RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

  - Proposal 4 (Nokia, MTK): Measurement skipping is reflected on RRM measurements via extension of measurement delay.

  - Proposal 5 (OPPO): To reflect the RRM impacts due to measurement cancellation/skipping, extend the delay and keep the accuracy requirements could be considered as the baseline.

  - Proposal 6 (Qualcomm): RAN4 to consider extending the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

    - For DCI based dynamic indication, follow the NR-U approach to extend the L3 delays by the number of MGs deactivated during the period."

  - Proposal 7 (vivo): RAN4 to study whether and how to define new core requirements when some of gaps that need to be used for measurements are enabled for data transmission/reception after solution for enabling Rx/Tx in gaps is clear.

    - When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

- Proposals related to measurement accuracy

  - Proposal 8 (Nokia): Reuse existing measurement accuracy requirements.

- Recommended WF

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period is extended by number of measurement occasions which are cancelled due to XR traffic.

        - Reuse existing measurement accuracy requirements.

      - FFS: discussion on UAI and maximum number of occasions is not limited by this agreement.

      - FFS if there is case without delay extension

- Companies’ views

  - Apple: It does not preclude other discussions like, limiting the maximum number of measurement skipping.

  - Vivo: We need a clear a picture on how this feature works, and than we discuss the requirements. For some cases if the UE reports some gaps for measurements and if that is enough for the network skipping, than no impact in measurement performance. Another case if that network indicates skipping of more occasions, than UE cannot meet the existing measurement requirements. Need to take different cases into account

  - CMCC: For extension of delay requirements it is also related to the UAI discussion. If we have UAI and this is an unused gap, than measurement delay doesn’t need ot be extended

  - ZTE: Need to discuss further. Maybe it depends on what type of gap occasions is skipped. In some occasions not any measurement is performed. Regarding measurement accuracy requirements, the existing requirements should be kept.

  - Huawei: Same as vivo, CMCC, ZTE. For some cases mobility is very important. When UE is need of mobility, we need to reuse measurement delay without impact.

  - Qualcomm: Agree with proposed WF.This is a simple solution as we we did in NR-U. some companies talk reporting unused occasions. It is up to the UE how it uses a gap. There is no one solution fits all for reporting unused gaps. Are we expecting that the UE is reporting all occasions not used?

  - Nokia: Same view as WF. This is a network controlled procedure, where network is controlling skipping of measurements. We still need ot see discussion on UAI. But this involves discussion on different implementation. It should be a simple mechanism, with impact on delay requirements, and accurary should not be modified.

  - Mediatek : Agree with WF. This is the obvious direction. If there is gap cancelation there is extension of the delay. This agreement does not preclude other agreement latter on, like limiting amount of gap cancelation for XR

  - Ericsson: Agree with proposed WF. Need clarify which FR it applies.

  - Apple: This is only baseline. We agre that sometimes cancelation of gap may not have impact on delay, but this is something that we will discuss as part of the UAI. Eventually it is also up to the network how we get the cancellation.

  - Moderation: add FFS

  - Huawei: can we discuss 2 cases. 1 case we use the same delay, and other case where there is delay extension

  - CMCC: we also support to consider 2 sets of requirements. 1 if the network follows UAI and another if the network doesn’t follow the UAI

  - Vivo: even for the extended delay we don’t know how it is extended. We agree on 2 cases, or we keep all open

  - Mediatek: How is the UAI related to this discussion? We have a measurement gap cancelation, some gaps are cancellation. Do we expect the same capability of the UE, meaning same number of samples, or better UE capability?

  - Qualcomm: Question about the UAI. Do we expect that the network has to respect the UAI?

  - Huawei: It is a handshake procedure. We can some cases where network takes the occasion that the UE reported and cases where it doesn’t.

  - Ericsson: The requirements depend on the conditions. The UAI is provided before the conditions will happen. So how will the UE in advance guarantee that the requirements will be? The UE cannot guarantee that the conditions will hold, regardless of whether the UAI reports the UAI.

  - Vivo: It is true that the UE cannot know which gaps can be cancelled by the network. If this feature is only used by XR devices, but I don’t believe that this is only for XR devices, so in some cases if the network has data it could use gaps even if no XR traffic.

  - ZTE: How does the network distinguish the UE suggestion in UAI? If the UE reports some gap occasions can be skipped, and some depend on CSSF and SMTC configuration. In other cases the channel condition is good and I don’t need so many samples to measure. We can’t reply whether network can schedule base don UAI

  - Nokia:  Agree that this is network controlled and the network has good understanding of requirements and mobility. DCI is up to network, UAI can be used, but it is general capability for UE, but not something that the network can guarantee to respect.

- Tentative agreement

  - None

- Moderator proposal for Online Discussion

  - Discuss based on the following Options:

    - Option 1:

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period is extended by number of measurement occasions which are cancelled due to XR traffic.

        - Reuse existing measurement accuracy requirements.

      - FFS: discussion on UAI is not limited by this agreement.

      - FFS if there is case without delay extension

    - Option 2:

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended by number of measurement occasions which are cancelled due to XR traffic.

        - Reuse existing measurement accuracy requirements.

      - Note: discussion on UAI is not limited by this agreement.

      - For the cases where measurement impact is not needed

        - FFS the feasible conditions

#### Issue 1-2-1: General approach for defining requirements for measurements with gaps

- Proposals

  - Proposal 1 (Huawei): RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

  - Proposal 2 (Nokia): RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Application of the requirements to other types of gap should be dealt with in a second stage.

  - Proposal 3: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

- Recommended WF

  - The selection of the measurement gap types is a topic that has been taking long discussion. Please consider if a combination of proposals 1 and 2 can be considered as a baseline.

- Companies views

- Tentative agreement

  - None

- Moderator proposal for online discussion

  - Discuss based on the options:

    - Option 1: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

    - Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

#### Issue 1-7-1: Response on RAN1 working assumption and time offset

- Proposals

  - Option 1:

    - Option 1a (CMCC): for RAN1 LS on solutions to enable transmission/reception in gaps/restrictions, it is proposed to confirm RAN1 working assumption.

    - Option 1b (Nokia): Alt 1-1 is feasible from RAN4 perspective and RAN4 is working on the related requirements on timeline

    - Option 1c (OPPO): Alt 1-1 in the LS R1-2407561 could be confirmed from RAN4 perspective.

  - Option 2 (Huawei) RAN4 reply LS that:

    - The dynamic approach (Alt.1) should be supported from RAN4 perspective, which can give more flexibility on NW scheduling.

    - RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period.

    - Whether UE supports both approaches can be up to UE capability discussion."

  - Option 3 (Ericsson, draft LS in R4-2416205) RAN4 replies that:

    - RAN4 to discuss the range of acceptable time offset values between DCI reception and the intended MG and will agree on:

      - the minimum time offset value, and

      - the maximum time offset value.

    - It is RAN4 understanding that the UE will expect XR data on the same carrier frequency, on which the corresponding DCI is received.

- Recommended WF

  - Please discuss the options.

- Companies’ views

- Tentative agreements

  - None

- Moderator proposal for online discussion

  - Consider the following questions

    - Can the Alt 1-1 from RAN1 be confirmed?

    - Semi-static approaches can be discussed in Issue 1-6-1

#### Issue 1-1-3: Measurements without gaps

- Proposals

  - Option 1 (Apple, ZTE): support measurements without gaps

  - Option 2 (Mediatek, OPPO, vivo): de-prioritize measurements without gaps

  - Option 3 (Nokia): measurement skipping do not apply for measurements without gaps

- Recommended WF

  - Please discuss the options regarding measurements without gaps.

- Moderator proposal for online discussion

  - Many companies support Option 2 and Option 3. Check if supporters of Option 1 can compromise.

#### Issue 1-1-4: L1 measurements

- Proposals

  - Option 1 (CMCC)– support skipping of L1 measurements

  - Option 2 (Apple)– support skipping of RLM and BFD measurements

  - Option 3 (OPPO)– de-prioritize L1 measurements

  - Option 4 (Huawei, Qualcomm, vivo, ZTE, MTK)- measurement skipping do not apply for L1 measurements

- Recommended WF

  - Please discuss the options regarding L1 measurements.

- Moderator proposal for online discussion

  - Only 2 companies supporting Option 1 and Option 2. Check if we can compromise to Option 3 or Option 4.

#### Issue 1-1-5: L1/L2 triggered mobility measurements

- Proposals

  - Option 1 (ZTE) – support skipping of L1 measurements for mobility within and outside gap

  - Option 2 – de-prioritize L1 measurements for mobility

  - Option 3 - measurement skipping do not apply for L1 measurements for mobility

- Recommended WF

  - Please discuss the options regarding L1 measurements for mobility.

- Moderator proposal for online discussion

  - Postpone discussion

#### Issue 1-4-1: Timeline Time offset requirement for measurement gap skipping

- Proposals degrading minimum time offset between indication to skip and skipped measurement gap

  - Proposal 1 (Apple): 7 ms

  - Proposal 2 (CMCC): N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window  (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214)

  - Proposal 3 (Interdigital): assume subclause 5.3.1 of TS38.214 as a baseline for X offset.

  - Proposal 4 (Nokia): Fraction of a slot

  - Proposal 5 (OPPO): DCI decoding time defined in TS 38.214 + [5]ms

  - Proposal 6 (Qualcomm, vivo): 5 ms

  - Proposal 7 (ZTE): besides the DCI decoding delay, no additional processing delay is needed.

  - Proposal 8 (MTK): Proposal 8: In order to maintain short time offset for gap cancellation, RAN4 RRM shall not consider 3G/2G for UE operating with XR data traffic.

- Recommended WF

  - Discussion needed

- Moderator proposal for online discussion

#### Issue 1-2-2: Types of measurement gaps

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Company proposals on the priorities for RAN4 to work are summarized using based on the following table

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | Apple, NTT DOCOMO, Ericsson, Interdigital, Mediatek, Nokia, OPPO, vivo, ZTE |  |  |
| Type 2 | Apple, OPPO, vivo | Interdigital, Nokia | NTT DOCOMO, ZTE |
| NCSG | Apple, OPPO | NTT DOCOMO, Nokia | ZTE |
| MUSIM |  |  | NTT DOCOMO, Ericsson, Nokia, OPPO, ZTE |
| Positioning |  | Nokia | NTT DOCOMO, Ericsson, OPPO, ZTE |
| Rel 18 concurrent gap |  | OPPO | NTT DOCOMO, Nokia, ZTE |
| PreConfigured | Apple, OPPO, vivo | NTT DOCOMO, ZTE | Nokia |

- Recommended WF

  - The selection of the measurement gap types is a topic that has been taking long discussion. Please consider if agreement on Issue 1-2-1 is possible.

- Moderator proposal for online discussion

  - Discussion first on Issue 1-2-1

#### Issue 1-5-1: General on UAI

- Proposals

  - Option 1 (Mediatek): RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution.

  - Option 2 (ZTE): The UAI is beneficial in the XR scenario, and the regular UAI mechanism could be applied, i.e. UE reports UAI when necessary, and NW makes decision.

- Recommended WF

  - Please discuss which option can be agreed.

- Moderator proposal for online session

  - The issue is too general. We can discuss based on Issue 1-5-3

#### Issue 1-5-2: Expected gNB behaviour

- Proposals

  - Proposal 1 (Ericsson): RAN4 will not specify any explicit or implicit requirement on gNB behaviour in relation to the UE assistance data.

  - Proposal 2 (Ericsson): UE assistance containing patterns still cannot guarantee that gNB configuration will follow the UE pattern, since the configuration is ultimately the network decision.

- Recommended WF

  - Please discuss if the proposal is agreed. Please have in mind that the LS from RAN1 already includes an agreement stating that there is no mandated gNB behaviour expected in response to any UE assistance information that might be specified in this WID.

#### Issue 1-5-3: UAI Information related to measurement occasions

- Proposals

  - Option 1 (Nokia):

    - When considering the feasibility of UAI related to measurement occasions, it should be ensured that measurement delay and measurement accuracy are not impacted when measurement skipping is influenced by UAI.

    - Any proposals for UAI related to measurement occasions should be justified by clear performance benefit either in terms of measurement accuracy and delay or XR capacity.

  - Option 2 (Apple): UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation.

  - Option 3 (NTT DOCOMO): No UE assistance information related to measurement occasion is needed.

  - Option 4 (Meta): UE assistance information related to measurement occasions should be considered.

  - Option 5 (vivo):

    - UE should provide information about gaps not used for measurement for enabling transmission/reception in these gaps.

    - If supported, UE should provide information about SMTC occasions not used for L3 measurement for enabling transmission/reception on the SMTC occasions.

- Recommended WF

  - Please discuss the options above

- Moderator proposal for online discussion

  - Questions to be discussed online

    - What is the benefit of UAI related to measurement occasions?

    - How can the UE collect information for UAI.

#### Issue 1-1-1: Deployment scenarios

- Proposals

  - Proposal 1 (Ericsson): RAN4 to discuss and agree on the applicable scenarios:

    - FR2-1,

      - FFS: FR1;

    - NR SA,

      - FFS: MR-DC, including NR-DC, EN-DC, and NE-DC;

      - FFS: Carrier aggregation."

  - Proposal 2 (Mediatek): In order to maintain short time offset for gap cancellation, RAN4 RRM shall not consider 3G/2G for UE operating with XR data traffic.

  - Proposal 3 (Nokia): Measurement skipping apply for FR1 and FR2-1 measurements

  - Proposal 4 (Ericsson): To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

- Moderator proposal for online discussion:

    - Frequency ranges

      - Agree on FR1 and FR2-1

      - Option 1: FR2-2

      - Option 2: Not include FR2-2

    - Scenarios

      - Agree on

        - NR-SA

        - Carrier aggregation

      - Discuss

        - NR-DC, EN-DC, NE-DC

    - 2G/3G Measurements

      - Discuss the proposal

        - Proposal 2a: XR requirement do not apply for UE configured to measure 2G/3G

#### Issue 1-1-2: Types of measurements to be considered

- Proposals

  - Proposal 1: Intra-frequency

  - Proposal 2: Inter- frequency

  - Proposal 3: Inter-RAT

    - Proposal 3a: LTE inter-RAT

    - Proposal 3b: 2G/3G inter-RAT

  - Proposal 4: specific-purpose measurements, e.g., positioning, etc.

- Recommended WF

  - Please discuss which measurement types RAN4 should discuss.

- Moderator proposal for online session:

  - Agree at least on

    - Intra-frequency

    - Inter-Frequency

    - LTE inter-RAT

  - 2G/3G interRAT

    - Option 1: Do not include 2G/3G inter-RAT

    - Option 2: Include 2G/3G inter-RAT

  - Specific-purpose measurements, e.g. positioning

    - Option 1: Do not include specific-purpose measurements, e.g. positioning

    - Option 2: Include specific-purpose measurements, e.g. positioning

#### Issue 1-6-1: Semi-static schemes

- Proposals

  - Proposal 1 (Qualcomm): In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1.

    - Proposal 1a: (Huawei): RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.3) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period.

  - Proposal 2 (vivo): To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC

  - Proposal 3: Follow RAN1 working assumption and consider DCI only

- Recommended WF

  - Please discuss the proposals, considering the existing agreements from RAN1.

- Moderator proposal for online session

  - Discuss the options with new Proposal 3.